3GPP TSG-RAN WG2 Meeting #129bis                                                                        R2-2503790

Malta, May 19th – 23rd , 2025

Agenda item:	8.4.2

Source:	China Telecom

Title:	Discussion on LP-WUS in RRC_IDLE and INACTIVE

Document for:	Discussion

- Introduction

In the RAN #102 meeting, a new work item [1] on Low-power wake-up signal and receiver for NR (LP-WUS/WUR) was approved. According to the objective of WID, RAN2#129bis meeting has agreed the following agreements on LP-WUS monitoring in RRC_IDLE/INACTIVE [2]:

LP-WUS is supported with eDRX, FFS on exact impact if any Use 5G-S-TMSI to determine the UE_ID in the formula of UE_ID based subgrouping for LP-WUS, i.e., UE_ID = 5G-S-TMSI mod X. X is based on 32 subgrouping number. Details can be discussed in the running CR. Send LS to RAN3 (CC SA2/SA3) to inform our agreements on UE ID based subgrouping. Correct the typo as following for the previous agreed formula of UE_ID based subgrouping for LP-WUS: - Np is the number of subgroupsNumForUEID for PEI, if configured and UE supports PEI; otherwise, Np is 1. Confirm the principle for determining CN assigned subgrouping or UE_ID based subgrouping for PEI is reused for LP-WUS subgrouping. Details will be discussed in the running CR. All the LP-WUS related configurations except for measurement configurations are provided in SIB1. FFS the details on measurement configurations. Dedicated configuration in RRC signaling is not needed for providing LP-WUS related configuration in RRC_IDLE/INACTIVE modes. Use existing Srxlev/Squal for all MR measurement based entry/exit condition evaluation. Use measured value for all LR measurement based entry/exit condition evaluation.

This paper aims to further discuss the detailed procedures and configurations to support LP-WUS in RRC IDLE/INACTIVE mode.

- Discussion

  - Entry/Exit Condition of LP-WUS Monitoring

LP-WUS/WUR was specifically designed as an energy-saving mechanism for devices operating in RRC_IDLE and RRC_INACTIVE modes. By using LP-WUS/WUR, the main receiver (MR) of UE can transition into an ultra-deep sleep state, resulting in significant energy savings. While in this ultra-deep sleep state, the UE can still use its LP-WUS/WUR functionality to monitor network signaling activities and wake up the MR when needed.

In previous RAN2 meetings, it was agreed that the evaluation of serving cell quality, with mandatory RSRP and optional RSRQ, would serve as the basis for determining the entry and exit conditions for LP-WUS monitoring.  The following are some related agreements:

Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed.

Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS.

The metrics for serving cell quality measured by MR/LR for entry condition includes (LP-)RSRP and optional (LP-)RSRQ.

The metrics for serving cell quality measured by LR for exit condition includes (LP-)RSRP and optional (LP-)RSRQ.

Use existing Srxlev/Squal for all MR measurement based entry/exit condition evaluation.

The current focus of discussion has shifted toward whether “low mobility” should be included as an additional criterion for LP-WUS operation. Since LP-WUS primarily targets scenarios in which UEs remain in RRC_IDLE or RRC_INACTIVE states, relying solely on measurement-based thresholds may lead to ping-pong effects for UEs with high mobility within a cell. For instance, a UE may meet the RSRP threshold and enter LP-WUS monitoring; however, due to its high-speed movement, it may quickly trigger regular MR paging. This would result in frequent transitions between LP-WUS and conventional paging, undermining the intended energy savings. It is evident that high-mobility UEs are not ideal candidates for LP-WUS, as repeated MR paging wake-ups can increase energy consumption—contradicting the core objective of LP-WUS. To improve overall efficiency and minimize unnecessary power consumption, we propose introducing a “low mobility” criterion as an optional, network-configurable condition for LP-WUS entry and exit. Additionally, RAN2 may consider using the low mobility criterion specified in Rel-16 as a baseline for the entry/exit conditions of LP-WUS monitoring.

- Proposal 1: RAN2 to introduce the entry/exit conditions of LP-WUS monitoring as Rel-16 the low mobility criterion.

Per TS 38.304, the existing relaxed measurement criteria are based on Srxlev and Squal, where Srxlev represents the cell selection RX level of the serving cell, and Squal represents the cell selection quality of the serving cell. Currently, LP-WUS relies on serving cell quality derived from LR measurements, at least for evaluating the exit condition. However, LR measurements do not yet have a defined mapping to cell selection values such as Srxlev or Squal. As a result, enhancements to both the "low mobility" criterion and the "not at cell edge" criterion—originally defined in Rel-16—are necessary to enable effective support for LP-WUS operations.

- Proposal 2: Measurement results derived from LR can represent serving cell quality for LP-WUS monitoring to ensure alignment and applicability.

  - Ping-Pong Issues

The following agreement has been achieved in RAN2#127bis meeting.

If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s).

The LPWUS monitoring exit condition does not include MR measurements.

If only the threshold is considered, the device may frequently switch between LR and MR usage, resulting in a "ping-pong" effect. As a result, the UE may not have sufficient time to transition between sleep states, leading to additional power consumption associated with these frequent transitions. To address this issue, we propose introducing a time window: the condition should be considered fulfilled only if the measured value remains above the threshold for the entire duration of this window.

- Proposal 3: A new configurable time window need to be introduced in addition to the existing threshold-based entry conditions for LP-WUS monitoring.

- Conclusions

Based on the discussion in the previous sections, we provide the following proposals:

- Proposal 1: RAN2 to introduce the entry/exit conditions of LP-WUS monitoring as Rel-16 the low mobility criterion.

- Proposal 2: Measurement results derived from LR can represent serving cell quality for LP-WUS monitoring to ensure alignment and applicability.

- Proposal 3: A new configurable time window need to be introduced in addition to the existing threshold-based entry conditions for LP-WUS monitoring.

- References

- RP-240801 Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

- R2-25xxxxx, Draft Report of 3GPP TSG RAN WG2 meeting #129bis, Wuhan

- 3GPP TS 38.304 V18.4.0, User Equipment (UE) procedures in Idle mode and RRC Inactive state.