3GPP TSG-RAN WG2 Meeting #129bis                                                                        R2-2503791

St.Julians, Malta,  May 19th – 23rd , 2025

Agenda item:	8.4.3

Source:	China Telecom

Title:	Discussion on RRM Measurement Relaxation and Offloading in RRC_IDLE and INACTIVE

Document for:	Discussion

- Introduction

In the RAN #102 meeting, a new work item on Low-power wake-up signal and receiver for NR (LP-WUS/WUR) was approved. According to the objective of WID, the entry/exit condition for serving cell RRM relaxation was discussed in RAN2#129bis meeting, and the following agreements were made:

RAN2 assumes for the entry/ exit conditions of serving cell measurement offloading and serving cell RRM measurement relaxation: separate MR thresholds (according to RAN1 agreement)/LR thresholds can be configured for different types of LP WUR if a cell supports both types of LRs (can revisit based on RAN1 and RAN 4 progress, if any).

RAN2 assumes the entry/exit thresholds for RRM relaxation/offloading for OFDM-based WUR measuring LP-SS only are the same as that for OOK-based WUR measuring LP-SS. It can be revisited based on RAN1/RAN4 process, if any. Network is allowed to provide either OOK based threshold or OFDM based WUR mesasuring SSB threhold or both.

It is up to NW to configure either serving cell relaxation or serving cell offloading or both in one cell.

The metrics for RRM measurement offloading/relaxation criteria include (LP-)RSRP and optional (LP-)RSRQ.

How to define LP-RSRP and LP-RSRQ is up to RAN1.

The duplication between RAN2 and RAN4 specification on RRM relaxation and offloading should be avoided, details up to running CR rapporteur and companies’ review.

Merge the entry/exit condition for Serving Cell RRM measurement relaxation and Rel-19 Neighbouring Cell RRM measurement relaxation (higher priority frequency is separate discussion).

This paper aims to further discuss the criteria for RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE modes.

- Discussion

According to the latest agreements in RAN4, only Case #1 and Case #3 have been approved for serving cell measurement by LP-WUR. Based on RAN4’s progress, RAN2 need to consider the criteria for RRM measurement cases that RAN4 has agreed upon, specifically Case #1 and Case #3. So far, RAN2 has discussed the conditions for LP-WUS monitoring, as illustrated in Figure 1, MR RRM measurement relaxation (Case #3 – Relaxed Case B in RAN4), and MR serving cell fully offloading (Case #1 – Fully Offloading Case in RAN4).

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

In previous RAN2 meeting, it was agreed that the entry condition for MR serving cell RRM relaxation could include both MR and LR measurements. Specifically, the entry condition is met when both MR and LR measurements exceed their respective configured thresholds. With both Case #1 and Case #3 supported, further evaluate the entry and exit conditions, particularly regarding the transition between these two cases, is needed.

  - Case#1: Fully Offloading

Given that the latency of MR startup can reach several hundred milliseconds, it need to configure a more stringent condition for offloading serving cell measurements to LR. If a UE is in a high mobility state, it is likely to reach the cell edge quickly, so the UE should use MR to perform neighbor measurements to prepare for possible handovers. If the UE performs measurement offloading and turns off MR at this time, on the one hand, due to the extremely high startup delay of the MR, the UE is likely to miss the best handover opportunity; on the other hand, it will also cause unnecessary conversion power consumption. Therefore, the mobility of UE should be considered for serving cell measurement offloading, that is, measurement offloading should not be performed when the UE is in a high-speed state. UEs with high-mobility should not perform serving cell measurement offloading. Then RAN2 need to add the low-mobility criterion for the entry condition of serving cell measurement offloading.

Figure 1: Relationship between Case #1 and Case #3

- Proposal 1: To reuse Rel-16 low-mobility criterion for the entry condition of serving cell measurement offloading to avoid high-mobility UE perform serving cell measurement offloading.

Per TS 38.304, the existing relaxed measurement criteria rely on Srxlev and Squal, where Srxlev represents the cell selection RX level of the serving cell, and Squal represents the cell selection quality of the serving cell. Currently, LP-WUS relies on serving cell quality derived from LR measurements at least for exit condition evaluation. However, LR measurements do not yet have a defined mapping to the cell selection values such as Srxlev or Squal. As a result, enhancements to both the "low mobility" criterion and the "not at cell edge" criterion—originally defined in Rel-16—are necessary to enable effective support for LP-WUS operations.

- Proposal 2: Measurement results derived from LR can represent serving cell quality for measurement relaxation and serving cell measurement offloading, to ensure alignment and applicability.

  - Case#3: RRM measurement relaxation

If the entry conditions for LP-WUS monitoring are fulfilled, the system should first determine whether the criteria for performing neighboring cell measurements are met before conducting serving cell measurements on MR. Consequently, based on the aforementioned cases, RRM relaxation and RRM full offloading should be configured with distinct entry conditions when operating with LP-WUS.Moreover, RRM full offloading requires halting both serving cell and neighboring cell measurements on MR; therefore, the criteria for full offloading should be more stringent than those for RRM relaxation.

- Proposal 3: RRM relaxation and RRM full offloading should have distinct entry conditions when configured to operate with LP-WUS.

- Proposal 4: The criteria required for full offloading should be more stringent than those for RRM relaxation.

RAN4 may further discuss case #2 and case #4:

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #2 Relaxed case a | On with relaxation measurement | Off | ON |
| #4 Relaxed case c | Off | On, FFS the condition and the details | ON |

RAN2 agreed that when an offloading condition is satisfied, serving cell measurement is performed by LR (fully offloaded from MR) and serving cell measurement via MR is not required. In this case, it is desired to deactivate MR for additional power saving. However, when the serving cell measurement is offloaded from MR to LR, there may be no serving cell measurement via MR.

Figure 2: One example of relationships among different conditions

Hence, the UE is not able to determine whether the legacy Rel-16 relaxation condition for neighboring cell measurements is satisfied due to absence of the serving cell measurement via MR (e.g., offloading case). To handle this issue, measured value (e.g., LP-SS) of the serving cell measurement via LR can be used to determine whether to satisfy the conditions of relaxed measurement for neighboring cell measurement (while the offloading condition is satisfied).

- Proposal 5: For offloading case, whether to satisfy MR neighbouring cell RRM measurement relaxation (i.e., R16 criteria) can be determined by serving cell measurement results via LR.

- Conclusions

Based on the discussion in the previous sections, we provide the following observation and proposals:

- Proposal 1: To reuse Rel-16 low-mobility criterion for the entry condition of serving cell measurement offloading to avoid high-mobility UE perform serving cell measurement offloading.

- Proposal 2: Measurement results derived from LR can represent serving cell quality for measurement relaxation and serving cell measurement offloading, to ensure alignment and applicability.

- Proposal 3: RRM relaxation and RRM full offloading should have distinct entry conditions when configured to operate with LP-WUS.

- Proposal 4: The criteria required for full offloading should be more stringent than those for RRM relaxation.

- Proposal 5: For offloading case, whether to satisfy MR neighbouring cell RRM measurement relaxation (i.e., R16 criteria) can be determined by serving cell measurement results via LR.

- References

- RP-240801 Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

- R2-25xxxxx, Draft Report of 3GPP TSG RAN WG2 meeting #128, Orlando, USA

- R4-2410296,WF for RRM core requirements for LP-WUSWUR

- TR 38.869, Study on low-power wake up signal and receiver for NR.