3GPP TSG-RAN WG2 Meeting #130	R2-2503792

St.Julians, Malta, May 19th – 23rd, 2025

Agenda item:	8.7.3

Source:	China Telecom

Title:	Discussion on RRM Measurement Gaps/Restrictions Related Enhancements

Document for:	Discussion and Decision

- Introduction

In the RAN2#129 meeting, the following agreements were made for RRM measurement gaps related enhancements.

Agreements on MG skipping - From MAC perspective, the UE behaves as if there is no activated measurement gap during a skipped/cancelled measurement gap occasion. - “Cancelled” or “skipped” terminology will be aligned with RAN1 specifications when implementing changes in MAC. - RAN2 will not work on semi-static MG skipping solutions unless requested by RAN4/RAN1

The intention of this paper is to discuss the remaining issues related to the RRM measurement.

2 Discussion

In the RAN1#118 meeting, a DCI-based solution was agreed to enable Tx/Rx transmission during particular gap/restriction caused by RRM measurements.

For solutions based on triggering/enabling by network signalling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI. Send an LS to RAN4 to inform them of the above working assumption and ask them if there is any issue with it.

The network can use DCI to indicate the UE to skip a particular gap/restriction caused by RRM measurement dynamically. Obviously, the DCI-based solution is suitable for dynamic data scheduling, such as uncertain data transmission or XR traffic with significant jitter.

However, for dynamic scheduling, the network needs to send DCI to the UE before each gap/restriction occasion. Consequently, the UE needs to monitor PDCCH continuously to determine whether to skip the upcoming gap/restriction, which may introduce huge signalling overhead and energy consumption. To overcome the problem, the UE can send the indication to the network to assist the network make the proper decision on MG cancellation.

On the other hand, the DCI based MG cancellation mechanism need the network has the full knowledge of the UE’s mobility state and the details of UE’s data delay conditions to determine whether an MG should be cancelled. Although the network can derive the UE’s mobility status and data’s delay condition, it may lack visibility into whether the UE requires urgent uplink transmission. Therefore, the UE can use UAI to send the indication of urgent uplink transmission to the network.

Proposal 1:​​ The UE can use UAI to send the indication of urgent uplink transmission to the network.

Since both DRX and MG are configured by the network in a semi-static way, semi-static solutions may be more suitable for resolving overlaps between the MG and DRX. For instance, the network can send a semi-static configuration to UE to indicate when MG skipping should occur. Based on this configuration, the UE can skip the MG without requiring a DCI reception. This approach ensures that the limited time between the MG and DRX on-duration will not affect MG skipping.

In addition, semi-static scheduling (i.e. CG or SPS) is used for periodic XR traffic to reduce the signalling overhead. With the DCI-based solution, the network has to schedule a DCI before the gap/restriction occasion coming. And the UE needs to monitor the PDCCH continuously. Obviously, dynamic DCI scheduling will result in huge extra signalling overhead and energy consumption, which is contrary to the original intention of semi-static scheduling (i.e. CG/SPS).

As MG configuration is also configured in a semi-static way, semi-static solutions are more suitable to address the collision between the DRX and MG, MG and CG/SPS configuration. In this way, there is no need for the network to schedule the DCI continuously, which can reduce the signalling overhead, energy consumption and implementation complexity.

Proposal 2: Semi-static solutions can be employed to resolve conflicts between DRX and MG, or CG/SPS and MG.

- Conclusions

Based on the above analysis, we have the following proposals:

Proposal 1:​​ The UE can use UAI to send the indication of urgent uplink transmission to the network.

Proposal 2: Semi-static solutions can be employed to resolve conflicts between DRX and MG, or CG/SPS and MG.

- References

- R2-25xxxxx, Report of 3GPP TSG RAN WG2 meeting #129, Athens, Greece.