3GPP TSG RAN WG2 Meeting #130	R2-2503818

St.Julians, Malta, May 19th – 23th, 2025

Agenda item:	8.7.5

Source:	Quectel

Title:	Details on XR RLC autonomous retransmission

Document for:	Discussion

- Introduction

In RAN2 #129bis meeting, following agreements are achieved [1]:

- For autonomous retransmission and polling, the remaining time is determined based on discardTimer at PDCP. FFS whether/what additional conditions are needed to prevent too early and/or unnecessary retransmission - Autonomous retransmission is triggered for an RLC SDU (segment) provided that the original RLC SDU has been submitted to lower layers. - Autonomous retransmission is not triggered if the RLC SDU (segment) is already pending for retransmission. FFS specifications impact.

In this contribution, some details on XR RLC autonomous retransmission are discussed furtherly.

- Discussion

Since the autonomous retransmission is designed for XR session, it shall be configured only for RB of XR session.

Proposal 1: The autonomous retransmission is configured per RB.

If the remaining time is determined based on discardTimer at PDCP, PDCP shall indicate to RLC when the remaining time is below the threshold. There are two alternatives:

- Alternative 1: PDCP indicates to RLC for the PDCP PDU which is not acknowledged by RLC status report.

- Alternative 2: PDCP indicates to RLC for every PDCP PDU, even if it is acknowledged by RLC status report.

In the current RLC specification, RLC shall indicate to upper layers of successful delivery of the RLC SDU when the positive acknowledgement is received via RLC status report [2]. So alternative 1 is a better than alternative 2 because of less indication from PDCP to RLC.

Proposal 2: when the remaining time is below the threshold, PDCP indicates to RLC for the PDCP PDU which is not acknowledged by RLC status report.

In Rel-18, a new parameter of discardTimerForLowImportance is introduced for XR. For the unimportant PDCP SDU, the PDCP discardtimer shall be set a less value than that of the other important data. Since this PDCP SDU is not important, it is not necessary to apply autonomous retransmission. Therefore, PDCP entity shall not indicate to RLC when the remaining time is below the threshold for this unimportant PDCP SDU.

Proposal 3: PDCP entity shall not indicate to RLC when the remaining timer is below the threshold for the unimportant PDCP SDU.

From RLC point of view, there are 3 cases when the indication is received from PDCP:

Table 1: the RLC action upon receiving indication from PDCP

|  | Whether this RLS SDU exist in buffer for initial transmission? | Whether this RLS SDU exist in buffer for retransmission? | UE action |
| --- | --- | --- | --- |
| Case 1 | Yes | No | null |
| Case 2 | No | Yes | null |
| Case 3 | No | No | Insert this RLC SDU into the buffer for retransmission |

Based on the agreements in RAN2 #129bis, autonomous retransmission is triggered for an RLC SDU (segment) provided that the original RLC SDU has been submitted to lower layers, so UE action for case 1 is not needed. Furthermore, autonomous retransmission is not triggered if the RLC SDU (segment) is already pending for retransmission, so the UE does not need any action for case 2. In case 3, this RLC SDU has been transmitted via Uu interface, but has not been acknowledged by the receiver, so this RLC SDU shall be inserted into the retransmission buffer for autonomous retransmission.

Proposal 4: Upon receiving the indication from PDCP, RLC shall insert this indicated RLC SDU into the retransmission buffer if this RLC SDU does not exist in buffer for initial transmission and buffer for retransmission.

For the uplink XR data, once the autonomous retransmission is triggered, when the actual retransmission happens depends on the gNB scheduling. If the positive acknowledgement is received before the uplink grant which is for autonomous retransmission, this autonomous retransmission shall be cancelled.

Proposal 5: If the positive acknowledgement is received before the uplink grant which is for autonomous retransmission, this autonomous retransmission shall be cancelled.

- Conclusion

Based on the discussion above, the following proposals are suggested:

Proposal 1: The autonomous retransmission is configured per RB.

Proposal 2: when the remaining time is below the threshold, PDCP indicates to RLC for the PDCP PDU which is not acknowledged by RLC status report.

Proposal 3: PDCP entity shall not indicate to RLC when the remaining timer is below the threshold for the unimportant PDCP SDU.

Proposal 4: Upon receiving the indication from PDCP, RLC shall insert this indicated RLC SDU into the retransmission buffer if this RLC SDU does not exist in buffer for initial transmission and buffer for retransmission.

Proposal 5: If the positive acknowledgement is received before the uplink grant which is for autonomous retransmission, this autonomous retransmission shall be cancelled.

- References

- RAN2 #129bis minutes

- 3GPP TS 38.322 V18.2.0