3GPP TSG-RAN2#130 	R2-2503821

St. Julian’s, Malta, 19th May – 23th May, 2025

Agenda item:		8.12.3 (NR_MIMO_Ph5-Core)

Source:	LG Electronics Inc.

Title: 	Discussion on UEI beam reporting impact

Document for:	Discussion and Decision

1.	Introduction

During post e-mail discussion [1], open issues were discussed, but some issues are remained. In this contribution, we discuss further discussion points.

2.	Discussion

In RAN1#116bis, RAN1 agreed to use UCI for UEI both Mode-A and Mode-B UEI beam reporting.

[RAN1#116bis] Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, following modes are supported: - Mode-A (dynamically scheduling UCI by gNB): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) to request a resource for a second UL channel to carry beam report - FFS: Request format, e.g., SR or a new UCI type. - Step 2: UE detects the DCI format to indicate a resource for a second UL channel to carry beam report. - Step 3: Beam report is transmitted in second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - This option is basic UE capability (i.e. all UE supporting UE-initiated/event-driven beam reporting should support this feature). - No new DCI format is introduced. - Mode-B (UCI in pre-configured resource(s) for second UL channel): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) notifying a second UL channel to carry beam report - FFS: Notification format, e.g., SR or a new UCI type. - Step 2: UE transmits the beam report in the second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - The notification in Step1 is in a separate reporting instance from the beam report in Step 2. FFS: Whether UE receives acknowledge information with response to each step for all options For above procedures, cross-CC beam reporting is supported for both options. - FFS: Details.

Meanwhile, in the current specification, the MAC procedure for Rel-15 UL skipping and Rel-16 enhanced UL skipping are specified, and Rel-15 UL skipping is not applied to aperiodic CSI reporting (i.e. MAC PDU is generated if there is aperiodic CSI reporting) and Rel-16 enhanced UL skipping is not applied to both UCI and aperiodic CSI reporting (i.e. MAC PDU is generated if there is either UCI or aperiodic CSI reporting).

[TS 38.321 – 5.4.3.1.3] The MAC entity shall: 1> if the MAC entity is configured with enhancedSkipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or if the MAC entity is configured with enhancedSkipUplinkTxConfigured with value true and the grant indicated to the HARQ entity is a configured uplink grant: 2> if there is no UCI to be multiplexed on this PUSCH transmission as specified in TS 38.213 [6]; and 2> if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and 2> if the MAC PDU includes zero MAC SDUs; and 2> if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR: 3> not generate a MAC PDU for the HARQ entity. 1> else if the MAC entity is configured with skipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or the grant indicated to the HARQ entity is a configured uplink grant: 2> if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and 2> if the MAC PDU includes zero MAC SDUs; and 2> if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR: 3> not generate a MAC PDU for the HARQ entity.

Based on this, during post e-mail discussion [1], RAN2 discussed whether to follow Rel-15/16 UL skipping procedure for Mode-A and Mode B UEI beam reporting.

For Mode-B, RAN1 already agreed that Type-1 CG PUSCH cannot carry UL-SCH and RAN2 confirmed this in the last meeting.

Confirm the following RAN2 understandings: - The CG type-1 PUSCH carrying the beam report of Mode-B does not carry MAC PDU (i.e. UL-SCH). - The DG PUSCH carrying the beam report of Mode-A carries MAC PDU (i.e. UL-SCH) as legacy.

Therefore, it is clear that Mode-B UEI beam reporting does not need to be considered in the MAC procedure for R15/16 UL skipping, and there was a consensus in [1].

Observation 1. RAN2 has a consensus that Mode-B UEI beam reporting is not considered in the MAC procedure for R15/16 UL skipping during post-email discussion.

However, Mode-A UEI beam reporting needs to be considered in the MAC procedure for R15/16 UL skipping. This is because RAN1 agreed to use DCI format comprising UL-grant in Step 2, and this means that not only Mode-A UEI beam reporting can be scheduled by the DCI, but also UL-grant can be also scheduled

[RAN1#117] Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-A, the DCI format in Step-2 comprises UL-grant DCI format, and the second channel in Step-3 is at least PUSCH. - The UL-grant DCI format at least comprises DCI format 0_1/0_2. - FFS: DCI format 0_3 - FFS: How to trigger the UEI beam report by the UL-grant DCI format - FFS: the DCI format in Step-2 comprises DL-grant DCI format, and the second channel in Step-3 is PUCCH. - 1-bit field in the DL-grant DCI format is introduced to indicate the transmission of the UEI beam report - The PUCCH resource for HARQ-ACK transmission can be reused to carry both the HARQ-ACK and UEI beam report. - The DL-grant DCI format at least comprises DCI format 1_1/1_2. - FFS: DCI format 1_3

During [1], companies thought that Mode-A UEI beam reporting is treated as UCI (yellow highlighted above) because RAN1 agreed Mode-A UEI beam reporting is reported using UCI. Thus, RAN2 concluded that Mode-A UEI beam reporting follows Rel-16 UL skipping procedure, but there was no conclusion on Rel-15 UL skipping procedure.

However, in our view, Mode-A UEI beam reporting can be treated as aperiodic CSI reporting (cyan highlighted above). This is because Mode-A UEI beam reporting is transmitted based on CSI request in the legacy DCI format which is used for aperiodic CSI reporting.

[RAN1#118bis] Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the triggering procedure in Step-2 of Mode-A - Reuse CSI request field in DCI format 0_1/0_2 to trigger the transmission of the UEI beam report - If a CSI trigger state associated with UEI beam report configuration(s) is indicated by the CSI request field in DCI format 0_1/0_2, the UE transmits the corresponding UEI beam report(s) in the second PUSCH scheduled by the DCI format 0_1/0_2 - FFS: DCI format 0_3 - FFS: Whether a CSI trigger state should be dedicated to UE-initiated/event-driven beam reporting, i.e., not associated with legacy AP-CSI report configuration.

In addition, RAN2 agreed not to report Mode-A UEI beam reporting for dormant BWP, and this means that Mode-A UEI beam reporting is aligned with aperiodic CSI reporting than other CSI reporting.

Therefore, we propose that RAN2 treats Mode-A UEI beam reporting as aperiodic CSI reporting, and Mode-A UEI beam reporting follows Rel-15 UL skipping.

Proposal 1. RAN2 treats Mode-A UEI beam reporting as aperiodic CSI reporting.

Proposal 2. Mode-A UEI beam reporting follows Rel-15 UL skipping procedure, as like aperiodic CSI reporting.

Meanwhile, when the UE is not in Active time, the UE does not report CSI reporting, except aperiodic CSI reporting. If Mode-A UEI beam reporting is treated as aperiodic CSI reporting, Mode-A UEI beam reporting should be able to reported in non-Active Time, i.e. Mode-A UEI beam reporting can be transmitted regardless Active Time.

Proposal 3. Mode-A UEI beam reporting can be transmitted regardless Active Time.

3.	Conclusion

In this document, we analysis RAN2 impacts of UEI beam reporting, and made observations and proposals as follows.

Observation 1. RAN2 has a consensus that Mode-B UEI beam reporting is not considered in the MAC procedure for R15/16 UL skipping during post-email discussion.

Proposal 1. RAN2 treats Mode-A UEI beam reporting as aperiodic CSI reporting.

Proposal 2. Mode-A UEI beam reporting follows Rel-15 UL skipping procedure, as like aperiodic CSI reporting.

Proposal 3. Mode-A UEI beam reporting can be transmitted regardless Active Time.

4.	Reference

- [Post129bis][215][ MIMO_Ph5] Running CR for 38.321 (Samsung)