3GPP TSG-RAN WG2 #130	R2-2503822

St Julian’s, Malta, 19-23 May 2025

Agenda item:		8.6.2	Inter-CU LTM

Title:		Discussion on inter-CU LTM

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

During the last meeting, RAN2 further discussed several aspects to support inter-CU LTM and some agreements for inter-CU LTM have been reached [1].

Agreements on inter-CU LTM

- For security key update in inter-CU LTM, RAN2 agree to include actual NCC value in the LTM cell switch command MAC CE.

- NCC is included in the LTM cell switch command MAC CE if the R19 set ID is different between the target cell and source cell. Conversely, if the R19 set ID is same for both cells, the NCC will not be included.

- NW configures the corresponding sk-Counter in all LTM candidate configurations, and UE uses the configured value for generating the SN key when security key update is performed in MCG.

- From RAN2 point of view, UE-based TA measurement can be also applicable for the synchronized inter-CU LTM scenario, and no additional RAN2 specification impact is expected. Final decision is up to RAN4.

- MIMO 2TA can be configured with inter-CU LTM using PDCCH-order based TA measurement.

- If UE-based TA measurement is supported for inter-CU LTM, it cannot be configured if candidate cell is configured with MIMO 2TA.

- An LTM target cell will not apply cell DTX/DRX until LTM cell switch is completed. RAN2 will not consider any specification impacts needed for LTM RACH-less cell switch to an NES candidate cell.

A list of sk-counters is linked to a Rel-19 ID configured by the SN. At RLF and reconfiguration with sync failure: - If RLF: If the selected candidate cell has the same Rel-19 ID as source (no security key change), the UE performs fast RLF recovery (same as in Rel-18). - if reconfiguration failure (inter-CU LTM): If the selected candidate cell has the same Rel-19 ID as target, the UE performs fast failure recovery. FFS if fast failure recovery with different Rel-19 IDs is allowed. How to indicate whether an LTM candidate configuration is a complete configuration is the same as Rel-18 (no different indication for MCG and SCG part of the candidate configuration). The indication on whether to allow or not the SN to configure an inter-SN candidate is included in the inter-node RRC message. We include this agreement in the LS to RAN3. Network can send an LTM Cell Switch Command MAC CE indicating a CLTM candidate configuration (no specification change).

In this contribution, we further discuss the remaining issues or details for inter-CU LTM and provide our consideration.

# 2. Discussion

2.1 LTM fast recovery for inter-CU LTM

On the issue of LTM fast recovery, RAN2 agreed the agreement that if reconfiguration failure for inter-CU LTM and if the selected candidate cell has the same Rel-19 ID as target, the UE performs fast failure recovery. It is still FFS if fast failure recovery with different Rel-19 IDs as target is allowed.

Take the below RAN3 agreement into consideration, we observed that the source gNB forwards the security key to each candidate gNB, i.e., not only the target gNB:

RAN3#127bis Agreements: - For the initial inter-CU LTM preparation, the source gNB computes the KgNB*(s) (per candidate cell) for the candidate gNB, and forwards the {KgNB*, NCC} pair(s) to the candidate gNB via a Handover Request message as legacy. - The new source gNB needs to provide the new KgNB* to candidate gNBs via LTM configuration procedure.

Therefore, if reconfiguration failure for inter-CU LTM happens, as long as the selected candidate cell has the different Rel-19 ID with the source cell, the LTM fast recovery can be supported. This is because the security key is available at the candidate gNB, and the UE can derive the corresponding security key based on the NCC received in the LTM cell switch command. And since the UE would perform PDCP re-establishment during the recovery, there is no need to revert back the PDCP variables in UE implementation, so the recovery can be performed easily.

But if the selected candidate cell has the same Rel-19 ID as the source cell, since the PDCP re-establishment has been performed during the failed LTM cell switch, and there will be no PDCP re-establishment during the LTM recovery, it is difficult to revert back the PDCP state variables, so the recovery should not be supported. Instead, the UE shall initiate transmission of RRCReestablishmentRequest message.

Proposal 1a (RRC-8): If reconfiguration failure for inter-CU LTM cell switch, and if the selected candidate cell has the different Rel-19 ID as the source cell, the UE performs fast failure recovery.

Proposal 1b (RRC-8): If reconfiguration failure for inter-CU LTM cell switch, and if the selected candidate cell has the same Rel-19 ID as the source cell, the UE does not perform fast failure recovery.

2.2 RRC FFS RRC-2

Editor’s Note: FFS if appliedLTM-CandidateId needs to be included in both MCG and SCG RRCReconfigurationComplete message.

We would like to discuss the above RRC FFS as summarised by the RRC rapporteur.

In CPAC, upon execution of CPAC procedure, the UE transmits a MN RRCReconfigurationComplete including SN RRCReconfigurationComplete message and the identity of the applied conditional reconfiguration. The motivation behind is to assist the MN to identify the target SN of the CPAC procedure. During previous meeting, it was agreed that upon execution of inter-SN SCG LTM, the UE sends an MN RRCReconfigurationComplete message to the MN, which includes an SN RRCReconfigurationComplete message. Similar as CPAC, to assisted the MN to identify the target SN, the UE should include the identity of the applied LTM candidate configuration in the MN RRCReconfigurationComplete message.

Proposal 2 (RRC-2): For inter-CU SCG LTM, to assist the MN to identify the target SN, UE includes the identity of applied LTM candidate configuration in the MN RRCReconfigurationComplete message.

Additionally, in Rel-18 intra-CU LTM, it was agreed to include the identity of the applied LTM candidate configuration in the RRCReconfigurationComplete message, with the motivation behind is to indicate to the gNB-CU that the RRCReconfigurationComplete is for the LTM cell switch procedure, in case that LTM and CHO are configured for the same candidate cell, or multiple LTM configuration are associated with the same candidate cell. The corresponding chairman’s note can be seen in the below table. For SN/SCG, it is also possible that LTM and CPAC are configured for the same candidate PSCell, or multiple LTM configuration are associated with the same candidate PSCell. Therefore, for inter-CU SCG LTM, to assist the target SN to identify the applied LTM candidate configuration, the UE should also include the identity of the applied LTM candidate configuration in the SN RRCReconfigurationComplete message.

R2-2403177 [E068][E231][E074][E240][S792]Resolution of remaining RILs for LTM Ericsson discussion Rel-18 NR_Mob_enh2-Core Late E74: “Transaction ID” - Apple agrees with Ericsson proposal, and think this is helpful for R19 as well. - CATT think it is not a valid case that multiple configurations are applicable for one target cell. - MTK think now we have some cases where this may apply and think this is helpful for R19 - HW think the only scenario is LTM and CHO config for the same cell, think only 1 bit is needed. - vivo prefer to not do any optimization, can configure different C-RNTI for different config. - LGE think that with extended transaction ID space this is resolved. - Xiaomi think that complete message already contains the conditional configuration ID. - Ericsson think there can be multiple LTM configs per target cell, and think that CRNTI check does not work. - Intel support this proposal, it seems to be a clean solution. Lenovo as well. - Nokia wonders if this is really a real problem .. Upon an LTM cell switch, the UE includes the applied LTM candidate configuration identifier within the RRCReconfigurationComplete message.

Proposal 3 (RRC-2): For inter-CU SCG LTM, to assist the SN to identify the applied LTM candidate configuration, UE includes the identity of applied LTM candidate configuration in the embedded SN RRCReconfigurationComplete message.

2.3 L2 handling not supported in Rel-18 due to too late

During discussion of Rel-18 mobility correction last meeting, triggering of PDCP recovery for AM DRB with RLC bearer released in case of LTM cell switch without L2 reset was discussed, and companies think that it is technical correct, but it is too late to support. The Rel-18 network implementation should ensure that the concerned case never happens.

R2-2410063 Remaining issues on L2 reset for LTM NEC discussion Rel-18 NR_Mob_enh2-Core Proposal: In case of LTM cell switch without L2 reset, UE triggers PDCP recovery for an AM DRB, if one of its associated RLC bearer is released. The draft CR in Annex is adopted. Not pursued. [Apple]: Wonders if the scenario is really valid one. [Ericsson]: Considering it is NBC change, prefer not to have it. We still can survive w/o this change. [Xiaomi]: If companies are not favourable with this change, it’s good to capture that NW ensures this situation doesn’t happen in the meeting minutes. [Samsung, Qualcomm]: Although we agree in technical point of view, also share the concern with Ericsson. It may be too late to specify it now. [Vivo]: It’s ok not to agree with this correction, but considering it is valid scenario since it was already considered in SCPAC and specified it, it would be better to capture that NW ensures this situation doesn’t happen. [Ericsson]: Prefer to do nothing. Reasonable NW implementation takes care of it. [MediaTek]: Agree with Ericsson. [Google]: Agree with Ericsson. We may consider for Rel-19.

For legacy handover procedure, RLC bearer release can happen in case of intra-CU handover case, which means that there is no such restriction compared with LTM. To the remove this restriction for LTM, the related UE behaviour can be supported in Rel-19.

Proposal 4: Support in Rel-19 that the UE triggers PDCP recovery for an AM DRB, if one of its associated RLC bearer is released in case of LTM cell switch without L2 reset. And RAN2 to agree the corresponding TP in Annex.

# 3. Conclusion

In this contribution we discussed some issues related to inter-CU LTM and made the following proposals.

Proposal 1a (RRC-8): If reconfiguration failure for inter-CU LTM cell switch, and if the selected candidate cell has the different Rel-19 ID as the source cell, the UE performs fast failure recovery.

Proposal 1b (RRC-8): If reconfiguration failure for inter-CU LTM cell switch, and if the selected candidate cell has the same Rel-19 ID as the source cell, the UE does not perform fast failure recovery.

Proposal 2 (RRC-2): For inter-CU SCG LTM, to assist the MN to identify the target SN, UE includes the identity of applied LTM candidate configuration in the MN RRCReconfigurationComplete message.

Proposal 3 (RRC-2): For inter-CU SCG LTM, to assist the SN to identify the applied LTM candidate configuration, UE includes the identity of applied LTM candidate configuration in the embedded SN RRCReconfigurationComplete message.

Proposal 4: Support in Rel-19 that the UE triggers PDCP recovery for an AM DRB, if one of its associated RLC bearer is released in case of LTM cell switch without L2 reset. And RAN2 to agree the corresponding TP in Annex.

# 4. References

[1] Draft_RAN2_129bis_Meeting_Report_v2

[2] 2500442, Discussion on Inter-CU LTM, ZTE

# 5. Annex (TS 38.331)

##### 5.3.5.18.6	LTM cell switch execution

<skip>

1>	else if the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers or for the selected cell in accordance with 5.3.7.3 does not contain the field ltm-NoResetID and if the UE does not have any value stored of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID; or

1>	if the value of field ltm-NoResetID contained within the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers or for the selected cell in accordance with 5.3.5.18.x or 5.3.7.3 is not equal to the value of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID:

2>	for each logicalChannelIdentity and logicalChannelIdentityExt that is part of the current UE configuration for the cell group for which the LTM cell switch procedure is triggered:

3>	if servedRadioBearer is set to drb-Identity:

4>	after the end of this procedure, re-establish the corresponding RLC entity as specified in TS 38.322 [4], after applying the LTM configuration in ltm-CandidateConfig within the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC;

2>	for each bh-LogicalChannelIdentity that is part of the current UE configuration for the cell group for which the LTM cell switch procedure is triggered:

3>	after the end of this procedure, re-establish the corresponding RLC entity as specified in TS 38.322 [4], after applying the LTM configuration in ltm-CandidateConfig within the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC;

2>	for each drb-Identity value that is part of the current UE configuration:

3>	if this DRB is an AM DRB:

4>	after the end of this procedure, trigger the PDCP entity of this DRB to perform data recovery as specified in TS 38.323 [5], after applying the LTM configuration in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC;

2>	if the value of field ltm-NoResetID contained within the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID:

3>	replace the value of ltm-ServingCellNoResetID in VarLTM-ServingCellNoResetID with the value of ltm-NoResetID in the LTM-Candidate in ltm-Config or ltm-ConfigNRDC indicated by lower layers or for the selected cell in accordance with 5.3.5.18.x or 5.3.7.3;

1> else:

2>	for each drb-Identity value included in each RadioBearerConfig for the cell group for which the LTM cell switch procedure is triggered:

3>	if an associated RLC bearer is released in the LTM candidate configuration to be applied:

4>	if the bearer is an AM DRB:

5>	after the end of this procedure, trigger the PDCP entity of the bearer to perform PDCP data recovery as specified in TS 38.323 [5];