3GPP TSG-RAN WG2 Meeting #130	R2-2503840

St. Julians, Malta, 19 – 23 May 2025

Agenda item:	8.11.2

Source:	Nokia

Title:	Random Access Operation of SBFD

WID/SID:	NR_duplex_evo-Core - Release 19

Document for:	Discussion and Decision

# Introduction

In RAN#102 meeting, a Work Item (WI) on Evolution of NR duplex operation: Subband full duplex (SBFD) was approved. SBFD is being specified in Rel-19 as a new mode of operation in unpaired spectrum where a separate set of RBs within a TDD carrier are used for UL and DL transmission simultaneously. The intention is to allow more UL transmission opportunities compared to the typical DL-heavy TDD configurations used in today’s TDD deployments. This bring benefits in terms of UL coverage and UL latency, especially in wide-area macro networks where many UEs are in power-limited conditions in the UL direction. The work item description has been updated in RAN#104 meeting and the objectives are mentioned in  RP-241614.

RAN2 start discussing on SBFD random access procedures and in RAN2#129bis, and the following agreements are made:

Agreements On RO type signaling for CFRA - 1. For CFRA triggered by BFR, the RO type is indicated in BeamFailureRecoveryConfig. - 2. For CFRA triggered by ReconfigurationwithSync, the RO type is indicated in RACH-ConfigDedicated. When both NW indication on RO type and RSRP threshold are absent, it is up to UE implementation to select the RO type. Random access procedure in SBFD symbols is supported for all the existing RACH trigger events except for SI request. FFS for LTM. NW indicate via explicit signaling whether the SBFD RO is selected when SSB RSRP are 'below' or 'above' the configured threshold. Working assumption: For SBFD-aware UE, the selection of RO type is suggested to be performed before the selection of the set of Random Access resources. RO-Type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols is supported when the number of PRACH transmission attempts exceed a threshold (we assume it is the same threshold with the fallback from additional RO to legacy RO). If fallback from legacy RO to additional RO occurs, no further fallback to legacy RO is supported.

In this contribution, we further discuss different aspects of the SBFD random access operation.

# SBFD Random Access Operation

For initial access, the UE selects the index of the preferred SSB index and decodes the associated PBCH for MIB, and obtains relevant information for decoding the SIB1, from where it obtains the RACH configuration for sending Msg1. The RACH configuration contains, among others, the PRACH configuration index, the frequency domain multiplexing of the RACH occasions (ROs), the frequency offset of the ROs, and information about the power ramp up during the RACH procedure. For UEs in RRC_CONNECTED, dedicated RACH configuration, including CFRA, can be indicated by the network. If the CFRA configuration is missing, UEs in RRC_CONNECTED use the common RACH configuration for CBRA.

To enable support of initial access in SBFD, i.e., in the UL subband, RAN1 has been discussing the above aspects of random access procedures in SBFD since RAN1#116. In fact, RAN1 observed that support of initial access in SBFD provides at least one or more following benefits: reduced random access latency, reduced PRACH collision probability or allowing more contiguous frequency resources for PUSCH in UL slots, improved coverage of PRACH with sparse UL resources, and increased cell range of PRACH with sparse UL resources. As mentioned in section 1, WI was updated in RAN#104 [RP-241614] to specify SBFD operation to support random access in SBFD symbols by UEs in RRC_IDLE/INACTIVE mode, along with RRC_CONNECTED mode in RAN1 and RAN2.

# Various Aspects of SBFD random access

In this section, we discuss the RAN2 aspects of the SBFD random access procedure for RRC_CONNECTED and IDLE/INACTIVE operations.  RAN1 agreements on SBFD random access is listed in Appendix 1.

## PRACH resource configuration

Since Option 2 necessitates support for two RACH configurations, it introduces some overhead, especially when redundant information is present in these configurations. Hence to reduce the signaling overhead, if the legacy ROs and the additional ROs have the same value for one of the RACH configuration parameters it would not be needed to include those parameters in the additional RACH configuration. In such a scenario, inclusion in the legacy RACH configuration would be enough.

Proposal 1: For Option 2, in case the legacy ROs and the additional ROs have the same value for one of the RACH configuration mandatory parameters, only the legacy RACH configuration will be configured with this parameter.  For Option 2, in case the SBFD aware UEs did not receive a dedicated mandatory parameter in the additional RACH configuration, the SBFD-aware UEs determine this dedicated parameter from the legacy RACH configuration.

## RO Type Indication for CBRA

During the Post129bis][212][SBFD] Running CR for 38.331 (Huawei), the following has been agreed by majority of the companies.

[Proposal for RRC-1] To use RRC signaling, e.g., additional field in RACH-ConfigCommon IE to indicate RO type for CBRA. (15/16 support, no need for meeting contributions on this issue.)

The main purpose of the network indication is to dynamically manage the distribution of UEs between additional and legacy ROs. Therefore, the network indication should support dynamic changes, based on the network load. If the NW  indicate a 1-bit value, then it would force all the SBFD UEs to use the indicated RO type (SBFD RO or legacy RO), which is not aligned with the load balancing goal. Meaning, what NW want to achieve is that the network indicate via broadcast signalling that a portion of the SBFD UEs to use a type of RO based on the network load condition.

Observation 1:  A 1-bit indication in RACH-ConfigCommon is not suitable for dynamically distributing  the RA attempts from the SBFD UEs in legacy and additional RO type.

Hence we propose:

Proposal 2.1: For CBRA RO type indication, that NW includes a value between 0 and 1 based on the NW load in RACH-ConfigCommon. FFS the granularity of the value generated (how many bits).

Proposal 2.2: Upon receiving the indicated value, the UE generate a uniformly distributed random number between 0 and 1(using pseudo random number generator algorithm), and if the generated value is less than the value indicated by the NW, the UEs use the SBFD RO for random access.

## RA-RNTI collision in SBFD

For the additional RACH configuration in option 2, designed for SBFD-aware UEs, a challenge is avoiding RA-RNTI confusion caused by having two sets of RACH resources. The RA-RNTI is used by a UE to receive the Random Access Response (RAR) once the PRACH is transmitted. Different RACH occasions within a system frame are assigned unique RA-RNTI values to ensure that the RAR for a given PRACH transmission is directed only to the UE that used that specific RACH occasion. If two RACH occasions share the same RA-RNTI value, there is a risk that two UEs using different RACH occasions could incorrectly act on the same RAR message.

More specifically, the UE calculates the RA-RANTI when sending the Msg 1 of the RA procedure. According to TS38.321, the RA-RNTI is computed as

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id

where,

- s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14)

- t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80)

- f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8)

- ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier)

For PRACH configuration Option 2 (i.e., an SBFD-aware UE is configured with a legacy and additional PRACH configurations), there is a possibility that the SBFD ROs configured by the additional configurations may overlapped with the legacy ROs. This could happen in flexible symbols (as both legacy and SBFD ROs can be configured in flexible symbols). When this overlapping happens, there is a non-negligible possibility that at least s_id and t_id of the overlapping legacy RO and SBFD RO are the same (and of course the ul-carrier_id is the same since we are talking about a same carrier).

Now the tricky part is the f_id. If the legacy specs is kept as is, then at the point when there is such overlapping, there is a mismatch between the frequency indices being used by legacy UEs and the one being used by SBFD UEs. Indeed, as shown in figure below, although there is no issue at the non-overlapping time instance, there exist the possibility that two independent legacy and SBFD ROs may end up having the same f_id. This leads to the same RA-RNTI and making confusion for these UE when receiving the same RA-RNTI. Please note that  contention resolution in the current spec (msg 4) handles the case when two or UEs use the same RO and preamble. Here RA-RNTI collision happens for the case when two different RO share the same RA-RNTI due to misinterpretation of the f_id.

More illustration is shown below

- SBFD f_id : is the frequency index of any RO seen by SBFD-aware UE (not the f_id of the SBFD ROs only).

- Legacy f_id: is the frequency index seen by legacy UE.

Observation 2: In PRACH configuration Option 2, RA-RNTI collision happens when the SBFD ROs configured by the additional configurations overlap with the legacy ROs in flexible symbols.

Proposal 3: RAN2 to confirm that RA-RNTI collision may happen when the SBFD ROs configured by the additional configurations overlap with the legacy ROs in flexible symbols.

Proposal 4: To avoid RA-RNTI collision scenario described above, RAN2 to down select a solution from the option 1 and option 2 listed below

- Option 1: Defining a new rule for indexing of f_id when there is overlapping without modifying the existing RA-RNTI formula. When there is overlapping, the SBFD-aware UE determines the frequency index (f_id) by:

- Firstly, determining the frequency index of the legacy ROs by ignoring the SBFD ROs (i.e., aiming at keeping the same legacy index for the legacy ROs).

- Secondly, determining the frequency index of SBFD ROs by continuing the indexing starting from the index used for the last legacy RO plus one (and ignoring the legacy ROs).

- Option 2: When there is an overlap between the SBFD ROs and additional ROs in flexible symbols (RACH configuration Option 2), the SBFD UEs uses  s_id + 1 for the calculation of the RA-RNTI.

Please note that s_id range is 0-13, the s_id can be used max value is 12 in spec 38.211 table 6.3.3.2, then using s_id+1 will not exceed the spec range. So, the change will not impact the current formula.  Also, the minimum interval for multiple ROs is 2 symbols, and since the minimum PRACH duration requires 2 symbols. The minimum interval for S_ID is 2 symbols in one PCI, therefore, using s_id+1 will not conflict with the current spec.

## RO type switching from non-SBFD symbols to SBFD symbols.

In RAN2#129bis RAN2 made following agreement on initial attempt and re-attempt of the RACH procedure for an SBFD-aware UE.

RAN2#129bis Agreements RO-Type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols is supported when the number of PRACH transmission attempts exceed a threshold (we assume it is the same threshold with the fallback from additional RO to legacy RO). If fallback from legacy RO to additional RO occurs, no further fallback to legacy RO is supported.

A successful RA may not only depend on the interference condition in the RO. A RA failure for SBFD UEs using the legacy symbols could be due to two main reasons:

- Case 1: The SBFD-aware UE did not receive an RAR response from the NW (Msg1 failed to be received by the NW). A failure in the RAR response is an indication of bad coverage of PRACH.

- Case 2: The SBFD-aware UE did not receive the contention resolution message (Msg4). A failure in the Msg4 response is an indication of a collision happening between different UEs.

  - Note that the probability of collision in the RACH procedure may depend at least on the number of users trying to perform random access on RACH resources. For SBFD, collision probability may be very different between SBFD ROs and legacy ROs considering only SBFD-aware (Rel-19 or later) UEs may be capable of transmitting in the SBFD ROs.

If the RA failure is due to Case 1, there is limited advantage in having SBFD-aware UEs switch to additional RACH Occasions (ROs), as the Cross-Link Interference (CLI) could be higher in those ROs. However, if the failure is due to Case 2, switching to additional ROs can be beneficial since these ROs are likely to be less congested than the legacy RO. The UEs are capable of distinguishing between these two failure scenarios. Therefore, we propose:

Proposal 5: The fallback from legacy RO to an additional RO in SBFD symbols is supported only when the contention resolution is not succeeded for the SBFD-aware UEs for its RA attempts in legacy RO.

The following agreements are made in RAN2#127bis.

RAN2#127Bis Agreements - For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first.

However, the agreed way of RO type switching has the issue mentioned below.

If a UE initially selects an SBFD RO, but after multiple attempts, the NW observes that the UL signal strength does not improve and is unlikely to change in the next several attempts (e.g., 5 attempts), or NW observes that there is preamble congestion in the selected RO type,  the NW realize that a legacy RO would provide a better chance of success. However, the current procedure requires the UE to exhaust all RA attempts in the SBFD RO (RRC configured value) before switching to the more suitable legacy RO.  This results in unnecessary latency, as the UE keeps retrying in an inefficient RO type before switching, even when the network has a better alternative available.

Proposal 6: Discuss whether to allow the NW to indicate to the SBFD UEs via RAR message to switch the RO type.

# Conclusion

Observation 1:  A 1-bit indication in RACH-ConfigCommon is not suitable for dynamically distributing  the RA attempts from the SBFD UEs in legacy and additional RO type.

Observation 2: In PRACH configuration Option 2, RA-RNTI collision may happen when the SBFD ROs configured by the additional configurations overlap with the legacy ROs in flexible symbols.

Proposal 1: For Option 2, in case the legacy ROs and the additional ROs have the same value for one of the RACH configuration mandatory parameters, only the legacy RACH configuration will be configured with this parameter.  For Option 2, in case the SBFD aware UEs did not receive a dedicated mandatory parameter in the additional RACH configuration, the SBFD-aware UEs determine this dedicated parameter from the legacy RACH configuration.

Proposal 2.1: For CBRA RO type indication, that NW includes a value  between 0 and 1 based on the NW load in RACH-ConfigCommon. FFS the granularity of the value generated (how many bits).

Proposal 2.2: Upon receiving the indicated value, the UE generate a uniformly distributed random number between 0 and 1(using pseudo random number generator algorithm), and if the generated value is less than the value indicated by the NW, the UEs use the SBFD RO for random access.

Proposal 3: RAN2 to confirm that RA-RNTI collision may happen when the SBFD ROs configured by the additional configurations overlap with the legacy ROs in flexible symbols.

Proposal 4: To avoid RA-RNTI collision scenario described above, RAN2 to down select a solution from the option 1 and option 2 listed below

- Option 1: Defining a new rule for indexing of f_id when there is overlapping without modifying the existing RA-RNTI formula. When there is overlapping, the SBFD-aware UE determines the frequency index (f_id) by:

- Firstly, determining the frequency index of the legacy ROs by ignoring the SBFD ROs (i.e., aiming at keeping the same legacy index for the legacy ROs).

- Secondly, determining the frequency index of SBFD ROs by continuing the indexing starting from the index used for the last legacy RO plus one (and ignoring the legacy ROs).

- Option 2: When there is an overlap between the SBFD ROs and additional ROs in flexible symbols (RACH configuration Option 2), the SBFD UEs uses  s_id + 1 for the calculation of the RA-RNTI.

Proposal 5: The fallback from legacy RO to an additional RO in SBFD symbols is supported only when the contention resolution is not succeeded for the SBFD-aware UEs for its RA attempts in legacy RO.

Proposal 6: Discuss whether to allow the NW to indicate to the SBFD UEs  via RAR message to switch the RO type.