3GPP TSG-RAN WG2 #130	R2-2503860

St. Julians, Malta, May 19th – 23rd, 2025

Agenda item:	8.6.4

Source: 	Ericsson

Title:  	Open issues for Conditional LTM

Document for:	Discussion and Decision

# Introduction

This contribution discusses resource usage for a configured grant, but also brings up an issue with L2 reset that needs discussion.

# Discussion

## Configured Grant Activation

The discussion so far in RAN2 has concluded that a configured grant in the target cell enables the UE to make a RACH-less conditional cell switch. Since a configured grant configuration needs to be included within each LTM candidate configuration, this represent a very big burden for the network as a lot of resources needs to be reserved in advance (at the time of the conditional LTM preparation) and the network would not know for how long, since the LTM cell switch procedure in this case is triggered by the UE itself. For this reason, a large number of resources might be occupied just for the case of configuring an LTM candidate, and this is especially true when the decision to execute the cell switch is under UE control as for conditional LTM. The consequence of this is that the network may not be able to configure many LTM candidate configurations, thus not exploiting fully the benefits of LTM.

Reserving network resource in advance for an LTM candidate configuration is not efficient from a resource usage and efficiency perspective.

Due to the scarcity of available resources, network may be able to configure only a few LTM candidate configurations.

According to this, it would be beneficial from a resource usage (and efficiency) perspective if a configured grant could be allocated during the LTM configuration procedure but activated just in time for the cell switch execution.

This means that the network can indeed configure a CG within an LTM candidate configuration, but this CG will not be used by the UE unless the network explicitly indicates the UE that can use it. If the network does not indicate that the UE can use that CG during an LTM cell switch procedure, this means that the LTM cell switch procedure will be RACH-based, even if a valid TA is available. This may sound not efficient, but the benefit of this solution is that the same CG may be configured for more than one LTM candidate configuration and the network may decide to “activate” such CG only for the most promising LTM candidate configuration (network can figures out e.g., based on the UE’s measurements). How the activation of this CG is done, it can be further discussed. Thus, we propose:

The network can activate or deactivate a CG which is part of an (conditional) LTM candidate configuration.

## TA values after CLTM cell switch execution

Previous agreements in RAN2 have concluded what should happen for TA values and associated timers that the UE might have for other candidate cells when a cell switch is executed by the UE to a target cell:

RAN2#129bis agreement: Upon C-LTM execution, UE performs the following for received TA values and TA timers for other candidate cells than the target candidate cell: Maintain the TA value and Keep the early TA timer running.

This means that the UE will keep the TA values and timers for other candidates than the target cell. However, it is not clear for what purpose these will be kept by the UE. For what is currently agreed the only use case would be if the UE executes another conditional cell switch to one of the cells for which it has a TA value with a timer still running.

The use for a TA value to another candidate cell after a cell switch execution is limited since the network does not know of this TA value.

From a network perspective, if the new serving cell (i.e., the target cell to which the UE is switching to) does not know which TA value are kept by the UE and what is the status of their TAT, the agreement taken in RAN2 would not be very useful. In fact, if such information is not known, the new serving cell should assume that no TA values are available at the UE and thus the early UL synchronization procedure should be done from scratch for all the LTM candidate cells.

If the new serving cell does now know which TA values are kept at the UE, the new serving cell will assume that no TA are available at the UE and the early UL synchronization procedure needs to be re-triggered (or done from scratch).

According to this, it would be good if network can inform the new serving cell about which TA values and TAT for other LTM candidate cells the UE has kept. The benefit would be that the new serving cell would not need to re-trigger the early UL synchronization procedure again, unless if e.g., a TAT is going to expire soon or is expired. This will save resources and signalling.

If the new serving cell knows which TA values and TAT for other LTM candidate cells the UE has kept, the early UL synchronization procedure does not need to be re-triggered again, and this saves network resources and signaling.

Therefore, we propose:

The UE informs the new serving cell, within the RRCReconfigurationComplete message, which TA values and TAT for other LTM candidate cells the UE has kept.

## L2 reset coordination

During non-conditional LTM cell switch the target DU will be aware of which the source cell is through the Cell Switch Notification message, and therefore the target DU can understand if the ltm-noResetID differs and thus if a L2 reset has been performed at the UE side.

For the conditional case however, there are no message to the DU serving the target cell since the execution is initiated by the UE. Since the source cell can be not only a certain cell, but one out of a number of candidates (and the candidate configuration does not depend on which the source cell is), the target DU needs to know if a L2 reset has been performed at the UE side to act accordingly.

When the UE has executed CLTM, the target DU needs to be aware if the UE has performed L2 reset or not.

To handle this issue, we see three possible options:

- Good Bye message; The UE tells the source DU which candidate cell it intends to switch to, before it executes the cell switch so the source DU can inform the target with the relevant information.

- Hello message; The UE tells the target in the first available grant what the reset ID is, or potentially if it has reset its MAC buffer or not, to let the target DU know the appropriate action.

- CU synchronization; The CU will know the configuration of all UEs and which have conditional configuration. It can continuously send this information to the DUs to let them know which is the current source cell for each UE. When a DU receives an incoming cell switch it will inform the CU so the CU can send an updated list to all DUs.

Option 1) above has already been discussed in RAN2 and there is no support to introduce it, mainly because the robustness aspects will be affected if the message doesn’t get through to the source. For this reason we don’t think it is a good option.

Option 2) would rely on either a new MAC CE with highest priority in the LCP procedure alternatively an indication in the UCI to let the target DU know if the buffer has been reset or not. It seems feasible but comes with some specification impact.

Option 3) sounds like a stable option since the CU will have information about current state of all candidate configurations. This falls within the RAN3 domain so we think it would be best to consult them with evaluating the feasibility for these options as RAN2s preferred choice.

Letting the CU informing the DUs pro-actively would mean the target DU has information about if a L2 reset has been performed or not.

For CLTM, the CU shall update the DU(s) of other LTM candidates about the current serving cell for a UE at every LTM cell switch to allow the DU for the LTM candidate to understand whether an L2 reset needs to be done or not. Send an LS to RAN3 to inform them.

## LTM cell switch MAC CE for a candidate with execution conditions

In the previous RAN2 meeting we made the following agreement:

RAN2#129bis agreement: Network can send an LTM Cell Switch Command MAC CE indicating a CLTM candidate configuration (no specification change).

However, a case not discussed yet is what should happen if the UE receives the cell switch command and already has a valid TA (i.e., because was delivered previously). In Rel-18 LTM, the network can signal the value ‘FFF’ as the Timing Advance Command which indicates that no TA value is available from the network. However, if value “FFF” is included in the LTM cell switch MAC CE and the UE has a valid TA, it would be reasonable to allow the UE to still perform a RACH-less LTM cell switch procedure. This is also in line with the behavior we have in Rel-18 for the case on when the UE is configured with UE-based TA measurements.

If value “FFF” is included in the LTM cell switch MAC CE and the UE has a valid TA, it would be reasonable to allow the UE to still perform a RACH-less LTM cell switch procedure. This is also in line with the behavior we have in Rel-18 for the case on when the UE is configured with UE-based TA measurements.

Therefore, we propose:

If the network sends a LTM cell switch MAC CE with value “FFF” for the TA value, and the UE has a valid TA (either because previously send by the network or because of UE-based TA), the UE performs a RACH-less LTM cell switch procedure.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Reserving network resource in advance for an LTM candidate configuration is not efficient from a resource usage and efficiency perspective.

Observation 2	Due to the scarcity of available resources, network may be able to configure only a few LTM candidate configurations.

Observation 3	The use for a TA value to another candidate cell after a cell switch execution is limited since the network does not know of this TA value.

Observation 4	If the new serving cell does now know which TA values are kept at the UE, the new serving cell will assume that no TA are available at the UE and the early UL synchronization procedure needs to be re-triggered (or done from scratch).

Observation 5	If the new serving cell knows which TA values and TAT for other LTM candidate cells the UE has kept, the early UL synchronization procedure does not need to be re-triggered again, and this saves network resources and signaling.

Observation 6	When the UE has executed CLTM, the target DU needs to be aware if the UE has performed L2 reset or not.

Observation 7	Letting the CU informing the DUs pro-actively would mean the target DU has information about if a L2 reset has been performed or not.

Observation 8	If value “FFF” is included in the LTM cell switch MAC CE and the UE has a valid TA, it would be reasonable to allow the UE to still perform a RACH-less LTM cell switch procedure. This is also in line with the behavior we have in Rel-18 for the case on when the UE is configured with UE-based TA measurements.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The network can activate or deactivate a CG which is part of an (conditional) LTM candidate configuration.

Proposal 2	The UE informs the new serving cell, within the RRCReconfigurationComplete message, which TA values and TAT for other LTM candidate cells the UE has kept.

Proposal 3	For CLTM, the CU shall update the DU(s) of other LTM candidates about the current serving cell for a UE at every LTM cell switch to allow the DU for the LTM candidate to understand whether an L2 reset needs to be done or not. Send an LS to RAN3 to inform them.

Proposal 4	If the network sends a LTM cell switch MAC CE with value “FFF” for the TA value, and the UE has a valid TA (either because previously send by the network or because of UE-based TA), the UE performs a RACH-less LTM cell switch procedure.

# References

R2-25xxxxx- Running RRC CR for L1 measurements, vivo, RAN2#130, May 2025.