3GPP TSG-RAN WG2 Meeting #130                                       R2-2503876

St Julian's, Malta, 19 - 23 May 2025

Source:        ZTE Corporation

Title:           Discussion on LCM for positioning use case

Agenda item:   8.1.2.3

Document for:  Discussion and Decision

- Introduction

The revised WID [1] of Rel-19 NR_AIML_air related to AI/ML positioning is given as follows:

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

This contribution provides our view on LCM for AI/ML positioning use case, focusing on use case 1.

- Discussion

The inference procedure of 3 AI/ML positioning use cases described in the WID are briefly depicted as follows:

Figure 1. Use case 1, model inference is at UE side (1st priority)

Figure 2. Use case 3a, model inference is at gNB side (1st priority)

Figure 3. Use case 3b, model inference is at LMF side (1st priority)

Training data collection request

For usecase 1, UE needs to get training data set for training. The training data set can be UE’s own channel PRS measurement (with the label of UE’s own location), or can be other PRU’s PRS channel measurement (with the label of other PRU’s location/intermediate feature).

To acquire the UE’s own PRS channel measurement and the UE’s own location, UE needs to get PRS configuration and/or TRP information in advance. So, the LMF should be able to provide PRS configuration and/or TRP information to a UE (i.e., green arrows in the figure 4).

LMF should also be able to provide other UE/PRU’s PRS channel measurement with PRU/UE location/intermediate feature to a UE, for AI/ML training or monitoring (see green arrows in the figure 4).

Figure 4. LMF provides PRS configuration, TRP information and other UE/PRU’s PRS channel measurement with PRU/UE location to target UE for AI training

The provision of such assistance data for AI/ML training can be embedded in ProvideAssistanceData message. And the request of such assistance data for AI/ML training can be embedded in RequestAssistanceData message.

Proposal 1: For usecase 1, UE can request assistance data for AI/ML training using RequestAssistanceData message, and LMF can provide assistance data for AI/ML training using ProvideAssistanceData message.

Different training data set associated with different TRP information will have different impact on UE AI model’s performance, for example, the model trained using DL-PRS measurement results from 20 TRPs will be more accurate than using DL-PRS measurement results from 3 TRPs. To suit the current UE’s condition or given QoS requirement for AI/ML positioning, UE can request some specific assistance data that will have significant impact on model training, e.g.:

- UE requests the number of TRPs that the UE wants to receive DL-PRS from;

- UE requests the area of TRPs that the UE wants to receive DL-PRS from;

LMF may also provide other PRU’s measurement and PRU label to target UE for training. So UE is better to request the number of PRUs that the UE wants to receive the corresponding measurement and labels from.

Proposal 2: For case 1, UE can request some specific assistance data for training:

- UE requests the number of TRPs that the UE wants to receive DL-PRS from;

- UE requests the area of TRPs that the UE wants to receive DL-PRS from;

- UE requests the number of PRUs that the UE wants to receive the corresponding measurement and labels from.

Consistency between training and inference

There is a discussion on how to ensure the consistency between training and inference. The basic assumption is that, each AI model that UE trains should be under some certain condition, and the trained AI model should also be used for inference under the same or similar condition to achieve higher accuracy. From specification perspective, this means the set of attribute/parameter/condition of the training data input of an AI model should be same or similar as the set of attribute/parameter/condition of the inference data input.

For AI/ML positioning usecase 1, UE receives and measures TRP’s PRS for training and inference, so the set of attribute/parameter/condition of the training data input or inference data input can include NW additional conditions such as TRP location, TRP synchronization error, TRP number, PRS configuration, etc. It can be seen that for AI/ML positioning, the aforementioned NW additional conditions can already be indicated to the UE in current LPP spec since Rel-16.

Since detailed NW side additional condition is already provided to UE without any privacy concern, there is no need to use associated ID to represent a specific kind of NW side additional conditions. In addition, it is hard to set up explicit relationship between NW additional conditions and AI models, for example, an AI model can have good generalization and it can work well under several different additional conditions.

Observation 1: For AI/ML positioning, the NW additional conditions (e.g., TRP location, TRP synchronization error, TRP number, PRS configuration) can already be provided to the UE in current LPP spec.

Proposal 3: For AI/ML positioning usecase 1, ensuring the consistency between training and inference does not need an associated ID.

LMF is the node to provide inference configuration to UE, UE should strictly following NW’s configuration to perform inference. But LMF does not know which kind of inference configuration best suits the training configuration using which UE trained its AI model, e.g., how many number of TRPs used during training, which area the used TRPs locate on, the synchronization error range that the used TRP belongs to, etc. Therefore, UE should be able to request some specific TRP attributes/NW side additional conditions (for inference) that matches UE’s current well-trained AI model to ensure the training-inference consistency. The specific request can include the wanted number of TRPs, the wanted area of TRPs, the wanted synchronization error between TRPs for inference.

Since different AI model has different generalization and it is only known by UE, so only the UE knows how much difference the training and inference owns that will not have big impact to the inference performance, and to ensure the QoS requirement. The UE is able to ensure the requested value of TRP attribute to be consistent with the value used during training, and the UE ensures such difference can be covered by the good generalization performance.

The above request can reuse the current on-demand PRS request procedure.

Observation 2: When providing inference configuration to UE, LMF may not know which kind of inference configuration best suits the training configuration that UE used to train an AI model.

Proposal 4: In order to ensure consistency between training and inference, UE should be able to request some specific assistance data for inference which suits the UE’s current trained AI model:

- UE can request the wanted number of TRPs/cells for inference;

- UE can request the wanted area of TRPs/cells for inference;

- UE can request the wanted synchronization error range between TRPs for inference.

PRU info transfer

RAN1 agreed to let LMF transfer/forward PRU’s label/channel measurements to the target UE for training or monitoring, and NR-PRU-DL-Info can be reused for such delivery. Since current NR-PRU-DL-Info is introduced for calibration in CPP positioning, it only contains one PRU’s location and measurement information:

- – NR-PRU-DL-Info The IE NR-PRU-DL-Info is used by the location server to provide the carrier phase measurements with associated measurements and additional information reported by a PRU for UE-based DL-TDOA to a target UE. -- ASN1START NR-PRU-DL-Info-r18 ::= SEQUENCE { nr-PRU-LocationInfo-r18 LocationCoordinates OPTIONAL, -- Need ON nr-PRU-DL-TDOA-MeasInfo-r18 NR-DL-TDOA-SignalMeasurementInformation-r16 OPTIONAL, -- Need ON nr-PRU-DL-AoD-MeasInfo-r18 NR-DL-AoD-SignalMeasurementInformation-r16 OPTIONAL, -- Need ON nr-PRU-RSCP-MeasInfo-r18 NR-PRU-RSCP-MeasurementInformation-r18 OPTIONAL, -- Need ON ... }

When the IE is reused for AI/ML training or monitoring purpose, only one PRU’s part A/part B information at a time is not enough nor efficient to build a AI data set. Therefore, we suggest to create a list in ProvideAssistanceData containing a list of PRU’s part A/part B information for AI/ML. The list can be either contained in the legacy NR-PRU-DL-Info IE or be parallel to NR-PRU-DL-Info IE.

Proposal 5: Create a list in ProvideAssistanceData containing a list of PRU’s part A/part B information for AI/ML.

Open issues

This section discusses the identified open issues for AI/ML positioning in [POST129bis][015][AI PHY] 37.355 Running CR (Qualcomm).

LPP-7: Applicability of NR-SelectedDL-PRS-IndexList

NR-SelectedDL-PRS-IndexList is used to select several PFL(s), TRP(s), PRS resource set(s) and PRS resource(s) from NR-DL-PRS-AssistanceData (which can be common to all positioning methods). The nr-DL-PRS-AssistanceData and nr-SelectedDL-PRS-IndexList can be both optionally present in the new AI/ML positioning method, so even if nr-DL-PRS-AssistanceData in AI method and the nr-DL-PRS-AssistanceData in legacy method are different, LMF can make the nr-SelectedDL-PRS-IndexList in AI method to be absent.

Proposal 6: Support to include NR-SelectedDL-PRS-IndexList in the new AI/ML positioning method.

LPP-10a: Applicability of "batch reporting" for AI/ML positioning

Batch reporting is introduced to let UE report multiple measurements and/or multiple location estimates in a single ProvideLocationInformation signaling to LMF. The multiple location estimates are historical, considering currently LMF only forwards the latest PRU/UE’s part A/part B information to the target UE for training/monitoring purpose, it is not useful to let PRU/UE perform batch reporting associated with historical timestamps. Further, this is not an essential function when the target UE reports inference results, since periodical reporting can be used to achieve same effect but with more timely AI/ML location estimate provision.

Proposal 7: Do not support batch reporting in AI/ML positioning method.

LPP-13/14: Error cause in the new positioning method

In current LPP specification, the Error message is to indicate an error occurs at LMF or UE side during an on-going positioning method. For this new AI/ML positioning method, the Error message can be reused with some new error cause indication dedicated for AI/ML positioning use cases. RAN2#128 has already agreed to use Error message when the AIML based positioning method becomes non-applicable.

Since the new AI/ML method are applicable to all the LCM procedures, the target device error cause should be able to indicate the error occurred during training/inference/monitoring. For example:

Error cause during training:

UE does not get enough training data to train a AI/ML model;

UE cannot download or acquire a AI/ML model;

Error cause during inference:

UE cannot generate an estimate location using a AI/ML model;

Error cause during performance monitoring:

UE cannot generate a performance monitoring metric.

Proposal 8: The target device error cause is indicated per LCM procedure (i.e.  training/inference/monitoring), E.g.:

- Error cause during training:

  - UE does not get enough training data to train a AI/ML model;

  - UE cannot download or acquire or train a AI/ML model;

- Error cause during inference:

  - UE cannot generate an estimate location using a AI/ML model;

- Error cause during performance monitoring:

  - UE cannot generate a performance monitoring metric.

LPP-15: Applicability of Positioning Integrity to AI/ML positioning

RAN1 has already agreed to deliver integrity related information from LMF to UE.

RAN1#118b Agreement For AI/ML positioning Case 1, regarding the assistance data provided from LMF to UE, for ensuring consistency between training and inference, - for each of the existing assistance data IE of UE-based DL-TDOA and/or UE-based DL-AoD, study whether it should be: (a) explicitly indicated, (b) implicitly indicated and/or (c) other; - Companies can provide inputs on further enhancements of existing assistance data, including new information - Note: this does not mean that training and inference phases are mentioned in assistance data. Table. Existing assistance data (supported up to Rel-18) that may be transferred from LMF to UE in UE-based DL-TDOA [1] or UE-based DL-AoD [2], as applicable. Information; UE-based DL-TdoA; UE-based DL-AoD. 1; Physical cell IDs (PCIs), global cell IDs (GCIs), ARFCN, and PRS IDs of candidate NR TRPs for measurement. 2; Timing relative to the serving (reference) TRP of candidate NR TRPs. 3; DL-PRS configuration of candidate NR TRPs. 4; Indication of which DL-PRS Resource Sets across DL-PRS positioning frequency layers are linked for DL-PRS bandwidth aggregation. 5; SSB information of the TRPs (the time/frequency occupancy of SSBs). 6; Spatial direction information (e.g. azimuth, elevation etc.) of the DL-PRS Resources of the TRPs served by the gNB. 7; Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs). 8; Fine Timing relative to the serving (reference) TRP of candidate NR TRPs. 9; PRS-only TP indication. 10; The association information of DL-PRS resources with TRP Tx TEG ID. 11; LOS/NLOS indicators. 12; On-Demand DL-PRS-Configurations, possibly together with information on which configurations are available for DL-PRS bandwidth aggregation. 13; Validity Area of the Assistance Data. 14; PRU measurements together with the location information of the PRU. 15; Data facilitating the integrity results determination of the calculated location. 16; TRP beam/antenna information (including azimuth angle, zenith angle and relative power between PRS resources per angle per TRP). 17; Expected Angle Assistance information. 18; PRS priority list. [1] Table 8.12.2.1.0-1 in 38.305, Use equipment (UE) positioning in NG-RAN (Release 18), v18.3.0 [2] Table 8.11.2.1.0-1 in 38.305, Use equipment (UE) positioning in NG-RAN (Release 18), v18.3.0 RAN1#119 Agreement For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE.

So based on the above, the integrity related assistance data can be provided to UE in AI/ML positioning. The integrity related assistance data for AI/ML should take the integrity related assistance data for DL-TDOA as baseline.

Proposal 9: Support LMF to provide integrity related assistance data to UE in AI/ML positioning method. The integrity related assistance data for AI/ML should take the integrity related assistance data for DL-TDOA as baseline.

LPP-16: Performance monitoring results transfer

RAN1#116-bis [5] has the following agreement on performance monitoring:

Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded.

Based on the agreement, there are 4 kinds of performance monitoring control procedure under RAN1’s decision:

- Option 1: UE generates monitoring metric, and UE makes monitoring decision;

- Option 2: LMF generates monitoring metric, and LMF makes monitoring decision;

- Option 3: UE generates monitoring metric, and LMF makes monitoring decision;

- Option 4: LMF generates monitoring metric, and UE makes monitoring decision.

Option 1 and option 2 appear to have no LPP specification on performance monitoring.

If option 3 is adopted, LMF needs to request UE to perform monitoring, and UE needs to report monitoring metric to LMF.

If option 4 is adopted, LMF needs to indicate monitoring metric to UE.

RAN1#119 further agrees the following to support the above option 1 or option 3 which requires spec impact:

Agreement For model performance monitoring of AI/ML positioning Case 1, support at least: - Option A. The target UE side performs monitoring metric calculation. - The target UE may signal the monitoring outcome to the LMF. - FFS: content of monitoring outcome - FFS: Option B

For the above potential spec impact, RAN2 needs to discuss whether a new set of signaling/procedure dedicated for performance monitoring is necessary, or to embed it in other existing procedures (e.g. Location Information transfer procedure).

Agreement (RAN1#120bis) For model performance monitoring of AI/ML positioning Case 1, “FFS: content of monitoring outcome” in RAN1#119 agreement is resolved by: • the content of monitoring outcome includes at least an indication that the target UE cannot perform the Case 1 positioning method.

According to the above agreement, the UE’s calculated performance monitoring metric/outcome contains an indicator that target UE cannot perform the Case 1 positioning method. However such indication is already covered by the error cause that RAN2 agreed for the new AI/ML method, so if RAN1 only agrees this indication in the monitoring outcome, no additional signaling is needed. Error message can be reused for this purpose.

Observation 3: Current content of performance monitoring outcome does not require a new LPP signaling. It can be covered by the error message.

So we suggest to wait for RAN1 to see if there will be other monitoring outcome before RAN2 decides to create a new signaling or reuse the old signaling.

Proposal 10: For performance monitoring signaling, wait for RAN1 to see if there is any other monitoring outcome, before RAN2 decides to create a new signaling or to reuse the legacy signaling.

LPP-17: Signalling of "ground-truth label" information

RAN1 has achieved the following agreements to enable LMF transferring UE’s label to the UE/PRU itself:

Agreement For AI/ML based positioning Case 1, from RAN1 perspective, when the label data of location is generated by LMF and transferred from LMF to UE, label and quality indicator of label can be provided by reusing existing IEs. - From RAN1 perspective, the existing IE can use one of the geographic shapes defined in TS 23.032. The location estimate uncertainty and confidence (if included with the geographic shapes) can serve as quality indicator of the label.

RAN1 does not have clear picture of whether the signalling from LMF to UE is LPP or others, e.g. NAS signaling. If the signalling is LPP, we suggest to add it in the NR-PositionCalculationAssistance message.

Proposal 11: The signalling from LMF to UE to transfer the UE’s label and quality indicator is added in the NR-PositionCalculationAssistance message.

- Conclusion

In this contribution, we propose the following observation and proposals:

Training data collection request

Proposal 1: For usecase 1, UE can request assistance data for AI/ML training using RequestAssistanceData message, and LMF can provide assistance data for AI/ML training using ProvideAssistanceData message.

Proposal 2: For case 1, UE can request some specific assistance data for training:

- UE requests the number of TRPs that the UE wants to receive DL-PRS from;

- UE requests the area of TRPs that the UE wants to receive DL-PRS from;

- UE requests the number of PRUs that the UE wants to receive the corresponding measurement and labels from.

Consistency between training and inference

Observation 1: For AI/ML positioning, the NW additional conditions (e.g., TRP location, TRP synchronization error, TRP number, PRS configuration) can already be provided to the UE in current LPP spec.

Observation 2: When providing inference configuration to UE, LMF may not know which kind of inference configuration best suits the training configuration that UE used to train an AI model.

Proposal 3: For AI/ML positioning usecase 1, ensuring the consistency between training and inference does not need an associated ID.

Proposal 4: In order to ensure consistency between training and inference, UE should be able to request some specific assistance data for inference which suits the UE’s current trained AI model:

- UE can request the wanted number of TRPs/cells for inference;

- UE can request the wanted area of TRPs/cells for inference;

- UE can request the wanted synchronization error range between TRPs for inference.

PRU info transfer

Proposal 5: Create a list in ProvideAssistanceData containing a list of PRU’s part A/part B information for AI/ML.

Open issues

Observation 3: Current content of performance monitoring outcome does not require a new LPP signaling. It can be covered by the error message.

Proposal 6: Support to include NR-SelectedDL-PRS-IndexList in the new AI/ML positioning method.

Proposal 7: Do not support batch reporting in AI/ML positioning method.

Proposal 8: The target device error cause is indicated per LCM procedure (i.e.  training/inference/monitoring), E.g.:

- Error cause during training:

  - UE does not get enough training data to train a AI/ML model;

  - UE cannot download or acquire or train a AI/ML model;

- Error cause during inference:

  - UE cannot generate an estimate location using a AI/ML model;

- Error cause during performance monitoring:

  - UE cannot generate a performance monitoring metric.

Proposal 9: Support LMF to provide integrity related assistance data to UE in AI/ML positioning method. The integrity related assistance data for AI/ML should take the integrity related assistance data for DL-TDOA as baseline.

Proposal 10: For performance monitoring signaling, wait for RAN1 to see if there is any other monitoring outcome, before RAN2 decides to create a new signaling or to reuse the legacy signaling.

Proposal 11: The signalling from LMF to UE to transfer the UE’s label and quality indicator is added in the NR-PositionCalculationAssistance message.

- Reference

- RP-250792, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, 2025-03