3GPP TSG RAN WG2 #130          						R2- 2503879

Malta, Malta, May 19th – 23rd, 2025

Source:	Tejas Networks Ltd.

Title:	Discussion on A-IoT message format for CBRA and CFRA

Agenda item:	8.2.3

Document for:     Discussion and Decision

# Introduction

As discussed in TR38.769 [1], the A-IoT devices receive Msg0 paging signal from reader or base station (BS), upon receiving Msg0, multiple A-IoT devices transmit Msg1 which carries a unique random ID of the device in response to Msg0. Transmission of Msg1 can be contention-based or contention-free random access. In case of contention-based random access, the devices transmit Msg1 using slotted ALOHA-based multi-access. In response to Msg1, the reader transmits Msg2 with scheduling information for the device to transmit the actual device ID and group ID. Based on the resource allocated to the devices, they send their data consists of device ID and group ID. Fig. 1 shows the timing diagram of A-IoT communication between the reader and a device. At every communication slot or occasion, the reader initiates the communication by sending a paging signal and the slot ends when the device received acknowledgement of msg3 or the command response.

The random-access triggering by sending the paging message called “msg0” can be either contention-free random access (CFRA) or contention-bases random access (CBRA).

The CFRA can be a 2-step process as shown in Fig. 1(a) is as follows:

- Step A: Msg0 (R2D): Device receives an inventory command from the reader

- Step B: Msg1 (D2R): Device transmits UL data (such as device identifier, device type, etc.) to the reader

The CBRA can be a 4-step process as shown in Fig. 1(b) is as follows:

- Step 0: Msg0 (R2D): Device receives an inventory command from the reader

- Step 1: Msg1 (D2R): Device transmits a device random ID to the reader

- Step 2: Msg2 (R2D): Device receives an acknowledgment of the device random ID from the reader and scheduling information required to transmit Msg3

- Step 3: Msg3 (D2R): Device transmits UL data (such as device identifier, device type, etc.) to the reader

- Step 4 (optional): Msg4 (R2D): Device receives an ACK/NAK signal for Msg3 from the reader

Fig. 1(a): 2-step CFRA/CBRA.                                                Fig. 1(b): 4-step CFRA/CBRA.

The timing diagram of A-IoT communication between the reader and a device is shown in Fig. 2. Each transaction cycle may consist of one or multiple communication slots. At every communication slot or occasion, the reader initiates the communication by sending a paging signal and the slot ends when the device received acknowledgement of msg3 or the command response.

Fig. 2: Timing diagram of A-IoT communication involved multiple transactions.

# A-IoT Msg0 Content/Format for Contention-based and Contention-Free Random Access

Access occasion: An opportunity of time-frequency resource for A-IoT device(s) to perform access (e.g., transmitting the A-IoT Msg1 by the device). A set of access occasion(s) for different A-IoT device(s) is scheduled via the R2D message (referring to the "R2D transmission triggering random access" in clause 6.1.4) by the reader.

When the A-IoT device is selected to respond in accordance to the clause 6.3.3, the A-IoT device performs the following procedure:

-	Step 1: Random access type (i.e., contention-free or contention-based) and access occasion/resource determination:

-	The A-IoT device determines the random access type from the A-IoT paging message, in accordance to clause 6.3.3. The reader can configure either contention-free access or contention-based random access (and the corresponding configurations). It needs to be further discussed on whether it is explicitly or implicitly.

-	If the random access is contention-free access:

-	Selects the indicated D2R occasion/resource;

-	Skips the contention resolution in Step 2 and performs the data transmission in accordance to clause 6.3.5.

-	If the random access is contention-based random access:

-	Performs access occasion/resource selection: as the baseline for CBRA, at least for TDMA case, the device can randomly select one access occasion for A-IoT Msg1 within the access occasions provided/assigned by the reader. It needs to be further discussed if this is applicable to FDMA case. Further enhancement option(s) can be also considered after more physical layer detailed design on TDMA and FDMA;

-	Performs the Step 2 for contention resolution.

The following has been studied in TR 38.769, section 6.3.4 on random access type [1]:

- Agreements

- The A-IoT paging message can include a number of msg1 resources

- From RAN2 perspective, after initial paging message, the R2D transmission which determines the Msg1 resource(s), can be achieved by one of the below two ways, unless RAN1 concludes to use L1 signaling later:

- Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or

- Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field

3.  The service type of A-IoT (e.g., inventory only, inventory + command) is not included in paging message.

The following agreements have been made in RAN2#129 [2] on one or multiple A-IoT paging messages:

As to the A-IoT paging message, the identifier may be required to identify the device/group of devices in this trigger message (e.g., for the case of reaching a single or a group of devices). Following cases are studied:

-	The A-IoT paging message containing an identifier of a single A-IoT device

-	The A-IoT paging message containing a group ID that maps to multiple A-IoT devices

-	The A-IoT paging message that does not contain any identifier, i.e., indicating all A-IoT devices that can receive the A-IoT paging message need to respond

-	The A-IoT paging message containing multiple identifiers of A-IoT devices. The need for this use case is still to be confirmed/dependent according to the conclusion in [151]. From RAN2 perspective, it is feasible to support paging multiple identifiers of A-IoT devices, pending on TB size and multiplexing design of A-IoT paging message.

The following has been studied in TR 38.769, section 6.3.3 for A-IoT paging message content [1]:

The following agreements were made on CFRA/CBRA in RAN2#129-bis [3]:

Agreements on CFRA

Introduce an explicit 1 bit indication to indicate whether it is CFRA or CBRA per paging message.

The following agreements were made on Msg1 resource in RAN2#129-bis [3]:

Agreements on new R2D message

- A new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling.   The R2D message indicates the start of a set of MSG1 resources that were configured in paging message.

- Assumption: The R2D message does not include slot number/count down number.

Unified Msg0 structure for all the cases

The R2D transmission in way-1 as a part of paging message can be a unique or unified message structure for all query types mentioned above in TR38.769 for both CBRA and CFRA.

Fig. 3: Unified Msg0 structure for all the paging/query types for 2/4-step CRFA/CBRA.

The PRDCH data part contains the R2D information bits from layer2 is shown in Fig. 3. The unified msg0 structure is proposed for the data part, where the reader conveys the query type to the device and accordingly allocate the time and frequency domain resources.

The field parameters of msg0 are described below:

- Message type

- Transaction ID

- Reader ID (if required)

- Paging ID presence indicator

- Paging ID length

- Paging ID

- CBRA/CFRA

- Number of R2D triggers

- Time domain resources (X=1 or X=2)

- Frequency domain resources

Paging ID: This field contains device ID(s), in case the reader sends query to one or few specific devices. This field contains group ID if the reader sends query to a group of devices. This field is either empty or contains ‘0’ when device sends query to all the devices within the coverage.

Time domain resources: The msg0 may carry 1-bit to convey the number of time domain resources to the device. Bit ‘1’ indicates X=1 within two R2D triggers. Bit ‘0’ indicates X= 2 within two R2D triggers.

Frequency domain resources:

- Option 1: List of frequency resources (F1 … FY): This field contains the frequency domain resources either dedicatedly allocated to the device(s) or available frequency domain resources for the devices to send msg1.

- Option 2: Total number of frequency resources: This indicates the total number of frequency resources that are allocated for sending msg1 for the devices being paged in this msg0.

Observation 1: The R2D transmission as a part of paging message or the msg0 can be a unique or unified message structure for all query/paging types for both CBRA and CFRA.

Proposal 1: The field parameters of msg0 includes:

- Message type

- Transaction ID

- Reader ID (if required)

- Paging ID presence indicator

- Paging ID length

- Paging ID

- CBRA/CFRA

- Number of R2D triggers

- Time domain resources (X=1 or X=2)

- Frequency domain resources

Proposal 2: Msg0 signal may carry 1-bit to convey the number of time domain resources for an R2D trigger to the device. Bit ‘1’ indicates X=1 within two R2D triggers. Bit ‘0’ indicates X= 2 within two R2D triggers.

Proposal 3: Msg0 should contain the Frequency domain resources for msg1 either implicitly or explicitly:

- Option 1: List of frequency resources (F1 … FY)

- Option 2: Total number of frequency resources

# A-IoT Msg1 Content/Format for Contention-based and Contention-Free Random Access

The following agreements were made on Msg1 resource in RAN2#129-bis [3]:

Agreements on msg1

      - In case of CBRA, only 16 bits random ID is included in Msg1.  FFS can be revisited if message type will be needed for other D2R messages purposes

      - RN16 is not included in the first D2R message in the CFRA procedure.  AS ID is the only ID needed for addressing the device in R2D command message assuming for CFRA no multiple devices are performing the procedures with the given reader.   FFS if we can assume or need to support multiple device scenario.

Unified Msg1 structure for A-IoT msg1 with or without data

Fig. 4: Unified Msg1 structure for 2/4-step CRFA/CBRA.

Unified message structure of D2R msg1 for all the query types for both 2-step and 4-step CBRA and CFRA is shown in Fig. 4. The field parameters of msg1 includes:

- Random ID

- Message type

- Data size

- Device energy status

- Data presence indicator

- Upper layer data

In case of CFRA Msg1 may also carry the device random ID to support future AIoT communications, where a reader can send command to multiple devices simultaneously. Reader may use this random ID to send command to the specific devices.

The following has been studied in TR 38.769, section 6.3.5 on A-IoT data transmission [1]:

It is beneficial for the reader to know an estimate of expected D2R message size. This D2R message size information would be beneficial but it is not essential. Two options are studied:

-	Option 1: The estimate of expected following D2R message size from the CN to the reader;

-	Advantages/disadvantages of option 1: If this information is available from CN, it saves the overhead/latency caused by Option 2 in AS layer. It can be also used by the reader as early as for the first D2R message, if needed.

-	Option 2: The simple message size indication from the device to the reader.

-	Advantages/disadvantages of option 2: It is useful in case the CN is not able to provide or does not provide the expected D2R message size. Limited granularity is supported considering the AS layer overhead. It requires D2R message to carry this information in advance, before the reader can actually use this size information for scheduling.

As mentioned in option 2, device can convey the msg3 size through msg1 to the reader. Accordingly, reader allocates resources to the devices. Thus, data size in msg1 is the size of upper layer data to be conveyed through msg3 in the 4-step process. In 2-step process, data size is either the size of data present in the field “upper layer data” or it is ‘0’ to convey that there is no msg3.

Device energy status: Device may send the device energy status either implicitly or explicitly to the reader as a scheduling parameter.

Data presence indicator: For 2-step process, the data presence indicator is set to ‘1’. The device sends the upper layer data, which is the actual information consists of device ID, group ID, etc. through msg1 only. For 4-step process, the data presence indicator is set to ‘1’. The device sends the upper layer data through msg3. In that case, the upper layer data field in msg1 is empty or no data present in that field.

Upper layer data: For 2-step process, the device sends the upper layer data, which is the actual information consists of device ID, group ID, etc. through msg1 only. For 4-step process, the device sends the upper layer data through msg3. In that case, the upper layer data field in msg1 is empty or no data present in that field.

Observation 2: The D2R transmission to send msg1 can be a unique or unified message structure for all query/paging types for both CBRA and CFRA.

Proposal 4: RAN2 to consider a unified msg1 structure for both 2-step and 4-step random access as well as rel-19 and rel-20 devices.

Proposal 5: The field parameters of msg0 includes:

- Random ID

- Message type

- Data size

- Device energy status

- Data presence indicator

- Upper layer data

Observation 3: In case of CFRA Msg1 may also carry the device random ID to support future AIoT communications, where a reader can send command to multiple devices simultaneously. Reader may use this random ID to send command to the specific devices.

Proposal 6: For 2-step process, msg1 contains the device generated unique random ID, data size (size of upper layer data present in msg1), device energy status, 1-bit data presence indicator (set to ‘1’), and upper layer data.

Proposal 7: For 4-step process, msg1 contains the device generated unique random ID, data size (size of upper layer data to be conveyed through msg3), device energy status, 1-bit data presence indicator (set to ‘0’), data field is not present.

Proposal 8: The device can transmit the device energy status to the reader through msg1 to inform the energy status of the device, which the reader can use for prioritizing/ deprioritizing the device for scheduling. If the energy available at the device is below certain threshold, reader may choose not to allocate resource to that device.

# Conclusion

This work includes unified message structure of the A-IoT Msg0 and Msg1 content/format for contention-based and contention-free random access for all the paging query types. We have made the following observations and proposals related to these aspects:

Observation 1: The R2D transmission as a part of paging message or the msg0 can be a unique or unified message structure for all query/paging types for both CBRA and CFRA.

Proposal 1: The field parameters of msg0 includes:

- Message type

- Transaction ID

- Reader ID (if required)

- Paging ID presence indicator

- Paging ID length

- Paging ID

- CBRA/CFRA

- Number of R2D triggers

- Time domain resources (X=1 or X=2)

- Frequency domain resources

Proposal 2: Msg0 signal may carry 1-bit to convey the number of time domain resources for an R2D trigger to the device. Bit ‘1’ indicates X=1 within two R2D triggers. Bit ‘0’ indicates X= 2 within two R2D triggers.

Proposal 3: Msg0 should contain the Frequency domain resources for msg1 either implicitly or explicitly:

- Option 1: List of frequency resources (F1 … FY)

- Option 2: Total number of frequency resources

Observation 2: The D2R transmission to send msg1 can be a unique or unified message structure for all query/paging types for both CBRA and CFRA.

Proposal 4: RAN2 to consider a unified msg1 structure for both 2-step and 4-step random access as well as rel-19 and rel-20 devices.

Proposal 5: The field parameters of msg0 includes:

- Random ID

- Message type

- Data size

- Device energy status

- Data presence indicator

- Upper layer data

Observation 3: In case of CFRA Msg1 may also carry the device random ID to support future AIoT communications, where a reader can send command to multiple devices simultaneously. Reader may use this random ID to send command to the specific devices.

Proposal 6: For 2-step process, msg1 contains the device generated unique random ID, data size (size of upper layer data present in msg1), device energy status, 1-bit data presence indicator (set to ‘1’), and upper layer data.

Proposal 7: For 4-step process, msg1 contains the device generated unique random ID, data size (size of upper layer data to be conveyed through msg3), device energy status, 1-bit data presence indicator (set to ‘0’), data field is not present.

Proposal 8: The device can transmit the device energy status to the reader through msg1 to inform the energy status of the device, which the reader can use for prioritizing/ deprioritizing the device for scheduling. If the energy available at the device is below certain threshold, reader may choose not to allocate resource to that device.

# References

[1] TR 38.769, Study on solutions for ambient IoT (Internet of Things) in RAN, V2.0.0.

[2] 3GPP RAN2 Chairman’s Notes, 3GPP TSG RAN2 meeting #129.

[3] 3GPP RAN2 Chairman’s Notes, 3GPP TSG RAN2 meeting #129-bis.