GPP TSG-RAN WG2#130   									           R2-2503880

St Julian’s, Malta, May 19th – 23th, 2025

Agenda Item:	8.9.3

Source: 	NEC

Title:	        Discussion on UL Capacity Enhancement for IoT-NTN

Document for:	 Discussion and Decision

- Introduction

In this contribution, we follow the RAN2#129bis agreements and further discuss the FFS points and other points for CB-msg3 EDT on the following topics:

- CB-msg3 resource configuration: DSA window, Frequency domain configuration and OCC

- Contention resolution: single RNTI / multiple RNTIs, CB-Msg4 monitoring window starting time

- CB-Msg4 design

- CB-msg3 EDT re-attempt and fallback handling

- Discussion

RAN1 has replied to RAN2 LS on OCC for CB-msg3 NPUSCH and confirmed that the OCC solution that RAN1 is working on might be applied to CB-msg3 NPUSCH format 1 single tone at least for OCC length 2, for the subcarrier spacing (s) for which CB-msg3 is supported when the power imbalance is small [1].

Fig. 1 below illustrates the CB-msg3 with multiple resources in the time, frequency, and code domains (if considered in R-19).

Fig. 1 CB-msg3

  - CB-msg3 PUSCH resources configuration

For the legacy NB-IoT random access procedure, the number of RUs and TBS for EDT are defined in Table 16.3.3-2 and Table 16.3.3-3 in [2], and EDT-TBS-InfoList-NB-r15 in [3] as below:

Table 16.3.3-2: EDT TBS for Msg3 NPUSCH with edt-SmallTBS-Enabled set to 'true' [2]

| edt-TBS | edt-SmallTBS-Subset | Allowable TBS values |
| --- | --- | --- |
| 408 | not configured | 328, 408 |
| 504 | not configured | 328, 408, 504 |
| 504 | enabled | 408, 504 |
| 584 | not configured | 328, 408, 504, 584 |
| 584 | enabled | 408, 584 |
| 680 | not configured | 328, 456, 584, 680 |
| 680 | enabled | 456, 680 |
| 808 | not configured | 328, 504, 680, 808 |
| 808 | enabled | 504, 808 |
| 936 | not configured | 328, 504, 712, 936 |
| 936 | enabled | 504, 936 |
| 1000 | not configured | 328, 536, 776, 1000 |
| 1000 | enabled | 536, 1000 |

Table 16.3.3-3: MCS index for Msg3 NPUSCH and EDT [2]

| MCS Index | Number of RUs |  |  |
| --- | --- | --- | --- |
|  | edt-TBS = 328, 408, 504, or 584 | edt-TBS = 680 | edt-TBS = 808, 936, or 1000 |
| '011' | 3 | 3 | 4 |
| '100' | 4 | 4 | 5 |
| '101' | 5 | 5 | 6 |
| '110' | 6 | 8 | 8 |
| '111' | 8 | 10 | 10 |

EDT-TBS-InfoList-NB-r15 ::=	SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF EDT-TBS-NB-r15

EDT-TBS-NB-r15 ::=	SEQUENCE {

edt-SmallTBS-Enabled-r15		BOOLEAN,

edt-TBS-r15						ENUMERATED {b328, b408, b504, b584, b680, b808, b936, b1000}

}

Observation 1: The candidate RU numbers for EDT are 3, 4, 5, 6, 8, and 10, with each RU number covering various TBSs in the range of 328 to 1000.

The duration of a RU in different SCS and tone configurations for frame structure type1 and NPUSCH format 1 are defined in Table 10.1.2.3-1 of [4] as below:

Table 10.1.2.3-1: Supported combinations of , , and  for frame structure type 1[4]

| NPUSCH format |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | 3.75 kHz | 1 | 16 | 7 |
|  | 15 kHz | 1 | 16 |  |
|  |  | 3 | 8 |  |
|  |  | 6 | 4 |  |
|  |  | 12 | 2 |  |
| 2 | 3.75 kHz | 1 | 4 |  |
|  | 15 kHz | 1 | 4 |  |

Observation 2: The duration of one replica depends on the required number of RUs, the SCS, and the number of subcarriers.

Fig. 2 Different CE levels with different RU numbers and repetition numbers

Observation 3: For NB-IoT, the time duration for one CB-msg3 resource could be up to 40.96 s (i.e. 3.75 kHz SCS, 10 RUs, 160 slots, 128 repetitions), which is longer than the duration of one hyper-frame.

Observation 4: For NB-IoT, the time duration for one CB-msg3 resource might neither divides one hyper-frame duration nor is divisible by one hyper-frame duration.

Proposal 1: For the time domain CB-msg3 resource configuration parameters, both the hyper-frame number (H-SFN) and frame number (SFN) are needed.

Proposal 2: For the DSA window starting time:

Option 1: DSA window starts with the CB-msg3 resource together.

Option 2: DSA window stars with an offset to the CB-msg3 resource.

Option 3: DSA window stars at a configured H-SFN and SFN.

A DSA window would allow UEs to select CB-msg3 resources across different time domains for replica transmissions when the number of replicas is larger than 1. The window size should be proportional to the CB-msg3 resource periodicity (i.e., scaled by the replica count). Thus, the DSA window size can be implicitly derived from replica numbers and CB-msg3 periodicity without explicit network signaling.

Proposal 3: The DSA window size may be:

Option 1: Implicitly indicated by the DSA replica number times the PUSCH resource periodicity.

Option 2: Explicitly indicated by the network.

Since CB-msg3 resources are periodic, the DSA window will recur with a periodicity scaled by the CB-msg3 resource period. Consequently, the DSA window periodicity can also be implicitly derived.

Proposal 4: The DSA window periodicity may be:

Option 1: Implicitly indicated by the DSA window size.

Option 2: Explicitly indicated by the network.

If both the DSA window size and DSA window periodicity are adopted, the network may configure the periodicity to be equal to or larger than the window size. This configuration would create temporal gaps between consecutive DSA windows, leading to inefficient utilization of time-domain resources.

Furthermore, according to RAN2's working assumption, the UE operation involves two sequential steps: first selecting the next DSA transmission window, then randomly choosing K replicas within the selected window. Configuring the periodicity to exceed the window size would not only result in resource fragmentation but also introduce unnecessary delays in DSA CB-msg3 transmissions.

Proposal 5: Eliminate the DSA window periodicity parameter by aligning it with the DSA window size (i.e., setting periodicity equal to window size).

For the CB-msg3 frequency domain resource configurations, the legacy NPRACH configures a set of contiguously allocated subcarriers as a pool for the UEs to choose one of the subcarriers to send the preambles by nprach-SubcarrierOffset and nprach-NumSubcarriers, and the legacy PUR configures a set of contiguously allocated subcarriers by the prb-AllocationInfo or npusch-SubCarrierSetIndex in PUR-Config.

Observation 5: For legacy PUR and NPRACH, the frequency domain resource is a set of contiguously allocated subcarriers.

Proposal 6: RAN2 further discusses the following frequency-domain configuration options:

Option 1: A single contiguous subcarrier set, i.e. one subcarrier starting point with one range

Option 2: Multiple contiguous subcarrier sets, i.e. several subcarriers starting points with corresponding ranges

NB-IoT has 1) periodicity fixed downlink subframes for NPBCH, NPSS, and SSS; and 2) non-periodicity downlink subframes for SIs. Due to the half-duplex Type B constraint, it will be complicated to configure/determine the CB-Msg3 EDT resources in the anchor carrier.

Proposal 7: RAN2 should configure non-anchor carrier resources for CB-msg3 EDT transmission in SIB22-NB.

For CB-msg3-EDT, UE should randomly choose the frequency resource to randomize the contention from each other. Carrier is a kind of frequency resource set, thus it is straightforward for a UE to randomly choose one of the configured carriers to transmit the NPUSCH when multiple carriers are configured for CB-Msg3-EDT.

Proposal 8: When multiple carriers are provided for CB-msg3-EDT resource for the same enhanced coverage level from the SIBx, it is up to the UE implementation to select any of the carriers.

In RAN2 #129, we have the following working assumption:

Working assumption: The UE first selects the next DSA transmission window and then randomly select K replicas inside the window.

When CB-msg3 EDT procedure is triggered, the UE will determine the indexes of the replica occasions randomly. If the determined first replica is after the triggering timing, the UE will transmit CB-msg3 since the replica occasion. Taking Fig. 3 as an example, the UE determines the indexes of replica occasions as 3, 5, and 6, when the CB-msg3 EDT procedure is triggered. As the timing of the first occasion (occasion 3) is after the triggering time, the UE transmits CB-msg3 since occasion 3. Taking Fig. 4 as an example, the UE determines the indexes of replica occasions as 1, 3, and 5, when the CB-msg3 EDT procedure is triggered. As the timing of the first occasion (occasion 1) is before the triggering time, the UE transmits CB-msg3 since occasion 1 in the N+1 DSA window.

Using this procedure, the UE could transmit CB-msg3 timely manner without any unnecessary latency.

Fig. 3 Determined replica occasion instance

Fig. 4 Another determined replica occasion instance

Proposal 9: The next DSA window is the first window instance containing all the selected replica occasions after CB-msg3 is triggered.

Given RAN1’s emphasis on power imbalance for OCC-based CB-msg3 NPUSCH, narrower RSRP thresholds between CE levels could mitigate power imbalance among UEs.

Observation 6: The network could control the CB-msg3 EDT power imbalance between different UEs through the RSRP threshold configuration.

Proposal 10: The network should configure at least two RSRP thresholds with a narrow gap to enable CB-msg3 EDT with OCC.

Proposal 11: RAN2 support OCC based CB-msg3 via narrow RSRP gaps and separate resources from the non-OCC based transmission.

RAN2 has agreed that only System Information is used to provide cell-specific CB-Msg3 PUSCH resources. However, CB-Msg3 PUSCH resources for a particular group of UEs will benefit both the network and operators. For the network, this could reduce the contention in CB-msg3, enhance the CB-msg3 success rate of particular UEs, and improve the whole system performance. For the operators, this could bring in new business (e.g., subscriptions). Thus, the network should have the flexibility to configure specific CB-msg3 PUSCH resources, separately from the cell-specific CB-Msg3 resources, through dedicated signalling (e.g. RRC RELEASE ).

Proposal 12: RAN2 to discuss the CB-msg3 resource, other than cell-specific CB-Msg3 PUSCH resources,   to reduce the contention, enhance the successful rate, and improve the whole system performance.

  - Contention resolution

RAN2 has agreed that a single Msg4 monitoring starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT (FFS NW/UE processing time is needed or not), and a single RNTI for the Msg4.

For the legacy RACH procedure, the processing time depends on the preamble format and the number of NPRACH repetitions in a predefined table. For CB-msg3, the network work has to blindly detect the CB-Msg3-EDT from different UEs, which requires time. As the CB-Msg3-EDT configuration is per CE level, the processing time will be different due to the CB-Msg3-EDT configurations (e.g., repetition number, RU size, with/without OCC and etc). Thus, the network should have the flexibility to configure processing time for the CB-msg3-EDT.

Proposal 13: When determining the starting point of the Msg4 monitoring window, a preconfigured or network indicated processing time is necessary.

As the RNTI is derived from the transmit resource for the transmission window and the transmission might be configured in different carriers, the RNTI calculation should consider the carrier ID.

As discussed in Section 2.1 and previous agreements, the CB-Msg3-EDT transmission window might belong to different H-SFNs, thus, the RNTI calculation should consider the H-SFN.

Proposal 14: The RNTI calculation should, at least, consider the carrier ID if non-anchor carrier transmission is supported and H-SFN related to the transmission window.

  - CB-Msg4 delivery

In the current remaining MAC open issues in IoT NTN [6], whether L2 ACK is supported is being discussed. For UP solution, there is no doubt that RRC message and/or data can exist. While for solution, if there is no data, RRC message is not needed; otherwise, the RRC message is needed. So L2 ACK should be supported in case there is no RRC message for the CP solution.

Proposal 15: For the CP solution, L2 ACK is supported.

For CB-Msg4, besides Contention Resolution ID, for the CP solution, if there is NAS signalling or DL data, RRCEarlyDataComplete message including NAS signalling or DL data can be transmitted to the UE. For the UP solution, the RRCConnectionRelease message and possible DL data can be transmitted to the UE. If CB-Msg4 from multiple UEs are multiplexed into one MAC PDU, considering there may be DL data besides RRC message for some UEs, CB-Msg4 will be very large, and the multiplexing is very complex. Besides, this multiplexing can also cause some decoding delay. Hence, if RRC message and/or DL data from multiple UEs exist, only one UE’s RRC message and/or DL data can be included in CB-Msg4, similar to MSGB of 2-step RACH procedure.

- Proposal 16: There is only one UE’s RRC message and/or DL data that could be included in CB-Msg4 if there are RRC messages and/or data from multiple UEs.

In the last meeting [7], HARQ feedback can be adopted to acknowledge that CB-Msg4 was agreed, but the details are FFS. In our understanding, since CB-Msg4 can target multiple UEs simultaneously, multiple HARQ feedback resources should be indicated. In the current specification, the HARQ feedback resource for Msg4 of the 4-step RACH can be indicated to the UE by DCI, which targets one UE. Hence, for CB-Msg4, HARQ feedback resource should be indicated to UE by MAC CE, such that each UE can have the corresponding HARQ feedback resource to acknowledge CB-Msg4.

- Proposal 17: HARQ feedback resource could be indicated to the UE by MAC CE.

Besides, timing alignment information can also be included in CB-Msg4 according to the agreement of last meeting. As we know, absolute TA or relative TA may be indicated to the UE. When the network receives the preamble, absolute TA can be indicated to the UE. Other cases can indicate relative TA to the UE. Then, for the CB-EDT procedure, considering the detection accuracy of CB-Msg3 PUSCH, the relative TA should be indicated to the UE, i.e., 6 bits TAC. In addition to TAC, when the UE enters RRC_CONNECTED state, TPC is also needed for UL power adjustment, which could also be indicated to the UE by the MAC CE.

Proposal 18: 6 bits relative TAC could be indicated to the UE by the MAC CE.

Proposal 19: TPC is also indicated to the UE by the MAC CE when the UE enters the RRC_CONNECTED state.

- Based on the analysis above, CB-Msg4 may include Contention resolution ID, C-RNTI, HARQ feedback resource, TA and TPC. For this information, in order to reduce header overhead and simplify MAC PDU design, one unified MAC CE can be designed to include them.

- Proposal 20: For CB-Msg4, one unified MAC CE could be introduced to include Contention resolution ID, C-RNTI, HARQ feedback resource, TAC, and TPC.

- For this information, considering RRC message occurrence, fallback and CB-Msg4 inclusion for RRC message/data, the following five cases may occur:

- Case 1: RRC message doesn’t exist

- Case 2: when the UE stays in RRC IDLE state, RRC message exists and can be included in CB-Msg4

- Case 3: when the UE stays in RRC IDLE state, RRC message exists but cannot be included in CB-Msg4, it can be delivered to the UE subsequently.

- Case 4: when the UE enters RRC CONNECTED state, RRC message can be included in CB-Msg4

- Case 5: when the UE enters RRC CONNECTED state and RRC message cannot be included in CB-Msg4, it can be transmitted to the UE subsequently.

For the above five cases, information requirement is listed in Table 1 below:

Table 1 information requirement for above five cases

|  | Contention resolution ID | HARQ feedback resource | C-RNTI | TA and TPC |
| --- | --- | --- | --- | --- |
| Case 1 | Y | Y | N | N |
| Case 2 | Y | Y | N | N |
| Case 3 | Y | Y | Y | N |
| Case 4 | Y | Y | Y | Y |
| Case 5 | Y | Y | Y | Y |

According to the above table, we can see that contention resolution ID and HARQ feedback resource are needed for all scenarios. For case 1 and case 2, C-RNTI, TA and TPC are not needed. For case 3, TA and TPC are not needed. Hence, considering the different information requirements for the different scenarios, the following two options may be considered:

- Option 1: the unified MAC CE is fixed size. If some information is not needed, it may be set to 0. When the UE decodes the corresponding field and it is set to 0, which means that the information is not needed.

- Option 2: the unified MAC CE is variable size. If some information is not needed, the corresponding fields can be absent.

For option 1, which is simple, but has more overhead due to the unnecessary information. For option 2, since MAC CE is variable size, which means that some unnecessary fields may be absent. Then more overhead can be reduced. Hence, we prefer option 2.

- Proposal 21: The unified MAC CE has a variable size considering the different information requirements for the different scenarios.

- When MAC CEs from multiple UEs are multiplexed into one MAC PDU, if the unified MAC CE is used, subheader design for MAC CE has the following two schemes:

- Scheme 1: one subheader for all MAC CEs, wherein the number of MAC CEs can be indicated in the subheader.

- Scheme 2: each subheader for each MAC CE as a legacy way

- Taking multiplexing from 10 UEs MAC CE as an example, the corresponding overhead for two schemes is listed in Table 2 below.

- Table 2 subheader overhead for MAC CE

|  | - Subheader overhead for MAC CEs of 10 UEs |
| --- | --- |
| - Scheme 1 | - One octet |
| - Scheme 2 | - 10 octets |

- From the above table, we can see that 9 octets can be reduced. Hence, from the perspective of reducing signaling overhead, we prefer scheme 1.

- If scheme 1 is adopted, for MAC PDU format design, the following two options for the orders of subheader, MAC CE, MAC SDU and padding may be considered:

- Option 1: BI subheader, MAC CE subheader, MAC SDU subheader, multiple MAC CEs, MAC SDU and padding. An example is shown in figure 5a.

- Option 2: BI subheader, MAC CE subheader, multiple MAC CEs, MAC SDU subheader, MAC SDU and padding. An example is shown in figure 5b.

Fig. 5a MAC PDU format for CB-Msg4

Fig.5b MAC PDU format for CB-Msg4

- Proposal 22: One MAC subheader with the number of MAC CEs for all multiplexed MAC CEs should be used to reduce the subheader overhead.

- Proposal 23: For MAC PDU format of CB-Msg4, the following two options for the orders of subheader, MAC CE and MAC SDU may be considered:

- Option 1: BI subheader, MAC CE subheader, MAC SDU subheader, multiple MAC CEs, MAC SDU, and padding.

- Option 2: BI subheader, MAC CE subheader, multiple MAC CEs, MAC SDU subheader, MAC SDU, and padding.

  - Failure handling and fallback

RAN2’s last meeting agreed to have backoff indication in CB-Msg4. Backoff indication in legacy RACH procedure is a 4 bits field, and the backoff values are defined as follows in ms. For CB-EDT, considering UE shall first choose DSA window and then choose CB-PUSCH transmission occasions within the DSA window, the unit of backoff shall be DSA window periodicity and potentially we reduce the size of backoff field to less than 4bits.

Table 7.2-1: Backoff Parameter values.

| Index | Backoff Parameter value (ms) |
| --- | --- |
| 0 | 0 |
| 1 | 10 |
| 2 | 20 |
| 3 | 30 |
| 4 | 40 |
| 5 | 60 |
| 6 | 80 |
| 7 | 120 |
| 8 | 160 |
| 9 | 240 |
| 10 | 320 |
| 11 | 480 |
| 12 | 960 |
| 13 | Reserved |
| 14 | Reserved |
| 15 | Reserved |

Table 7.2-2: Backoff Parameter values for NB-IoT.

| Index | Backoff Parameter value (ms) |
| --- | --- |
| 0 | 0 |
| 1 | 256 |
| 2 | 512 |
| 3 | 1024 |
| 4 | 2048 |
| 5 | 4096 |
| 6 | 8192 |
| 7 | 16384 |
| 8 | 32768 |
| 9 | 65536 |
| 10 | 131072 |
| 11 | 262144 |
| 12 | 524288 |
| 13 | Reserved |
| 14 | Reserved |
| 15 | Reserved |

- Proposal 24: RAN2 considers defining backoff value as number of DSA window periodicity and the parameter size can be less than 4bits.

Last meeting during at meeting email discussion, it was discussed but not concluded whether we can introduce a fallback MAC CE. Fallback MAC CE allows NW to instruct failed users to fallback RACH-based EDT. We think this is a useful way to offload random access from CB-PUSCH back to RACH -based EDT when NW detects high load on CB-PUSCH temporarily. For long-term load balance, it could be achieved by system configuration.

- Proposal 25: A fallback MAC CE could be included in CB-Msg4, and a UE will fall back to 4-step RACH procedure, when it has received a fallback MAC CE and failed to receive Msg4 response.

At the last meeting, as follows, we could not conclude on UE behavior upon CB-Msg3 procedure fails i.e., when the maximum times of attempts has reaches without receiving a valid response:

Proposal 13: UE can initiate the legacy 4-step RA when the CB-Msg3 procedure fails. FFS on the condition for the UE to initiate the fallback to legacy 4-step RA. - Ericsson thinks this should be formulated in a way that failure of the procedure is indicated to upper layers. MTK agrees. ZTE wonders which UL Continue in the next meeting

For the legacy RACH procedure, UE MAC will indicate a random-access problem to RRC, and based on the timer in RRC, it may or may not trigger random access again. So, some companies mentioned that UE MAC shall do the same. From modeling point of view, we could follow the legacy RACH procedure to indicate this to RRC layer, and then RRC layer will determine following up action. In the RRC layer, if the procedure can continue i.e. the corresponding timer is still running, legacy RACH-based EDT rather than CB-EDT should be triggered, because it would be getting better chance to success if the procedure switches to more conservative way.

- if the Contention Resolution is considered not successful the MAC entity shall: - flush the HARQ buffer used for transmission of the MAC PDU in the Msg3 buffer; - if the notification of power ramping suspension has not been received from lower layers: - increment PREAMBLE_TRANSMISSION_COUNTER by 1; - if the UE is an NB-IoT UE, a BL UE or a UE in enhanced coverage: - if PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax-CE + 1: - indicate a Random Access problem to upper layers; - if NB-IoT: - consider the Random Access procedure unsuccessfully completed. - else: - if PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1: - indicate a Random Access problem to upper layers.

- Proposal 26: UE MAC can indicate CB-EDT failure to RRC, and if the EDT is allowed to continue, RRC shall trigger RACH-based EDT rather than CB-EDT.

- When the max re-attempt number for the current CE level has been reached, the congestion in the current CE Level must be heavy. If UEs in the same situation move to the next CE Level, it will cause heavy congestion in the next CE Level. Moving to the next CE Level could not solve the congestion problem and introduce other system delay.

- Proposal 27: It is a failure when the max re-attempt number for the current CE level has been reached.

- Conclusion

This contribution provides our observation and proposals for UL capacity enhancement as listed below:

Observation 1: The candidate RU numbers for EDT are 3, 4, 5, 6, 8, and 10, with each RU number covering various TBSs in the range of 328 to 1000.

Observation 2: The duration of one replica depends on the required number of RUs, the SCS, and the number of subcarriers.

Observation 3: For NB-IoT, the time duration for one CB-msg3 resource could be up to 40.96 s (i.e. 3.75 kHz SCS, 10 RUs, 160 slots, 128 repetitions), which is longer than the duration of one hyper-frame.

Observation 4: For NB-IoT, the time duration for one CB-msg3 resource might neither divides one hyper-frame duration nor is divisible by one hyper-frame duration.

Observation 5: For legacy PUR and NPRACH, the frequency domain resource is a set of contiguously allocated subcarriers.

Observation 6: The network could control the CB-msg3 EDT power imbalance between different UEs through the RSRP threshold configuration.

Proposal 1: For the time domain CB-msg3 resource configuration parameters, both the hyper-frame number (H-SFN) and frame number (SFN) are needed.

Proposal 2: For the DSA window starting time:

Option 1: DSA window starts with the CB-msg3 resource together.

Option 2: DSA window stars with an offset to the CB-msg3 resource.

Option 3: DSA window stars at a configured H-SFN and SFN.

Proposal 3: The DSA window size may be:

Option 1: Implicitly indicated by the DSA replica number times the PUSCH resource periodicity.

Option 2: Explicitly indicated by the network.

Proposal 4: The DSA window periodicity may be:

Option 1: Implicitly indicated by the DSA window size.

Option 2: Explicitly indicated by the network.

Proposal 5: Eliminate the DSA window periodicity parameter by aligning it with the DSA window size (i.e., setting periodicity equal to window size).

Proposal 6: RAN2 further discusses the following frequency-domain configuration options:

Option 1: A single contiguous subcarrier set, i.e. one subcarrier starting point with one range

Option 2: Multiple contiguous subcarrier sets, i.e. several subcarriers starting points with corresponding ranges

Proposal 7: RAN2 should configure non-anchor carrier resources for CB-msg3 EDT transmission in SIB22-NB.

Proposal 8: When multiple carriers are provided for CB-msg3-EDT resource for the same enhanced coverage level from the SIBx, it is up to the UE implementation to select any of the carriers.

Proposal 9: The next DSA window is the first window instance containing all the selected replica occasions after CB-msg3 is triggered.

Proposal 10: The network should configure at least two RSRP thresholds with a narrow gap to enable CB-msg3 EDT with OCC.

Proposal 11: RAN2 support OCC based CB-msg3 via narrow RSRP gaps and separate resources from the non-OCC based transmission.

Proposal 12: RAN2 to discuss the CB-msg3 resource, other than cell-specific CB-Msg3 PUSCH resources, to reduce the contention, enhance the successful rate, and improve the whole system performance.

Proposal 13: When determining the starting point of the Msg4 monitoring window, a preconfigured or network indicated processing time is necessary.

Proposal 14: The RNTI calculation should, at least, consider the carrier ID if non-anchor carrier transmission is supported and H-SFN related to the transmission window.

Proposal 15: For CP solution, L2 ACK is supported.

- Proposal 16: There is only one UE’s RRC message and/or DL data that could be included in CB-Msg4 if there are RRC messages and/or data from multiple UEs.

- Proposal 17: HARQ feedback resource could be indicated to the UE by MAC CE.

Proposal 18: 6 bits relative TAC could be indicated to the UE by MAC CE.

Proposal 19: TPC is also indicated to the UE by MAC CE when the UE enters RRC_CONNECTED state.

- Proposal 20: For CB-Msg4, one unified MAC CE could be introduced to include Contention resolution ID, C-RNTI, HARQ feedback resource, TAC, and TPC.

- Proposal 21: The unified MAC CE has a variable size considering the different information requirements for the different scenarios.

- Proposal 22: One MAC subheader with the number of MAC CEs for all multiplexed MAC CEs should be used to reduce the subheader overhead.

- Proposal 23: For MAC PDU format of CB-Msg4, the following two options for the orders of subheader, MAC CE and MAC SDU may be considered:

- Option 1: BI subheader, MAC CE subheader, MAC SDU subheader, multiple MAC CEs, MAC SDU, and padding.

- Option 2: BI subheader, MAC CE subheader, multiple MAC CEs, MAC SDU subheader, MAC SDU, and padding.

- Proposal 24: RAN2 considers defining backoff value as number of DSA window periodicity and the parameter size can be less than 4bits.

- Proposal 25: A fallback MAC CE could be included in CB-Msg4, and a UE will fall back to 4-step RACH procedure, when it has received a fallback MAC CE and failed to receive Msg4 response.

- Proposal 26: UE MAC can indicate CB-EDT failure to RRC, and if the EDT is allowed to continue, RRC shall trigger RACH-based EDT rather than CB-EDT.

- Proposal 27: It is a failure when the max re-attempt number for the current CE level has been reached.

- References

[1] R1-2410895

[2] TS 36.213

[3] TS 36.331

[4] TS 36.211

[5] RAN2#129 chairman notes

[6] Remaining MAC open issues in IoT NTN, MTK

[7] RAN2#129bis chairman notes