3GPP TSG-RAN WG2 Meeting #130	R2-2503896

Saint Julian’s, Malta, 19 – 23 May 2025

Agenda item:	8.11.3

Source:	Nokia

Title:	Other Aspects of SBFD

WID/SID:	NR_duplex_evo-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In RAN#102 meeting, a Work Item (WI) on Evolution of NR duplex operation: Subband full duplex (SBFD) was approved [1]. The work item description has been updated in RAN#104 meeting and the objectives are mentioned in [RP-241614].

In this contribution we discuss RAN2-aspects relevant to SBFD which are not covered in the other SBFD agenda item on Random Access considering emergency call handling and CSI-RS measurements as well as also issues brought by rapporteur in the email discussion [Post129][217][SBFD] List of open issues of RRC impact (Huawei)

# 2	SBFD signalling aspects

So based on the decision in RAN2 and RAN1 we should add signaling of SBFD symbols within TDD-UL-DL pattern configuration in SIB1 – and this will naturally mean that also the corresponding information elements are included in the “common” signaling part of dedicated signaling to allow RRC_CONNECTED mode UEs to be signalled SBFD parameters without requirement of reading SIB1 before utilizing RACH.

Proposal  1: RAN2 needs to add signaling of SBFD parameters (up to RAN1) within SIB1 which implicitly also means they should be added to ServingCellConfigCommon

# 3	RLM handling

There was discussion in the RAN2 list of open issues email discussion regarding whether network needs/can configure the valid symbol type for CSI-RS based RLM/BFD/CBD measurement.

In the last RAN1 meeting, an LS was sent to RAN4 (R1-2501560), where in Issue 2 RAN1 answered that configuration 1 and configuration 2 are not applicable to CSI-RS configuration for RLM/BFD. The configuration of a valid symbol type is applicable only to L1-RSRP and SINR measurement reporting. Therefore, we believe that this issue should be discussed in RAN1 and RAN4.

Proposal 2: It is up to RAN1 and RAN4 to discuss the configuration of a valid symbol type for RLM/BFD measurements

# 4	Mobility aspects of SBFD

Mobility measurements, e.g. RSRP and RSSQ, can be based on SBFD or CSI-RS signals. CSI-RS are impacted by SBFD operation as noted in the following agreements made by RAN1:

RAN1#117: Working Assumption For frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs, support one contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL usable PRBs. - No impact on CSI-RS sequence generation - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured) - RAN1 to further study the impact on CSI processing timeline in SBFD symbols to process the CSI-RS across the two DL subbands Agreement For a contiguous CSI-RS resource which overlaps with SBFD subband boundaries, only CSI-RS frequency resources within DL usable PRBs are valid for SBFD-aware. - No impact on CSI-RS sequence generation - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured)

In the email discussion “[Post129bis][212][SBFD] Running CR for 38.331 (Huawei)” it was also discussed whether RAN2 should consider to support separate Layer3 measurement report for CSI-RS resources in SBFD symbol. The conclusion from the offline discussion is:

[Summary] There is clear majority to support Option 1. [Proposal for RRC-4] RAN2 confirms that the separate Layer 3 measurement report for CSI-RS resources in SBFD symbol is not supported, no RRC spec impact is expected. (13/15 support. No contributions for May meeting are needed as the opponents only propose to wait for RAN4 conclusion.)

Option 1 is an acceptable way forward considering the limited remaining time of the work item. On the other hand, Option 1 could have some impact on RAN4 RRM measurements as the UE may need to average measurements from different symbol types. Nevertheless, RAN2 could conclude to support Option 1 in this RAN2#130 meeting and communicate the decision to RAN4 to see whether there are concerns regarding the RAN2 agreement.

Proposal 3: RAN2 to agree that separate Layer 3 measurement report for CSI-RS resources in SBFD symbol is not supported and ask for concerns to RAN4 (if any).

# 5 	SBFD with Dual Connectivity

In R2-2503191, RAN2 sent an LS to RAN1 regarding simultaneous configuration of SBFD and DC. While waiting for the RAN1 reply, we think RAN2 can still discuss and (and potentially agree) on which type of DC should be supported (e.g. EN-DC vs NR-DC), if any. From our perspective, considering the limited remaining time of the Rel-19 work item and expected standardization impact in RAN2 and RAN4 test cases, our preference is that EN-DC is prioritized over other DC types as this is the scheme most deployed in the field.

Proposal 4: On the potential support of SBFD with dual connectivity (DC), RAN2 prioritizes EN-DC over other DC types.

# 6	Emergency call handling

Emergency calls are critical, thus it is imperative to minimize the risk of interruptions or temporarily too poor quality in the voice transmission. In cells utilizing SBFD this can be an issue if not taken into account when allocating resources for emergency calls or mission critical services with a low data rate. This is particularly important for non SBFD aware UEs. Assuming that the cell operates with a mix of DL-only, UL-only and SBFD slots, the DL resource allocation for a UE making an emergency call should take place in DL-only slots, and the UL resource allocation should take place in UL-only slots for legacy UEs but likely also for SBFD aware UEs as the NW may not be aware of the UEs' SBFD capability at the call setup procedure.

Proposal 5: The NW should ensure that when establishing voice calls and especially emergency calls (at least for non SBFD aware UEs), resources are not allocated to SBFD symbols (i.e. resources should be DL-only or UL-only symbols).

# 7	Conclusion

Proposal  1: RAN2 needs to add signaling of SBFD parameters (up to RAN1) within SIB1 which implicitly also means they should be added to ServingCellConfigCommon

Proposal 2: It is up to RAN1 and RAN4 to discuss the configuration of a valid symbol type for RLM/BFD measurements

Proposal 3: RAN2 to agree that separate Layer 3 measurement report for CSI-RS resources in SBFD symbol is not supported and ask for concerns to RAN4 (if any).

Proposal 4: On the potential support of SBFD with dual connectivity (DC), RAN2 prioritizes EN-DC over other DC types.

Proposal 5: The NW should ensure that when establishing voice calls and especially emergency calls (at least for non SBFD aware UEs), resources are not allocated to SBFD symbols (i.e. resources should be DL-only or UL-only symbols).