3GPP TSG-RAN WG2 Meeting #130	R2-2503900

Malta, May 19th – 23th, 2025

Agenda item:	8.4.2

Source:	Huawei, HiSilicon

Title:	Further discussion on the LP-WUS in RRC_IDLE/INACTIVE mode

Document for:	Discussion and Decision

# Introduction

In this contribution, we further discuss the open issues related to LP-WUS in RRC_IDLE/INACTIVE mode considering the progress in RAN1/2/4.

# Discussion

## The remaining issue of UE_ID based subgrouping

In the latest meeting, RAN2 agreed that in the formula of UE_ID based subgrouping, the UE ID shall be determined by 5G-S-TMSI, and also agreed that the LP-WUS can be operated with eDRX simultaneously.

Agreement in RAN2-129b - Use 5G-S-TMSI to determine the UE_ID in the formula of UE_ID based subgrouping for LP-WUS, i.e., UE_ID=5G-S-TMSI mod X. - X is based on 32 subgrouping number. Details can be discussed in the running CR. - LP-WUS is supported with eDRX, FFS on exact impact if any

The current specification defines that up to 13 bits and 15 bits from the UE ID are used for determining the PF/PO and PEI subgrouping with and without eDRX configured respectively, as specified in TS 38.304. Considering RAN1 has agreed that the maximum number of subgroups per PO is 31, and the current agreed formula of UE_ID based subgrouping ensures that the bits in UE ID used for LP-WUS will not overlap with those used for PEI, additional 5 bits are required in UE ID for LP-WUS subgrouping. Consequently, at least 18 bits (if not configured with eDRX) and 20 bits (if configured with eDRX) are used in UE ID to cover PO/PF determination, PEI subgrouping, and LP-WUS subgrouping requirements while maintaining the agreed separation between functional bit usages. Correspondingly, the value of X in the formula of UE_ID based subgrouping for LP-WUS shall be 218 = 262144 if eDRX is not configured and 220 = 1048576 for the eDRX case.

Proposal 1: The UE_ID in the formula of UE_ID based subgrouping for LP-WUS is determined by UE_ID = 5G-S-TMSI mod X, where X is 1048576, if eDRX is applied; otherwise, X is 262144.

## Enabling of LP-WUS

If LP-WUS functionality is enabled for all the UEs supporting LP-WUS in a camped cell (for example, when a UE receives LP-WUS configuration via SIB from the camped cell, it can use LP-WUS functionality), it is simple but may lead to a lot of UEs using LP-WUS, which means the false paging probability increases and causes more transitions from LP-WUR to MR. This will reduce the power saving gain or even result in negative power saving gain considering the maximum number of subgroups per PO for LP-WUS has been decided by RAN1 to be 31. Hence there can be UE/group specific control for enabling LP-WUS.

To reduce false paging probability, the network (CN or RAN) decides whether a LP-WUS capable UE is allowed to use the LP-WUS functionality based on the conditions in the network (e.g., radio resources) and/or the characteristics of the UEs (e.g., paging probability, shorter DRX cycle, etc.). For example, the CN can use NAS signaling to indicate whether a specific UE can use LP-WUS functionality which is similar to Rel-17 CN-assigned subgrouping method for PEI. The UE capability reporting of LP-WUS Paging Subgrouping Support Indication is already captured in SA2 [1] as shown below:

#### 5.4.12a.2 Core Network Assistance for LP-WUSPS To support the LP-WUSPS, LP-WUS Paging Subgrouping Support Indication and the LP-WUSPS Assistance Information is used by the AMF and NG-RAN to help determine whether LP-WUSPS applies to the UE and which paging subgroup is used when paging the UE (see TS 38.300 [27]). In the Registration Request message, the UE includes the LP-WUS Paging Subgrouping Support Indication if the UE supports LP-WUS Paging Subgrouping. If the UE includes LP-WUS Paging Subgrouping Support Indication, the UE may also include the paging probability information to assist the AMF. The AMF supporting LP-WUSPS stores the UE provided LP-WUS Support Indication in the UE context and the AMF determines AMF LP-WUSPS Assistance Information based on this indication. The AMF may further consider the local configuration, the UE's paging probability information if provided, information provided by the RAN (e.g. any of the Information On Recommended Cells And RAN nodes For Paging), and/or previous statistical information for the UE to determines AMF LP-WUSPS Assistance Information. The AMF LP-WUSPS Assistance Information includes the Paging Subgroup ID.

To minimize the impact for CN deployment, the UE by default can use LP-WUS functionality unless explicit indication to disable the functionality is received from the CN. The UE can include the indication of supporting LP-WUS reception (or UE-ID subgrouping) in addition to including the support of CN subgroup ID. Figure 1 shows the procedure to enable LP-WUS functionality implicitly with NAS signalling.

Figure 1: Implicit enabling of LP-WUS functionality with NAS signalling

Figure 2 shows the procedure to disable LP-WUS functionality with NAS signalling. CN indicates explicitly to the UE to disable LP-WUS functionality. CN can send this explicit disable indication during either initial registration or the registration triggered by mobility. If CN disables LP-WUS functionality to the UE, it also indicates to RAN to disable LP-WUS functionality. Note that only the procedure of enabling and disabling is shown in the figures and the details of this procedure can be left to SA2/CT1/RAN3. The advantage with explicit indication to disable is to minimize the impact to CN. If the CN does not implement this feature, LP-WUS can still be implemented as an AS solution.

Figure 2: Explicit disabling of LP-WUS functionality with NAS signalling

Fig 1. LP-WUS disabling control via NAS signaling

Proposal 2: (RRC-12) The CN indicates whether LP-WUS capable UE(s) is/are allowed to use the LP-WUS functionality by NAS signaling: the absence of indication means UE is allowed to use LP-WUS functionality, and presence of indication to disable means UE is not allowed to use LP-WUS functionality.

Proposal 2a: If above proposal is agreed, send LS to SA2/CT1/RAN3 to inform the agreement and to update the signalling between CN and RAN.

## LP-SS configuration

RAN1 agreed to support both OOK-based and OFDM-based receivers for LP-WUS monitoring. Since the OFDM-based LP-WUR has stronger capability for LP-WUS signal reception and processing, it is possible for NW to configure a shorter LP-WUS to wake up only OFDM-based receiver to reduce the resource overhead. In this case, the UE with OFDM-based receiver can benefit from the low-complexity operation on LP-WUS, e.g., to cancel any OOK sequence/pattern detection or implement termination of LP-WUS procedure by early detection. The UE with OOK-based receiver will skip the current LP-WUS. Thus, the NW should be able to support the scenario that only UEs with OFDM-based LP-WUR are paged, e.g., by sending a shorter LP-WUS that cannot be received by OOK-based receiver.

Observation 1: It is possible that only OFDM-based LP-WUR is supported in a cell based on network configuration.

Currently there is consensus that the OFDM-based LP-WUR can also receive LP-SS for DL synchronization and measurement since RAN1 agreed that LP-SS reuses the overlaid OFDM sequence(s) specified for LP-WUS. Compared to SSB, LP-SS provides significant power-saving advantages due to its measurement efficiency. Since one LP-SS cycle contains larger number of occasions for LP-SS reception, a single LP-SS based measurement is sufficient to meet performance requirements. In contrast, SSB-based measurements may require multiple receptions to achieve the same reliability, leading to additional power consumption. Therefore, it should be allowed to provide LP-SS configuration even if only OFDM-based LP-WUR is supported in a cell. How to configure LP-SS configuration can be up to the network implementation.

Proposal 3: It is up to NW implementation to configure LP-SS for OOK-based and/or OFDM-based LP-WUR.

## LP-WUS entry/exit condition

RAN2 has discussed whether the entry condition can be also based on the measurement result of LP-WUR in RAN2#127 and RAN2#127bis, and made the following working assumption and agreements.

Working Assumption in RAN2#127 - Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds. Agreement in RAN2#127bis - If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s). - The LPWUS monitoring exit condition does not include MR measurements.

If the NW configures the LP-WUS entry thresholds for both MR and LP-WUR measurements, it should be up to UE implementation to obtain the measurement results on LP-WUR, i.e., the metrics and requirement of LR measurement should wait for RAN4’s discussion and there is no need to define in RAN2 spec the specific UE behavior on LP-WUR measurement for LP-WUS entry condition.

Proposal 4: If the LP-WUS entry threshold based on LP-WUR is configured, it is up to UE implementation how to obtain the LP-WUR measurement results used for LP-WUS entry condition.

# Conclusion

In this contribution, we discuss procedure and configuration of LP-WUS for IDLE/INACTIVE modes, and have the following proposals:

Proposal 1: The UE_ID in the formula of UE_ID based subgrouping for LP-WUS is determined by UE_ID = 5G-S-TMSI mod X, where X is 1048576, if eDRX is applied; otherwise, X is 262144.

Proposal 2: (RRC-12) The CN indicates whether LP-WUS capable UE(s) is/are allowed to use the LP-WUS functionality by NAS signaling: the absence of indication means UE is allowed to use LP-WUS functionality, and presence of indication to disable means UE is not allowed to use LP-WUS functionality.

Proposal 2a: If above proposal is agreed, send LS to SA2/CT1/RAN3 to inform the agreement and to update the signalling between CN and RAN.

Proposal 3: It is up to NW implementation to configure LP-SS for OOK-based and/or OFDM-based LP-WUR.

Proposal 4: If the LP-WUS entry threshold based on LP-WUR is configured, it is up to UE implementation how to obtain the LP-WUR measurement results used for LP-WUS entry condition.

# References

3GPP TS 23.501, System architecture for the 5G System (5GS); Stage 2 (Release 19).