3GPP TSG-RAN WG2 Meeting #130	R2-2503905

St. Julians, Malta, May 19th – 23rd, 2025

Agenda item:		8.8.2

Source:		Lenovo

Title:		On DL-CE aspects for NR NTN

Document for:		Discussion

- Introduction

The latest RAN2 meetings made the following agreements regarding DL-CE (Downlink Coverage Enhancement) in NTN:

RAN2#129 Agreements - RAN2 assumes it will be possible to have different SSB periodicity among neighbour cells in the same frequency layer - RAN2 assumes that in a NR NTN cell, SSB beam sweeping in different spatial directions is possible as in a NR TN cell: the whole cell is covered by the different SSB beams in half-frame - RAN2 also assumes that, with the current status of RAN1 discussion, if one cell is defined by multiple “satellite beams”, the satellite beams are all simultaneously active or inactive (“beam hopping” applies equally to all the satellite beams of a given cell) - The number of SMTC/gaps a UE needs to consider at any time will not be increased further RAN2#129bis Agreements - From SSB extension point of view, RAN2 assumes there is no need to introduce new barring bits - We wait for further progress in RAN1 on link level enhancements before further discussing the possible impacts on access barring - RAN2 considers to support configuring two different SMTC periodicities (with different offsets) for SMTCs in one frequency layer for idle, inactive and connected mode. We ask RAN4 whether it is feasible to support this in Rel-19 timeframe (also include previous agreement that at any time the UE will not use more SMTCs in parallel than in previous releases). - We support configuring more than 4 SMTCs per frequency (e.g. 6) for idle/inactive UEs. It will be up to UE implementation to select which of the SMTCs to consider (send this RAN2 decision to RAN4 for checking) - Network can provide assistance information (for Rel-19 UEs, not necessarily supporting DL CE) on the association between SMTC and location to help UE to perform SMTC selection for idle/inactive mode. FFS on the details of location information, e.g. serving cell SSB index, reference location, etc. In any case it is up to UE implementation on how to utilize the assistance information for SMTC selection in idle/inactive mode.

In this contribution we discuss some remaining issues and potential solutions for DL-CE in NTN.

- Discussion

## Assistance information for SMTC reselection

As per RAN2#129bis discussion, it has been agreed that gNB provides assistance information to help idle/inactive mode UE in SMTC selection, and the remaining detail is how to represent the association between SMTC and location.

The first option is to provide a reference location of neighbour cell so that UE can select SMTC based on its distance to the reference location. In this option a UE in idle/inactive mode needs to maintain valid UE location information and perform distance calculation. Although in some cases (e.g., UE is configured with distance-based neighbour cell measurement triggering) the UE already needs to maintain valid UE location information that can be reused, this option still requires provisioning of neighbour cell reference location in system information as well as UE calculation, especially that UE may need to calculate multiple distances timely. With such complexity and power consumption introduced there seems to be no obvious benefit comparing with autonomous SMTC selection.

Observation 1: To implement location/distance-based SMTC selection, UE needs to obtain at least one neighbour cell reference location and timely perform at least one distance calculation to determine which SMTC is to be selected.

The second option is to provide serving cell SSB index so that UE can select SMTC based on selected SSB. In this option the UE location and distance calculation is no longer necessary and thus less complexity and power consumption. Although it has been mentioned that the benefit may not be obvious when there are only 2 SMTCs or the information is useless when there is only one SSB in the serving cell, the UE still has the choice of autonomous selection as in RAN2#129bis it has been agreed that “it is up to UE implementation on how to utilize the assistance information for SMTC selection in idle/inactive mode.”

Proposal 1: The assistance information to help idle/inactive mode UE in SMTC selection from gNB is serving cell SSB index. It is up to UE implementation whether to use the assistance information for SSB-based SMTC selection.

## Msg4 PDSCH repetition support

For DL-CE link level enhancements, RAN1 have confirmed support of Msg4 PDSCH repetition which may have further RAN2 impact. For instance, the following agreements were made in the latest RAN1#120bis:

Agreement For enabling/disabling Msg4 PDSCH repetition, RAN1 to down-select among the following options: - Option 1: UE specific PDSCH with Msg4 repetition activation indicated via PDCCH- DCI Format 1_0 - FFS: indication details. - FFS: whether/how network is informed by the UE that certain conditions are met to trigger Msg4 PDSCH repetition (e.g. RSRP detected at UE is less than x dB) - Option 2: The enabling/disabling of Msg4 PDSCH repetition is implicitly indicated by the enabling/disabling of SIB1 PDSCH repetition. - Option 3: The enabling/disabling is indicated by SIB1 configuration - FFS: whether/how network is informed by the UE that certain conditions are met to trigger Msg4 PDSCH repetition (e.g. RSRP detected at UE is less than x dB) FFS: Whether UE reports its capability

And for Option 2 referring to SIB1 PDSCH repetition:

Agreement For enabling/disabling SIB1 PDSCH repetition, RAN1 to consider the following options: - Option 1: Using reserved bit(s) in PBCH payload. - Option 2: Using scheduling PDCCH. - Option 3: The enabling/disabling of SIB1 PDSCH repetition is implicitly indicating by the enabling/disabling of Type-0 CSS PDCCH repetition.

It is observed that RAN1 have not made the final down-selection, and for the same reason RAN2 decided to wait for further RAN1 progress in RAN2#129bis. However, considering that the Rel-19 function freezing is approaching, some common issues can be discussed in RAN2, e.g. UE request/capability indication for Msg4 PDSCH repetition in Msg3 and corresponding threshold.

Proposal 2: If there is a need of UE request/capability indication for Msg4 PDSCH repetition (i.e. from RAN1), UE can include such request/capability indication in Msg3. FFS a RSRP threshold to trigger such indication.

## Cell reselection for Rel-19 NTN UEs

As per RAN2#129bis discussion, RAN2 assumes there is no need to introduce new barring bits from SSB extension point of view, and will wait for further progress in RAN1 on link level enhancements before further discussing the possible impacts on access barring. In our understanding, regardless of introducing cell barring for DL-CE or not, a UE not supporting DL-CE will experience performance degrade in a DL-CE NTN cell. It would be beneficial if such UE has a better chance to camp on a non-DL-CE cell.

Proposal 3: If a neighbor NTN cell is operating in DL-CE, an NTN-capable UE not supporting DL-CE may deprioritize the frequency of that cell for cell reselection.

## Timing of satellite beam activation/deactivation

With the agreement that “beam hopping” applies equally to all the satellite beams of a given cell, the beam activation/deactivation pattern is the same within a cell, which is a mechanism similar to cell DTX/DRX. Due to the large propagation delay and propagation delay difference in NTN, the configuring gNB and the configured UEs may have different understanding about the timing of satellite beam activation/deactivation. For DL satellite beams being activated or deactivated, UE may either miss possible transmission from network or monitor PDCCH too early with unnecessary power consumption.

Proposal 4: If a beam activation/deactivation pattern is configured by gNB, UE shall consider the propagation delay (e.g. half of UE-gNB RTT) when determine the timing of satellite beam activation/deactivation.

- Conclusion

In this contribution we discuss issues and potential solutions in cell barring, cell reselection and timing of satellite beam activation/deactivation for DL-CE in NTN. The following observations are given:

Observation 1: To implement location/distance-based SMTC selection, UE needs to obtain at least one neighbour cell reference location and timely perform at least one distance calculation to determine which SMTC is to be selected.

And we propose:

Proposal 1: The assistance information to help idle/inactive mode UE in SMTC selection from gNB is serving cell SSB index. It is up to UE implementation whether to use the assistance information for SSB-based SMTC selection.

Proposal 2: If there is a need of UE request/capability indication for Msg4 PDSCH repetition (i.e. from RAN1), UE can include such request/capability indication in Msg3. FFS a RSRP threshold to trigger such indication.

Proposal 3: If a neighbor NTN cell is operating in DL-CE, an NTN-capable UE not supporting DL-CE may deprioritize the frequency of that cell for cell reselection.

Proposal 4: If a beam activation/deactivation pattern is configured by gNB, UE shall consider the propagation delay (e.g. half of UE-gNB RTT) when determine the timing of satellite beam activation/deactivation.