3GPP TSG-RAN WG2 Meeting #130	R2-2503908

St. Julians, Malta, May 19th – 23rd, 2025                       Revision of R2-2502355

Agenda item:		8.9.2

Source:		Lenovo

Title:		Mode transition time for S&F operation

Document for:		Discussion

- Introduction

The latest RAN2 meetings made the following agreements regarding Store and Forward (S&F) operations in IoT NTN:

RAN2#129 agreements - RAN2 currently assumes that a S&F mode changes does not necessarily imply a MME change - The already agreed time information on transition from S&F operation mode to normal mode is provided in SIB31 (can work on signalling optimizations in case the new time information and T-service would carry the same information) - Use absolute UTC time to broadcast the (S&F-> normal mode) time information. When this information is broadcast and indicates a time in the future the UE can assume the NW is currently operating in S&F mode (until the indicated S&F-> normal mode transition time). - A change to the (S&F-> normal mode) time indication shall not result in system information change notification. Legacy rules for acquiring SIB31 apply RAN2#129bis agreements - For UEs supporting S&F, the UE AS indicates the information on transition time (if any) from current "S&F operation mode" to "normal mode" (if received) to the upper layers. Whether/how it is used by the upper layers is up to CT1. - We introduce an indication in system information for the normal mode to S&F mode transition, at least for NAS use. FFS on the details (e.g. whether we can link this to other existing information). The information on transition time for the normal mode to S&F mode transition is sent from AS to NAS - We don’t introduce any new release cause related to S&F operation

In this contribution we discuss some further issues and enhancements for the mode transitions in S&F operation.

- Discussion

In the following we discuss the details of mode transition time and potential impacts, separately for “S&F to normal” mode transition time and “normal to S&F” mode transition time.

## “S&F to normal” mode transition time

In RAN2#129 the “S&F to normal” mode transition time has been agreed to be introduced in SIB31 with format of absolute UTC time, and the remaining issue is whether t-Service can be reused to indicate this or a new indication is needed.

Option 1-1: t-Service is reused to indicate “S&F to normal” mode transition time in S&F operation

In Rel-17 and Rel-18 before S&F operation, the t-Service indicates the time information on when a cell provided via NTN is going to stop serving the area it is currently covering. This field applies for both service link switches in NTN quasi-Earth fixed cell and feeder link switches for both NTN quasi-Earth fixed and Earth-moving cell. In case t-Service is used for multiple purposes, a first issue is how to recognize that the t-Service indicates “S&F to normal” mode transition time rather than cell stop serving time. Based on current progress and discussions, the following alternatives are possible:

- The S&F operation indication is present in SIB1 of serving cell;

- UE receives an indication that the cell supports S&F operation (e.g. from NAS signalling and NAS-AS interaction);

- UE receives an explicit indication indicating the purpose of t-Service.

Proposal 1: If t-Service is reused to indicate “S&F to normal” mode transition time in S&F operation, RAN2 to discuss how to identify its purpose between “S&F to normal” mode transition time and cell stop serving time.

For t-Service, another fact that cannot be ignored is that UE is required to trigger neighbour cell measurement for cell reselection before t-Service if indicated, as specified in TS36.304. In case t-Service is used for multiple purposes, the triggering of neighbour cell measurement may be differentiated for different purposes.

If t-Service is used to indicate the “S&F to normal” mode transition time, considering that it is just the stop serving time of a service mode (mode change) instead of the stop serving time of the cell (cell change), it is unnecessary to trigger neighbour cell measurement as the cell is still available. Only in case that some S&F service cannot supported in normal mode (which is rare in our understanding) shall the S&F-capable UEs trigger neighbour cell measurement before t-Service.

Proposal 2: If t-Service is reused to indicate “S&F to normal” mode transition time in S&F operation, at least S&F-incapable UE is not required to trigger neighbour cell measurement for cell reselection before t-Service. FFS for S&F-capable UE.

Option 1-2: t-Service is NOT reused to indicate “S&F to normal” mode transition time in S&F operation, a new indication is introduced (e.g. t-SFtoNORMAL)

If a new indication is to be introduced, the followed work can be easier as there is no need to consider the behaviors regarding t-Service. For example it is unnecessary to trigger neighbour cell measurement before t-SFtoNORMAL. Only in case that some S&F service cannot supported in normal mode (which is rare in our understanding) shall the S&F-capable UEs trigger neighbour cell measurement before t-Service.

Proposal 3: If a new indication (e.g. t-SFtoNORMAL) is introduced to indicate “S&F to normal” mode transition time in S&F operation, at least S&F-incapable UE is not required to trigger neighbour cell measurement for cell reselection before t-SFtoNORMAL. FFS for S&F-capable UE.

## “Normal to S&F” mode transition time

In RAN2#129bis the “normal to S&F” mode transition time has also been agreed while the other details are FFS.

Similar to the “S&F to normal” mode transition time indication, it can be provided in SIB31 either with UTC time format or a duration from “S&F to normal” mode transition time, and there is no need to raise SI update when it changes.

Proposal 4: The time information on transition from normal mode to S&F operation mode is provided in SIB31.

Proposal 5: Use absolute UTC time or relative time duration to “S&F to normal” mode transition time to broadcast the “normal to S&F” mode transition time information. When this information is broadcast and indicates a time in the future the UE can assume the NW is currently operating in normal mode (until the indicated “normal to S&F” mode transition time).

Proposal 6: A change to the “normal to S&F” mode transition time indication shall not result in system information change notification. Legacy rules for acquiring SIB31 apply.

As whether to reuse t-Service for the “S&F to normal” mode transition time indication has not been decided, for “normal to S&F” mode transition time both options of reusing t-Service or introducing a new indication are possible.

Option 2-1: t-Service is reused to indicate “normal to S&F” mode transition time in S&F operation

Similarly, in case t-Service is used for multiple purposes, a first issue is how to recognize that the t-Service indicates “normal to S&F” mode transition time rather than cell stop serving time. The following alternatives are possible:

- The S&F operation indication was once present and is now absent in SIB1 of serving cell;

- UE receives an indication that the cell supports S&F operation (e.g. from NAS signalling and NAS-AS interaction);

- UE receives an explicit indication indicating the purpose of t-Service.

Proposal 7: If t-Service is reused to indicate “normal to S&F” mode transition time in S&F operation, RAN2 to discuss how to identify its purpose between “normal to S&F” mode transition time and cell stop serving time.

If t-Service is used to indicate the “normal to S&F” mode transition time, it is also unnecessary to trigger neighbour cell measurement as the cell is still available for S&F-capable UEs.

Proposal 8: If t-Service is reused to indicate “normal to S&F” mode transition time in S&F operation, S&F-capable UE is not required to trigger neighbour cell measurement for cell reselection before t-Service. S&F-incapable UE is required to trigger neighbour cell measurement for cell reselection before t-Service.

Option 1-2: t-Service is NOT reused to indicate “normal to S&F” mode transition time in S&F operation, a new indication is introduced (e.g. t-NORMALtoSF)

If a new indication is to be introduced, the followed work can be easier as there is no need to consider the behaviors regarding t-Service.

Proposal 9: If a new indication (e.g. t-NORMALtoSF) is introduced to indicate “normal to S&F” mode transition time in S&F operation, S&F-capable UE is not required to trigger neighbour cell measurement for cell reselection before t-NORMALtoSF, and S&F-incapable UE is required to trigger neighbour cell measurement for cell reselection before t-NORMALtoSF.

- Conclusion

In this contribution we discuss further issues and enhancements for the mode transitions in S&F operation.. The following proposal are provided:

Proposal 1: If t-Service is reused to indicate “S&F to normal” mode transition time in S&F operation, RAN2 to discuss how to identify its purpose between “S&F to normal” mode transition time and cell stop serving time.

Proposal 2: If t-Service is reused to indicate “S&F to normal” mode transition time in S&F operation, at least S&F-incapable UE is not required to trigger neighbour cell measurement for cell reselection before t-Service. FFS for S&F-capable UE.

Proposal 3: If a new indication (e.g. t-SFtoNORMAL) is introduced to indicate “S&F to normal” mode transition time in S&F operation, at least S&F-incapable UE is not required to trigger neighbour cell measurement for cell reselection before t-SFtoNORMAL. FFS for S&F-capable UE.

Proposal 4: The time information on transition from normal mode to S&F operation mode is provided in SIB31.

Proposal 5: Use absolute UTC time or relative time duration to “S&F to normal” mode transition time to broadcast the “normal to S&F” mode transition time information. When this information is broadcast and indicates a time in the future the UE can assume the NW is currently operating in normal mode (until the indicated “normal to S&F” mode transition time).

Proposal 6: A change to the “normal to S&F” mode transition time indication shall not result in system information change notification. Legacy rules for acquiring SIB31 apply.

Proposal 7: If t-Service is reused to indicate “normal to S&F” mode transition time in S&F operation, RAN2 to discuss how to identify its purpose between “normal to S&F” mode transition time and cell stop serving time.

Proposal 8: If t-Service is reused to indicate “normal to S&F” mode transition time in S&F operation, S&F-capable UE is not required to trigger neighbour cell measurement for cell reselection before t-Service. S&F-incapable UE is required to trigger neighbour cell measurement for cell reselection before t-Service.

Proposal 9: If a new indication (e.g. t-NORMALtoSF) is introduced to indicate “normal to S&F” mode transition time in S&F operation, S&F-capable UE is not required to trigger neighbour cell measurement for cell reselection before t-NORMALtoSF, and S&F-incapable UE is required to trigger neighbour cell measurement for cell reselection before t-NORMALtoSF.