3GPP TSG-RAN WG2 Meeting #130	R2-2503909

St. Julians, Malta, May 19th – 23rd, 2025                       Revision of R2-2502356

Agenda item:		8.9.3

Source:		Lenovo

Title:		EDT for uplink capacity enhancement in NTN

Document for:		Discussion

- Introduction

The latest RAN2#129bis meeting made plentiful agreements regarding uplink capacity enhancements in Rel-19 IoT NTN, most of which focus on CB-msg3-EDT transmission mechanisms. In this contribution we discuss some other issues for EDT enhancements in IoT NTN not discussed so far, including fallback, collision with GNSS position fix or ephemeris acquiring, and efficient delivery of Msg4.

- Discussion

Considering the need of supporting massive capacity in terms of number and types of UE, there is a need to identify methods to reduce the necessary uplink and downlink signaling. From this perspective there are two directions that are to be studied: Msg3 transmission without Msg1/2 and reduced overhead reduction for Msg4 delivery.

## Msg3 transmission without Msg1/2 for EDT

In legacy EDT, UE fallbacks to RRC connection establishment if EDT operation is not successful. If Msg3 transmission without Msg1(preamble)/Msg2(RAR) is supported and not successful, the followed operation including possible backoff or fallback needs to be specified, with more options than the legacy fallback mechanism, including:

- Option 1: UE suspends the EDT operation without Msg1(preamble)/Msg2(RAR) and waits for a next attempt e.g., when UE receives indication of contention from eNB or UE has a timer for contention resolution (Msg4 reception) expired. UE may backoff for a time duration to use the next available resource.

- Option 2: UE fallbacks to normal EDT (with random access) e.g., when UE receives indication of contention or fallback from eNB or UE has a timer for contention resolution (Msg4 reception) expired.

- Option 3: As in legacy EDT, UE directly fallbacks to connection establishment to transit to CONNECTED state e.g., when UE receives indication of contention or fallback from eNB or UE has a timer for contention resolution (Msg4 reception) expired.

Proposal 1: RAN2 to discuss the fallback schemes for Msg3 transmission failure in CB EDT.

Another fact for NTN is that UE needs to pre-compensate the timing advance between UE and the network reference point for UL synchronization before initiating random access. For an EDT operation in such deployment, it is also required for the UE to calculate the pre-compensated timing advance, while the procedures to acquire ephemeris or UE position may interrupt the initiation of EDT.

For a CB-EDT initiation that collisions with GNSS position fix or ephemeris acquiring, wither the initiation of an EDT operation without Msg1(preamble)/Msg2(RAR) can delayed until UE completes GNSS position fix or ephemeris (SIB31) acquiring, or GNSS position fix or ephemeris is suspended (no UL sync loss after validity duration) when an EDT operation without Msg1(preamble)/Msg2(RAR) is triggered.

Proposal 2: RAN2 to discuss how to handle the CB-EDT initiation that occurs during GNSS position fix or ephemeris (SIB31) acquiring.

## Efficient delivery of Msg4 for EDT

In legacy EDT the delivery of Msg4 (e.g., RRCEarlyDataComplete for CP-EDT) is unicast via dedicated RRC signalling, and its included fields (e.g., dedicatedInfoNAS, extendedWaitTime, idleModeMobilityControlInfo, redirectedCarrierInfo) are specified as optional. From network perspective eNB can reduce overhead simply by omitting some unnecessary fields or restricting the optional fields in Msg4.

Moreover, these fields of Msg4 are not all UE-specific e.g., extendedWaitTime, idleModeMobilityControlInfo, and redirectedCarrierInfo could be common for multiple or even all UEs in the cell, and dedicatedInfoNAS could also be common when it is empty or zero (i.e., no DL data for EDT from core network). Therefore it is not necessary to always unicast all these fields with duplicated signalling overhead. Based on the above analysis, at least the following two directions can be considered for efficient Msg4 delivery:

- Case A: At least part of Msg4 is common for multiple UEs initiated EDT in the cell. In this case the common part of Msg4 may be delivered in a non-unicast manner.

- Case B: No DL data or other configuration is to be delivered to UE (i.e., empty Msg4). In this case an alternative indication other than Msg4 via RRC can be considered e.g., the Layer 1 or MAC CE acknowledgement used for PUR.

Therefore the following directions can be further discussed:

Proposal 3: RAN2 to discuss the following enhancements for Msg4 delivery in EDT:

- Msg4 delivery in non-unicast ways (e.g., multicast);

- Msg4 delivery using lower layer indication (e.g., Layer 1 indication or MAC CE as in PUR).

- Conclusion

In this contribution we discuss the potential issues and solutions for EDT enhancements in IoT NTN. The following proposals are given:

Proposal 1: RAN2 to discuss the fallback schemes for Msg3 transmission failure in CB EDT.

Proposal 2: RAN2 to discuss how to handle the CB-EDT initiation that occurs during GNSS position fix or ephemeris (SIB31) acquiring.

Proposal 3: RAN2 to discuss the following enhancements for Msg4 delivery in EDT:

- Msg4 delivery in non-unicast ways (e.g., multicast);

- Msg4 delivery using lower layer indication (e.g., Layer 1 indication or MAC CE as in PUR).