3GPP TSG-RAN WG2 Meeting #130	R2-2503912

St. Julian, Malta, 19th – 23rd May 2025

Agenda item:	8.7.4.2

Source:		Lenovo

Title:			Enhanced delay status reporting for XR

Document for:	Discussion and Decision

- Introduction

In this contribution we discuss delay status reporting enhancements for XR traffic.

- Discussion

In RAN2#129bis the following agreements were reached on DSR enhancements:

- To avoid the case that there is no delay-reporting data for DSR, at least one configured reporting threshold should be no lower than the DSR triggering threshold. This means we do not have to address “empty DSR” issue, but it is FFS whether we need to capture this restriction in specifications - During DSR data volume calculation the remaining time of retransmitted is not considered, i.e. it is always put either in the smallest configured or smallest reported threshold. FFS which one. - Clarify RAN2#128 agreement as “the UE may also support including non-delay-reporting data ahead of delay-reporting data for buffer size calculation of Rel-19 DSR, based on the capability indication” (the exact terminology to be discussed as part of CR review) - We will try to find a way to describe how the UE determines non-delay-reporting data ahead of delay-reporting data for delay-reporting data volume calculation. The aim is to have consistent UE behaviour to avoid fairness issues, but also consider different UE implementations.

RAN2 has agreed in RAN2#129bis that the data volume of retransmitted data is always reported in  either the smallest configured or smallest reported DSR reporting range, i.e. dsr-ReportingThreshold. Furthermore, RAN2 has agreed that there needs to be at least some delay-reporting data associated with one configured reporting range (dsr-ReportingThreshold) if a R19 DSR has been triggered. If the one configured reporting threshold is not the smallest configured threshold, UE needs to report information for two ranges . Therefore, in our view retransmission PDCP PDUs/SDUs to be retransmitted should be reported in smallest reported range/threshold for simplicity.

Proposal 1: RAN2 to agree to report PDCP Data PDUs/SDUs to be retransmitted in the delay-reporting data volume in smallest reported range/threshold.

Since PDCP control PDUs don’t have an associated PDCP discard timer, also for simplicity, we agree to report them in the smallest reported threshold.

Proposal 2: RAN2 to agree to report PDCP control PDUs in the smallest reported threshold.

- Conclusion

In this contribution, we discuss delay status reporting enhancements. We have the following proposals:

Proposal 1: RAN2 to agree to report PDCP Data PDUs/SDUs to be retransmitted in the delay-reporting data volume in smallest reported range/threshold.

Proposal 2: RAN2 to agree to report PDCP control PDUs in the smallest reported threshold.

- References

### TS 38.321 v18.0.0