3GPP TSG-RAN WG2 Meeting#130	R2-2503914

Malta, MT, May 19–23, 2025

Agenda item:	 8.10.2

Source:		Lenovo

Title:		Discussion on MRO enhancements for LTM

Document for:		Discussion and Decision

1	Introduction

RAN2#129bis meeting agreed:

Agreements - We will not add support for including the unnecessary LTM configuration, e.g. LTM candidate cell(s) unused, in SON reports, for both failure and near failure cases. - Do not specify support for reporting difference between the TA value received from the LTM cell switch command and TA value measured by the UE - Do not specify support for reporting of elapsed time between UE obtains the target cell TA and UE receives the cell switch command, or an indication to indicate whether the UE obtains TA before UE receives LTM cell switch command - For LTM-related RA reports, the UE sets the cellId to the CGI if available, otherwise sets it based on ltm-CandidatePCI and ssb-Frequency in corresponding LTM-Candidate.

In this paper, we further discuss MRO enhancements for LTM.

2	Discussion

In a LTM procedure, depending on the availability of a valid TA value, the UE performs either a RACH-less LTM or RACH-based LTM cell switch:

- the UE performs RACH-less LTM cell switch upon receiving the LTM cell switch command:

  - if the TA value is provided in the cell switch command, the UE applies the TA value as instructed by the network. For this case, before LTM cell switch command, the UE performs RACH to an LTM candidate cell triggered by PDCCH order for early TA acquisition/UL synchronization; when RACH-less LTM cell switch fails, the UE performs RACH based LTM failure recovery or RRC-re-establishment.

  - if UE-based TA measurement is configured, but no TA value is provided in the cell switch command, the UE applies the TA value by itself if available; when RACH-less LTM cell switch fails, the UE performs RACH based LTM failure recovery or RRC-re-establishment.

- if no valid TA value is available, the UE performs RACH-based LTM cell switch upon receiving the LTM cell switch command:

  - the UE performs CFRA to the target cell, if contention-free Random Access Resources are provided in LTM Cell Switch Command MAC CE;

  - or, the UE performs CFRA to the target cell, if no contention-free Random Access Resource is provided in LTM Cell Switch Command MAC CE, but contention-free Random Access Resources are provided in the RRC Reconfiguration message for the LTM procedure;

  - or, the UE performs CBRA to the target cell, if no contention-free Random Access Resource is provided.

Currently, RA report is used to log RACH information related with RA procedure, and at most eight RA reports can be included in a RACH report list. In an LTM procedure, the UE may perform both RACH for early TA acquisition and RACH based LTM failure recovery/RRC-re-establishment. Besides RACH information for RACH based access (e.g. LTM failure recovery or RRC-re-establishment), it is beneficial to include RACH information for early TA acquisition in RA report, in order to optimize RACH related parameters for early TA acquisition/UL synchronization.

Proposal 1: RACH information for early TA acquisition can be included in RA report.

When both RACH information for RACH based access (e.g. LTM failure recovery or RRC-re-establishment) and RACH information for early TA acquisition need to be logged in the RA report, RAN2 should discuss how to log them:

- Option 1: One entry can be used to log both RACH information related with early TA acquisition and RACH information related with RACH based access (e.g. LTM failure recovery or RRC re-establishment) in the RA report.

- Option 2: Separate entries can be used to log RACH information related with early TA acquisition and RACH information related with RACH based access (e.g. LTM failure recovery or RRC re-establishment) in the RA report.

One entry for logging is the better solution to reduce spec impacts on legacy RA report structure and RACH information logging behaviour at UE side. Therefore, Option 1 is preferred.

Proposal 2: One entry can be used to log both RACH information related with early TA acquisition and RACH information related with RACH based access (e.g. LTM failure recovery or RRC re-establishment) in RA report.

Since the UE may perform both RACH for early TA acquisition and RACH based LTM failure recovery/RRC re-establishment in an LTM procedure, current RA purpose e.g. ReconfigurationWithSync can’t be reused, since network can’t distinguish RACH information related with early TA acquisition from RACH information related with RACH based access (e.g. LTM failure recovery or RRC re-establishment). Therefore, a new RA purpose to indicate the RA procedure was triggered for early TA acquisition can be introduced in RA report.

Proposal 3: A new RA purpose to indicate the RA procedure was triggered for early TA acquisition can be introduced in RA report.

3	Conclusion

In this paper, MRO enhancements for LTM are discussed. We have the following proposals:

Proposal 1: RACH information for early TA acquisition can be included in RA report.

Proposal 2: One entry can be used to log both RACH information related with early TA acquisition and RACH information related with RACH based access (e.g. LTM failure recovery or RRC re-establishment) in RA report.

Proposal 3: A new RA purpose to indicate the RA procedure was triggered for early TA acquisition can be introduced in RA report.