3GPP TSG-RAN WG2 Meeting#130	R2-2503915

Malta, MT, May 19 – 23, 2025

Agenda item:	 8.10.3

Source:		Lenovo

Title:		Discussion on MRO for intra-NTN mobility

Document for:		Discussion and Decision

1	Introduction

RAN2#129 meeting agreed that:

Existing IEs for CHO are used to report the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition” or “location and measurement based trigger condition”.

RAN2#129bis meeting further agreed that:

Agreements: - RAN2 assumes that the signaling structure agreed for encoding geographical area in case of MBS NTN can be used as a reference for encoding geographical area based logged MDT for NTN. - Support reference location/radius and polygon-based area indication. - Use dedicated signalling to provide area scope configuration to the UE - Introduce a UE capability to support geographic area scope checking. FFS if with or without signalling. - Only geographic area scope is used to indicate applicable logging area to the UE. - Only if the UE can obtain location information in IDLE/Inactive state, the UE performs geographical area checking for logged MDT. FFS if, when the UE is in IDLE/INACTIVE, the UE logs or not if the UE cannot obtain its location.

In this paper, we would further discuss MRO for intra-NTN mobility.

2	Discussion

For time-based CHO in NTN:

RAN3#127 meeting agreed there is no need to identify whether RLF happens before or after T1 for time-based CHO, and existing timers can be reused to identify when RLF happens with 100ms accuracy. Since MRO analysis is performed based on multiple statistical reports rather than only one RLF report, 100ms accuracy is sufficient for MRO purpose, and there is no critical issue to be solved. Last RAN3#127bis meeting achieved common understanding that existing timers can be reused to identify when RLF happens with 100ms accuracy, and no more accuracy is needed. Thus, RAN3 finally agreed no further discussion on time-based CHO, e.g. the UE does not need to report the absolute UTC time when RLF/HOF happens in RLF report for time-based CHO in NTN.

Observation: RAN3 agreed no further discussion on time-based CHO, e.g. the UE does not need to report the absolute UTC time when RLF/HOF happens in RLF report for time-based CHO in NTN.

For location-based CHO in NTN:

For earth fixed case, RAN3 agreed network-based solution, and there is no need to report the measured distance from the UE, since reference location for earth fixed case is fixed, network can know it based on its stored or retrieved UE context. Further, network can calculate the distance information based on UE location info reported in RLF report and the derived reference location.

However, for earth moving case, it is another story, network based solution for earth fixed case can’t work well for earth moving case, the accuracy and complexity issue of the network-based solution is acknowledged by RAN3. That is why RAN3 send an LS in [1] to RAN2 to ask which option to be adopted in the RLF report from UE, in order to optimize the distance threshold for location-based CHO:

- Option 1: to introduce the absolute UTC time when RLF/HOF happens, based on which the network computes the distance.

- Option 2: to introduce measured distance from UE to moving reference location of serving cell, and measured distance from UE to moving reference location of candidate cell when RLF/HOF happens.

In Option 1, based on the reported absolute UTC time of when RLF/HOF happens, network can derive the (moving) reference location, and further based on UE location reported in the RLF report, network can further calculate the distance from UE to serving cell or candidate cell when RLF/HOF occurred. We can find that it is optional to include location in the RLF report, Option 1 can work only for the UEs supporting to log and report its location information. On the other hand, the size of absolute time (measured UTC time) is big, and reporting absolute time (measured UTC time) when RLF/HOF occurred in the RLF report would cause complexity at both UE and network side.

Compared with Option 1, Option 2 is more straightforward, and cause less complexity from implementation point of view. RAN2 can reply to RAN3 that Option 2 is adopted.

Proposal 1: For location-based CHO in NTN, RLF report can be enhanced to include measured distance from UE to moving reference location of serving cell, and measured distance from UE to moving reference location of candidate cell when RLF/HOF occurred.

3	Conclusion

In this paper, MRO enhancements for intra-NTN mobility are discussed. We have the following proposal:

Observation: RAN3 agreed no further discussion on time-based CHO, e.g. the UE does not need to report the absolute UTC time when RLF/HOF happens in RLF report for time-based CHO in NTN.

Proposal 1: For location-based CHO in NTN, RLF report can be enhanced to include measured distance from UE to moving reference location of serving cell, and measured distance from UE to moving reference location of candidate cell when RLF/HOF occurred.

References

- R3-252382, LS on SONMDT for NTN, CMCC