3GPP TSG-RAN WG2 Meeting #130	                                              	R2- 2503916

St.Julians, Malta, May. 19th – 23rd, 2025

Agenda Item:	8.5.2

Source: 	Lenovo

Title: 	Issues on the procedure of on-demand SSB SCell operation

Document for:	Discussion and decision

# Introduction

In RAN2#127bis, some agreements were achieved [1].

Agreements on OD-SSB SCell

- No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

- Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

- NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

And the issue on L3 RRM framework is FFS including two potential options:

- Option1: Based on different measurement configuration when OD-SSB is transmitted

- Option2: Based on OD-SSB pattern ignoring SMTC when OD-SSB is transmitted

RAN2 reached the following agreements in RAN2#129bis:

Agreements on OD-SSB

- For explicit activation/deactivation, the OD-SSB MAC-CE includes fixed sized bitmap to indicate whether OD-SSB is activated in each SCell (i.e., similar to legacy SCell A/D MAC-CE).

- A/D bit: “1” means activation, “0” means deactivation for explicit deactivation case.

- For explicit activation/deactivation, the OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells.

- OD-SSB MAC-CE includes a configuration index for each SCell activating OD-SSB.

- RAN2 aims to define single MAC CE format for both explicit and implicit OD-SSB deactivations. Details are FFS.

- Working assumption: When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB. FFS if we allow multiple OD-SSBs with the different frequencies for a given SCell.

In RAN1#120, some agreements were achieved [3].

Agreement

- For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameter (i.e., od-ssb-config) for on-demand SSB configuration, for both Case #1 and Case #2.

- For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameters to configure each of the followings, for both Case #1 and Case #2.

  - Frequency of the on-demand SSB (i.e., od-ssb-absoluteFrequency)

    - FFS: Whether this parameter can be absent for Case #2

  - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst)

    - FFS: Whether this parameter can be absent for Case #2

  - Periodicity of the on-demand SSB (i.e., od-ssb-Periodicity)

  - Sub-carrier spacing of the on-demand SSB (i.e., od-ssbSubcarrierSpacing)

    - For Case #2, this parameter is absent and sub-carrier spagcing of on-demand SSB is the same as that of always-on SSB.

  - Physical Cell ID of the on-demand SSB (i.e., od-ssb-physCellId)

    - For Case #2, this parameter is absent and physical cell ID of on-demand SSB is the same as that of always-on SSB.

  - Time location of on-demand SSB burst (i.e., od-ssb-sfn-Offset and od-ssb-halfFrameIndex)

  - Downlink transmit power of on-demand SSB (i.e., od-ss-PBCH-BlockPower)

    - For Case #2, this parameter is absent and downlink transmit power of on-demand SSB is the same as that of always-on SSB.

  - Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofTx)

  - FFS: Which of the above parameters are a list

Agreement

Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB,

- Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB.

  - On-demand SSB is not on sync raster

  - AO-SSB and OD-SSB are located in the same BWP

  - FFS: Additional conditions

  - Subject to separate UE capability

  - Note: UE is not required to measure both AO-SSB and OD-SSB

In RAN1#118b, some agreements were achieved [3].

Agreement

- For a cell supporting on-demand SSB SCell operation, deactivation of on-demand SSB transmission is supported. In order to deactivate on-demand SSB transmission from a UE perspective, support at least one of the following options.

  - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication

  - Option 1A: Explicit indication of deactivation for on-demand SSB via RRC for on-demand SSB transmission indication

  - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated

  - Option 3: Configuration/indication of the duration of on-demand SSB transmission window

  - Option 4: On-demand SSB transmission, if any, is deactivated when UE receives SCell deactivation MAC-CE for the activated SCell

  - Option 4A: On-demand SSB transmission, if any, is deactivated when the timer for SCell deactivation is expired

  - Option 5: On-demand SSB transmission, if any, is deactivated when SCell activation is completed

  - Option 6: Explicit indication of deactivation for on-demand SSB via [group-common] DCI

  - FFS: Each option is applicable to which Cases or Scenarios

  - FFS: Details related to each of the above options

In this contribution, we provide analysis and proposals on procedures and signalling method(s) to support on-demand SSB SCell operation for UE in CONNECTED mode configured with CA, for both intra-/inter-band CA, also we discuss the impact on UE in IDLE/INACTIVE mode.

# Discussion

Configuration framework of OD-SSB

Referring to RAN1 progress, it was agreed to provide at least Frequency of the on-demand SSB, SSB positions within an on-demand SSB burst by using signalling similar to ssb-PositionsInBurst and Periodicity of the on-demand SSB via higher layer RRC signalling. Also, in RAN1#118, it was agreed to configure multiple candidate values by RRC for on-demand SSB at least including Periodicity of the on-demand SSB and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. Since the existing RRC Reconfiguration message can configure cell specific parameters of a UE's serving cell, e.g. when configuring a UE with an SCell or with an additional cell group (SCG), we can reuse the RRC Reconfiguration message to configure the on-demand SSB related information as well.

Proposal 1: Reuse existing RRC Reconfiguration message to configure the on-demand SSB related information to UE.

Considering the existing configuration of periodicity for SSB is enumerated which cannot provide multiple values, new parameters have been agreed to be added to provide on-demand SSB configuration including frequency, periodicity, Sub-carrier spacing of the on-demand SSB, etc. for both Case #1 and Case #2. However, how to provide multiple configurations of the on-demand SSB is not clear. On the other hand, the details for what can be indicated by MAC CE for on-demand SSB e.g. other relevant parameters except periodicity of the on-demand SSB, have not been concluded. There could be different signalling structures corresponding to different contents that the MAC CE includes. For example, to configure the OD-SSB in RRC, a new IE using “SEQUENCE” structure for each of the multiple configurations IEs of OD-SSB can be used e.g.

od-ssb-config-r19 ::=      SEQUENCE {

od-ssb-physCellId	PhysCellId                                                OPTIONAL, -- Cond case#1,

od-ssb-halfFrameIndex-List-r19             SEQUENCE (SIZE (1..maxSSB-r19)) OF ENUMERATED {zero, one},

od-ssb-sfn-Offset-List-r19             SEQUENCE (SIZE (1..maxSSB-r19)) OF INTEGER (0..1023),

od-ss-PBCH-BlockPower-List-r19             SEQUENCE (SIZE (1..maxSSB-r19)) OF INTEGER (-60..50)                                                  OPTIONAL  -- Cond case#1,

od-ssb-Periodicity-List-r19             SEQUENCE (SIZE (1..maxSSB-r19)) OF od-ssb-Periodicity-r19       OPTIONAL,

od-ssb-absoluteFrequency-List-r19	        SEQUENCE (SIZE (1..maxSSB-r19)) OF ARFCN-ValueNR       OPTIONAL, -- Cond case#1,

od-ssbSubcarrierSpacing-List-r19                SEQUENCE (SIZE (1..maxSSB-r19)) OF od-ssbSubcarrierSpacing-r19                                                   OPTIONAL -- Cond case#1,

od-ssb-PositionsInBurst-List-r19             SEQUENCE (SIZE (1..maxSSB-r19)) OF od-ssb-PositionsInBurst-r19       OPTIONAL,

od-ssb-nrofTx             SEQUENCE (SIZE (1..maxSSB-r19)) OF INTEGER (0..tbd) 		OPTIONAL,

...

}

Then it can work if a bit (or 2 bits depending on the final RAN1 outcome) will be included in the MAC CE to indicate which parameters are included corresponding to each field in the MAC CE e.g., if the periodicity, SCS and frequency is being included. A corresponding field in the MAC CE will indicate the index value of the current received configuration for each of the included field(s).

Alternatively, a new RRC structure is used to provide multiple configurations of the on-demand SSB e.g.

od-ssb-addModList-r19        SEQUENCE {(SIZE (1..maxSSB-r19)) OF od-ssb-config-r19

od-ssb-config-r19 ::=      SEQUENCE {

ssb-configId                INTEGER (0.. maxConfigIDs),

od-ssb-physCellId	PhysCellId                                                OPTIONAL,

od-ssb-halfFrameIndex             ENUMERATED {zero, one},

od-ssb-sfn-Offset             INTEGER (0..1023),

od-ss-PBCH-BlockPower             INTEGER (-60..50)                            OPTIONAL  -- Cond case#1,

od-ssb-Periodicity-r19             ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 }       OPTIONAL,

od-ssb-absoluteFrequency-r19	ARFCN-ValueNR       OPTIONAL,

od-ssbSubcarrierSpacing-r19                SubcarrierSpacing-r19                                                   OPTIONAL -- Cond case#1,

od-ssb-PositionsInBurst-r19             od-ssb-PositionsInBurst-r19       OPTIONAL,

od-ssb-nrofTx		INTEGER (0..tbd) 		OPTIONAL,

...

}

Then, it can work if an index (of the ssb-configId being activated) is indicated by MAC CE. We can wait for RAN1 progress to determine the detailed RRC signalling for OD-SSB configuration. Comparing the two alternatives,  it is better to provide an AddModList to configure the OD-SSB in the RRC configuration to simplify the design of MAC CE.

Proposal 2: Use an AddModList to configure the OD-SSB parameters in the RRC configuration and include a corresponding index (config Id) in the MAC CE once RAN1 has determined the detailed RRC parameters for OD-SSB configuration.

To support explicit or implicit OD-SSB deactivation for a UE, an indication may be added in the RRC configuration which includes the OD-SSB parameters. For example, when using an AddModList to configure the OD-SSB parameters,

od-ssb-addModList-r19        SEQUENCE {(SIZE (1..maxSSB-r19)) OF SSB-r19

od-ssb-deactivation-mode	ENUMERATED { explicit, implicit }

multiple OD-SSBs with the different frequencies

From the perspective of configuration, it can be allowed that a UE is configured with multiple OD-SSBs with the different frequencies for a given SCell e.g., one OD-SSB with the different frequency is configured for each BWP. On the other hand, it is not necessary to configure multiple OD-SSBs with the different frequencies for a given BWP. The corresponding SMTC may be needed if the different (additional) center frequencies can even lie across different BWPs. while for the BWP which is not activated, the UE may follow the configuration to determine the behaviour of measurement.

Proposal 3: it can be allowed to configure a UE with multiple OD-SSBs with the different frequencies for a given Scelland at most one OD-SSB with the different frequency for each BWP.

Further, considering the measurement and the related configuration, it has been agreed in RAN1 that UE is not required to measure both AO-SSB and OD-SSB. In our understanding, there could be two possibilities. One is that only one MO and one measurment report are configured to measue AO-SSB or OD-SSB. And RRC reconiguration is used to change the measurement and report between AO-SSB and OD-SSB.Another is that multiple MOs and corresponding measurement reports are configured, and one pair of them are associated and reconfiguered by RRC (alternatively, some new principles are made) to activate the measurment of AO-SSB or OD-SSB.  Comparing of the two possibilities, the first one introduces less spec impact.

Proposal 4: A UE is configured with one MO for a sering cell to measure AO-SSB or OD-SSBs.

# Conclusion

In summary, we have following observations and proposals on the operation of on-demand SSB for SCell:

Proposal 1: Reuse existing RRC Reconfiguration message to configure the on-demand SSB related information to UE.

Proposal 2: Use an AddModList to configure the OD-SSB parameters in the RRC configuration and include a corresponding index (config Id) in the MAC CE once RAN1 has determined the detailed RRC parameters for OD-SSB configuration.

Proposal 3: it can be allowed to configure a UE with multiple OD-SSBs with the different frequencies for a given Scelland at most one OD-SSB with the different frequency for each BWP.

Proposal 4: A UE is configured with one MO for a sering cell to measure AO-SSB or OD-SSBs.

# References

- RAN2#127bis chairman note.

- RAN1#120 chairman note.

- RAN1#118b chairman note.

- RAN2#129bis chairman note.