3GPP TSG-RAN WG2 Meeting #129bis	R2- 2503917

St.Julians, Malta, May. 19th – 23rd, 2025

Agenda item:	8.12.2

Source:	Lenovo

Title:	Discussion on PL offset

WID/SID:	NR_MIMO_Ph5-Core - Release 19

Document for:	Discussion and Decision

1	Introduction

For asymmetric DL sTRP/UL mTRP deployment scenarios, the updated subject (addition of two TA) was outlined in the WID of NR MIMO Phase 5” [1].

- Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources.

To support UL transmission with 2 TAs for asymmetric DL sTRP/UL mTRP deployment scenarios, some agreements on PL offset were achieved including

RAN1#117

Agreement

For the association between PL offset and joint/UL TCI state, support the following

- Alt1b: One PL offset value is configured in a joint or UL TCI state by RRC, where different PL offset values can be configured to different joint or UL TCI states. A MAC CE can update the PL offset value(s) for joint or UL TCI state(s).

RAN1#118bis

Agreement

Support 2TA for the asymmetric DL sTRP/UL mTRP deployment scenarios:

- Remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature.

- One downlink reference timing is supported and applied to both TAGs.

  - (FFS) Note: UE autonomous TA adjustment is only applicable to the first TAG

- One single n-TimingAdvanceoffset is configured and applied to both TAGs.

- Any of the TCI states can be associated with any one of the two TAGs.

- The RAR carrying TA adjustment for those 2 TAGs is reused for Rel-19 2TA

- The MAC CE based TA adjustment for 2 TAGs is reused for Rel-19 2TA.

- Introduce the optional UE capability of “Overlapping UL transmission reduction” for Rel-19 2TA

  - If UE does not report this UE capability, UE does not expect two UL transmissions associated with different TAGs are overlapped.

- FFS: UE does not expect that in intra-slot TDM PUSCH type-B repetition transmission, two consecutive repetitions associated with different TAGs are overlapped.

RAN2#129

Agreements on Asymmetric DL sTRP/UL mTRP

One PL offset value is indicated for each TCI state included in the new MAC CE. The new MAC CE contains one serving cell ID and one BWP ID TCI state ID is used to indicate a TCI state in the new MAC CE (i.e., no bitmap for TCI states is needed) The new MAC CE can include flexible number of PL offset values. - Working assumption: UE applies the latest PL offset value received in RRC or MAC CE. Can revisit if new issue is found. - Agreement RAN2 understands that if a joint/UL TCI state is configured with a PL offset, PHR trigger is based on the PL change of the PL-RS associated to the joint/UL TCI, where the PL change takes into account the PL offset. FFS whether/how to capture this.

RAN2#129bis

Agreements on Asymmetric DL sTRP/UL mTRP

From RAN2 point of view, UE applies the latest PL offset value received in RRC or MAC CE.

And some issues are left to furhter discuss e.g. the supporting of PL offset in LTM.

In this document, we discuss the RAN2 impact of RAN1 progress.

2	Discussion

TAT expiry

Since two TAGs have been agreed for the asymmetric DL sTRP/UL mTRP deployment scenarios, it is natural to configure TAT for each of the TAG. Then once the TAT expires esp. the TAT on the UL only TRP expires, the UE behaviour needs to be discussed.

According to the existing specification, the MAC entity shall not perform any uplink transmission except the Random Access Preamble and MSGA transmission on a Serving Cell using TCI state(s) associated with a TAG for which the timeAlignmentTimer is not running.

On the other side,  RAN2 agreed that PL offset value can be updated for any configured TCI states with RRC configured PL offset, i.e., not limited to the activated TCI states.

when a timeAlignmentTimer corresponding to a TAG associated with a TCI state expires, and the TCI state is associated with a PL offset(this is for UL only TRP), maintaining the PL offset is beneficial for quickly recovery of UL time alignment for the UL only TRP.

Proposal 1: UE maintains the PL offset when a timeAlignmentTimer corresponding to a TAG associated with a TCI state expires.

PL offset update

It was agreed that UE applies the latest PL offset value received in RRC or MAC CE. If it is initially configured by RRC and updated by MAC CE, there is no problem in our understanding. However, if both RRC and MAC CE can be used to update PL offset, many signalling will be introduced between CU and DU in the case of CU-DU split. To reduce signalling overhead, PL offset updated by RRC signalling is not allowed.

Proposal 2: It is not allowed to update PL offset by RRC signalling.

Selection of UL TRP

If the preconfigured UL grants are configured on both UL TRPs, for example, CG type 1 on UL only TRP and CG type 2 on UL&DL TRP, and the data can be carried on any CG, the UE shall select an UL TRP for the data transmission. Considering the reason for the introduction of UL only TRP, the UL performance on UL only TRP may be better than that on the UL&DL TRP. The UE may prioritize the UL only TRP for UL transmission if the UL only TRP is not overloaded.

Proposal 3: UL only TRP is selected if both of the UL TRPs can perform a UL transmission.

State transition

To save UE power consumption and resume the RRC connection quickly, the UE may be released to RRC_INACTIVE state. Then, the UE may resume the RRC connection to the same cell.

If the cell supports asymmetric DL sTRP/UL mTRP deployment, it’s better for the UE to store the PL offset(s) when the UE is released to RRC_INACTIVE state in order for quick recovery of better performance. The PL offset(s) is resumed once recovering to RRC_CONNECTION in the same cell.

Proposal 4: UE stores PL offset(s) when UE is released to RRC_INACTIVE state.

Proposal 5: The stored PL offset(s) is resumed if the UE resumes RRC connection to the same cell as the cell the UE being released to.

LTM target cell

Referring to the existing specification, 2TA case is supported by LTM e.g. for the serving cell configured with 2TA, or for the candidate cell configured with two TAG IDs and one TAG ID indicated to the UE to perform a RACH-less LTM cell switch. It is natural to support asymmetric DL sTRP and UL mTRP as 2TA case. Namely, the asymmetric DL sTRP and UL mTRP with PL offset can be configured in LTM candidate cell configuration and the configuration is up to network implementation.

Proposal 6: Support asymmetric DL sTRP and UL mTRP in the case of LTM.

Further for RACH-less LTM, the UE accesses the target cell using either a configured grant or a dynamic grant via DCI. The configured grant is provided in the LTM candidate configuration, and the UE selects the configured grant occasion associated with the beam indicated in the cell switch command.

To support asymmetric DL sTRP and UL mTRP as a target cell in the case of LTM, the TCI state associated with the configured grant and the TCI state associated with the PL offset are indicated in the cell switch command.

Proposal 7: To support asymmetric DL sTRP and UL mTRP as a target cell in the case of LTM, the PL offset value are indicated in the cell switch command.

3	Conclusion

In summary, we have following proposals for PL offset:

Proposal 1: UE maintains the PL offset when a timeAlignmentTimer corresponding to a TAG associated with a TCI state expires.

Proposal 2: It is not allowed to update PL offset by RRC signalling.

Proposal 3: UL only TRP is selected if both of the UL TRPs can perform a UL transmission.

Proposal 4: UE stores PL offset(s) when UE is released to RRC_INACTIVE state.

Proposal 5: The stored PL offset(s) is resumed if the UE resumes RRC connection to the same cell as the cell the UE being released to.

Proposal 6: Support asymmetric DL sTRP and UL mTRP in the case of LTM.

Proposal 7: To support asymmetric DL sTRP and UL mTRP as a target cell in the case of LTM, the PL offset value are indicated in the cell switch command.

4	References

[1] RP-242394	WID revision: NR MIMO Phase 5	Samsung

[2] RAN1#117 chairman notes.

[3] RAN1#118bis chairman notes.

[4] RAN2#129 chairman notes.

[5] RAN2#129bis chairman notes.