3GPP TSG-RAN WG2 Meeting #130	R2-2503926

Saint Julian’s, Malta, 19 – 23 May 2025

Agenda item:	8.8.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Open issues for DL coverage enhancements

WID/SID:	NR_NTN_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In RAN2#130 it was agreed that

Agreements;

1.	From SSB extension point of view, RAN2 assumes there is no need to introduce new barring bits

2.	We wait for further progress in RAN1 on link level enhancements before further discussing the possible impacts on access barring

3.	RAN2 considers to support configuring two different SMTC periodicities (with different offsets) for SMTCs in one frequency layer for idle, inactive and connected mode. We ask RAN4 whether it is feasible to support this in Rel-19 timeframe (also include previous agreement that at any time the UE will not use more SMTCs in parallel than in previous releases).

4.	We support configuring more than 4 SMTCs per frequency (e.g. 6) for idle/inactive UEs. It will be up to UE implementation to select which of the SMTCs to consider (send this RAN2 decision to RAN4 for checking)

5.	Network can provide assistance information (for Rel-19 UEs, not necessarily supporting DL CE) on the association between SMTC and location to help UE to perform SMTC selection for idle/inactive mode. FFS on the details of location information, e.g. serving cell SSB index, reference location, etc. In any case it is up to UE implementation on how to utilize the assistance information for SMTC selection in idle/inactive mode.

Although the main feasibility is up to RAN4, based on the LS in R2-2503068, RAN2 should still prepare final solutions w.r.t. the SSB extensions when RAN4 returns with a reply. Furthermore, RAN2 should be ready to provide additional information to RAN4 in case it is requested. This Tdoc handles the remaining open issues on topic.

# 2	Discussion

As seen in the introduction, it has already been agreed that the network can provide assistance information such as location to assist the UE in associating the related SMTC configuration to the UEs current position. Such solution does come with both complexity and overhead, as the UE needs to maintain information about the position, and be up to date with the latest SMTC assistance information, in order to be fully aware of the optimal SMTC configuration.

In Rel-17 a new SMTC list was introduced, to consider the different propagation offsets between cells from different satellites in the measurement configuration. A UE capability called paralellSMTC-r17 was introduced as follows (TS 38.306):

| parallelSMTC-r17 Indicates whether the UE supports NTN SSB based RRM measurements on target cells belonging to 4 SMTC-s on a single frequency carrier. If a UE does not include this field but includes nonTerrestrialNetwork-r17, the UE supports NTN SSB based RRM measurements on target cells belonging to 2 SMTC-s on a single frequency carrier. | UE | No | FDD only | FR1 only |
| --- | --- | --- | --- | --- |

From TS 38.133, Clause 9.2C.1: SSB based measurements are configured along with one or more measurement timing configuration(s) (SMTC(s)) which provides periodicity, duration and offset information on a window of up to 5 ms where the measurements are to be performed. For intra-frequency connected mode measurements, - when SSB-MTC4List-r17 is not configured, up to two measurement window periodicities may be configured with SSB-MTC and SSB-MTC2 - when SSB-MTC4List-r17 is configured, multiple measurement window offsets may be configured with SSB-MTC and SSB-MTC4List-r17, and the requirements in clause 9.2C apply provided that the total number of measurement window offsets does not exceed the UE capability parallelSMTC-r17

RAN4 may not agree to a solution within the timeframe, when a UE is configured with more SMTCs than it supports, measurement requirements are not applicable according to RAN4 specifications. The reason for this is that RAN4 considered that if the UE had to select a subset of measurements at will, it would not be possible to evaluate which SMTCs the UE had selected to check if the measurements were completed in time. Besides, if the UE decides to split the measurement occasions across multiple SMTC, the total time to measure would be much longer.

Observation 1: RRM requirements are not applicable in case more than 2 SMTCs are configured.

In Rel-17, the capability parallelSMTC-r17 was related to the measurements performed by the UE in different satellites. So the problem was somehow confined to the “inter-satellite” handover. In Rel-19, RAN2 discusses configuring more SMTCs offsets even for cells served by the same satellite. If the same capability from release 17 is reused, even for UEs that support the location assisted SMTC selection, we could end up in a situation where the RAN4 requirements are not applicable for neighbour cell measurements from cells served by the same or different satellites. In the table below we summarize the expected UE behavior in different cases, if the parallelSMTC-r17 is reused:

|  |  |  | UE behavior |
| --- | --- | --- | --- |
| gNB configures location assistance information | gNB configures more than 4 SMTCs | UE supports location assistance information | UE indicates parallelSMTC-r17: UE is expected to measure 4 SMTCs according to the assistance information. According to 38.133 requirements, no requirements apply. UE does not indicate parallelSMTC-r17: UE is expected to measure 2 SMTCs according to the assistance information. According to 38.133 requirements, no requirements apply. |
|  |  | UE does not support location assistance information | UE is expected to measure 2 or 4 SMTCs, depending on whether the UE indicates or not parallelSMTC-r17. Not clear which SMTCs will be used. According to 38.133 requirements, no requirements apply |
|  | gNB configures up to 4 SMTCs | UE supports location assistance information | UE indicates parallelSMTC-r17: UE is expected to measure up to 4 SMTCs UE does not indicate parallelSMTC-r17: UE is expected to measure up to 2 SMTCs according to the assistance information. According to 38.133 requirements, no requirements apply. |
|  |  | UE does not support location assistance information | UE indicates parallelSMTC-r17: UE is expected to measure up to 4 SMTCs UE does not indicate parallelSMTC-r17: UE is expected to measure up to 2 SMTCs. Not clear which SMTCs will be used. According to 38.133 requirements, no requirements apply. |
| gNB does not configure location assistance information | gNB configures more than 4 SMTCs | All UEs | UE is expected to measure 2 or 4 SMTCs, depending on whether the UE indicates or not parallelSMTC-r17. Not clear which SMTCs will be used. According to 38.133 requirements, no requirements apply |
|  | gNB configures up to 4 SMTCs | All UEs | UE indicates parallelSMTC-r17: UE is expected to measure up to 4 SMTCs UE does not indicate parallelSMTC-r17: UE is expected to measure up to 2 SMTCs. Not clear which SMTCs will be used. According to 38.133 requirements, no requirements apply if the UE is configured with more than 2 SMTCs. |

## 2.1	Considerations on the current RAN2 solution

RAN2 has discussed whether the additional SMTCs should be configured in a new list i.e. SMTC5, or how to include the list. In legacy, the UE defaults to SMTC1 in case it does not support SMTC4 i.e. it does not support more than 2 SMTCs. In order to follow this concept, we believe that RAN2 should consider adding SMTC5 as an additional extension to SMTC4, where the UE can default to SMTC4 in case it does not support SMTC5.

Proposal 1: SMTC4 is the fallback in case SMTC5 is not supported by the UE similarly as SMTC1 is fallback in case the UE does not support SMTC4.

From the above table, we see that the network in general will be very uncertain which SSB will be reported, as RAN2 agreed to give the UE complete freedom to utilise the assistance as it finds suitable. There is a risk that RAN4 will reply that the solution provided is feasible only in case there is a well defined procedure for which SMTC the UE should be using. As of such RAN2 can discuss in this meeting whether or not we will drop the location based solution, or apply i.e. a prioritisation of SSBs in case RAN4 replies that such information is needed to design the solution.

Proposal 2: RAN2 to discuss how to restrics the UE freedom when legacy SSB and SMTC is configured in order to prepare for RAN4 reply that such prioritisation is needed for the solution to be acceptable.

## 2.2	Coordinated network deployment

As of the considerations mentioned in the previous sections, we think that alternative working solution should be discussed, enabling energy savings at the network with through an alternate solution, not including RAN4 to the same extend w.r.t. requirements for measurement reqiurements. The alternative to the location based approach, enables the network to configure the availability of the neighbouring cells SSBs (from the same satellite) in a coordinated manner and provide simpler, assistance information may be more optimal. For example, the network could configure time offsets in which the SSBs of the different neighbor cells are available as a multiple of 20 ms. This situation is illustrated in Figure 3.

Figure 1 – Example of one SMTC being configured to measure SSBs with regular time offsets

In such case, we assume that the SSBs of DL-CE cells will be available for measurements at least with a default periodicity supported in NR: 20ms or 160 ms. Based on the deployment in Figure 1 and that all the supported SSB periodicities longer than 20 ms in NR are multiple of 20 ms. It is possible that with a SMTC periodicity shorter than the SSB periodicity the UE can measure SSBs from the neighbour cells that are not active at the same time if the time offset between the SSBs is regular. However, the UE would need to be provided with assistance information, such as the SSBs periodicity, and the time offsets between the SSBs measured in the same SMTC. It is a matter of network configuration and of the network providing the necessary information for the UE to measure the neighbor cells.

Observation 2: The relevant information for configuring the UE to measure neighbor cells that are active in a scattered manner in the same or different satellites are: periodicity and time offset.

Proposal 3: gNB may provide the neighbor cells SSB periodicities and time offsets in relation to the serving cell SSB in addition or as a replacement to the location.

In case both solutions are adopted, the UE may determine the current configuration based on the assistance information provided, and act accordingly.

Proposal 4: In case both location based and the coordinated procedure is adopted, RAN2 to discuss whether the assistance information type indicated is used to determine the network configuration.

## 2.3	On location assistance information

One remaining FFS is on the details of location information, e.g. serving cell SSB index, reference location, etc. Since at least the serving cell SSB index can only be seen if the serving cell uses multiple beams to cover the cell area. Thus using the SSB index will restrict the deployment and we believe that the most obvious solution is to have the location defined as a center and a radius, and the reference to be the index of the SMTC configuration.

Proposal 5: Location assistance information should consist of a center and a radius of a number of circles, each referring to one of the SMTCs in the new list.

## 2.3 Number of SMTCs

Another open point is whether there should be a total of 6, or another number of SMTC configurations. When the UE is in idle mode, it is already measuring the serving cell using SMTC1. Thus 6 additional SMTCs in one carrier frequency should be enough to cover worst case network configuration.

Proposal 6: Only 6 additional SMTCs are needed.

# 3	Conclusion

This document has made the following observations and proposals:

Observation 1: RRM requirements are not applicable in case more than 2 SMTCs are configured.

Proposal 1: SMTC4 is the fallback in case SMTC5 is not supported by the UE similarly as SMTC1 is fallback in case the UE does not support SMTC4.

Proposal 2: RAN2 to discuss how to restrics the UE freedom when legacy SSB and SMTC is configured in order to prepare for RAN4 reply that such prioritisation is needed for the solution to be acceptable.

Observation 2: The relevant information for configuring the UE to measure neighbor cells that are active in a scattered manner in the same or different satellites are: periodicity and time offset.

Proposal 3: gNB may provide the neighbor cells SSB periodicities and time offsets in relation to the serving cell SSB in addition or as a replacement to the location.

Proposal 4: In case both location based and the coordinated procedure is adopted, RAN2 to discuss whether the assistance information type indicated is used to determine the network configuration.

Proposal 5: Location assistance information should consist of a center and a radius of a number of circles, each referring to one of the SMTCs in the new list.

Proposal 6: Only 6 additional SMTCs are needed.