3GPP TSG RAN WG2 #130		  R2-2503928

Malta, MT, 19th - 23th May 2025

Title:	Discussion on NW-side Data Collection for BM

Agenda item:	8.1.3

Source:	Google

Document for:	Discussion and Decision

# 1 Introduction

In RAN2#129bis meeting, there were some agreements and FFSs for NW-side data collection.

Agreements on Idle/inactive and HO

Introduce 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration during/before HO.  Source gNB decides whether the data should be kept.  The indication is provided in RRCReconfiguration (i.e. not in RRC Reconfiguration from target cell).   FFS signaling details.

Upon going to RRC_IDLE, RLF, or RRC_INACTIVE, UE discards any logged data

Agreements on availability indication - Availability indication can be triggered due to: - Full buffer being reached (if configured) - Buffer threshold being reached (if configured). - Low power (if configured) - The UE send a UAI that indicates: - Data is available - Reason for trigger (full buffer, threshold) - Low power indication - The encoding of the data is available/UAI and the cause value is FFS NOTE: it is up to UE Implementation how buffer threshold reached and low power is determined

Next meeting proponents should work together and bring complete proposals to show specification impact and consider future use cases.

Agreements on data collection configuration

- The measurement configuration of AI/ML data collection can configure measurements for multiple sets of resources and use cases (e.g. BM, Mobility, etc)

Agreements on content of collected data:

- For temporal domain, the network is made aware whether there is a gap between two consecutive samples.   FFS amount of gap and whether this is implicit or explicit

- New SRB can be configured for NW-side data collection  (with lower priority)

In the post-meeting email discussion [POST129bis] [016] [AI PHY], there are some FFSs and open issues. In this paper, we further discuss the possible issues and give related considerations.

# 2 Discussion

## Open Issues provided by Rapporteur

### Open issue RRC-7: NW control on retaining logged data at HO

Issue description: The signaling details of the network control on how data should be retained at handover are FFS, based on the RAN2#129bis agreement:

Introduce 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration during/before HO.  Source gNB decides whether the data should be kept.  The indication is provided in RRCReconfiguration (i.e. not in RRC Reconfiguration from target cell).   FFS signaling details.

To configure the UE to retain the logged data, the 1-bit indication can be sent before or during the HO. Technically speaking, it is simpler to send the 1-bit indication before the HO, e.g., in an RRCReconfigration from the source cell. This will not involve the inter-node interaction. Additionally, if the 1-bit indication can be configured during HO, it might bring additional complexity for the UE to decide whether the 1-bit indication is configured by the target NW or the source one.

Proposal 1: (RRC-7) The 1-bit indication on whether to release or retain un-retrieved data is sent in RRCReconfiguration before the HO.

### Open issue RRC-8: Reporting assistance information related to logged measurements

Issue description: It is not yet clear what the otherConfig should contain to enable the UE to report assistance information via UAI, related to logging of radio measurements. For instance, should the low power, buffer full, and buffer threshold reached indications be all configured with a single bit, or should the configurations be separated?

This issue is related to the RAN2#129bis agreement:

Agreements on availability indication - Availability indication can be triggered due to: - Full buffer being reached (if configured) - Buffer threshold being reached (if configured). - Low power (if configured) - The UE send a UAI that indicates: - Data is available - Reason for trigger (full buffer, threshold) - Low power indication - The encoding of the data is available/UAI and the cause value is FFS NOTE: it is up to UE Implementation how buffer threshold reached and low power is determined

In RAN2#129bis, three types of triggering conditions for data availability indication were agreed. However, even if the triggering condition is met, the UE may not have any, or may have insufficient logged data for the node which configured the triggering condition.

As mentioned in the following section 2.2.2, the UE memory might be shared across various data collection types, including NW-side data collection (initiated by gNB, OAM or LMF) and UE-side data collection. It is possible that the UE is configured by the LMF to perform data collection for positioning and stores positioning related logging data in its buffer. Subsequently, the gNB configures the UE to perform event-triggered data collection for beam management. It is assumed that the gNB configures the triggering condition for availability indication to be buffer full or buffer threshold. In this situation, it is possible that a significant volume of logged positioning data causes the UE’s buffer to reach the buffer full or buffer threshold. Consequently, the UE might trigger the data availability indication, even if there is no logged data for BM. For example, the logging event for BM data collection is not triggered at all.

Similar issue occurs for low power case. Firstly, the UE is configured with event-based data collection for BM. It is possible that no event has been triggered when the UE detects low power state. According to current agreement, the UE will trigger a data availability indication if configured with low power as the triggering condition for data availability indication.

Observation 1: When the triggering condition (buffer full, buffer threshold or low power) is met, there may be no required logged data in the UE buffer.

Based on the preceding analysis, one potential solution is to decouple the specific triggering conditions from being the triggers for data availability indication. Instead, these conditions could be configured to trigger the UAI. Correspondingly, the UE can include a specific triggering cause in the UAI. To achieve this, the NW could configure a low power state. If the UE detects a low power state, it would transmit a UAI message including a “low power state” indication. Similarly, the NW could configure buffer full or buffer threshold as separate reporting criteria to report the buffer status at UE side. When the UE detects these buffer conditions reached, it includes “buffer full” or “buffer threshold triggered” information in the UAI message. This approach allows the UAI to convey precise reasons for the UE’s current state.

Proposal 2a: (RRC-8) The NW configures:

- low power as the triggering of low power state reporting in UAI

- Buffer full or buffer threshold as the triggering of buffer state reporting in UAI

If configured with the reporting of low power state, buffer full or buffer threshold, the UE indicates the related reasons to the NW upon detecting the conditions triggered. Additionally, if there is logged data collected for BM, the UE can further include a data availability indication in the UAI.

Proposal 2b: (RRC-8) The UE includes the following information in the UAI:

  - Low power state

  - buffer state set to full buffer or buffer threshold triggered

  - availability indication if there is logged data for the gNB

### Open issue RRC-9: Further procedures for UE assistance information related to logging

Issue description: It has not yet been discussed whether further procedures for UE reporting assistance information related to logging is need. Such further procedures may be, e.g. prohibit timers, indication that battery state is not low any longer, indication that the memory is not full any longer, etc.

Following previous discussion, the UE can be configured to report low power state, full buffer, or reached buffer threshold to the NW during data collection. The NW is then expected to deactivate or release the relevant data collection configuration for the UE.

Consequently, it is also reasonable for the UE to inform the NW when it exits such previously reported limiting states (e.g., when the battery state is not low any longer, or the buffer is no longer full). This notification would allow the NW to potentially configure new data collection configuration or resume the previous one.

To prevent unnecessary reporting when these states change, the NW should be able to configure the specific triggers for these "exit" reports. Thus, if configured to do so, the UE would report the resolution of the low power state or the full buffer condition.

Proposal 3: (RRC-9) If configured, the UE reports the exit of low power state, or full buffer state.

### Open issue RRC-10: Time related content of logged data

Issue description: It has not yet been clarified what information needs to be included with the logged data, to indicate a time gap between the logged data entries (i.e. a gap that is longer than the logging data periodicity).

This issue refers to the RAN2#129bis agreement:

- For temporal domain, the network is made aware whether there is a gap between two consecutive samples.   FFS amount of gap and whether this is implicit or explicit

It is known that both periodical and event-triggered logging configuration are introduced for data collection. For periodical logging, it was argued that the NW can determine the time information of the logged data. However, RAN2 also agreed that the UE can retain and report the logged data for the source node after handover to the other node. After successful handover, the source node releases UE context. It is questionable whether the source node can determine the time information of the logged data if receiving it from the other node.

Observation 2:  The source NW cannot determine the time information of periodical logged data when receiving logged data from the other node after handover.

Accounting for the handover case, the unified solution is to introduce the explicit time information of logged data. Additionally, for event-triggered logging scheme, the NW cannot know when the event is triggered. Literally, the time information should be provided with the logged data.

Proposal 4: (RRC-10) Time information is included for both periodical and event-based logged data.

For the logged data, there may be multiple samples. It is preferred to reuse the time related content in the logged MDT report. The UE includes an absolute timestamp and relative ones for the remaining samples.

Proposal 5: (RRC-10) The UE includes absolute and relative timestamps for the logged data.

### Open issue RRC-13: Where to include the logging configuration from NW to UE

Issue description: It is FFS whether the logging configuration is included in the CSI framework (whether in CSI-ReportConfig or directly under CSI-MeasConfig) or at L3.

In the existing draft CR, it is known that the parameters for data collection configuration can include the CSI resource configuration and corresponding logging configurations.

#### –	CSI-LoggedMeasurementConfig

The IE CSI-LoggedMeasurementConfig defines a group of one or more CSI resources for which the UE logs the associated L1 radio measurements.

CSI-LoggedMeasurementConfig information element

-- ASN1START

-- TAG-CSI-LOGGEDMEASUREMENTCONFIG-START

CSI-LoggedMeasurementConfig-r19 ::=          SEQUENCE {

csi-LoggedMeasurementConfigId-r19         CSI-LoggedMeasurementConfigId-r19,

csi-LoggedResourceConfig-r19              CSI-ResourceConfigId,

eventTriggeredConfig-r19                  EventTriggeredConfig-r19                    OPTIONAL, --Need R

...

}

As indicated in the above section, it is noticed that there is the CSI resource configuration ID, e.g., CSI-ResourceConfigId, indicating the parameters of the CSI resource configuration in data collection configuration. The CSI resource configuration ID links a set of parameters defined for the CSI resource configuration in the CSI framework. If we include the data collection configuration in the existing CSI framework, this avoids redundant definitions for the CSI resource configurations.

Additionally, for NW-side data collection, the reporting configuration should be provided to the UE. However, for UE-side data collection, the reporting configuration is not needed. From the perspective of simplicity, RAN2 should consider a unified framework for both NW-side and UE-side data collection.

Proposal 6: (RRC-13) A unified framework is for both NW-side and UE-side data collection.

Proposal 7: (RRC-13) The data collection configuration is included directly under the CSI-MeasConfig in the CSI framework.

## Other Open Issues

### FFS on the Structure of Data Collection Configuration

In RAN2#127bis, periodical logging was supported for NW-side data collection. In RAN2#128 meeting, it was agreed that the NW only configures periodic CSI resources for measurements for NW-sided data collection.

- Measurements on aperiodic CSI resources are not reported for NW sided data collection.

Literally, for periodic logging, the UE logs data periodically. There is an issue on the setting of the logging interval for periodic logging scheme.

One approach is to implicitly set the periodic logging interval to the period of the configured CSI resource. However, this approach lacks flexibility. For example, if the NW wants to adjust the logging interval for the previous CSI resource, it would need to modify the existing CSI resource configuration to the new period. This would involve additional signalling overhead as well.

Alternatively, to enable flexible periodic data logging for a configured CSI resource, we propose introducing explicit logging interval configuration. This is similar to the legacy L3 periodic reporting configuration. This would allow the NW to change the logging interval without modifying the pervious CSI resource.

Proposal 8: Define an explicit logging interval for periodic data logging.

If the above proposal is agreed in RAN2, for the logging configuration in the data collection configuration, the NW should provide detailed parameters for both periodical logging and event-triggered logging. Specifically, we could define data logging configurations where each data logging configuration assigned with a unique ID, e.g., CSI-LoggingConfigId, corresponds to a set of logging configuration parameters.

This structure provides a flexible way to correlate CSI resource configuration with logging configurations. For each logging configuration associated with a CSI-LoggedMeaurementConfigId, the NW can directly configure a CSI-LoggingConfigId linked with a CSI-ResourceConfigId, instead of defining detailed logging configuration parameters. This approach allows the flexibility as the legacy L3 measurement framework, reducing complexity and signalling overhead.

Proposal 9: Define a list of data logging configurations including logging parameters for both periodic and event-triggered logging. Each data logging configuration is associated with a unique ID, e.g., CSI-LoggingConfigId.

Proposal 10: For data collection configuration, one CSI-LoggedMeaurementConfigId links one CSI-ResourceConfigId and one CSI-LoggingConfigId.

### FFS on UE Memory Size and Sharing

In RAN2#127 meeting, it was agreed that the AS layer memory is across all use cases.

- UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size.  This is across all use cases

For the minimum memory size, we can follow the definition for the MDT feature. The minimum memory size can be defined as 64KB.

Currently, there are discussions to support data collections for NW-side model training (e.g., initiated by OAM, gNB or LMF) and UE-side model training. Correspondingly, there will be logged training data stored at UE memory for various model trainings. To reduce the complexity on the UE, it is straightforward that the UE memory should be shared for the data collections for all the above cases.

Proposal 11: The minimum memory size is 64 KB.

Proposal 12: The UE memory is shared for data collections for both NW-side model training (e.g., initiated by OAM, gNB or LMF) and UE-side model training.

### FFS on the cause in data collection content

When the UE indicates data availability indication to the NW (i.e., gNB for BM case), the NW can retrieve the logged data collected based on the measurement configuration from the NW. According to the data collection report in the current CR on TS 38.331, the UE can report the logged data for each CSI-LoggedMeasurementConfigId.

CSI-LogMeasReport-r19 ::=            SEQUENCE {

csi-LogMeasInfoList-r19              CSI-LogMeasInfoList-r19,

csi-LogMeasAvailable-r19             ENUMERATED {true}                                       OPTIONAL,

FFS

...

}

CSI-LogMeasInfoList-r19 ::=          SEQUENCE (SIZE (1..maxLogCSI-MeasReport-r19)) OF CSI-LogMeasInfo-r19

CSI-LogMeasInfo-r19 ::=              SEQUENCE {

cellId-r19                              FFS,

refCSI-LoggedMeasurementConfigId-r19    CSI-LoggedMeasurementConfigId-r19,

csi-RS-MeasResultList-r19               SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF CSI-MeasResults-r19    OPTIONAL,

csi-SSB-MeasResultList-r19              SEQUENCE (SIZE (1..maxNrofSSBs)) OF CSI-MeasResults-r19                    OPTIONAL,

FFS

...

}

CSI-MeasResults-r19 ::=              SEQUENCE {

rs-Index-r19                         CHOICE {

csi-RS-Index-r19                     NZP-CSI-RS-ResourceId,

ssb-Index-r19                        SSB-Index

}

l1-RSRP-r19                          RSRP-Range

...

}

As discussed in the above sections, it is known that the UE may support multiple data logging configurations simultaneously. It is possible that for certain CSI-LoggedMeasurementConfigId, the UE cannot log sufficient data due to various reasons. For example, for event-triggered tasks, the triggering conditions for the events might not be satisfied at all, or be satisfied infrequently. In addition, the UE may lack sufficient memory to store all the data required for data logging of one CSI-LoggedMeasurementConfigId. Moreover, there might be abnormal UE state, e.g., low power, overheating, which may cause the UE to stop data logging potentially based on NW control. Furthermore, RAN2 agreed that the UE can retain logged data during handover. Generally, the UE would stop the data logging configured by the source cell after handover. For all the above cases, they may result in no or insufficient logged data for some configured asks at UE side.

Therefore, When the UE reports the logged data to the NW, it is essential to include a cause indication for the related CSI-LoggedMeasurementConfigId where the data logging is incomplete or the logged data is limited. If the cause is set to event not triggered or event triggered infrequently, the NW can optimize the triggering conditions for the events.

Proposal 13: When reporting the logged data content, the UE includes a cause indication (e.g., event not triggered/event triggered infrequently, others) for the CSI-LoggedMeasurementConfigId with no or insufficient logged data.

### FFS on data handling after consecutive handover

When the UE receives the retaining indication from the source cell and completes successful handover to the target cell, the UE can indicate the availability indication to the target cell. There is the situation that the target cell doesn’t retrieve the logged data for the source cell and configures a second handover command to the UE.

Observation 3: The logged data for source cell may not be retrieved by the target cell before a subsequent handover command.

This introduces the issue of how to manage logged data retaining for source cell when the UE receives a subsequent handover command. One straightforward solution is for the UE to discard the logged data for source cell upon receiving any new handover command from the target cell. This is simple for UE implementation.

Alternatively, the decision could depend on the target cell specified in the new handover command. If the UE is handed over back to the source cell from the target one, it could retain the logged data for source cell. Otherwise, the UE would discard the logged data for source cell, similar to the first solution. The latter approach allows for the potential reuse of the logged data retained at the UE side. This conserves UE power and memory by preventing redundant data collection.

Proposal 14: Upon reception of a subsequent handover in the target cell, the UE manages the logged data for the source cell in the following options:

- Opt 1: Discard the logged data for the source cell upon receiving the subsequent HO CMD from the target cell

- Opt 2:

  - If the new target cell is the source one, the UE retains the logged data for source cell

  - Otherwise, the UE discards the logged data for the source cell

# 3 Conclusion

In this contribution, we discussed the potential solutions for the remaining issues on NW-side data collection for BM. The related observations and proposals are summarized below:

Observation 1: When the triggering condition (buffer full, buffer threshold or low power) is met, there may be no required logged data in the UE buffer.

Observation 2:  The source NW cannot determine the time information of periodical logged data when receiving logged data from the other node after handover.

Observation 3: The logged data for source cell may not be retrieved by the target cell before a subsequent handover command.

Proposal 1: (RRC-7) The 1-bit indication on whether to release or retain un-retrieved data is sent in RRCReconfiguration before the HO.

Proposal 2a: (RRC-8) The NW configures:

- low power as the triggering of low power state reporting in UAI

- Buffer full or buffer threshold as the triggering of buffer state reporting in UAI

Proposal 2b: (RRC-8) The UE includes the following information in the UAI:

  - Low power state

  - buffer state set to full buffer or buffer threshold triggered

  - availability indication if there is logged data for the gNB

Proposal 3: (RRC-9) If configured, the UE reports the exit of low power state, or full buffer state.

Proposal 4: (RRC-10) Time information is included for both periodical and event-based logged data.

Proposal 5: (RRC-10) The UE includes absolute and relative timestamps for the logged data.

Proposal 6: (RRC-13) A unified framework is for both NW-side and UE-side data collection.

Proposal 7: (RRC-13) The data collection configuration is included directly under the CSI-MeasConfig in the CSI framework.

Proposal 8: Define an explicit logging interval for periodic data logging.

Proposal 9: Define a list of data logging configurations including logging parameters for both periodic and event-triggered logging. Each data logging configuration is associated with a unique ID, e.g., CSI-LoggingConfigId.

Proposal 10: For data collection configuration, one CSI-LoggedMeaurementConfigId links one CSI-ResourceConfigId and one CSI-LoggingConfigId.

Proposal 11: The minimum memory size is 64 KB.

Proposal 12: The UE memory is shared for data collections for both NW-side model training (e.g., initiated by OAM, gNB or LMF) and UE-side model training.

Proposal 13: When reporting the logged data content, the UE includes a cause indication (e.g., event not triggered/event triggered infrequently, others) for the CSI-LoggedMeasurementConfigId with no or insufficient logged data.

Proposal 14: Upon reception of a subsequent handover in the target cell, the UE manages the logged data for the source cell in the following options:

- Opt 1: Discard the logged data for the source cell upon receiving the subsequent HO CMD from the target cell

- Opt 2:

  - If the new target cell is the source one, the UE retains the logged data for source cell

  - Otherwise, the UE discards the logged data for the source cell