3GPP TSG-RAN WG2 #130	R2-2503931

St. Julian's, Malta, 19th – 23rd May 2025

Source:		NEC

Agenda item:		8.6.4	Conditional intra-CU LTM

Title:	Remaining Open Issues for CLTM

Document for:		Discussion and decision

- Introduction

During the last RAN2 meeting, the following agreements were made for the support of CLTM [1]:

Agreements on C-LTM

- Upon C-LTM execution, UE performs the following for received TA values and TA timers for other candidate cells than the target candidate cell: Maintain the TA value and Keep the early TA timer running.

- C-LTM TAT for target cell is not stopped upon C-LTM cell switch execution to the target cell if it is running, and starts the PTAG using the remaining time from the C-LTM timer.

- RAN2 confirm CLTM with Rel-18 MIMO 2TA is supported, introducing TAG ID field in LTM candidate TAC MAC CE.

- For CLTM with Rel-18 MIMO 2TA, two TA timer length (i.e. ltm-TimeAlignmentTimer) can be configured via RRC.

- Same as Rel-18: When the C-LTM execution is triggered, if the TA of the target cell acquired by early RACH is valid, UE applies the TA acquired by early RACH, otherwise if UE has successfully measured the TA of the target cell, UE applies the measured TA.

In this paper, we provide our consideration on the remaining issues of conditional intra-CU LTM.

- Discussion

2.1 valid CG resource determination

One issue to be discussed is that CSI-RS could be applicable beam of an event, however the CG resource is only associated with SSBs, hence how to CG resource selection that the beam fulfilling the execution condition is a CSI-RS.

There can be two options for to handle the case that beam meets the execution condition is CSI-RS:

Option 1. The UE determine whether a CG resource is valid based on the SSB associated with the CSI-RS which fulfills the execution condition

In our understanding, this option is aligned with Rel-18 handling for RACH-less LTM. In Rel-18, the TCI state indicated in the LTM cell switch command MAC CE can be associated with SSB or CSI-RS.

Agreement of RAN1 #114: In R18 LTM, on the QCL source of the TCI state before/during the cell switch command, - SSB or TRS can be configured in a TCI state for the candidate cell(s) before/during cell switch command - Whether the TRS can be used for the candidate cell(s) before/during cell switch command is up to UE capability

And if the TCI state is associated with a CSI-RS, the UE determines whether a CG resource is valid based on the SSB associated with the CSI-RS of the TCI state, selects the SSB and indicates to the lower layer the SSB. More specifically, the UE needs to check the corresponding NZP-CSI-RS-Resource of the CSI-RS. If the NZP-CSI-RS-Resource indicates a further TCI state of which the reference signal is an SSB, the UE shall determine whether the CG resource is valid or not based on the SSB. Otherwise, if the reference signal provided by the further TCI state is another CSI-RS, the UE needs to further look in the TCI state indicated by the another CSI-RS, and so on, until the UE finds an SSB.

TS 38.213 v18.4.0 clause 21.1:

## 21.1 Configured-grant PUSCH transmission in RACH-less LTM cell switch <skipped> If the UE is provided cg-LTM-Configuration, the UE performs configured grant Type 1 PUSCH transmission on the valid PUSCH occasions associated with the SS/PBCH block indexes same as the SS/PBCH block indexes provided by or associated with QCL RS of the CandidateTCI-State or CandidateTCI-UL-State indicated by the LTM Cell Switch Command MAC CE.

Therefore, similar as in Rel-18, if the beam fulfilling the execution condition is CSI-RS, the UE determines whether a CG resource is valid based on the SSB associated with the CSI-RS which fulfills the execution condition. In other words, if the SSB associated with the CG resource is the same as the SSB associated with the CSI-RS fulfilling the execution condition, the UE shall consider the CG resource as valid, and indicate the SSB to the lower layer.

Option 2. Extend CG resource for LTM to be associated with CSI-RS

For this option, if the CSI-RS associated with the CG resource is the same as the beam fulfilling the execution condition, the UE consider the CG resource as valid, and indicate the CSI-RS to the lower layer. To support this, the CG resource configuration needs to be extended, and some additional efforts are needed to specify how to support the association between CG resource and CSI-RS, including impact to RAN2 signalling and RAN1 specification.

Option 3. RACH-less CLTM is not triggered

For this option, the UE perform RA-based CLTM instead. We think is option is too passive and not aligned with Rel-18 behavior.

Since RAN1 haven’t started the discussion to support CG resource associated with a list of CSI-RS for RACH-less LTM, and this meeting is the last RAN1 meeting for Rel-19 LTM, it is better to go with Option 1. Additionally, even RAN1 may support CG resource associated with CSI-RS, it is possible that the UE is only configured with CG resource associated with SSB, and in this case we prefer to go with Option 1.

Proposal 1. In case CG resource is only associated with SSB, if the beam meeting the execution condition is CSI-RS, the UE determines whether a CG resource is valid based on the SSB associated with the CSI-RS.

2.2 Expiry of timeAlignmentTimer for PTAG

During last meeting, it was agreed that C-LTM TAT for target cell is not stopped upon C-LTM cell switch execution to the target cell if it is running, and UE starts the time alignment timer associated with PTAG using the remaining time from the C-LTM timer.

In Rel-18 LTM cell switch, the time alignment timer PTAG is started upon initiation of RACH-less LTM cell switch procedure, and the network configuration ensures that the time alignment timer of PTAG will not expire during the execution of RACH-less LTM cell switch procedure, for example the maximum value of the time alignment timer of PTAG is configured longer than T304. However, in case of CLTM, since the time alignment timer for PTAG starts using the remaining time or CLTM timer, and therefore may be expired during the LTM cell switch procedure. And RAN2 should discuss the corresponding UE behavior in that case.

There can be three candidate options:

- Opt1) the UE terminates the CLTM cell switch procedure

- Opt2) the UE fallbacks from RACH-less LTM to RACH-based C-LTM

- Opt3) the UE does nothing

For option 1, the LTM cell switch procedure is terminated, and the RRC layer can thereafter trigger a LTM recovery procedure, or RRC re-establishment procedure. For this option, the UE does not need to wait until the expiry of T304 expiry, and the interruption caused by the expiry of TAT can be reduced.

For option 2, the LTM cell switch procedure keeps on-going, since uplink transmission cannot be performed anymore due to expiry of TAT, the UE fallbacks from RACH-less LTM to RACH-based LTM by initiating a random access procedure directly.

For option 3, the LTM cell switch procedure keeps on-going, since the network may be not aware of execution of the CLTM cell switch procedure and therefore cannot transmits TA command to the UE in time, the UE waits for the T304 timer to expire which results in LTM failure.

Proposal 2. In case of timeAlignmentTimer associated with PTAG expiry during RACH-less LTM cell switch, discuss the following options:

- Opt1) the UE terminates the CLTM cell switch procedure

- Opt2) the UE fallbacks from RACH-less LTM to RACH-based C-LTM

- Opt3) the UE does nothing

2.3 Co-existence between CLTM and (e)RedCap and CovEnh

At least, we would like to discussion whether to support co-existence between CLTM and (e)RedCap and CovEnh. We think that it is beneficial to support such co-existence. And since there is no LTM cell switch command transmitted from the network to the UE, such co-existence can be supported easily without additional specification efforts.

Proposal 3. Support co-existence between CLTM and (e)RedCap and CovEnh without additional specification effort.

# 3. Conclusion

In this paper, we discussion on the remaining issues of RACH-less CLTM, and have the following proposals:

Proposal 1. In case CG resource is only associated with SSB, if the beam meeting the execution condition is CSI-RS, the UE determines whether a CG resource is valid based on the SSB associated with the CSI-RS.

Proposal 2. In case of timeAlignmentTimer associated with PTAG expiry during RACH-less LTM cell switch, discuss the following options:

- Opt1) the UE terminates the CLTM cell switch procedure

- Opt2) the UE fallbacks from RACH-less LTM to RACH-based C-LTM

- Opt3) the UE does nothing

Proposal 3. Support co-existence between CLTM and (e)RedCap and CovEnh without additional specification effort.

# Reference

[1] meeting report of RAN2 #129bis.