3GPP TSG-RAN WG2 Meeting #130	R2-2503944

St. Julian’s, Malta, 19 - 23 May, 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on on-demand SSB SCell operation for NES

Agenda Item:	8.5.2

Document for:	Discussion and decision

Introduction

In this contribution, we discuss the remaining issues of on-demand SSB SCell operation, including MAC CE design and L3 measurements.

Discussion

2.1 MAC CE design

On MAC CE design, the following progress has been made in RAN1 and RAN2 respectively:

From RAN1 perspective (related RAN1 agreements provided in the Annex):

- OD-SSB activation by MAC CE, at least these parameters can be configured by RRC and indicated by MAC CE: OD-SSB periodicity, SSB positions in burst, N of OD-SSB bursts to be transmitted

- OD-SSB deactivation can be either explicitly indicated by MAC CE, or implicitly indicated by number N of OD-SSB bursts to be transmitted after activation, number N is configured by RRC.

- OD-SSB activated by RRC is expected to be transmitted after UE receives RRC config (based on time location of on-demand SSB burst i.e., od-ssb-sfn-Offset and od-ssb-halfFrameIndex); OD-SSB activated by MAC CE is expected to be transmitted after a minimal time defined by RAN1, the actual time location still follows RRC configuration.

- RAN1 expects some parameters to be configured by RRC for OD-SSB SCell, including OD-SSB frequency, SSB positions in a burst, OD-SSB periodicity, OD-SSB SCS, PCI, time location of OD-SSB, DL transmit power of OD-SSB, number N for implicit deactivation.

From RAN2 perspective:

- OD-SSB activation/deactivation can be indicated by RRC, including initial status and reconfigured status.

- The new MAC CE is not used for SCell activation/deactivation.

- The new MAC CE can indicate OD-SSB transmission for multiple SCells.

- The new MAC CE contains a fixed size bitmap corresponding to 7/31 SCells, and include a configuration index for each SCell activating OD-SSB. The new MAC CE addresses both explicit and implicit deactivation.

Take 7 SCells as an example, the MAC CE may look like:

The new MAC CE is a variable size MAC CE. For byte alignment, some R bits are reserved. The configuration index (and the corresponding R bits) is only present when the A/D bit of the corresponding SCell is set to 1 (activation).

There is still one open issue about how to use the new MAC CE applies when implicit deactivation is used. The controversial scenario is that: one SCell activates OD-SSB by the new MAC CE and uses implicit deactivation, then the new MAC CE is sent again to activate the OD-SSB of another SCell, and in this case how to handle the corresponding bits in the MAC CE related to the SCell which previously activated OD-SSB. During online discussion, there are several understandings:

- Understanding 1: For SCell using implicit deactivation, A/D bit is set to 0 (deactivation), UE ignores the configuration index field and only follows N configured by RRC and indicated by MAC CE when the OD-SSB of this SCell was activated. No configuration index is included for the SCell.

- Understanding 2: For SCell using implicit deactivation, A/D bit is set to 1 (activation), UE restarts counter N for the SCell when receiving the MAC CE, and N is configured by RRC. No configuration index is included for the SCell.

- Understanding 3: For SCell using implicit deactivation, A/D bit is set to 1 (activation) and configuration index is included to update the N (e.g. NW implementation to choose N to the remaining numbers of OD-SSB bursts to be transmitted, or NW implementation to choose N to previous value so that OD-SSB will be sent N times).

Note that RAN1 has made the following agreements in RAN1#120bis that the OD-SSB configuration can be updated in Scenario 3B:

Agreement For a cell supporting on-demand SSB SCell operation, the following combinations of scenarios and cases are supported for indicating OD-SSB using a MAC-CE. - Scenario #3B and Case #1 - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication. - Scenario #3B and Case #2 - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication.

Since RAN1 has agreed to use MAC CE to update the configuration (including N), we think RAN2 cannot assume N is always reset when MAC CE is received. Therefore, Understanding 2 should not be pursued.

Understanding 1 has the smallest overhead since the configuration index can be omitted when A/D bit is set to 0 (deactivation). Understanding 3 is aligned with RAN1 agreement to update the OD-SSB parameters. Therefore, we think Understanding 1 and Understanding 3 can be combined: if the A/D bit set to 0 (deactivation) UE ignores the configuration index field (an alternative is to restrict the NW configuration, if A/D bit is set to 0, configuration index is not included), if A/D bit is set to 1 (activation) the UE follows the configuration index field.

Proposal 1: RAN2 to down-select from the following alternatives:

- Understanding 1: For SCell using implicit deactivation, A/D bit is set to 0 (deactivation), UE ignores the configuration index and only follows N configured by RRC. No configuration index is included for the SCell.

- Understanding 2: For SCell using implicit deactivation, A/D bit is set to 1 (activation), UE restarts counter N for the SCell when receiving the MAC CE, and N is configured by RRC. No configuration index is included for the SCell.

- Understanding 3: For SCell using implicit deactivation, A/D bit is set to 1 (activation) and configuration index is included to update the N to the remaining numbers of OD-SSB bursts to be transmitted.

- Combined Understanding 1 and 3: For SCell using implicit deactivation, if the A/D bit set to 0 (deactivation) UE ignores the configuration index field, if A/D bit is set to 1 (activation) the UE follows the configuration index field.

Besides, following the aforementioned RAN1 agreement, in Scenario 3B (after SCell is activated), the OD-SSB can only be updated, it cannot be deactivated. To ensure that, either 1) RRC configured value N includes “infinity”, and the new MAC CE can be used to update N to indicate a configuration config pointing to N = “infinity”; or 2) RRC reconfigures the UE to disable the OD-SSB feature when SCell is activated.

Proposal 2: To ensure that OD-SSB is not deactivated after SCell is activated (i.e., Scenario 3B), RAN2 to down-select from the following alternatives:

- Option 1: RRC configured N includes a value set to infinity

- Option 2: It is up to NW implementation to reconfigure the UE to disable the OD-SSB feature for an SCell when the SCell is activated.

Another aspect related to the OD-SSB MAC CE is the (e)LCID. The new MAC CE requires an LCID or eLCID. Since RAN2 agreed to support two formats corresponding to 7 and 31 SCells respectively, two LCIDs or eLCIDs are needed. Currently we only have 12 reserved values for LCID (values 35-46), while there are plenty of reserved values for one-octet eLCID (values 0 to 215) for DL-SCH. It is preferred to use eLCID for the OD-SSB transmission MAC CE.

Proposal 3: Use two eLCIDs for the OD-SSB transmission MAC CE, corresponding to 7 SCells and 31 SCells respectively.

2.2 L3 measurements

In the last meeting, RAN2 concluded on the following working assumption for inter-frequency AO-SSB and OD-SSB:

Agreements on OD-SSB

- Working assumption: When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB. FFS if we allow multiple OD-SSBs with the different frequencies for a given SCell.

On the FFS, we think there is so far no related RAN1/RAN4 agreements to support multiple OD-SSBs with different frequencies for a given SCell. Furthermore, RAN1 has agreed in RAN1#120 that AO-SSB and OD-SSB are located in the same BWP, which means the BWP should be no smaller than 40 PRBs. If multiple OD-SSB with different frequencies are supported, the BWP needs to be even larger, making the implementation too restrictive. Besides, one OD-SSB frequency can already serve the purposes of synchronization, measurements etc, there is no motivation for having multiple OD-SSBs with different frequencies associated to the same SCell.

Proposal 4: RAN2 will not further discuss the case of multiple OD-SSBs with different frequencies for a given SCell, unless this scenario is agreed by RAN1 or RAN4.

On inter-frequency AO-SSB and OD-SSB, in the last meeting RAN2 agreed to have separate servingCellMOs.

On top of servingCellMO configuration, we think RAN2 also need to figure out the following aspects (some issues need to be discussed for both intra-frequency cases and inter-frequency cases):

1) Impacts on serving cell measurements: whether to rely on AO-SSB only or OD-SSB only or both

2) Impacts on neighbour cell measurements: whether to consider OD-SSB, how to differentiate between intra-f and inter-f neighbour cell measurements

3) Impacts on gap: how to determine gap-less measurements and gap-based measurements

4) Impacts on measurement reporting: whether to allow OD-SSB MAC CE based measurement triggering

In the following discussion, we provide our views on these issues.

- 2.2.1 Serving cell measurements

In case 1 (no always-on SSB), the L3 measurements can only rely on OD-SSB.

In case 2 (with always-on SSB), we still think the L3 measurements can also be based on OD-SSB, otherwise there is no use of transmitting OD-SSB. Besides, OD-SSB can have a smaller SSB periodicity compared with AO-SSB, making the measurement delay smaller.

Another issue for case 2 is whether the measurement results of OD-SSB and AO-SSB can be combined.

- In intra-frequency cases, RAN4 agreed the measurement results of OD-SSB and AO-SSB can be combined under some conditions.

---- cited from RAN4#114 WF R4-2502598 Issue 2-3: Combine OD-SSB and AO-SSB in measurement Agreement: - In scenario 4, the measurement requirement is defined based on the effective measurement periodicity. - The requirement is only applied in the FMW which includes both OD-SSB and AO-SSB, and - The effective measurement periodicity is half of OD-SSB periodicity - AO-SSB and OD-SSB are on the same frequency with the same spatial relation ---- cited from RAN4#114bis WF R4-2502598 Issue 1-2-2: Requirement for RAN1 Alt Time-C1 Agreement: - RAN4 to update the agreement of EMP for deactivated SCell measurement and SCell activation to apply to RAN1 Alt Time-C1 scenario. - The EMP is the periodicity of the union of AO-SSB and OD-SSB and equals one of legacy SSB periodicities defined in the specification. - FFS for other procedures.

- In inter-frequency cases, the combination has not been discussed in RAN4, because RAN1 agreed that UE is not required to measure both:

---- cited from RAN1#120 agreements Agreement Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB, - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - On-demand SSB is not on sync raster - AO-SSB and OD-SSB are located in the same BWP - FFS: Additional conditions - Subject to separate UE capability - Note: UE is not required to measure both AO-SSB and OD-SSB

Proposal 5: For serving cell measurements:

- In case 1 (no AO-SSB), the serving cell measurements are based on OD-SSB.

- In case 2 (with AO-SSB), in intra-frequency cases, OD-SSB can be used for serving cell measurements, how AO-SSB is utilized for serving cell measurements and how to combine AO-SSB and OD-SSB is up to RAN4 discussion; in inter-frequency cases, only OD-SSB is used for serving cell measurements.

After concluding on that serving cell measurement is based on OD-SSB, a related issue is whether the serving cell measurement is based on SMTC of OD-SSB or directly based on OD-SSB pattern.

In legacy mechanism, L3 measurement is based on the measConfig, which uses measIds to link measObjects (which provide SMTC for UE to monitor SSB) and reportConfigs, and UE measurement reporting is also based on measIds. For OD-SSB, at least the OD-SSB pattern of the serving cell will be informed to the UE. Therefore, RAN2 needs to determine whether the L3 measurement is still based on SMTC or it is based on the OD-SSB pattern or both.

For both Case 1 (no always-on SSB) and Case 2 (always-on SSB), as long as the OD-SSB pattern is informed to the UE, UE can perform serving cell measurements. There is no need to modify the legacy SMTC or make up a new dedicated SMTC for serving cell measurement purpose. If the specification forces the NW implementation to configure an SMTC for OD-SSB measurements, this configuration is anyway aligned with the OD-SSB pattern, which is already known to the UE, and an unnecessary step is defined at the NW side.

RAN4 agreed to introduce fast L3 measurements for deactivated SCell. In fast mode the measurement periodicity is based on OD-SSB periodicity rather than measCycleSCell. After fast mode, UE falls back to a slow mode (based on measCycleSCell).

---- cited from RAN4#113 WF R4-2420098 Issue 1-2-1: OD-SSB based deactivated SCell measurement – measurement periodicity in fast mode measurement Agreement: For the case of CSSF = 1: - OD-SSB periodicity regardless of measCycleSCell and DRX cycle. - FFS whether a lower bound will be specified. FFS for the other cases of CSSF. Issue 1-2-3: OD-SSB based deactivated SCell measurement – Procedure Agreement: - After the fast time window, the UE shall meet the legacy deactivated SCell measurement requirement as long as OD-SSB is activated/transmitted from UE’s perspective. - The agreement can be revisited based on further RAN1 agreement about OD-SSB deactivation mechanism

The above RAN4 agreements already indicate that in fast measurement window, RAN4 requirements will be based on OD-SSB pattern instead of the configured SMTC and measCycleSCell.

After fast mode, RAN4 requirements are based on measCycleSCell. In terms of UE behaviour, there are still two possibilities: 1) based on SMTC; 2) based on OD-SSB pattern but the measurement periodicity is adjusted based on measCycleSCell.

We prefer the latter, because this enables a single mechanism for OD-SSB L3 measurements at different phases. If SMTC is used for serving cell measurements after the fast window, this SMTC needs to be pre-configured and its pattern will be the same as OD-SSB pattern.

Proposal 6: For both Case 1 (no always-on SSB) and Case 2 (always-on SSB), the measurements of OD-SSB is based on OD-SSB pattern rather than SMTC. For deactivated SCell, after the fast window, OD-SSB is measured based on measCycleSCell in combination with OD-SSB pattern.

In the last meeting, RAN2 achieved the working assumption of introducing a separate servingCellMO for OD-SSB. However, the UE behaivour on how to utilize the servingCellMO is unclear. There could be two possibilities:

Option 1: When OD-SSB is activated, UE uses servingCellMO-OD to measure serving cell and intra-f neighbour cells; when OD-SSB is deactivated, UE uses servingCellMO-AO (i.e., legacy servingCellMO) to measure serving cell and intra-f neighbour cells.

Option 2: Regardless of the activation status of OD-SSB, UE uses both servingCellMO-OD and servingCellMO-AO (i.e., legacy servingCellMO) to measure serving cell and intra-f neighbour cells. FFS how the network determines the serving cell measurement results come from which MO.

One aspect that needs to be addressed in Option 2 is that, the available serving cell measurement result is piggybacked in all the measurement report (regardless of triggered by which measID). In other words, when a measurement report is triggered by a certain measID (not linked to the servingCellMO), the network will have no idea whether the included serving cell results come from OD-SSB or AO-SSB. This should be further considered if Option 2 is adopted.

Proposal 7: On how to utilize the new servingCellMO, RAN2 to down-select from the following:

- Option 1: When OD-SSB is activated, UE uses servingCellMO-OD to measure serving cell and intra-f neighbour cells; when OD-SSB is deactivated, UE uses servingCellMO-AO (i.e., legacy servingCellMO) to measure serving cell and intra-f neighbour cells.

- Option 2: Regardless of the activation status of OD-SSB, UE uses both servingCellMO-OD and servingCellMO-AO (i.e., legacy servingCellMO) to measure serving cell and intra-f neighbour cells. FFS how the network determines the serving cell measurement results come from which MO.

- 2.2.2 Neighbour cell measurements

For neighbour cells measurements, we think on-demand SSB status of neighbour cells should be transparent to the UE (otherwise large signalling overhead in Uu interface and Xn interface are incurred), and it is up to gNB implementation how to handle the reported neighbour cell measurement results which could be degraded based on the intermittent SSB transmission status of neighbour cell.

However, there is one spec impact when serving cell OD-SSB and AO-SSB are inter-frequency, that is how to determine whether neighbour cell measurements are intra-frequency or inter-frequency measurements. In legacy spec, whether an SSB based measurement is considered as intra-frequency or inter-frequency measurement depends on whether the neighbour cell SSB is of the same frequency with the SSB of serving cell, and whether the SCS is the same as serving cell SSB. If two servingCellMOs are configured for serving cell measurements, RAN2 needs to clarify the SSB frequency in which servingCellMO will be used as baseline to identify intra-frequency/inter-frequency measurements.

Proposal 8: For neighbour cell measurements:

- OD-SSB status of neighbour cell is transparent to UE, and no spec impact is needed.

- When serving cell OD-SSB and AO-SSB are on different frequencies, the UE determines whether neighbour cell measurements is intra-f or inter-f measurements based on AO-SSB servingCellMO when OD-SSB is not transmitted, based on OD-SSB servingCellMO when OD-SSB is transmitted.

- 2.2.3 Measurement gaps

In existing spec, other than the cases where UE reports gap requirement (e.g. via NeedForGapsInfoNR reporting), if any of the configured BWPs do not contain serving cell SSB (which refers to the SSB of servingCellMO) and are not configured with NCD-SSB for serving cell measurement (which was introduced in RedCap), the network should configure gaps to the UE for intra-frequency measurements. However, if the SCell can be configured with AO-SSB and an inter-frequency OD-SSB, it is unclear whether gaps are needed if a BWP contains AO-SSB but not OD-SSB.

---- cited from 38.300 v18.5.0 - For SSB based intra-frequency measurement, if the measurement gap requirement information is reported by the UE, a measurement gap configuration may be provided according to the information. Otherwise, a measurement gap configuration is always provided in the following case: - Other than the initial BWP, if any of the UE configured BWPs do not contain the frequency domain resources of the SSB associated to the initial DL BWP, and are not configured with NCD-SSB for serving cell measurement.

Proposal 9: For SSB based intra-f measurement, if gap requirement is not reported by the UE: Other than the initial BWP, if any of the UE configured BWPs do not contain the frequency domain resources of the SSB indicated by OD-SSB servingCellMO when OD-SSB transmission is activated, a measurement gap is always provided.

- 2.2.4 Measurement reporting

In the existing spec, the report types of RRM measurements include the following:

ReportConfigNR ::=                          SEQUENCE {

reportType                                  CHOICE {

periodical                                  PeriodicalReportConfig,

eventTriggered                              EventTriggerConfig,

...,

reportCGI                                   ReportCGI,

reportSFTD                                  ReportSFTD-NR,

condTriggerConfig-r16                       CondTriggerConfig-r16,

cli-Periodical-r16                          CLI-PeriodicalReportConfig-r16,

cli-EventTriggered-r16                      CLI-EventTriggerConfig-r16,

rxTxPeriodical-r17                          RxTxPeriodical-r17,

reportOnScellActivation-r18                 ReportOnScellActivation-r18

}

}

Event triggered reporting and periodical reporting are widely used to trigger serving/neighbour cell measurement reports.

For OD-SSB based SCell measurements, considering OD-SSB is transmitted in a limited period (e.g. from time instance A to time instance B), and there could be no intra-frequency neighbour cell SSBs during the period or there are fewer SSB occasions of neighbour cells compared with serving cell OD-SSB, it is difficult to define a proper report interval.

Looking at the existing RRM measurement events, event A1 and A2 only concern serving cell, and can be used for triggering OD-SSB measurement reporting. For event A3 and A5, the neighbour cell measurements need to be compared with serving cell measurements to determine whether the event is triggered. Considering the different characteristics of serving cell OD-SSB (which is denser, in a confined period of time) and neighbour cell always-on SSB (which is sparser), it is unclear whether A3 and A5 can be triggered properly.

Proposal 10: RAN2 to discuss how periodical reporting and/or event-triggered reporting is applied to OD-SSB measurements.

In the last meeting, some companies raised the issue of whether the OD-SSB MAC CE can trigger L3 measurement reporting. R18 introduced NW triggered report (of L3 measurement result) during FR2 unknown SCell activation. To be specific, SCell Activation/Deactivation MAC CE can be used to trigger the UE to report available measurement results of the SCell.

The R18 mechanism is a bit different from the OD-SSB MAC CE triggered measurement report: in the R18 scheme, the measurement is pre-configured to the UE and UE may have performed measurements already, the MAC CE triggered reporting is to report the measurement results that are already available at the UE side. While in the R19 scenario, before OD-SSB MAC CE is sent, the OD-SSB is not transmitted, and the UE needs to start measurements after receiving OD-SSB MAC CE.

From our perspective, in scenario 2a (upon SCell activation), the R18 mechanism can still be reused, there is no need to exclude the R18 mechanism to be used by OD-SSB SCell. In scenario 2, there is no existing mechanism to rely on, and it is beneficial to introduce OD-SSB MAC CE triggered measurement and reporting, for instance, if the NW wants to check the signal quality of the deactivated SCell. It may be argued that the network can always configure a very low value in Event A1 to trigger serving cell measurement reporting, but this implementation may bring excessive measurement reporting (especially for case 2, in which the AO-SSB will also trigger the measurement reports).

Proposal 11: OD-SSB MAC CE can be used to trigger serving cell measurements and reporting.

Conclusion

In this contribution, we discussed on-demand SSB for intra/inter-band CA, and propose the following:

Proposal 1: RAN2 to down-select from the following alternatives:

- Understanding 1: For SCell using implicit deactivation, A/D bit is set to 0 (deactivation), UE ignores the configuration index and only follows N configured by RRC. No configuration index is included for the SCell.

- Understanding 2: For SCell using implicit deactivation, A/D bit is set to 1 (activation), UE restarts counter N for the SCell when receiving the MAC CE, and N is configured by RRC. No configuration index is included for the SCell.

- Understanding 3: For SCell using implicit deactivation, A/D bit is set to 1 (activation) and configuration index is included to update the N to the remaining numbers of OD-SSB bursts to be transmitted.

- Combined Understanding 1 and 3: For SCell using implicit deactivation, if the A/D bit set to 0 (deactivation) UE ignores the configuration index field, if A/D bit is set to 1 (activation) the UE follows the configuration index field.

Proposal 2: To ensure that OD-SSB is not deactivated after SCell is activated (i.e., Scenario 3B), RAN2 to down-select from the following alternatives:

- Option 1: RRC configured N includes a value set to infinity

- Option 2: It is up to NW implementation to reconfigure the UE to disable the OD-SSB feature for an SCell when the SCell is activated.

Proposal 3: Use two eLCIDs for the OD-SSB transmission MAC CE, corresponding to 7 SCells and 31 SCells respectively.

Proposal 4: RAN2 will not further discuss the case of multiple OD-SSBs with different frequencies for a given SCell, unless this scenario is agreed by RAN1 or RAN4.

Proposal 5: For serving cell measurements:

- In case 1 (no AO-SSB), the serving cell measurements are based on OD-SSB.

- In case 2 (with AO-SSB), in intra-frequency cases, OD-SSB can be used for serving cell measurements, how AO-SSB is utilized for serving cell measurements and how to combine AO-SSB and OD-SSB is up to RAN4 discussion; in inter-frequency cases, only OD-SSB is used for serving cell measurements.

Proposal 6: For both Case 1 (no always-on SSB) and Case 2 (always-on SSB), the measurements of OD-SSB is based on OD-SSB pattern rather than SMTC. For deactivated SCell, after the fast window, OD-SSB is measured based on measCycleSCell in combination with OD-SSB pattern.

Proposal 7: On how to utilize the new servingCellMO, RAN2 to down-select from the following:

- Option 1: When OD-SSB is activated, UE uses servingCellMO-OD to measure serving cell and intra-f neighbour cells; when OD-SSB is deactivated, UE uses servingCellMO-AO (i.e., legacy servingCellMO) to measure serving cell and intra-f neighbour cells.

- Option 2: Regardless of the activation status of OD-SSB, UE uses both servingCellMO-OD and servingCellMO-AO (i.e., legacy servingCellMO) to measure serving cell and intra-f neighbour cells. FFS how the network determines the serving cell measurement results come from which MO.

Proposal 8: For neighbour cell measurements:

- OD-SSB status of neighbour cell is transparent to UE, and no spec impact is needed.

- When serving cell OD-SSB and AO-SSB are on different frequencies, the UE determines whether neighbour cell measurements is intra-f or inter-f measurements based on AO-SSB servingCellMO when OD-SSB is not transmitted, based on OD-SSB servingCellMO when OD-SSB is transmitted.

Proposal 9: For SSB based intra-f measurement, if gap requirement is not reported by the UE: Other than the initial BWP, if any of the UE configured BWPs do not contain the frequency domain resources of the SSB indicated by OD-SSB servingCellMO when OD-SSB transmission is activated, a measurement gap is always provided.

Proposal 10: RAN2 to discuss how periodical reporting and/or event-triggered reporting is applied to OD-SSB measurements.

Proposal 11: OD-SSB MAC CE can be used to trigger serving cell measurements and reporting.

References

- RP-242354, Revised WID: Enhancements of network energy savings for NR, Ericsson

Annex – related RAN1 agreements

@RAN1 118:

Agreement

For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell.

- Periodicity of the on-demand SSB.

- FFS: Any other relevant parameters.

@RAN1 118bis:

Agreement

The previous RAN1 agreement is partly confirmed and further revised as follows.

- For SSB burst(s) indicated by on-demand SSB SCell operation via a MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows.

  - Alt 3-1: Time instance A is the beginning of the first slot containing [candidate SSB index 0 or the first actually transmitted SSB index] ofwithin the first “possible” on-demand SSB burst which is at least T slots after the slot where UE receives a signalling from gNB to indicate on-demand SSB transmission

    - The SSB time domain positions of on-demand SSB burst are configured by gNB.

      - The location(s) (e.g., SFN offset, half frame index) in the time domain of “possible” on-demand SSB burst and SSB position within the burst should be configured by the gNB

  - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation

  - (Working assumption): T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to indicate on-demand SSB transmission ending in slot n, and Nslotsubframe,μ is as defined in current specification.

    - RAN4 to confirm that T_min can be equal to m+3Nslotsubframe,μ+1

  - (Working assumption) T=T_min

- Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology.

@RAN1 119:

Agreement

- New periodicity value for on-demand SSB other than the legacy values (i.e., 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms) is NOT introduced in Rel-19.

Agreement

For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective.

- Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication

  - Deactivation by RRC is up to RAN2

  - FFS: Which scenario Option 1 is used

- Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated

  - FFS: Whether Option 4, 4a is needed in addition to Option 2

  - FFS: Whether the value of N can be implicitly determined using a timer

@RAN1 120:

Agreement

- For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameter (i.e., od-ssb-config) for on-demand SSB configuration, for both Case #1 and Case #2.

- For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameters to configure each of the followings, for both Case #1 and Case #2.

  - Frequency of the on-demand SSB (i.e., od-ssb-absoluteFrequency)

    - FFS: Whether this parameter can be absent for Case #2

  - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst)

    - FFS: Whether this parameter can be absent for Case #2

  - Periodicity of the on-demand SSB (i.e., od-ssb-Periodicity)

  - Sub-carrier spacing of the on-demand SSB (i.e., od-ssbSubcarrierSpacing)

    - For Case #2, this parameter is absent and sub-carrier spagcing of on-demand SSB is the same as that of always-on SSB.

  - Physical Cell ID of the on-demand SSB (i.e., od-ssb-physCellId)

    - For Case #2, this parameter is absent and physical cell ID of on-demand SSB is the same as that of always-on SSB.

  - Time location of on-demand SSB burst (i.e., od-ssb-sfn-Offset and od-ssb-halfFrameIndex)

  - Downlink transmit power of on-demand SSB (i.e., od-ss-PBCH-BlockPower)

    - For Case #2, this parameter is absent and downlink transmit power of on-demand SSB is the same as that of always-on SSB.

  - Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofTx)

  - FFS: Which of the above parameters are a list

Agreement

Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB,

- Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB.

  - On-demand SSB is not on sync raster

  - AO-SSB and OD-SSB are located in the same BWP

  - FFS: Additional conditions

  - Subject to separate UE capability

  - Note: UE is not required to measure both AO-SSB and OD-SSB

@RAN1 120bis:

Agreement

For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s) (in addition to agreed ones), multiple candidate values can be configured (includes the case where no candidate values are configured) by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell.

- SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst) for the following cases

  - The case where center frequency of AO-SSB and OD-SSB are different

  - Case 1

- Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb- nrofBurst)

FFS: Additional restrictions

Agreement

For a cell supporting on-demand SSB SCell operation, the following combinations of scenarios and cases are supported for indicating OD-SSB using a MAC-CE.

- Scenario #3B and Case #1

  - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication.

- Scenario #3B and Case #2

  - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication.