3GPP TSG-RAN WG2 Meeting #130	R2-2503951

St.Julians, Malta, May 19th – 23rd, 2025

Agenda item:	8.20.1

Source:	Huawei, HiSilicon

Title:	Discussion on simultaneous Tx-Rx capability for TDD-SDL and TDD-FDD combinations

Document for:	Discussion and Decision

- Introduction

RAN4 sent a LS to RAN2 for simultaneous Tx-Rx capability for TDD-SDL and TDD-FDD combinations [1].

In Athens meeting, RAN4 had agreed on TDD-SDL inter-band CA band combination that - To clarify the handling of simultaneous Rx-Tx interaction between network and UE for band combinations with different types of duplex mode - For TDD-SDL band combination, simultaneous Rx-Tx is mandatory with signalling. Consider to reuse the capability of simultaneousRxTxInterBandCA referring to TDD-FDD.. And the clarification for FDD-TDD CA band combination has been captured in the general part of sub-clause 5.2A.2 Inter-band CA in TS38.101-1: Unless stated otherwise, simultaneous Rx/Tx capability is mandatory for FR1+FR1 FDD-TDD CA combinations.. Meanwhile, Note 1 attached to some FDD-TDD band combinations in Table 5.2A.2.1-1on inter-band CA operating bands involving FR1 (two bands) is removed. NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. RAN4 notice that currently there is no simultaneous Rx-Tx capability signaling defined for the TDD-SDL band combination in TS38.306. Therefore, RAN4 would like to ask RAN2 to take necessary actions based on the above RAN4 agreements.

In this contribution, we discuss the RAN2 impacts based on this RAN4 LS.

- Discussion

  - Clarification for FDD-TDD CA band combination

RAN4 further clarified simultaneous Rx/Tx capability for FDD-TDD CA band combination, by removing the NOTE1 and add a general description. From UE perspective, it is mandatory with signaling. On network side, it always relies on the capability reporting from the UE. Thus, no backward compatibility issue is foreseen. There is no further RAN2 specification impacts for FDD-TDD CA band combination.

Proposal 1: RAN2 confirms that no RAN2 specification impacts for FDD-TDD CA band combination.

  - Capability for TDD-SDL inter-band CA band combination

In previous xDD differentiation discussion, it was agree that the corresponding capabilities indicated by the FDD capability is applied to SDL if SDL band is supported by the UE. This is captured in TS 38.306.

The UE may support different functionalities between FDD and TDD, and/or between FR1 and FR2. The UE shall indicate the UE capabilities as follows. In the table of UE capability parameter in subsequent clauses, "Yes" in the column by "FDD-TDD DIFF" and "FR1-FR2 DIFF" indicates the UE capability field can have a different value for between FDD and TDD or between FR1 and FR2 and "No" indicates if it cannot. "(Incl FR2-2 DIFF)" in the column by "FR1-FR2 DIFF" indicates the UE capability field can have a different value for between FR2-1 and FR2-2. Regarding to the per UE capabilities that are FDD/TDD differentiated(i.e. capabilities indicated as "Yes" in the column by "FDD-TDD DIFF"), the corresponding capabilities indicated by the FDD capability is applied to SUL/SDL if SUL/SDL band is supported by the UE. "FD" in the column indicates to refer the associated field description. "FR1 only" or "FR2 only" in the column indicates the associated feature is only supported in FR1 or FR2 and "TDD only" indicates the associated feature is only supported in TDD and not applicable to SUL/SDL carriers. "N/A" in the column indicates it is not applicable to the feature (e,g. the signalling supports the UE to have different values between FDD and TDD or between FR1 and FR2).

The WF [2] approved by RAN4 confirms it:

- For SDL band combination, the SDL band can be viewed as derived from FDD band. - The handling of FDD-SDL BC refers to that for FDD-FDD - The handling of TDD-SDL BC refers to that for TDD-FDD.

Observation 1: For SDL band combination, the SDL band can be viewed as derived from FDD band. The handling of TDD-SDL BC refers to that for TDD-FDD.

As suggested by RAN4 LS, reusing the capability of simultaneousRxTxInterBandCA referring to TDD-FDD can be considered for TDD-SDL BC. simultaneousRxTxInterBandCA referring to TDD-FDD can cover the capability for TDD-SDL BC. Thus, the simultaneousRxTxInterBandCA can be extended to cover TDD-SDL BC. If UE supports simultaneous transmission and reception for a TDD-SDL inter-band NR CA, UE includes this field for such TDD-SDL BC.

If the legacy gNB validates simultaneousRxTxInterBandCA for TDD-SDL, it is compatible if this field is reused. If the legacy gNB does not validate simultaneousRxTxInterBandCA for TDD-SDL, as an alternative, introducing a new Rel-19 simultaneous Rx-Tx capability for TDD-SDL can be considered. However, the gNB anyway needs to be upgraded to support simultaneous Rx-Tx capability for TDD-SDL, no matter if the existing field is reused or a new capability field is introduced, there is not much difference in the gNB’s impacts. Reusing existing field provides better compatibility for legacy UEs that already implement simultaneousRxTxInterBandCA for TDD-SDL.

Thus, reusing simultaneousRxTxInterBandCA is preferred to support simultaneous Rx-Tx capability for TDD-SDL. Since the band combination is release independent, the update for simultaneousRxTxInterBandCA can be adopted from Rel-15.

Proposal 2: The simultaneousRxTxInterBandCA is extended to cover TDD-SDL BC from Rel-15.

In addition to TDD-SDL inter-band NR CA band combination, RAN4 also defines TDD-SDL inter-band (NG)EN-DC/NE-DC band combination, e.g., DC_1A_n75A-n78A. Thus, the same logic can be applied to simultaneousRxTxInterBandENDC to cover TDD-SDL BC. Moreover, for the enhanced signaling simultaneousRxTxInterBandCAPerBandPair and simultaneousRxTxInterBandENDCPerBandPair, the similar revision can be introduced.

Proposal 3: If the Proposal 2 can be agreed, the similar changes for simultaneousRxTxInterBandCA are applied to simultaneousRxTxInterBandENDC, simultaneousRxTxInterBandCAPerBandPair and simultaneousRxTxInterBandENDCPerBandPair.

The potential spec changes according to Proposal 2 and 3 are provided in Appendix.

- Conclusion

In this paper, we the RAN2 impacts based on RAN4 LS on simultaneous Tx-Rx capability for TDD-SDL and TDD-FDD combinations, and have the following proposals.

Proposal 1: RAN2 confirms that no RAN2 specification impacts for FDD-TDD CA band combination.

Proposal 2: The simultaneousRxTxInterBandCA is extended to cover TDD-SDL BC from Rel-15.

Proposal 3: If the Proposal 2 can be agreed, the similar changes for simultaneousRxTxInterBandCA are applied to simultaneousRxTxInterBandENDC, simultaneousRxTxInterBandCAPerBandPair and simultaneousRxTxInterBandENDCPerBandPair.

- References

R4-2505123, LS on simultaneous Tx-Rx capability for TDD-SDL and TDD-FDD combinations.

R4-2502840, WF on simultaneous Rx-Tx, Huawei.

- Appendix – TP for TS 38.306

- [Text omitted]

| simultaneousRxTxInterBandCA Indicates whether the UE supports simultaneous transmission and reception in TDD-TDD, TDD-FDD and TDD-SDL inter-band NR CA. If this field is included in ca-ParametersNR-ForDC, it indicates the UE supports simultaneous transmission and reception between any UL/DL band pair within a cell group and across MCG and SCG in TDD-TDD, TDD-FDD and TDD-SDL inter-band NR-DC. It is mandatory for certain TDD-FDD, TDD-TDD and TDD-SDL band combinations defined in TS 38.101-1 [2], TS 38.101-2 [3] and TS 38.101-3 [4]. This capability does not apply to the following components within TDD-TDD, TDD-FDD and TDD-SDL inter-band NR-CA or NR-DC combinations: - Intra-band NR-CA or NR-DC component - Inter-band NR-CA or NR-DC component where the frequency range of one TDD band is a subset of the frequency range of the other NR TDD band (as specified in TS 38.101-1 [2]). | BC | CY | N/A | N/A |
| --- | --- | --- | --- | --- |
| simultaneousRxTxInterBandCAPerBandPair Indicates whether the UE supports simultaneous transmission and reception in TDD-TDD, TDD-FDD and TDD-SDL inter-band NR CA for each band pair in the band combination. Encoded as a bitmap with size L * (L – 1) / 2, and bit N (leftmost bit is indexed as bit 0) is set to "1" if the UE supports simultaneous transmission and reception for band pair (x, y), where L is the number of band entries in the band combination, x and y are the indices of the band entry in the band combination (the first band entry is indexed as 0), x < y, and N = x*(2*L – x – 1)/2 + y – x – 1. If this field is included in ca-ParametersNR-ForDC, each bit of this field indicates whether the UE supports simultaneous transmission and reception between each band pair, within a cell group and across MCG and SCG in TDD-TDD, TDD-FDD and TDD-SDL inter-band NR-DC. The UE does not include this field if the UE supports simultaneous transmission and reception for all applicable band pairs in the band combination (in which case simultaneousRxTxInterBandCA is included) or does not support for any band pair in the band combination. It is mandatory for certain band pairs as specified in TS 38.101-1 [2], TS 38.101-2 [3] and TS 38.101-3 [4]. The UE shall consistently set the bits which correspond to the same band pair. | BC | CY | N/A | N/A |

- [Text omitted]

| simultaneousRxTxInterBandENDC Indicates whether the UE supports simultaneous transmission and reception in TDD-TDD, TDD-FDD and TDD-SDL inter-band (NG)EN-DC/NE-DC. It is mandatory for certain TDD-FDD, TDD-TDD and TDD-SDL band combinations defined in TS 38.101-3 [4]. This capability does not apply to the following components within TDD-TDD, TDD-FDD and TDD-SDL inter-band (NG)EN-DC/NE-DC combination: - Intra-band (NG)EN-DC/NE-DC component - Inter-band (NG)EN-DC/NE-DC component where the frequency range of the E-UTRA band is a subset of the frequency range of the NR band (as specified in Table 5.5B.4.1-1 of TS 38.101-3 [4]). | BC | CY | N/A | N/A |
| --- | --- | --- | --- | --- |
| simultaneousRxTxInterBandENDCPerBandPair Indicates whether the UE supports simultaneous transmission and reception in TDD-TDD, TDD-FDD and TDD-SDL inter-band (NG)EN-DC/NE-DC for each band pair in the band combination. Encoded in the same manner as simultaneousRxTxInterBandCAPerBandPair. The UE does not include this field if the UE supports simultaneous transmission and reception for all applicable band pairs in the band combination (in which case simultaneousRxTxInterBandENDC is included) or does not support for any band pair in the band combination. It is mandatory for certain band pairs as specified in TS 38.101-3 [4]. The UE shall consistently set the bits which correspond to the same band pair. Each bit of the capability only applies to TDD-TDD, TDD-FDD and TDD-SDL Inter-band (NG)EN-DC/NE-DC band pairs, except for the band pairs where the frequency range of the E-UTRA band is a subset of the frequency range of the NR band (as specified in Table 5.5B.4.1-1 of TS 38.101-3 [4]). | BC | CY | N/A | N/A |

- [Text omitted]