3GPP TSG-RAN WG2 Meeting #130	R2-2503953

St.Julians, Malta, May 19th – 23rd, 2025

Agenda item:	8.4.2

Source:	Huawei, HiSilicon, vivo, CMCC, CATT, Nokia, Samsung, LG Electronics Inc., Apple, Ericsson, OPPO, Sharp, NEC

Title:	Discussion on prioritizing the frequencies supporting LP-WUS

Document for:	Discussion and Decision

- Introduction

Based on RAN Plenary discussion and way forward [1], RAN2 discussed the following solutions.

- Solution 1: MR is prioritized (via SIBs) to camp on the same band as LR (System information, Paging, RACH on LR band).

- Solution 2: MR is prioritized to camp on the same band as LR (System information, Paging on LR band; and RACH on another MR band based on reselection criteria).

- Solution 3: MR is not prioritized to camp on the same band as LR but can be redirected via dedicated signalling depending on LR band load (up to gNB implementation).

The pros and cons of the solutions above were provided during offline discussion in RAN2#129 meeting [2].

|  | Overload issue, including RACH/data | Latency | Limited usage of LP-WUS (reduce PS gain) | Specification impact |
| --- | --- | --- | --- | --- |
| Solution 1 | Yes | As normal/legacy | No | RAN2: Medium (e.g. configure separate priority for LR band in SI, UE behavior on cell reselection based on separate priority) RAN4: No RAN1: No |
| Solution 2 | No | Longer than normal/legacy (Rapp: Not as long as requirements in RAN4. ) | No | RAN2: Higher than solution 1/3 (solution 1 or 3 is part of solution 2, and new procedure at UE to perform cell reselection after LP-WUS and paging) RAN4: Potential discussion on new requirements may be needed RAN1: No |
| Solution 3 | No (flexible by NW implementation) Maybe after release | As normal/legacy | Limited usage than solution1/2 | RAN2: Already supported, and resolve potential issue, if any RAN4: No RAN1: No |

The conclusions in RAN2#129 meeting were:

RAN2 understands that UE can report which band(s) is supported by LR to NW. RAN2 understands that any potential overload issues could be addressed by current mechanism in spec.

- Discussion

How to avoid potential overload issues was discussed and there was a consensus that it could be addressed by existing mechanism, e.g., relying on dedicated cellReselectionPriorities configured in RRCRelease message. However, RRCRelease message exclusively governs Connected state UEs. Once such UEs are released to Idle/Inactive state, NW control via RRCRelease message becomes challenging (paging a UE solely to transmit a RRCRelease message with dedicated configurations is inefficient and increasing signalling overhead).

Considering the following cases:

- Cell 1 in 700MHz is overloaded, the NW configures LP-WUS UEs to prioritize 2.6GHz via RRCRelease message. After the overload issue in 700MHz Cell 1 is relieved/resolved, the NW is not able to update these priorities in efficient manner (Network needs to move UEs to the Connected state to configure new dedicated priorities with RRCRelease message or NW needs to configure a timer indicating how long time dedicated priorities should be applied, which is also challenging since NW cannot know how long time the overload issue may last). Moreover, the other cells in 700MHz without overload issued cannot be prioritized either. So LP-WUS UEs are not able to camp on/reselect to cells in 700MHz to use LP-WUS function even when no overload exists.

- Cell 1 in 700MHz is not overloaded, if the NW configures LP-WUS UEs to prioritize 700MHz via RRCRelease message, when the NW detects overload for 700MHz Cell 1 afterwards, it has no efficient way to distribute the Idle/Inactive UEs to other cells to solve the overload issue. Thus, for real deployment, the conservative NW implementation may never be to let LP-WUS UEs prioritize 700MHz given the inability to adjust it.

Based on above analyses, relying on RRCRelease message or dedicated priority timer is hard to balance the load and LP-WUS utilization due to lack of flexibility for Idle/Inactive UEs, which leads to limited usage of LP-WUS feature and degrades power saving gain.

Observation 1: Relying on existing mechanisms is hard to balance the load and LP-WUS utilization due to lack of flexibility for Idle/Inactive UEs, which leads to limited usage of LP-WUS feature and degrades power saving gain.

SIB is a more efficient and flexible signaling to control Idle/Inactive UEs. If LP-WUS cells in 700MHz are not overloaded, the NW can indicate LP-WUS UEs to prioritize 700MHz via SIB to get the power saving benefits from LP-WUS feature. If the overload issue is detected, the NW can adjust the configuration in SIB to prioritize 2.6GHz, so that LP-WUS UEs camping on 700MHz will re-select to the cells in 2.6GHz.

Fig 1. Illustration for SIB configuration to balance load and LP-WUS utilization

Observation 2: SIB is a more efficient and flexible way to control Idle/Inactive UEs, the NW can configure LP-WUS UEs whether to prioritize the frequencies supporting LP-WUS via SIB dynamically based on the detected load situation.

Table 1. The Pro and Cons for the flag in SIB in different granularity

| Granularity | Pros and Cons |
| --- | --- |
| Per UE (common for all frequencies) | A flag broadcast in SIB, with minimal overhead; Too restrictive, there is the case that only one/some cell(s) for one LP-WUS frequency congested, other LP-WUS frequencies do not need to be impacted; UE has to know whether an inter-frequency supporting LP-WUS or not by implementation, due to lack of information from NW configuration. |
| Per frequency | A flag broadcast for each inter-frequency in SIB, with medium-level overhead; Flexible enough since the typical LP-WUS deployment may be consistent for all cells in one frequency; UE directly relies on the flag to know whether an inter-frequency supporting LP-WUS or not. |
| Per frequency per cell | A flag broadcast for a cell-list for each inter-frequency in SIB, with more overhead; Very flexible but would be over-designed if the typical LP-WUS deployment is consistent for all cells in one frequency; UE directly relies on the flag to know whether a cell of an inter-frequency supporting LP-WUS or not. |

For the detailed SIB signaling design, based on the analyses in Table 1 above, a flag can be introduced for each inter-frequency comprehensively considering signaling overhead, flexibility and implementation complexity. NW can set the flag for the frequency with LP-WUS deployed if there is no overload. In this case, UEs supporting LP-WUS on that frequency can consider that frequency as highest priority for cell reselection. The flag will be absent if LP-WUS is not deployed or there is overload issue for a frequency, which means legacy operation is applied. It is fully under NW control.

Proposal 1: Support 1-bit flag for each inter-frequency in SIB. If the flag is set to true for a frequency and UE supports LP-WUS on this frequency, UE can consider this frequency to be the highest priority.

The potential specification impacts are provided below.

TS 38.331, SIB4

InterFreqCarrierFreqInfo-v19xy ::=  SEQUENCE {

lpwusFreq-r19                  ENUMERATED {true}                     OPTIONAL    -- Need R

}

lpwusFreq If present, it indicates that UEs supporting LP-WUS are allowed to prioritize this frequency in cell reselection evaluation as specified in TS 38.304 [20].

TS 38.304, 5.2.4.1	Reselection priorities handling

The LP-WUS capable UE shall/may (to be discussed in running CR) consider the frequency for which lpwusFreq-r19 is present to be the highest priority, if the UE supports LP-WUS on the frequency.

To be noted, if the LP-WUS case is added in 5.2.4.1, the following NOTE 0c is also applicable.

NOTE 0c:	The prioritization among the frequencies which UE considers to be the highest priority frequency is left to UE implementation unless otherwise stated.

- Conclusion

In this paper, we discuss to prioritize the frequencies supporting LP-WUS, and suggest to agree the following proposal.

Proposal 1: Support 1-bit flag for each inter-frequency in SIB. If the flag is set to true for a frequency and UE supports LP-WUS on this frequency, UE can consider this frequency to be the highest priority.

- References

RP-243266, Way forward on Rel-19 LP-WUS separate band issues.

R2-2501443, Discussion report on [AT129][203][LPWUS], vivo.