3GPP TSG-RAN WG2 Meeting #130	R2-2503957

St Julian’s, Malta, 19th – 23rd May 2025

Agenda Item:	8.7.4.1

Source:	ITRI

Title:	Discussion on enhanced LCP for XR

Document for:	Discussion and Decision

1 Introduction

In RAN2 #129bis, the following agreements of enhanced LCP had been reached.

- Working assumption (to be revisited next meeting): No Bj enhancement is introduced.

It will only be considered provided that gains are proven and that we have a common understanding on the change that is needed in specifications

- We will check the impact of using additional LCP priority for SR priority determination during intra-UE prioritization in specifications and make a final decision on whether to have it based on that, i.e. it should be simple enough.

- It should be explicitly specified which priority, e.g. “default” or additional priority, to use for a LCH in order to determine the priority of the corresponding UL grant, i.e. highest priority used/to be used during LCP.

- For shared spectrum case, UE shall apply same rules for the LCH priority being used for determining the priority of a HARQ process as for determining the priority of an UL grant.

- Understanding in RAN2 is that we will not address any other NR-U specific issues to support XR.

- All UEs falls back to default priority in the 2nd round of the LCP procedure if there is no LCH-priority adjusted data left, i.e. no capability is needed (this reverts previous RAN2 agreement on this).

In this document, we discuss the two FFSs,  Bj enhancement and Schedulling Request (SR) priority determination during intra-UE prioritization. Because we care about the fairness of Logical Channel Prioritization (LCP) more than the unsured latency/capacity gain,  the Bj relaxation for logical channel (LCH) with priority-adjusted data is suggested not to be considerd in Rel-19 LCP. As to the SR priority determination, since RAN2 had agreed multiple reporting ports for Rel-19 Delay Status Reporting (DSR), we think that an optimization on the SR priority determination caused by adjusted priority is not needed. However, a compromise (e.g., a note clarification) is accepted based on the majority’s decision.

2 Discussion

2.1 Bj enhancement

We worried about the fairness issue of LCP with Bj relaxation. We understand that if an LCH with an upgraded priority is allowed to be transmitted even if Bj is negative, while the remaining time is less the configured threshold, these delay-critical data may be multiplexed into a MAC PDU as soon as possible and transmitted in time. However, if the data volume of these delay-critical data is very large, the Bj relaxation may cause a severe penalty to LCHs which priorities are higher than the default priority of the LCH with delay-critical data but lower than the corresponding upgraded priority. In addition, for LCHs with lower default priorities, it can also cause starvation to them.

Although the proponents had presented the scenarios and simulations to prove the need of Bj relaxation, we think that the transmission characteristics of simulated traffic and setting of parameters of LCP may not be realistic enough. Therefore, we suggest that maybe the Bj relaxation can be revisited in the coming Rel-20 XR enhancement for the Mobile AI. From our point of view, it is enough for XR traffic that each LCH is configured by network with an appropriate PBR, a default priority and an adjusted priority in Rel-19 LCP.

Proposal 1: Not to consider the Bj relaxation for LCH with priority-adjusted data in Rel-19 LCP.

2.2 SR priority determination during intra-UE prioritization

We wonder how much gain SR priority determination will do during intra-UE prioritization. We understand that some companies thought that SR priority determination needs to be specified during intra-UE prioritization as this allows the network to know which priority is applied by the UE in different situations. For example, the UE can select the SR configuration that matches the additional priority of the corresponding LCH.

However, the SR is used for requesting UL-SCH resources for new transmission, and for a LCH, at most one PUCCH resource for SR is configured per BWP. RAN2 had agreed multiple reporting ports for Rel-19 DSR, and UE may transmit Rel-19 DSR in the following UL grant for limited PUSCH resource case. We think that SR priority determination during intra-UE prioritization is not so necessary because the gNB can get enough delay-critical data information relying on the Rel-19 DSR. Besides, we should note that when an SR is triggered, there is actually no LCP performed simultaneously. From our point of view, an optimization on the SR priority determination caused by adjusted priority is not needed during intra-UE prioritization. Nevertheless, if the majority’s view is a compromise and to clarify the prevailing logical channel priority, it is an accepted decision on a simple added note to minimize the standard impact.

Proposal 2: There is no impact on SR priority determination during intra-UE prioritization due to adjusted priority. However, a compromise (e.g., a note clarification) is accepted based on the majority’s decision.

3 Conclusion

Based on the discussion in the previous section we propose the following:

Proposal 1: Not to consider the Bj relaxation for LCH with priority-adjusted data in Rel-19 LCP.

Proposal 2: There is no impact on SR priority determination during intra-UE prioritization due to adjusted priority. However, a compromise (e.g., a note clarification) is accepted based on the majority’s decision.

4 References

- R2_129bis R19 XR session notes (Dawid) 2025-04-10_EOM