3GPP TSG-RAN WG2 Meeting #130	R2-2503960

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda item:	8.2.3

Source:	Spreadtrum, UNISOC

Title:	Discussion on A-IoT random access

Document for:	Discussion and Decision

1 Introduction

In RAN2#129bis meeting, RAN2 discussed the random access for A-IoT and achieved the following agreements [1]:

MSG1 contents for CBRA/CFRA • In case of CBRA, only 16 bits random ID is included in Msg1. FFS can be revisited if message type will be needed for other D2R messages purposes • RN16 is not included in the first D2R message in the CFRA procedure. AS ID is the only ID needed for addressing the device in R2D command message assuming for CFRA no multiple devices are performing the procedures with the given reader. FFS if we can assume or need to support multiple device scenario. MSG1 Resource Definition • A new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling. The R2D message indicates the start of a set of MSG1 resources that were configured in paging message. • Assumption: The R2D message does not include slot number/count down number. Content and format of MSG2 for CBRA • A-IoT Msg2 contains one or multiple echoed random ID(s) from A-IoT Msg1 of different A-IoT devices. • Same Msg2 format is used for initial transmission and retransmission of Msg2. NACK feedback • For CBRA, as a baseline, NACK based mechanism is applied only to the Msg3. May come back for D2R data, if the NACK feedback indication is needed for the purpose to stop/terminate the “on-going procedure” and release the AS ID accordingly (depending on other later discussion). • For msg3, we rely on whether the device receives NACK indication before subsequent R2D message to determine re-access. No need for a timer. FFS whether subsequent R2D message is trigger message or paging • For CFRA, NACK feedback and re-access is not supported. FFS how to achieve • FFS on end of procedure

In this contribution, we will continue discussion the left issues of Ambient IoT random access.

2 Discussion

## 2.1 Contention-based random access

RAN2 agreed that only 3-step CBRA is supported for A-IoT in Rel-19. For CBRA procedure, slotted-ALOHA based access is baseline. 3-step CBRA random access for A-IoT is shown in Figure 1.

Figure 1 3-step CBRA for A-IoT

For Msg3 transmission, RAN2 agreed that we rely on whether the device receives NACK indication before subsequent R2D message to determine re-access. For the further study whether subsequent R2D message is trigger message or paging.

First, the NACK indication should be an explicit R2D NACK feedback message, where the random ID is included to indicate the failure for a given device. Since for inventory only case, there is no AS ID. For inventory and command case, if reader did not receive Msg3, it may be that device did not receive Msg2 successfully (device did not receive the AS ID include in Msg2) resulting in Msg3 was not sent. The device can recognize only if the NACK indication contains random ID (i.e. RN16) rather than AS ID.

Second, whether the NACK indication is received before subsequent R2D trigger message or before subsequent paging, the device always can decide to re-access. And no device behavior difference between “before subsequent R2D trigger message” and “before subsequent paging”, since device can only re-access upon subsequent paging anyway. However, if in a paging round, two devices connected to the network successively with the same RN16 (device 1 successfully sends Msg3, but device 2 fails to send Msg3), then a collision occurs when NACK message carries RN16 is received before subsequent paging. The NACK feedback is actually sent to device 2, but device 1 will assume that the NACK feedback is for itself and that it failed to transmit Msg3, but in fact it succeeded. This is not a problem if the NACK is received before subsequent R2D trigger message.

Proposal 1: The random ID of device should be carried in NACK indication.

Proposal 2: If NACK indication is received from the reader before subsequent R2D trigger message, device determine to re-access.

## 2.2 Contention-free random access

For dedicated inventory or command for a identified A-IoT device, the specific A-IoT device ID will included in trigger signal in Msg0. Only the device ID of A-IoT device match can initiate access, so it can be a CFRA. CFRA based access procedure is shown in Figure 2.

Figure 2 CFRA for A-IoT

Msg 0: Access trigger. The reader sends trigger signal to trigger A-IoT device to perform access. The trigger signal may be paging or command message which the device ID and resource for RACH are include in.

Msg 1: A-IoT device sends device ID and any other upper layer data to reader.

In last meeting, RAN2 agreed that NACK feedback and re-access is not supported for CFRA, for the further study how to achieve. In our view, the simplest way is that reader resends Msg0 (may potentially with updated resource or parameters allocation) to trigger device transmit Msg1 again. Furthermore, the device must consistently reply with Msg1 for duplicated CFRA Msg0 since the previous one remains unconfirmed successfully.

Proposal 3: In CFRA, if Msg1 is not received successfully by reader, reader can resend Msg0 to trigger device to retransmit Msg1.

3 Conclusion

In this contribution we discuss the random access procedure of A-IoT device, with the following proposals:

Proposal 1: The Random ID of device should be carried in NACK indication.

Proposal 2: If NACK indication is received from the reader before subsequent R2D trigger message, device determine to re-access.

Proposal 3: In CFRA, if Msg1 is not received successfully by reader, reader can resend Msg0 to trigger device retransmit Msg1.

- References

- Draft report of 3GPP TSG RAN WG2 #129bis, RAN2 chair (interDigtal), Wuhan, China, 7 - 11 April, 2025.