3GPP TSG-RAN WG2 Meeting #130	R2-2503962

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda item:	8.13.2

Source:	Spreadtrum, UNISOC

Title:	Discussion on notification message

Document for:	Discussion and Decision

1 Introduction

In this contribution, we will continue discuss the notification message to support multi-hop L2 U2N relay.

2 Discussion

In single U2N and U2U, notification message is used by a Relay UE to send notification to the connected Remote UE on the other side link status.

Figure 1 notification message

In TS 38.331, relay UE can send a notification message if the following conditions are met.

##### 5.8.9.10.2 Initiation The Relay UE may initiate the procedure when one of the following conditions is met: 1> if the UE is acting as U2N Relay UE: 2> upon Uu RLF as specified in 5.3.10; 2> upon reception of an RRCReconfiguration including the reconfigurationWithSync; 2> upon cell reselection; 2> upon L2 U2N Relay UE's RRC connection failure including RRC connection reject as specified in 5.3.3.5 and 5.3.13.10, and T300 expiry as specified in 5.3.3.7, and RRC resume failure as specified in 5.3.13.5;

The action upon receipting notification message at the remote UE is shown in following.

##### 5.8.9.10.4 Actions related to reception of NotificationMessageSidelink message Upon receiving the NotificationMessageSidelink, the Remote UE shall: 1> if the UE is acting as U2N Remote UE: 2> if the indicationType is included: 3> if the UE is L2 U2N Remote UE in RRC_CONNECTED: 4> if MP is configured and MCG transmission (i.e. direct path) is not suspended; 5> if the indicationType is relayUE-HO; 6> suspend indirect path transmission; 5> else: 6> initiate the indirect path failure information procedure as specified in 5.7.3c to report indirect path failure; 4> else if T301 is not running, initiate the RRC connection re-establishment procedure as specified in 5.3.7; 3> else (the UE is L3 U2N Remote UE, or L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE): 4> if the PC5-RRC connection with the U2N Relay UE is determined to be released: 5> indicate upper layers to trigger PC5 unicast link release; 4> else (i.e., maintain the PC5 RRC connection): 5> if the UE is L2 U2N Remote UE and the indicationType is relayUE-HO or relayUE-CellReselection: 6> consider cell re-selection occurs;

## 2.1 Notification message for relay reselection

In RAN2#129bis meeting, it discussed the notification exceptional cases and details, a conclusion was reached as following [1]:

Agreement:

When the parent UE in idle/inactive performs reselection, RAN2 assume that an indication of the reselection is needed for the child UE.

FFS detailed information in the indication and child UE handling.

FFS if there are exceptional cases where the indication can be suppressed, e.g., reselection under the same serving cell without changing the hop count.

FFS if the notification message is used or we rely on upper layer signalling (e.g., discovery).

In multi-path U2N relay, the last relay UE should initiate the notification procedure as in single hop U2N relay. But in the first relay UE, it is a little bit different. When the first relay UE occurs relay reselection and reselects a relay UE belong to another cell, it also should initiate the notification message to its connected remote UE with new indication type (i.e. relayUE-RelayReselection).

Upon receiving the notification message from the first relay UE, the remote UE should initiate the RRC connection re-establishment procedure when it is in RRC_CONNECTED. The RRC_IDLE/INACTIVE remote UE can freely decide whether to maintain or release the PC5 link with the first relay UE. If remote UE decides to release the PC5 link, it indicates upper layers to trigger PC5 unicast link release. If remote UE decides to maintain the PC5 link, remote UE considers that a cell reselection has occurred.

Proposal 1: When the first relay UE occurred relay reselection and reselected relay UE belong to another cell, it should send Notification message to its connected remote UE.

Proposal 2: The notification message includes a new indication Type (i.e. relayUE-RelayReselection).

## 2.2 Notification message from upstream relay UE

Moreover, the last relay UE, as illustrated in the following figure 2 for a 2-hop relay case, can directly detect the status of Uu link. While the Uu link failure (Uu RLF or RRC connection failure) or last relay UE performs HO/cell reselection, the last relay UE can send NotificationMessageSidelink to its connected first relay UE as in R17.

Figure 2 2-hop U2N relay

Upon receiving the NotificationMessageSidelink from the last relay UE, the first relay UE how to handle it. In last RAN2 meeting, a conclusion about this issue was reached as following:

Agreement:

When the intermediate relay UE receives a notification message from the last relay UE indicating a failure on Uu, the intermediate relay UE may transmit a notification message downstream (towards the remote UE).

FFS if the notification message is a forwarded copy of the original notification (same cause) or a regenerated message from the intermediate relay (cause might reflect its own reaction such as relay reselection or re-establishment).  This does not change the agreement that it is up to relay implementation whether to release the downstream link, and if the link is released there is no downstream notification message.

Actually, the first relay UE also act as a remote UE. Upon receiving the NotificationMessageSidelink from the last relay UE, the first relay UE should initiate the RRC connection re-establishment procedure when it is in RRC_CONNECTED, then generate NotificationMessageSidelink with indicationType (e.g. relayUE-PC5-RLF, relayUE-Uu-RLF or relayUE-RRC-Restablishment) to its connected remote UE.

Proposal 3: Upon receiving NotificationMessageSidelink, the RRC_CONNECTED first relay UE will perform the RRC Reestablishment and then send NotificationMessageSidelink to its connected remote UE indicating the occurred RRC Reestablishment event.

Upon receiving the NotificationMessageSidelink from the last relay UE, the RRC_IDLE/INACTIVE first relay UE can freely decide whether to maintain or release the PC5 link with last relay UE. If first relay UE decides to release the PC5 link and perform relay reselection, then it sends NotificationMessageSidelink to its connected remote UE with relayUE-RelayReselection indication type.

If first relay UE decides to maintain the PC5 link and the indicationType indicated in the NotificationMessageSidelink is relayUE-HO or relayUE-CellReselection, first relay UE considers that a cell reselection has occurred, so the indicationType in the NotificationMessageSidelink sent to the Remote UE is relayUE-CellReselection.

Proposal 4: Upon receiving NotificationMessageSidelink, if the RRC_IDLE/INACTIVE first relay UE performs the PC5 link release and relay reselection, then it will send NotificationMessageSidelink to its connected remote UE indicating relay reselection.

Proposal 5: Upon receiving NotificationMessageSidelink, if the RRC_IDLE/INACTIVE first relay UE maintains the PC5 link, it will send NotificationMessageSidelink to its connected remote UE indicating cell reselection.

3 Conclusion

In this contribution we discuss the notification message for multi-hop relay, with the following proposals

Proposal 1: When the first relay UE occurred relay reselection and reselected relay UE belong to another cell, it should send Notification message to its connected remote UE.

Proposal 2: The notification message includes a new indication Type (i.e. relayUE-RelayReselection).

Proposal 3: Upon receiving NotificationMessageSidelink, the RRC_CONNECTED first relay UE will perform the RRC Reestablishment and then send NotificationMessageSidelink to its connected remote UE indicating the occurred RRC Reestablishment event.

Proposal 4: Upon receiving NotificationMessageSidelink, if the RRC_IDLE/INACTIVE first relay UE performs the PC5 link release and relay reselection, then it will send NotificationMessageSidelink to its connected remote UE indicating relay reselection.

Proposal 5: Upon receiving NotificationMessageSidelink, if the RRC_IDLE/INACTIVE first relay UE maintains the PC5 link, it will send NotificationMessageSidelink to its connected remote UE indicating cell reselection.

- References

- Draft report of 3GPP TSG RAN WG2 #129bis, RAN2 chair (interDigtal), Wuhan, China, 7 - 11 April, 2025.