3GPP TSG-RAN WG2 Meeting #130 	R2-2503963

St.Julians, Malta, May, 19th – 23rd, 2025

Agenda Item:	8.2.2

Source:	Spreadtrum, UNISOC

Title:	Discussion on A-IoT paging

Document for:	Discussion and Decision

Introduction

In RAN2 #129-bis meeting, the following agreements were achieved for AIoT paging aspects, with yellow highlights added to the FFSs [1].

Agreements

FFS which solution if any for device behavior if it gets a new service request while one procedure is still ongoing or leave it to implementation.

RAN2 aims to design Rel-19 AIoT R2D messages extensible to accommodate devices and features of future release.

Agreements on CFRA

Introduce an explicit 1 bit indication to indicate whether it is CFRA or CBRA per paging message.

Agreements on paging ID length

- A field indicating Paging ID length information is always included together with the paging ID field in the A-IoT paging message, except the case where no ID is included in the A-IoT paging message.

- The number of bits required for paging ID length field should be as small as possible.  This would require the number of different Paging ID lengths to be small.

- Send an LS to SA2 to take this into account for their design.

In this contribution, we analyze the interleaved services, transaction ID and paging message contents and provide our opinions.

Discussion

2.1 Interleaved Services from Multiple readers

After RAN2#129bis meeting, the device behaviour if it gets a new service request while one procedure is still ongoing was discussed. Two options were considered:

Option a: ignore all new requests and R2D messages addressed to itself but not associated with the ongoing procedure;

Option b: terminate the ongoing procedure and respond to the latest request;

It was agreed that RAN2 aims to design Rel-19 AIoT R2D messages extensible to accommodate devices and features of future release. However what device behaviour still was left as FFS.

For option a, the device needs to determine whether the ongoing procedure is finished. Some potential solutions to address the above were raised during online discussion in last meeting. Such as:

-	Timer based autonomous release by the device.

o	Concerns: supporting timer for such low complexity devices increases complexity. Hard to find exact useful value of the timer.

-	Explicit release (RRC-release-like) message by the Reader to all the devices.

o	Concerns: new message needed to be defined; may not be possible to indicate release to specific devices and release-all-devices-within-reader-range may not be desirable. This new message may also be lost.

-	Turn off CW source and force the devices to drain the energy.

o	Concerns: this assumes Reader has control over the CW source. Forcing the device to lose all of the volatile memory and trigger power-cycle may not be ideal. This would impact all the devices under coverage. Pure NW implementation may not be sufficient.

-	Using the trigger message as an implicit end indicator.

o	Concerns: For inventory and command procedure, if the command procedure is performed in different access slot or even different paging round, the reader needs to indicate command type in the paging message. This has been ruled out during the first WI meeting.

From the above analysis, all the solutions of options cannot solve the problem completely. And another concern is that R2D messages need to include some ID to identify the session, which cause significant overhead.

In the last RAN2 meeting, the following agreements were reached. The device is expected to perform only one procedure at a time. Therefore, we need to discuss the device behaviour when multiple requests are received in parallel from different readers. This is because the same reader will not trigger a new service request if there is an ongoing procedure.

For option b, in R19, the coverage of reader is very limit. The interleaving service from multiple-reader will not happen frequently. And even there are interleaving service from multiple-reader, the resource utilization efficiency is reader low for both option a and option b. So reader can not trigger the session in interleaving way if the session are different for the same device. The coordination between readers is left to implementation.

Proposal 1: If a device receives a new service request while a procedure is still ongoing, the device will terminate the ongoing procedure and respond to the latest request.

2.2 Transaction ID

- The “transaction ID” can be generated by reader based on CN corelation ID.  FFS how reader will generate “transaction ID”.  FFS the size of transaction ID

- 1 bit solution is excluded.   FFS the size.  Aim to have a reasonable size.

For transaction ID, we agreed that the transaction ID can be generated by the reader based on CN correlation ID. But how to generate the transaction ID is FFS. In our opinion, Somehow network needs to guarantee the transaction ID different if the correlation ID is different. Regarding the multi-reader scenario for the proximity service, the device is expected to respond to multiple readers for the same service. As the agreement enlightened, the CN can assign distinct correlation IDs to different readers. Therefore these readers need to generate different transaction IDs. There are several ways to generate the transaction ID. For example, 1) reader can truncate correlation ID as transaction ID, e.g., take the lowest few bits of correlation ID; 2) reader randomly select X bits from the correlation ID to generate transaction ID.

We can rely on predefined rules or reader implementation to generate the transaction ID. But if we just rely on reader implementation to generate the transaction ID without the coordination between the readers, it is difficult to ensure the transaction ID between multiple readers is unique. Because the different readers may use different rules to generate transaction ID. Therefore, it is preferable to specify a simple generation rule to generate the transaction ID. But we can wait for the RAN3/SA2 progress.

For the size of transaction ID, considering to avoid wraparound and the maximum number of neighbouring readers covering an overlapping area. 3 bits can be reasonable size.

Proposal 2: Specify a simple rule to generate the transaction ID.

Proposal 3: For the size of transaction ID, 3 bits can be reasonable size.

2.3 Paging message content

- Identifier of A-IoT device(s)

In last meeting, RAN2 agreed that length field of paging ID is introduced in AIoT paging message. And for mask/filter/group ID, it also needs to wait for SA2 progress. Therefore, the paging message include a Length Field to indicate the length of paging identifier. The number of bits required for paging ID length can be determined until SA2 determine the device ID and filter/mask’s length.

Agreements on paging ID length

- A field indicating Paging ID length information is always included together with the paging ID field in the A-IoT paging message, except the case where no ID is included in the A-IoT paging message.

- The number of bits required for paging ID length field should be as small as possible.  This would require the number of different Paging ID lengths to be small.

- Send an LS to SA2 to take this into account for their design.

Proposal 4: The number of bits required for paging ID length field can wait for SA2 progress.

- Number of access occasions field

In WI previous meeting, RAN2 has agreed that AIoT paging message can include a number of Msg1 resources. But there is no conclusion on how to indicate the number of Msg1 resource. There are the two options:

Option 1: A-IoT paging message includes a parameter Q (like RFID mechanism, the total number of access occasion set is 2Q), such as 4 bits, value range as [1, 215].

Option 2: A parameter N indicating the total number of access occasion set, such as 15 bits, value range as [1, 215].

Agreements

- The A-IoT paging message can include a number of msg1 resources.

The only difference between the two parameter is that the indication methods are different and the occupied bits are different. The parameter Q can only indicate available access occasion set resources in multiples of 2 by using fewer bits. So the option 1 Option 1 cannot accurately indicate the number of resources. And option 2 can indicate any number of slots but takes up more bits.

Proposal 5: RAN2 supports that A-IoT paging message includes a parameter N indicating the total number of access occasion set, such as 15 bits, value range as [1, 215].

- Paging content for CFRA

In Rel-19, the CFRA procedure only focus on one paging identifier for a single device. So for the CFRA procedure, subsequent paging is not needed. The situation is the same for multiple device IDs with CFRA. If some of the devices fail to complete the inventory or command successfully, the Reader can initiate a new CFRA procedure to perform the inventory or command. And for CFRA procedure, there is no subsequent paging, the transaction ID is also not needed in AIoT paging message.

Proposal 6: The transaction ID in AIoT paging message is not needed for CFRA procedure.

Conclusions

In this contribution, we discuss the Rel-19 paging procedure and message content. And the proposals are given as follows:

Proposal 1: If a device receives a new service request while a procedure is still ongoing, the device will terminate the ongoing procedure and respond to the latest request.

Proposal 2: Specify a simple rule to generate the transaction ID.

Proposal 3: For the size of transaction ID, 3 bits can be reasonable size.

Proposal 4: The number of bits required for paging ID length field can wait for SA2 progress.

Proposal 5: RAN2 supports that A-IoT paging message includes a parameter N indicating the total number of access occasion set, such as 15 bits, value range as [1, 215].

Proposal 6: The transaction ID in AIoT paging message is not needed for CFRA procedure.

References

[1] RAN2 #129b_ChairNotes_25-04-11 final.