3GPP TSG-RAN WG2 Meeting #130						        											R2-2503969

St.Julians, Malta, May 19 – 23, 2025

Source: 			ZTE Corporation, Sanechips

Title: 	Open issues for A-IoT random access

Agenda item:	8.2.3

Document for: Discussion and Decision

# Introduction

In this contribution, we discuss the remaining open issues for the A-IoT random access procedure and propose a way forward for these.

# Open issues for CBRA

## 2.1 Procedure for MSG1 transmission

At RAN2#129-bis, the following agreements were made:

Agreements on new R2D message

- A new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling.   The R2D message indicates the start of a set of MSG1 resources that were configured in paging message.

- Assumption: The R2D message does not include slot number/count down number.

Based on the above, it can be seen that the configuration of the MSG1 resources within a paging frame is fixed. The R2D trigger message simply indicates the start of the MSG1 resources. Furthermore, the Access Occasions (AOs) triggered by a given R2D trigger message may span time domain (X) and frequency domain (Y) and RAN1 has agreed that X can be 1 or 2. This is depicted in Figure 1 below.

Figure 1: MSG1 configuration options

With the above procedure in mind, we then need to agree how the device selects a given access occasion (), within a paging frame.

The total number of AOs within a paging frame is given by: #AOs = X×Y×Q, where:

- Q is the total number of MSG1 slots within a paging frame (configurable – RAN2 to determine the max value)

- X is the number of time domain slots within a MSG1 slot (X can be either 1 or 2 as agreed by RAN1),

- Y is the number of frequency domain slots within a MSG1 slot and (Y is configurable – up to RAN1)

The reader will configure the values of Q, X and Y such that the total number of AOs is suitable for a given total number of contending devices.

From the device perspective, there are two options to select the AO.

Option 1: The device selects a random integer (RAND) between 1 and X×Y×Q, then the device determines the number of R2D triggers it has to countdown before the selected MSG1 slot arrives:

- In this option, the device has to wait for a total number (Qd) of R2D trigger messages that is given by: Qd=ceilRANDX×Y

- Then, the device also needs to determine the actual resource within the MSG1 slot as:  AO=modRAND,  X×Y

Option 2: The device first selects a random MSG1 slot number (between 1 and Q) and then within this slot, the device then selects a random AO (between 1 and X×Y)

Firstly, since X and Y remains constant throughout the paging frame, option 1 and option 2 will result in the same collision probability for a given access occasion. So, we can then select the exact device procedure based on other considerations. Option 1 seems to require the device to calculate the access occasion based on the ceil and modulo operations as depicted above and this may add to the complexity of the devices. Thus, we propose that option 2 is adopted.

Proposal 1: A MSG1 slot consists of X time domain (X = 1 or 2) and Y frequency domain AOs

Proposal 2: The paging message indicates the total number of MSG1 slots (Q) and the number of AOs in time domain (X) and number of AOs in frequency domain (Y) within each MSG1 slot

Proposal 3: The device selects a random number Qd: between 1 and Q

Proposal 4: The device decrements Qd value by 1 for each R2D trigger message received by the device and when Qd = 0, the device selects the MSG1 slot for transmission of Random ID

Proposal 5: The device transmits the Random ID in a randomly chosen AO within the selected MSG1 slot

## 2.2 MSG2 transmission window

It was agreed that MSG2 echos the Random ID included in MSG1. Furthermore, it was also agreed that MSG2 may also be repeated by the reader to make the device retransmit MSG3. However, it is not clear when the MSG2 may be repeated. As such, it is possible that the reader may repeat the MSG2 transmission or its repetition within the whole paging frame. Since there is no ACK for MSG3 (only NACK was agreed), whenever the device receives MSG2, it will compare the MSG2 contents with its own Random ID echoed in the MSG2 and the device will retransmit MSG3. It should be noted that even if the MSG3 was transmitted, the device will still have to retransmit MSG3 since it cannot differentiate between the cases where MSG2 is lost and MSG3 is lost as there is no explicit ACK. This causes problems as the collision probability then increases progressively towards the end of the paging frame. This is because the probability of two devices having picked the same RN16 value will increase as there will be more devices that would have transmitted MSG1 in the paging frame towards the end. The issue may be severe if the paging frame is very long (e.g. very large Q value). This issue is depicted in Figure 2 below.

Figure 2: High RN collision probability towards the end of the paging round if MSG2 can be retransmitted any time before next paging message

Observation 1: The collision probability for MSG3 will increase progressively towards the end of the paging frame if MSG2 transmission/retransmission is allowed at any point before the next paging message.

There are two ways to resolve this issue:

Option 1: time window for MSG2 reception:

- This means that the reader will transmit/retransmit the MSG2 within a predefined time window (similar to RAR).

- This option needs the devices to support a timer-based mechanism to monitor the MSG2.

- If these devices cannot support running a timer, then we need some other means for this.

Option 2: MSG2 transmission and any retransmission of MSG2 happens before the subsequent trigger message from the reader

- This option is similar to the option A in [1] (i.e. there is a pre-defined end point for MSG2 reception).

- This means that the reader has to complete the transmission and any retransmissions of MSG2 before the subsequent R2D trigger.

It should be noted that this problem would not exist if option A is adopted for the overall procedure – see [1] (since anyway, the reader would then have to complete the whole procedure before the subsequent R2D trigger). However, for option B, RAN2 should discuss this issue and agree one of the above options.

Proposal 6: RAN2 should discuss and agree one of the following options for MSG2, if option B is adopted for the overall MAC procedure:

Option 1: MSG2 transmission and any retransmission of MSG2 happens within a predefined time window or

Option 2: MSG2 transmission and any retransmission of MSG2 happens before the subsequent trigger message from the reader

# Open issues for CFRA

## 3.1 Transaction ID for CFRA

One open issue is whether the paging message for CFRA includes transaction ID or not.

Firstly, if transaction ID is not included in the paging message for CFRA, this would require a paging message format where the transaction ID is optional. This also needs the UE procedure to be defined with and without transaction ID in the paging message. This is not desirable. Furthermore, it should be noted that when the reader initiates CFRA for a subset of devices for a given service (correlation ID), there may be other UEs for which CBRA procedure may be initiated. If transaction ID is not included in the paging message for CFRA, then these devices (that already responded to CFRA for a given service) will respond unnecessarily again for the same service (i.e. same transaction ID) even after successfully completing the procedure using CFRA. Thus, the gains of CFRA will be lost in this case. Furthermore, having a transaction ID in paging message enables the device to determine whether to ignore the paging or not depending on any other ongoing procedure. Without the transaction ID, the device behaviour is a bit unclear. Given these considerations, it is proposed to support inclusion of transaction ID also for CFRA.

Proposal 7: transaction ID is included in the paging message for CFRA

## 3.2 NACK for CFRA

For CBRA, NACK Based retransmission is adopted for the inventory report message (i.e. MSG3). However, in case of CFRA, there is no NACK for inventory report. This means the device will transmit the inventory report and then waits for next paging message. However, if the inventory report message is not received successfully, then the reader has to send another paging message to the device. The size of the paging message is clearly much larger than the NACK. So, the advantage of having a different retransmission for inventory report for CFRA which has slightly more overhead than the CBRA mechanism for MSG3 is a bit unclear. Based on this, it is proposed to support NACK based retransmission for inventory report also for CFRA. In this case, the reader can provide a D2R grant along with the NACK indication to facilitate the retransmission of the inventory report message.

Proposal 8: NACK based retransmission for inventory report is also supported for CFRA

## 3.3 Multi-device scenario for CFRA

During RAN2#129bis meeting, it was agreed that RN16 is not included in the first D2R message in CFRA procedure, and AS ID is the only ID needed for addressing the device in R2D command message. In other words, the first R2D command message is used to assign AS ID for device, and there is no other ID (e.g. device ID) to identify the target device. This agreement was made under the assumption that for CFRA no multiple devices are performing the procedures with the given reader. And it is FFS if we can assume or need to support multiple device scenario.

Agreements on CFRA

RN16 is not included in the first D2R message in the CFRA procedure.  AS ID is the only ID needed for addressing the device in R2D command message assuming for CFRA no multiple devices are performing the procedures with the given reader.   FFS if we can assume or need to support multiple device scenario.

In our view, assuming only AS ID is included in R2D command for addressing the device, that means it needs to be avoided that there are multiple devices expecting the R2D command (with AS ID assignment) after sending inventory response. Otherwise, the devices expecting R2D command cannot differentiate whether the command and AS ID in the R2D command is for itself or not. In other words, for inventory and command case, the reader shall finish the whole inventory and command procedure for one device, and then it can proceed to the next device, start the inventory and command procedure for another device, which is similar as command option A-1 in [1].

However, if the reader adopts option A-2 i.e. complete the inventory first and then proceed to command later [1], then a different procedure is needed. In this case, the reader may assign the AS ID immediately after it receives inventory response if the reader determines that there is follow on command for this device (RAN3 already agreed that there will be a follow-on command indicator from AIOTF from RAN node in the inventory request message). With this approach, the reader can proceed to perform another procedure for another device whilst waiting for the command to arrive.

Proposal 9: If multiple device scenario for CFRA is to be supported for command use case, the reader may indicate follow-on command in the paging message and send the R2D message with ASID immediately after receiving the inventory response

## 3.4 R2D trigger for CFRA

It was agreed that a new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources. This discussion is mainly focused on CBRA case and we haven’t discussed whether R2D trigger message is used to determine the MSG1 resources for CFRA case. In our view, the R2D trigger message can be used for CFRA case as well. In CBRA case, the trigger message is used to determine the start time of each MSG1 slot and also used the counting down of MSG1 slots at the device, while for CFRA, the  trigger message is used only to determine the start time of MSG1 resource.

The benefit is that it simplifies the device behaviour, i.e. the device sends MSG1 after a R2D trigger message after paging regardless of CBRA or CFRA. Moreover, another advantage is that RAN1 doesn't need to define two different behaviours/or timing offset for MSG1 after paging and R2D trigger message respectively, especially considering that the time required to send MSG1 after paging and R2D trigger message are not likely to be the same since the device needs to process the paging message including the device ID and mask and determine whether it is paged or not and the response time to paging may hence be larger than the response time for an R2D trigger message.

Agreements on R2D trigger message A new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling. The R2D message indicates the start of a set of MSG1 resources that were configured in paging message. Assumption: The R2D message does not include slot number/count down number.

Proposal 10: R2D trigger message is used to determine the starting point for MSG1 resource for CFRA.

# Conclusion

The following observations and proposals are made:

MSG1 transmission details

Proposal 1 (Issue 2-1): A MSG1 slot consists of X time domain (X = 1 or 2) and Y frequency domain AOs

Proposal 2 (Issue 2-1): The paging message indicates the total number of MSG1 slots (Q) and the number of AOs in time domain (X) and number of AOs in frequency domain (Y) within each MSG1 slot

Proposal 3 (Issue 2-1): The device selects a random number Qd: between 1 and Q

Proposal 4 (Issue 2-1): The device decrements Qd value by 1 for each R2D trigger message received by the device and when Qd = 0, the device selects the MSG1 slot for transmission of Random ID

Proposal 5 (Issue 2-1): The device transmits the Random ID in a randomly chosen AO within the selected MSG1 slot

MSG2 aspects

Observation 1: The collision probability for MSG3 will increase progressively towards the end of the paging frame if MSG2 transmission/retransmission is allowed at any point before the next paging message.

Proposal 6 (Issue 2-4, 2-5): RAN2 should discuss and agree one of the following options for MSG2, if option B is adopted for the overall MAC procedure:

Option 1: MSG2 transmission and any retransmission of MSG2 happens within a predefined time window or

Option 2: MSG2 transmission and any retransmission of MSG2 happens before the subsequent trigger message from the reader

CFRA aspects

Proposal 7 (Issue 2-8): transaction ID is included in the paging message for CFRA

Proposal 8 (Issue 2-8, 2-10): NACK based retransmission for inventory report is also supported for CFRA

Proposal 9 (Issue 2-9): If multiple device scenario for CFRA is to be supported for command use case, the reader may indicate follow-on command in the paging message and send the R2D message with ASID immediately after receiving the inventory response

Proposal 10 (Issue 2-2): R2D trigger message is used to determine the starting point for MSG1 resource for CFRA.

# References

- R2-2504112, Details of option A for overall A-IoT MAC procedure, ZTE Corporation, Sanechips, Continental Automotive, Tejas Networks, Nokia, Ofinno, Samsung, Fujitsu, Kyocera, Sony, ETRI, III, Ericsson 	RAN2#130