3GPP TSG-RAN2#130									         R2-2503973

St Julian, Malta, 19th – 23rd May, 2025

Source: 			ZTE Corporation, Sanechips

Title: 	DSR enhancements for XR

Agenda item:	8.7.4.2

Document for: 	Discussion and Decision

# Introduction

Rel-19 XR WI[1] has an objective for RAN2 to specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG, to enable high XR capacity while meeting delay requirements/avoiding too late PDUs.

In this contribution we will further investigate the remaining issues related to DSR enhancements per this objective.

# Discussion

In RAN2#129bis meeting[2], there is a following:

- During DSR data volume calculation the remaining time of retransmitted is not considered, i.e. it is always put either in the smallest configured or smallest reported threshold. FFS which one.

We understand the smallest configured threshold is the smallest reporting threshold of the configured reporting thresholds, and the smallest reported threshold is the smallest reporting threshold in the Multiple Entry DSR MAC CE with Buffer Size larger than zero. The differentiation between putting retransmitted data in the smallest configured and smallest reported threshold is only in the case that there is no new data with remaining time below some lower threshold, and only data volume in included in some higher reported threshold. In which case, if the remaining time of retransmitted data is less than the remaining time of the smallest reported threshold and the retransmitted data is put in the smallest reported threshold, it may lead the NW to schedule the retransmitted data based on larger delay requirement than remaining time, which may lead the retransmitted data more easily outdated. Considering that the DSR reporting is to improve the delay critical data transmission, if is more suitable to put the retransmitted data in the smallest configured threshold.

Observation 1: If the remaining time of retransmitted data is put in the smallest reported threshold, it may lead the retransmitted data to be more easily outdated.

Based on the current Multiple Entry DSR MAC CE, even there is no new data with remaining time less than the smallest configured threshold, the smallest configured threshold is also included in the Multiple Entry DSR MAC CE with Buffer Size set to zero and the EXT0,i field is set to 1, thus, including the retransmitted data in the smallest configured threshold will not increase the size of Multiple Entry DSR MAC CE.

Observation 2: Including the retransmitted data in the smallest configured threshold will not increase the load of Multiple Entry DSR MAC CE.

Proposal 1: During DSR data volume calculation, the retransmitted data is put in the data volume related to the smallest configured threshold.

In RAN2#129bis meeting[2], there is a following agreement:

- To avoid the case that there is no delay-reporting data for DSR, at least one configured reporting threshold should be no lower than the DSR triggering threshold. This means we do not have to address “empty DSR” issue, but it is FFS whether we need to capture this restriction in specifications

Considering that the UE behaviour is unclear if the DSR triggering threshold is larger than the configured reporting threshold, the restriction should be captured in field description of DSR triggering threshold.

Proposal 2: In field description of DSR triggering threshold, capture the restriction that “at least one configured reporting threshold should be no lower than the DSR triggering threshold”.

Considering that the data volume with remaining time larger than the largest reporting threshold can be reported by BSR, it is not necessary to report data volume with remaining time larger than the largest reporting threshold in the DSR.

Proposal 3: UE shall not report data volume with remaining time larger than the largest reporting threshold in the DSR.

In RAN2#129 meeting, there is an “FFS New DSR MAC CE will (always) be used when at least one LCG is configured with multiple thresholds”. Considering that it has been agreed in RAN2#129 meeting that “One extension bit (e.g. by redefining the reserved R bit) can be used to indicate whether a further pair of remaining time and buffer size information is present for the associated LCG in the enhanced DSR MAC CE”, based on this agreement, the new DSR MAC CE is compatible with the legacy DSR MAC CE, e.g. the UE can identify whether the DSR MAC CE is a new DSR MAC CE or legacy DSR MAC CE based on the value of the reserved bit if the UE supports the new DSR MAC CE and at least one LCG is configured with multiple thresholds, new LC-ID is not necessary for the new DSR MAC CE.

Observation 3: The new DSR MAC CE is compatible with the legacy DSR MAC CE. The UE can identify whether the DSR MAC CE is a new DSR MAC CE or legacy DSR MAC CE based on the value of the reserved bit if the UE supports the new DSR MAC CE, no new LC-ID is necessary for the new DSR MAC CE.

Proposal 4: New DSR MAC CE will (always) be used when at least one LCG is configured with multiple thresholds, and not define new LC-ID for the new DSR MAC CE.

In the [POST129bis][502][XR] MAC running CR and open issues (Qualcomm), there is the following option issue related to DSR enhancement:

- DSR cancelation in DC configuration

In the rel-18 DSR, the same issue has been discussed and it has been agreed that the DSR MAC CE is not included and/or canceled as following[3]:

5.4.9 Delay status reporting ... A MAC PDU shall contain at most one DSR MAC CE. A MAC PDU shall not contain a DSR MAC CE if it includes all PDCP SDUs associated with all the pending DSRs. After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, when all the PDCP SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR (as described in the clause 6.1.3.72), or when a MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR.

The same rules can be reused for rel-19 DSR, e.g. the pending DSR is canceled in the following case:

- when all the PDCP SDUs associated with the DSR have been discarded. This case is clear for DC configuration, e.g. as long as all the PDCP SDUs associated with the DSR have been discarded by PDCP entity.

- when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR. This case is clear also for DC configuration, e.g. as long as a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR is sent, the pending DSR is canceled. .

- when a MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR. This case may need for some clarification for DC configuration, e.g. the DSR should be canceled if all the PDCP SDUs associated with the DSR have been transmitted, whatever they are transmitted in one MAC PDU or multiple MAC PDUs.

Observation 4: only the condition “when a MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR” in the DSR cancellation cases need to be clarified for DC configuration.

Proposal 5: The pending DSR is canceled in the following cases:

- when all the PDCP SDUs associated with the DSR have been discarded.

- when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR.

- when all the PDCP SDUs associated with the DSR have been transmitted.

# Conclusion and proposals

The following observations/proposals are made:

Observation 1: If the remaining time of retransmitted data is put in the smallest reported threshold, it may lead the retransmitted data more easily outdated.

Observation 2: Including the retransmitted data in the smallest configured threshold will not increase the load of Multiple Entry DSR MAC CE.

Proposal 1(RLC-5): During DSR data volume calculation, the retransmitted data is put in the data volume related to the smallest configured threshold.

Proposal 2(RRC-7): In field description of DSR triggering threshold, capture the restriction that “at least one configured reporting threshold should be no lower than the DSR triggering threshold”.

Proposal 3(RRC-7): UE shall not report data volume with remaining time larger than the largest reporting threshold in the DSR.

Observation 3: The new DSR MAC CE is compatible with the legacy DSR MAC CE. The UE can identify whether the DSR MAC CE is a new DSR MAC CE or legacy DSR MAC CE based on the value of the reserved bit if the UE supports the new DSR MAC CE, no new LC-ID is necessary for the new DSR MAC CE.

Proposal 4(RRC-7): New DSR MAC CE will (always) be used when at least one LCG is configured with multiple thresholds, and not define new LC-ID for the new DSR MAC CE.

Observation 4: only the condition “when a MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR” in the DSR cancellation cases need to be clarified for DC configuration.

Proposal 5(MAC-03): The pending DSR is canceled in the following cases:

- when all the PDCP SDUs associated with the DSR have been discarded.

- when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR.

- when all the PDCP SDUs associated with the DSR have been transmitted.

# References

- RP-243318, Revised WID on XR (eXtended Reality) for NR Phase 3, RAN#106

- Draft_RAN2_129bis_Meeting_Report_v2

- 3GPP TS 38.321 V18.4.0 (2024-12), NR;Medium Access Control (MAC) protocol specification