3GPP TSG-RAN WG2 Meeting #130                                  R2-2503984

St.Julians, Malta, May 19th – 23rd, 2025

Agenda item:		8.3.2

Source:				Samsung

Title:					Discussion on configuration and report of inference input for NW-side model

Document for:		Discussion & Decision

1	Introduction

For study on the general spec. impact for AI/ML for mobility, the following table was agreed in RAN2#129bis.

Things to continue study for specification and RAN2 impact

|  | UE sided model | Network sided model |
| --- | --- | --- |
| Measurement event prediction | Yes, | No (up to NW implementation) |
| Temporal domain case A | Yes, all sub cases | Yes |
| Temporal domain case B | Yes , all sub cases | Yes |
| Frequency domain | Yes , all sub cases | Yes |
| Spatial domain | Pending | Pending |
| L3 beam level measurement prediction | Yes, | Yes |

In this paper, we would like to discuss the configuration and report of inference input for NW-side model for the use cases agreed as in the table above (i.e., Temporal domain Case A & B, Frequency domain, L3 beam level measurement prediction).

2	Discussion

For NW-side model inference, UE can provide the input data for inference at NW-side. Then, the NW can perform the inference with the input data and use the outcome of the inference for HO. How to use the outcome for HO is totally up to NW implementation. Therefore, for the spec. impact, we can focus on the configuration and report of inference input data between UE and NW.

Also, at the last RAN2#129bis, RAN2 agreed the inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework. In our view, the existing RRM measurement configuration consisting of MeasObject/MeasId/ReportConfig can be easily extended/enhanced to support the use cases of AI for mobility. Therefore, in this part, we propose some potential enhancement based on the existing RRM measurement configuration and report.

Common part:

For the common part in the use cases (i.e., frequency/temporal domain RRM measurement prediction, measurement event prediction), the UE should provide the input data for NW every time NW performs the inference. Even though it is unclear how often NW will perform the inference, it is evident that there should be the concern on the signalling overhead from the input data report. For example, for temporal domain Case A, the UE should report the input data (i.e., the measurement results for the multiple time instances within OW) which is quite a lot compared to the legacy MR including only one latest measurement result. Moreover, when we consider the L3 beam level prediction, the signaling overhead will be further increased in proportion to the number of beams. However, considering that the motivation is to trigger the HO in proactive manner, NW does not need to perform the inference continuously. Instead, NW can perform the inference only when HO is likely to happen (e.g., when UE moves fast or moves to cell edge area), which can allow UE not to report the input data continuously/periodically. Thus, we need to study ways to reduce the signalling overhead (e.g., event-triggered reporting, adaptive reporting based on mobility status, …).

Proposal. 1: For NW-side model inference, RAN2 should consider the ways to reduce the signalling overhead for the report of inference input (e.g., event-triggered reporting, adaptive reporting based on mobility status, …).

Temporal domain Case A:

For temporal domain Case A, the input data would be the historical L3 Cell/beam-level measurement results of multiple time instances within OW (Observation Window). However, with the legacy measurement report, the UE can report only one latest measurement result for each cell. Therefore, there should be some enhancement on measurement configuration and report to allow UE to log the historical measurement results within OW and report them to network. Regarding the need of beam-level measurement result for input data, RAN2 already observe that sub case 2 using cell-level input data shows better accuracy than the other sub cases using beam-level input. Also, beam-level input data will increase the signalling overhead for inference input report for NW-side model. Thus, in our view, the beam-level inference input report is not necessary.

Proposal. 2: For temporal domain Case A, UE can log the L3 Cell-level measurement results of N recent time instances and report them to NW for inference input report of NW-side model. The N (i.e., OW length) can be configured by NW.

Temporal domain Case B:

For temporal domain Case B, the motivation is to reduce UE’s measurement effort (for saving energy). Specifically, with 50% reduction rate, UE can skip measurement once out of every two instances and predict the measurement values at the skipped time instance. In our evaluation in FR1, we assume that UE measures SSB every 40 msec as baseline and thus for Case B with MRRT 50%, UE performs the measurement every 80 msec. Then, we evaluate HO performance with the UE-side model case where the UE initiates the event-triggered MR based on the actual and predicted measurement results. The evaluation result shows that the performance degradation is marginal even with the measurement reduction at UE-side. As we can see from the evaluation scenario, for UE-side model case, the event-triggered measurement reporting can be the baseline, which means the UE can report the measurement report only when it detects some event.

Observation. 1: For temporal domain Case B, the motivation is to reduce UE’s measurement effort for energy saving.

Observation. 2: For temporal domain Case B, for UE-side model case, the event-triggered measurement reporting can be used. I.e., UE can report the measurement report only when it detects some event.

On the other hands, if we consider the case with NW-side model, the UE should report the measurement results every 80 msec continuously to provide the inference input data for NW. Thus, for NW-side model case, the event-triggered reporting can not be used and the periodical measurement reporting is essential. Considering that Case B is for goal 1(i.e., UE’s measurement effort reduction for energy saving) the frequent measurement reporting from UE with very small interval does not make sense. Therefore, for the Case B prediction, the use of NW-side model can be excluded.

Observation. 3: For temporal domain Case B, for NW-side model case, only the periodical measurement reporting can be used. I.e., UE should periodically report the measurement report with very short interval (e.g., 80 ms), which can increase the energy consumption of UE.

Based on the discussion above, we would like to propose the following.

Proposal. 3: For temporal domain Case B, RAN2 exclude the NW-side model case.

Frequency domain:

For frequency domain prediction, the input data would be the L3 Cell/beam-level measurement results at a certain time instance for input cell(s). With the legacy measurement report, the UE can already report one latest measurement result for each cell/beam. Therefore, we do not see any need of enhancement on the legacy measurement reporting to support the inference input report for frequency domain prediction for NW-side model.

Proposal. 4: For frequency domain prediction, UE can report one latest measurement result for each cell/beam to NW as in the legacy for inference input report of NW-side model. No further enhancement is needed.

L3 beam-level prediction:

For L3 beam-level prediction, the input data would be the historical L1/L3 beam-level measurement results of multiple time instances within OW. However, with the legacy measurement report, the UE can report only one latest L3 beam-level measurement result for each beam. Therefore, there should be some enhancement to allow UE to log the historical L1/L3 beam-level measurement results within OW and report them to network. By the way, that kind of beam-level logging and reporting will increase the signalling overhead quite a lot. Thus, RAN2 can study the way to reduce the signalling overhead especially for the beam-level prediction.

Proposal. 5: For L3 beam-level prediction, UE can log the L1/L3 Beam-level measurement results of N recent time instances and report them to NW for inference input report of NW-side model. The N (i.e., OW length) can be configured by NW.

Observation. 4: L1/L3 Beam-level logging and reporting will increase the signalling overhead.

3	Conclusion

Based on the above, RAN2 is requested to discuss and agree on the following proposals:

Common part:

Proposal. 1: For NW-side model inference, RAN2 should consider the ways to reduce the signalling overhead for the report of inference input (e.g., event-triggered reporting, adaptive reporting based on mobility status, …).

Temporal domain Case A:

Proposal. 2: For temporal domain Case A, UE can log the L3 Cell-level measurement results of N recent time instances and report them to NW for inference input report of NW-side model. The N (i.e., OW length) can be configured by NW.

Temporal domain Case B:

Observation. 1: For temporal domain Case B, the motivation is to reduce UE’s measurement effort for energy saving.

Observation. 2: For temporal domain Case B, for UE-side model case, the event-triggered measurement reporting can be used. I.e., UE can report the measurement report only when it detects some event.

Observation. 3: For temporal domain Case B, for NW-side model case, only the periodical measurement reporting can be used. I.e., UE should periodically report the measurement report with very short interval (e.g., 80 ms), which can increase the energy consumption of UE.

Proposal. 3: For temporal domain Case B, RAN2 exclude the NW-side model case.

Frequency domain:

Proposal. 4: For frequency domain prediction, UE can report one latest measurement result for each cell/beam to NW as in the legacy for inference input report of NW-side model. No further enhancement is needed.

L3 beam-level prediction:

Proposal. 5: For L3 beam-level prediction, UE can log the L1/L3 Beam-level measurement results of N recent time instances and report them to NW for inference input report of NW-side model. The N (i.e., OW length) can be configured by NW.

Observation. 4: L1/L3 Beam-level logging and reporting will increase the signalling overhead.