3GPP TSG RAN WG2 #130	R2-2503986

St. Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.8.5
Source:	Toyota ITC
Title:	Regenerative payload for NTN for NR Ph3

Document for:	Discussion and Decision

# 1	Introduction

The Work Item on NR for NTN Ph3 was initially approved in December 2023. The latest approved revision was in RAN#106 in [1]. It includes the objective on support of regenerative payload as follows:

Support of regenerative payload [RAN3, RAN2, RAN4] - Specify the support of gNB on board in TS 38.300 - Specify, if needed, any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. [RAN3] Note: if any additional necessary stage-3 specifications impact for e.g. NGAP is identified, RAN3 will handle it.

For the regenerative payload objective, the following agreement was made in RAN2#129bis [2]:

Agreements:

1.	Specific configurations of common TA and Kmac in regenerative architecture are not captured in the specs.

In this document, we discuss remaining issues for this objective.

# 2	Discussion

## 2.1	Satellite switch with resynch

With the Rel-18 transparent architecture, satellite switch with resync is supported because the same PCI may apply to the two satellites as they may belong to the same cell. However, with the Rel-19 regenerative payload scenario, the two satellites belong to two different cells.

This means that there is likely to have SA3 impacts e.g. in terms of changing security keys. Depending on the impact potentially required by SA3, it is therefore possible that additional signalling would be required for the procedure. Having additional signalling for the satellite switch with resynch procedure would mean that the benefit of satellite switch with resynch compared to L3 handover may decrease.

Observation 1: In a regenerative payload scenario, for satellite switch with resynch, the two satellites belong to different cells. Due to security reasons, this may lead to additional signalling required for the satellite switch with resynch procedure. It is possible that this may reduce the usefulness of the satellite switch with resynch procedure compared to the L3 handover procedure.

Given the above, RAN2 should not pursue the work on satellite switch with resynch for regenerative payload until SA3 has clarified the impact required for the procedure, and RAN2 concluded in turn that there are still benefits for this procedure. While RAN2#126 recorded the following agreement [4]:

Companies interested to support satellite switch with resync with regenerative payload are invited to bring this discussion to the RAN plenary.

There is a need to state a decision about satellite switch with resync in the Rel-19.

Proposal 1: RAN2 should not pursue the work on satellite switch with resynch for regenerative payload until SA3 has clarified the impact required for the procedure and RAN2 has concluded in turn that there are still benefits for this procedure.

    - Network verified UE positioning

The UE location verification procedure may be initiated by the network and assisted by a positioning method.

NGSO satellites have limited in-service duration. Consequently, the positioning procedure may be interrupted when the satellite is involved in the positioning procedure.

For the regenerative scenario, it is important to guarantee that the UE can receive the positioning signal from the involved TRP(s). This was stated in the contribution [3] from the RAN2-129 meeting. One reasoning is that for the regenerative scenario, different gNBs are involved as TRPs in the positioning procedure. Therefore, longer interruptions may happen and inter-gNB interaction is required for the positioning measurement configuration update. This means that the positioning procedure may be delayed, or even terminated depending on the number of active TRP(s).

Observation 2: TRP change for regenerative scenario may have adverse effects on positioning procedure.

To solve this, reconfiguration of positioning measurements appears as an inefficient solution. Inter-gNB coordination may lead to additional overhead, or may not be feasible economically-wise in the network.

Observation 3: In a regenerative scenario which includes gNB change, inter-gNB coordination appears inefficient to reduce positioning procedure interruption effects.

In order to solve the above issue, contribution [3] from RAN2-129 suggested that the positioning procedure is not initiated if the time before gNB change would not be enough to complete it. This can be done by the UE reporting the gNB change timing to the LMF via LPP, or to the gNB via Uu. The LMF can in turn make the decision whether or not to initiate the positioning procedure. The gNB can also use this information to guarantee the completion of the incoming positioning procedure.

The gNB change timing may be reported in absolute or relative time. As from TS 38.331, coding absolute time requires 48 bits, while coding relative time requires 7 bits. Therefore, coding the latest seems much more efficient, i.e. the remaining time compared to when the message was sent.

Observation 4: To prevent interruption of positioning procedure, the UE reporting of incoming TRP expiration is useful. Using of relative time compared to the sending of the message appears the most efficient for this.

Given the above, the following is suggested:

Proposal 2: The UE indicates incoming TRP expiration to the LMF.

Proposal 3: The UE indicates incoming TRP expiration to the LMF in a relative time compared to the sending of the message.

# 3.	Conclusion

In this document, we analyse some remaining issues for regenerative scenario for NTN for NR Ph3, with the following observations and proposals:

Satellite switch with resynch:

Observation 1: In a regenerative payload scenario, for satellite switch with resynch, the two satellites belong to different cells. Due to security reasons, this may lead to additional signalling required for the satellite switch with resynch procedure. It is possible that this may reduce the usefulness of the satellite switch with resynch procedure compared to the L3 handover procedure.

Proposal 1: RAN2 should not pursue the work on satellite switch with resynch for regenerative payload until SA3 has clarified the impact required for the procedure and RAN2 concluded in turn that there are still benefits for this procedure.

Network verified UE positioning:

Observation 2: TRP change for regenerative scenario may have adverse effects on positioning procedure.

Observation 3: In a regenerative scenario which includes gNB change, inter-gNB coordination appears inefficient to reduce positioning procedure interruption effects.

Observation 4: To prevent interruption of positioning procedure, the UE reporting of incoming TRP expiration is useful. Using of relative time compared to the sending of the message appears the most efficient for this.

Proposal 2: The UE indicates incoming TRP expiration to the LMF.

Proposal 3: The UE indicates incoming TRP expiration to the LMF in a relative time compared to the sending of the message.

# 4.	References

- RP-243300, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3 3GPP TSG RAN Meeting #106 Madrid, Spain, 09-12 December 2024.

- R2-2502983, RAN2-129bis Report from Break-out session on NR-NTN and IoT-NTN. Session Chair (ZTE Corporation).

- R2-2500617, RAN2-129. UE location verification in NTN regenerative architecture, Lenovo.

- R2-2405703, RAN2-126 Report from Break-out session on NR-NTN and IoT-NTN  Session Chair (ZTE Corporation).