3GPP TSG-RAN WG2 Meeting #130	R2-2503990

St Julian’s, Malta, 19th –23rd May, 2025

Agenda Item:	8.2.2

Source:	Huawei, HiSilicon

Title:	A-IoT paging

Document for: Discussion and Decision

# 1	Introduction

In the last RAN2 meetings, some related agreements on A-IoT paging were achieved in the following:

FFS which solution if any for device behavior if it gets a new service request while one procedure is still ongoing or leave it to implementation. RAN2 aims to design Rel-19 AIoT R2D messages extensible to accommodate devices and features of future release. Introduce an explicit 1 bit indication to indicate whether it is CFRA or CBRA per paging message. A field indicating Paging ID length information is always included together with the paging ID field in the A-IoT paging message, except the case where no ID is included in the A-IoT paging message. The number of bits required for paging ID length field should be as small as possible. This would require the number of different Paging ID lengths to be small.

Based on the current RAN2 progress, and focusing on the issues without conclusions, this contribution discusses multi-reader paging, transaction ID, and paging message content.

# 2	Discussion

## 2.1	Multi-reader paging (Issue 1-1)

The last RAN2 meeting leaves it as FFS for below issue:

R2-2502211 Email discussion report: [POST129][035][AIoT] Paging Qualcomm Incorporated discussion Rel-19 Ambient_IoT_Solutions-Core Proposal 1: Discuss and agree on the device behavior if it gets a new service request while one procedure is still ongoing (i.e. not completed or failed yet): (a) ignore all new requests and R2D messages addressed to itself but not associated with the ongoing procedure, or (b) terminate the ongoing procedure and respond to the latest request. (a/b/ffs = 17/8/4) Agreements FFS which solution if any for device behavior if it gets a new service request while one procedure is still ongoing or leave it to implementation.

The offline discussion “[AT129bis][010][AIoT] Paging” identified several issues with some concerns for option (a):

1. Deadlock issue for how to determine the end of procedure

- Timer based autonomous release by the device. - Concerns: supporting timer for such low complexity devices increases complexity. Hard to find exact useful value of the timer. - Explicit release (RRC-release-like) message by the Reader to all the devices. - Concerns: new message needed to be defined; may not be possible to indicate release to specific devices and release-all-devices-within-reader-range may not be desirable. This new message may also be lost. - Turn off CW source and force the devices to drain the energy. - Concerns: this assumes Reader has control over the CW source. Forcing the device to lose all of the volatile memory and trigger power-cycle may not be ideal. This would impact all the devices under coverage. Pure NW implementation may not be sufficient.

2. Identify the R2D messages associated with the ongoing procedure

Rapp Summary: If device needs to be able to determine whether a R2D message is associated to an ongoing procedure or not, some kind of indication (e.g. transaction ID, reader ID, some other ID) would be needed in the R2D messages.

Based on our previous calculation, including 3-bit reader/transaction ID in all R2D messages may raise 28% R2D signalling overhead and 19% inventory latency [See R2-2502268].

Observation 1:	After two meetings’ discussion, the option (a) still:

has the diverse solutions to address the deadlock issue for end of procedure determination, each of which may have some concerns; and

suffers the overhead concern by including some ID in all R2D messages.

On the other hand, the offline discussion “[AT129bis][010][AIoT] Paging” successfully addressed the concern for option (b), i.e. the so-called forward compatible issue, which should be common issue for any option.

RAN2 aims to design Rel-19 AIoT R2D messages extensible to accommodate devices and features of future release.

Observation 2:	The concern for forward compatibility that was raised for option (b) has been addressed.

During the Come Back in the last meeting, another point raised to option (b) is that it may require some network behavior or implementation assumption.

For the readers under the control of the same Base Station, the BS can configure separate resources for different readers to ensure no multiple requests are received by devices, and then the BS implementation can also avoid sending parallel/interleaved services to the devices.

Then we should consider the neighbour readers belonging to different BSs. In R3-250815, the TP to TS 38.300 BL CR has captured the following agreement.

- In this version of the specification, no A-IoT specific communication takes place between gNBs.

According to the above discussion in RAN3, it is not supported to have any inter-BS coordination. Therefore, we understand there are two potential methods to avoid parallel/interleaved services from the readers belonging to different BSs. In the first method, CN implementation ensures that a reader starts a new service after a reader from another BS successfully finishes its own service. In the other method, OAM can configure isolated resources for different BSs. Then the device will not receive another service request when it is already involved in one service under one BS.

Figure 2.1-1 Example of OAM configuration with isolated resources

Observation 3a:	Network implementation for some coordination is anyway inevitable, to address the physical layer interference among neighbour readers.

Observation 3b:	Network implementation can avoid parallel services from the neighbour readers that belong to different BSs, e.g., OAM can allocate/configure non-overlapped time-domain resources.

Observation 3c:	Inter-BS coordination is not really critical for the indoor scenario of Rel-19.

Observation 4:	In Rel-19, without physical layer design to address interference issue in overlapped area, there is no point to introduce any RAN2 solution to address the overlapped area cases, which is not realistic either (i.e., most of the R2D messages cannot be correctly decoded at all due to interference).

Proposal 1:	(Issue 1-1) Agree option (b): If the “transaction ID” received in the paging message differs from the one currently maintained by the device, the device shall update its currently maintained “transaction ID” to the received value and act consequently.

## 2.2	Transaction ID

### 2.2.1	Transaction ID generation (Issue 1-2)

According to the discussion in RAN3/SA2, there may be a Correlation Identifier transmitted over NGAP interface. The reader can use this Correlation Identifier to associate the response(s) from devices to a specific service request from CN. If we use this Correlation Identifier as the transaction ID, the length of the transaction ID could be large, since the Correlation Identifier is used to identify a service among all the services from different CN nodes. On the other hand, considering the AS overhead and the buffer size required by the device storage for transaction ID, the transaction ID in the paging message should not be the same size as the Correlation Identifier from CN.

There could be multiple methods for reader implementation to generate the transaction ID.

- A first possible method could be for the transaction ID to use the right-most truncated part of the Correlation Identifier from CN, which is shorter than the Correlation Identifier.

- A second method could be that the reader can try to use different transaction IDs for different CN Correlation Identifiers, according to different implementation manners (i.e. different truncated ways or reader’s mapping table).

- In yet another method, the reader can generate the transaction ID not considering the CN Correlation Identifier on purpose. In such case, the reader may not intend to avoid the redundant responses.

How the reader will generate "transaction ID" can be implemented in different manners mentioned above, which should be left to the network implementation. Therefore, it is suggested to not specify how the reader generates the transaction ID.

Proposal 2:	(Issue 1-2) It is proposed to NOT specify how the reader generates the transaction ID, while some reader implementation manners can be the following:

The transaction ID is the right-most truncated part of CN Correlation Identifier; or

The reader tries to use different transaction IDs for different CN Correlation Identifiers; or

Reader generates the transaction ID without considering the CN Correlation Identifier at all (intentionally).

### 2.2.2	Transaction ID size (Issue 1-2)

In RAN2#129 meeting, the following agreements on transaction ID were achieved:

- The “transaction ID” can be generated by reader based on CN correlation ID. FFS how reader will generate “transaction ID”. FFS the size of transaction ID - 1 bit solution is excluded. FFS the size. Aim to have a reasonable size.

Considering that there may be a case that several readers are deployed near each other, as shown in Figure 2.2-1, a 1-bit transaction ID may be insufficient to distinguish old and new services from different BSs. In detail, if the transaction ID is too short, there could be different services from multiple different neighbour BSs that use the same transaction ID. Then the device will miss a service from one of the multiple readers. Therefore, more than one bit transaction ID may be needed. Besides, multiple-bit transaction ID can also reduce the possibility of wrap-round issue. If the device failed to receive the paging messages for the new (the second) service, the device can still process the third service instead of regarding it as an old service (the first one).

To balance the purpose of reducing the required stored information size, 2-bit transaction ID in paging message seems enough to distinguish the old and new services from different BSs. Nevertheless, 3-bit transaction ID in paging message seems an acceptable compromise.

Proposal 3:	(Issue 1-2) 2-bit transaction ID in paging message seems enough, while 3-bit transaction ID in paging message seems a reasonable compromise.

### 2.2.3	Need for transaction ID in CFRA (Issue 1-5, 2-8)

The main function of transaction ID in paging messages is to avoid duplicated response for the same service request when the reader provides more paging opportunities for the devices that fail to receive previous paging messages. By this transaction ID, device can distinguish whether the provided access occasions are for re-access for the same service or initial access of one new service.

RAN2#129bis agreements: For CFRA, NACK feedback and re-access is not supported. FFS how to achieve

As for CFRA cases, NACK-based re-access is not supported. Therefore, the transaction ID is unnecessary, given that the re-access will not happen. When initializing CFRA, each access occasion is allocated to the dedicated device from the reader. So, the reader knows whether the selected device successfully responds to the current paging message. If the device successfully responds to the paging message, the reader will not initialize a subsequent paging procedure. Otherwise, when the D2R message transmission failure occurs due to bad link quality, the reader needs to send a subsequent paging message triggering the device to resend D2R message (e.g., device ID).

Observation 5:	Transaction ID is introduced in subsequent paging for re-access purpose. It will be unnecessary, since CFRA does not support re-access at all.

Proposal 4:	(Issue 1-5, 2-8) For CFRA, transaction ID is unnecessary in the paging message.

## 2.3	Paging message content

### 2.3.1	Paging ID length field (Issue 1-3)

This sub clause analyses the length of the Paging ID length field from two perspectives. First, from the perspective of analysing all possible lengths for Paging ID, the paging ID length field is discussed as follows.

Filtering information: The filtering information defined in TS 23.369 is as follows:

The filtering information is used to identify or filter multiple AIoT Devices and is constructed by one or multiple components of the AIoT Device Permanent Identifier as defined in clause 5.7. The filtering information includes a list of filtering element information. The filtering element information includes: - Information indicating a which component of the AIoT Device Permanent Identifier that is used to match the bitstring. - A bitstring which is used to compare with the component. Each bitstring is corresponding to one component of the AIoT Device Permanent Identifier. NOTE: The bitstring match can start at any position in the corresponding component.

From the above definitions, filtering information includes a list of filtering elements, i.e., including one or more filtering element information. The component in the filtering element information is one of the four components from PLMN ID, NID, the third-party ID, and Identification Information in Device Permanent Identifier. The bit string in the filtering element information can start at any position in the corresponding component. In order to indicate the bit string, the filtering information should contain: the starting position indication of the bit string, the length information of the bit string and the bit string itself.

Figure 5.7.2-1: AIoT Device Permanent Identifier Structure

NOTE 3: The length of ID Type, PLMN Identifier (if present), NID (if present) and the third party identifier (if present) components is fixed. The length of the Identification Information is variable. The details are specified in TS 23.003 [6] and TS 29.xxx [xx]. The following lengths are supported for the Identification Information in an AIoT Device Permanent Identifier: 96 bits, and 128 bits.

The filtering information may consist of the following:

- 2-bit information to indicate which component is selected.

- The starting position indication of the bitstring needs 7 bits. According to the analysis in Device Permanent Identifier, the maximum length of a component is 128 bits. The length of the starting position needs 7 bits to indicate which bit is the starting position in the indicated component.

- The length information of the bitstring needs 9 bits. Considering the bitstring length is unknown to devices, the length of the bitstring should be indicated. The maximum length of a bit string is the entire Device Permanent Identifier. As analysis of the Device Permanent Identifier, the maximum length of Device Permanent Identifier is much less than 512 bits, so 8 or at most 9 bits are enough.

- Bitstring at most is about 236 bits. The bitstring may change from 0 bit to the length of the entire Permanent ID. According to the definition of Permanent ID, the maximum length of the bitstring is equal to the length of the Permanent ID :

- 24-bit PLMN ID: From TS 23.003, the PLMN ID needs 24 bits (3-digit MCC (4 bits) and 3-digit MNC (4 bits)).

- 44-bit NID: From TS 23.003, the NID needs 44 bits (1-hex digit (4 bits) assignment mode and 3-hex digit (40 bits) NID value).

- Third part ID: assuming to be 40 bits which can be similar to NID.

- 96 or 128-bit Identification Information.

The length of filtering information is 254 bits even though the bit string includes the entire Device Permanent Identifier. Besides, if filtering information includes a list of filtering elements, it will only add several starting position indications of the bit string and length information of the bit string. Then 300 bits could be a possible upper bound of filtering information. Hence, 9 bits are needed to indicate the length of the filtering information in the unit of bit.

Temporary ID: The Temporary ID is another format of a paging ID. Referencing the length of the existing GUTI (Globally Unique Temporary UE Identity) defined in TS 23.003, Temporary ID can be assumed to be 80 bits. The bit length of an 80-bit Temporary ID needs 7-bit length indication. On the other hand, the length of a temporary ID should not be larger than the entire Device Permanent Identifier. Then, 9 bits are enough to indicate the bit length of the temporary ID field as well.

Therefore, from the perspective of the definitions of paging ID, 9 bits is a rather reasonable number for indicating the length of the paging ID field.

Second, from the perspective of the maximum R2D message size, the length of the Paging ID length field is discussed as follows.

Each R2D message size is not larger than 1000 bits:

RAN1#118bis agreements: From RAN1 perspective, a maximum TB size of around 1000 bits in PHY for R2D and D2R directions can be supported. - How large TB that can be transported at a given time depends on target coverage/data rate, energy consumption/device availability, etc. In this paper, we propose CFRA and CBRA different paging formats.

Accordingly, the paging ID field in a paging message should also be less than 1000 bits. With a granularity of bit, at most 10 bits can be used to indicate the length of the paging ID.

Observation 6:	Based on the above calculation, the maximum size of paging ID in case of mask/filter can be up to 300 bits. Then, 512 or 1024 can be the max value for paging ID length.

Besides, there is a possible enhancement for CFRA. Considering that an entire Permanent Identifier rather than filtering information is used for CFRA paging, the paging ID length field only needs to indicate the length of Device Permanent Identifier for CFRA cases. That means, for CFRA, 4-bit length indication is enough, which can save 5 bits.

However, with this definition, the AS layer needs to be aware of the ID type information, which introduces the cross-layer information exchange. For CFRA, about a hundred bits of a Permanent ID (e.g., at least 96 or 128-bit Identification Information) are included, 5 bits are much less than the total length of a paging message. The additional spec impacts make the overhead of saving 5 bits not so necessary.

Observation 7:	No need to have different length filed for paging ID length, i.e., not to do further size reduction for CFRA.

As analyzed above, the indication field includes 9 or 10 bits for paging ID length in the unit of bit.

Proposal 5:	(Issue 1-3) A 9-bit or 10-bit field for the “paging ID length” in the unit of bit is used.

### 2.3.2	Access occasion number field (Issue 1-4)

The paging message will indicate the number of access occasions for the following random access procedure. The number of access occasions is supposed to cover the huge number of devices for one inventory procedure, e.g., more than 10,000 devices.

There are two candidate options to use one field to indicate up to 10,000 access occasions.

- Option 1: The normal linear way, 14 bits, value range as [1, 214] in the unit of 1.

- Option 2: The RFID exponential way, 4 bits, value range as [1, 215] with code points as {20, 21, 22, 23, 24, 25, ... 215}.

We have the observations below to compare those options:

Observation 8a:	The linear representation, at the cost of 10 more bits overhead in paging message, can achieve optimized Slotted-ALOHA procedure by configuring the similar number of AO based on number of devices.

Observation 8b:	The exponential representation can save the overhead about 10bits, while it cannot configure relatively accurate number of AO according to number of devices.

Observation 9:	The difference between NR A-IoT design and RFID may be: A-IoT paging can tolerate more overhead, while the inventory procedure efficiency may be decreased due to many excessively allocated access occasions.

Figure 2.3.2-1	Illustrative figures of two options

Proposal 6:	(Issue 1-4) RAN2 to discuss the format and the size for the field of number of access occasions:

Option 1: linear way, 14 bits, value range as [1, 214-1] in the unit of 1; (suggested)

Option 2: exponential way, 4 bits, value range as [1, 215] with code points as {20, 21, 22, 23, 24, 25, ... 215}.

### 2.3.3	Forward compatibility

The Rel-19 WID clarifies the motivation to consider the forward compatibility is the possible multiple paging IDs case.

- A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes.

If RAN2 intends to use one paging message type to support the Rel-19 single paging ID and Rel-20 possible multiple paging IDs case, for example, the Rel-19 paging ID can be put at the end of paging message. This would allow the Rel-19 device to ignore the following unknown fields/bits. In addition, R bit can be used for any other new information extension in paging message for Rel-20.

Observation 10:	Rel-19 device can ignore R bit and any other unknown fields in paging message.

Proposal 7:	A single R bit is adequate for future extensions of the paging message.

### 2.3.4	Paging content for CFRA (Issue 1-5)

In the last meeting, RAN2 agreed that in paging message, an explicit indication is introduced to indicate whether CBRA or CFRA is going to be performed by the selected device(s).

- Introduce an explicit 1 bit indication to indicate whether it is CFRA or CBRA per paging message

In the previous discussion, RAN2 agreed that Paging ID, transaction ID, and RACH related configuration are going to be included in paging message. However, the discussion has been mainly focused on CBRA, and some of them may not be useful for CFRA case.

Paging ID presence indication:

- For CBRA, the paging ID could be per-device ID/per-group ID/no paging ID. So the length of paging ID would be with more complex/long format, and one more bit is needed to indicate whether the paging ID is present.

- For CFRA, the paging ID is the per-device ID, and always present. Then, there is no need to indicate present or not.

Transaction ID:

- For CBRA, the transaction ID can be used to identify paging/subsequent message for the same service, accordingly device can determine whether to perform re-access using the resources provided by subsequent paging message.

- For CFRA, re-access is not supported, and transaction ID is unnecessary based on the previous discussion in section 2.2.3.

RACH related configuration:

- For CBRA, the number of access occasions (i.e., number of resources for Msg1 transmission) needs to be indicated. For instance, in RFID, a 4-bit field indicates maximum1024 slots). The number of time domain (i.e., value of X) and frequency domain (i.e., value of Y) needs to be configured as well.

- For CFRA, only one device is scheduled, so the number of resources is one by default. The corresponding explicit configuration in paging is not needed.

Then, for the paging message format/content, there could be two directions:

- Direction 1: using the unified message format to accommodate both CBRA and CFRA, which means, in case of CFRA, the selected device needs to ignore the useless configuration.

- Direction 2: using separate message format for CBRA and CFRA (still one message type), and the RA type indication can also indicate the format. The format examples are shown in the following figures.

Figure 2.3.4-1:	An example of Paging message format for CBRA

Figure 2.3.4-2:	An example of Paging message format for CFRA

So, we can see direction 2 can allow less signaling overhead for CFRA, and the devices do not need to handle/ignore the configurations which are not useful, so the device behavior would be clearer. In this case, we prefer to go with direction 2.

Proposal 8:	(Issue 1-5) If the RA type indicates CFRA in paging message, the CBRA specific configurations/fields (e.g., access occasions number, transaction ID, paging ID presence indication) will be absent.

# 3	Conclusion

This contribution makes the following proposals:

Observation 1:	After two meetings’ discussion, the option (a) still:

has the diverse solutions to address the deadlock issue for end of procedure determination, each of which may have some concerns; and

suffers the overhead concern by including some ID in all R2D messages.

Observation 2:	The concern for forward compatibility that was raised for option (b) has been addressed.

Observation 3a:	Network implementation for some coordination is anyway inevitable, to address the physical layer interference among neighbour readers.

Observation 3b:	Network implementation can avoid parallel services from the neighbour readers that belong to different BSs, e.g., OAM can allocate/configure non-overlapped time-domain resources.

Observation 3c:	Inter-BS coordination is not really critical for the indoor scenario of Rel-19.

Observation 4:	In Rel-19, without physical layer design to address interference issue in overlapped area, there is no point to introduce any RAN2 solution to address the overlapped area cases, which is not realistic either (i.e., most of the R2D messages cannot be correctly decoded at all due to interference).

Observation 5:	Transaction ID is introduced in subsequent paging for re-access purpose. It will be unnecessary, since CFRA does not support re-access at all.

Observation 6:	Based on the above calculation, the maximum size of paging ID in case of mask/filter can be up to 300 bits. Then, 512 or 1024 can be the max value for paging ID length.

Observation 7:	No need to have different length filed for paging ID length, i.e., not to do further size reduction for CFRA.

Observation 8a:	The linear representation, at the cost of 10 more bits overhead in paging message, can achieve optimized Slotted-ALOHA procedure by configuring the similar number of AO based on number of devices.

Observation 8b:	The exponential representation can save the overhead about 10bits, while it cannot configure relatively accurate number of AO according to number of devices.

Observation 9:	The difference between NR A-IoT design and RFID may be: A-IoT paging can tolerate more overhead, while the inventory procedure efficiency may be decreased due to many excessively allocated access occasions.

Observation 10:	Rel-19 device can ignore R bit and any other unknown fields in paging message.

Multi-reader paging (Issue 1-1)

Proposal 1:	(Issue 1-1) Agree option (b): If the “transaction ID” received in the paging message differs from the one currently maintained by the device, the device shall update its currently maintained “transaction ID” to the received value and act consequently.

Transaction ID (Issue 1-2, 1-5, 2-8)

Proposal 2:	(Issue 1-2) It is proposed to NOT specify how the reader generates the transaction ID, while some reader implementation manners can be the following:

The transaction ID is the right-most truncated part of CN Correlation Identifier; or

The reader tries to use different transaction IDs for different CN Correlation Identifiers; or

Reader generates the transaction ID without considering the CN Correlation Identifier at all (intentionally).

Proposal 3:	(Issue 1-2) 2-bit transaction ID in paging message seems enough, while 3-bit transaction ID in paging message seems a reasonable compromise.

Proposal 4:	(Issue 1-5, 2-8) For CFRA, the transaction ID is unnecessary in the paging message.

Paging message content (Issue 1-3, 1-4, 1-5)

Proposal 5:	(Issue 1-3) A 9-bit or 10-bit field for the “paging ID length” in the unit of bit is used.

Proposal 6:	(Issue 1-4) RAN2 to discuss the format and the size for the field of number of access occasions:

Option 1: linear way, 14 bits, value range as [1, 214-1] in the unit of 1; (suggested)

Option 2: exponential way, 4 bits, value range as [1, 215] with code points as {20, 21, 22, 23, 24, 25, ... 215}.

Proposal 7:	A single R bit is adequate for future extensions of the paging message.

Proposal 8:	(Issue 1-5) If the RA type indicates CFRA in paging message, the CBRA specific configurations/fields (e.g., access occasions number, transaction ID, paging ID presence indication) will be absent.

# 4	References

- R2-2502268, A-IoT paging, Huawei, HiSilicon