3GPP TSG-RAN WG2 Meeting #130	R2-2503999

St. Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.5.3

Source:	OPPO

Title:	Consideration on on-demand SIB1

Document for:	Discussion, Decision

Introduction

This paper addresses the open issues related to on-demand SIB1.

- Whether to include cell access information in UL-WUS configuration.

- Scenarios where K_SSB < 24 for FR1 or K_SSB< 11 for FR2.

- FFS Co-existence of SDT and OD-SIB1.

Discussion

Whether to include cell access information

In the legacy cell selection/reselection procedure, a UE can be barred from accessing a cell by using cellBarred in MIB and/or cell access information in SIB1. The cell access information in SIB1 includes cell access-related parameters (e.g. plmn-IdentityList, trackingAreaCode, npn-IdentityInfoList, cellReservedForOtherUse, etc.) and feature-specific barring parameters for various UE capabilities (e.g., cellBarredNTN for NTN-capable UEs, cellBarredNES for UEs supporting nes-CellDTX-DRX, etc.).

In RAN2#129bis, RAN2 discussed whether to include cell access information in the UL-WUS configuration. The proponent companies argue that this inclusion can avoid the latency and power consumption for the UE attempting to access a specific cell. For example, if a Rel-19 OD-SIB1 UE supports nes-CellDTX-DRX and can determine the cellBarredNES information of a specific cell via the UL-WUS configuration, this UE can directly try another cell instead of acquiring the OD-SIB1 for this cell first. However, the increase in the signalling overhead to include cell access information in the UL-WUS configuration is not marginal.

- As specified in TS 38.304, many feature-specific barring parameters have already been introduced in SIB1. With iterations of the specification version, the number of such parameters is expected to rise. We need to update SIB-xx for each new parameter, which continuously increases the RAN2 workload and system information overhead.

[38.304] - cellBarredATG (IE type: "barred" or "not barred") Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs. This field is only applicable to ATG UEs. - cellBarred2RxXR (IE type: "barred") Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs. This field is only applicable to 2Rx XR UEs. - cellBarred-eRedCap1Rx (IE type: "barred" or "not barred") Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs. This field is only applicable to eRedCap UEs. - cellBarred-eRedCap2Rx (IE type: "barred" or "not barred") Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs. This field is only applicable to eRedCap UEs. - cellBarredFixedVSAT (IE type: "barred" or "not barred") Indicated in SIB1 message. In case of multiple PLMNs indicated in SIB1, this field is common for all PLMNs. This field is only applicable to VSAT UEs using NTN access. - cellBarredMobileVSAT (IE type: "barred" or "not barred") Indicated in SIB1 message. In case of multiple PLMNs indicated in SIB1, this field is common for all PLMNs. This field is only applicable to VSAT UEs using NTN access. - cellBarredNES (IE type: "not barred") Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs. This field is only applicable to UEs indicating any of the values in nes-CellDTX-DRX as specified in TS 38.306 [24]. - cellBarredNTN (IE type: "barred" or "not barred") Indicated in SIB1 message. In case of multiple PLMNs indicated in SIB1, this field is common for all PLMNs. This field is ignored if the UE does not support NTN access. - cellBarredRedCap1Rx (IE type: "barred" or "not barred") Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs. This field is only applicable to RedCap UEs. - cellBarredRedCap2Rx (IE type: "barred" or "not barred") Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs. This field is only applicable to RedCap UEs.

- Currently, the payload size of cellAccessRelatedInfo exceeds 400 bits. The greater the number of NES cells provided, the larger the payload size of cellAccessRelatedInfo. Additionally, cellAccessRelatedInfo contains PLMN/TAC-related parameters, and each cell associates with its specific PLMN info/TAC value, making it challenging for the gNB to compile a list of cells associated with a single UL-WUS configuration. Therefore, the benefit of introducing the mapping between multiple NES cells and a single UL-WUS has been limited.

[38.331] #### – CellAccessRelatedInfo The IE CellAccessRelatedInfo indicates cell access related information for this cell. CellAccessRelatedInfo information element -- ASN1START -- TAG-CELLACCESSRELATEDINFO-START CellAccessRelatedInfo ::= SEQUENCE { plmn-IdentityInfoList PLMN-IdentityInfoList, cellReservedForOtherUse ENUMERATED {true} OPTIONAL, -- Need R ..., [[ cellReservedForFutureUse-r16 ENUMERATED {true} OPTIONAL, -- Need R npn-IdentityInfoList-r16 NPN-IdentityInfoList-r16 OPTIONAL -- Need R ]], [[ snpn-AccessInfoList-r17 SEQUENCE (SIZE (1..maxNPN-r16)) OF SNPN-AccessInfo-r17 OPTIONAL -- Need R ]] } SNPN-AccessInfo-r17 ::= SEQUENCE { extCH-Supported-r17 ENUMERATED {true} OPTIONAL, -- Need R extCH-WithoutConfigAllowed-r17 ENUMERATED {true} OPTIONAL, -- Need R onboardingEnabled-r17 ENUMERATED {true} OPTIONAL, -- Need R imsEmergencySupportForSNPN-r17 ENUMERATED {true} OPTIONAL -- Need R } -- TAG-CELLACCESSRELATEDINFO-STOP -- ASN1STOP

- If the cell access information in SIB1 is not semi-static, frequent exchanges between Cell A and the OD-SIB1 cells, as well as the corresponding SI change notification, are required.

- Even if the parameter(s) originally provided in SIB1 can be configured as optional in the UL-WUS configuration, we assume that all cell access information should be treated consistently, meaning all should be included or excluded. Conversely, if all information in these fields is absent, it is equivalent to no cell access information being included in the UL-WUS configuration.

In Rel-19 NES, do not include cell access information originally provided in SIB1 in the UL-WUS configuration.

Scenarios where K_SSB < 24 for FR1 or K_SSB< 12 for FR2

A cell supporting OD-SIB1 may broadcast SIB1 upon the SI change notification or OD-SIB1 request reception. Regardless of the case, RAN2 assumes that the Kssb of the corresponding NES cell would change as <24/12 and therefore concludes with the following.

- When the cell supporting on demand SIB1 is broadcasting SIB1 (e.g. upon SIB1 request), legacy UE can camp on the cell if the legacy UE is able to acquire the broadcasted SIB1. No specification impact is foreseen.

However, regarding the latter case, the conclusion of RAN1 implies a different understanding from RAN2 about how to set Kssb. That is, the Kssb value of a certain NES cell may still be maintained as invalid, even if the Rel-19 UE has requested OD-SIB1 for this cell. On the other hand, if the Kssb value is changed,

- Either the network needs to invoke the legacy system change notification procedure (which not only leads to an unnecessary SIBx reading, but also leads to a different time window mechanism since MP is used for legacy SI modification but OD-SIB1 relies on new monitoring window), for notifying the UE the change of MIB.

- Or without SI modification, it would lead to the result that a legacy UE camps on a NCD-SSB without knowing that.

Agreement in RAN1#117 At least for Case-2: For further study of type 0 PDCCH monitoring occasions for on demand SIB1, after UE transmits the UL WUS in idle/inactive mode, RAN1 assumes following as a starting point: - Option 1: One or more type 0 PDCCH monitoring occasions for on demand SIB1 within a time window - FFS: How the search space zero configuration is provided (e.g. from searchSpaceZero in MIB or from a new search space that is indicated by UL-WUS configuration) - FFS: Details of the time window, including at least the starting time and duration - FFS: Whether/how to support transmission of on-demand SIB1 with the association with SSB(s) based on a received UL-WUS Agreement in RAN1#118 At least for Case-2: For further work on type 0 PDCCH monitoring occasions for on demand SIB1, on the starting time and duration of the time window of type 0 PDCCH monitoring occasions, RAN1 to down select from the following two options: - Option 1: starting time and duration are indicated in RAR of the UL-WUS transmission - Option 2: starting time and duration are indicated in the UL WUS configuration Agreement in RAN1#119 The duration of the time window for on-demand SIB1 is indicated in the UL WUS configuration. - Unit of the duration is in slot - FFS: Starting time Agreement in RAN1#120bis If a UE has SIB1 request configuration of a cell and before transmitting UL WUS, - If the UE detects a SSB where K_SSB>=24 for FR1 or K_SSB>=12 for FR2, select the following: - Alt. 3: It is up to UE implementation on whether to monitor Type 0 PDCCH for SIB1 transmission

Thus, further clarification is needed to avoid the misalignment between RAN1 and RAN2. As RAN1 is an expert in the usage of Kssb, we prefer RAN2 to rely on R1 for Kssb setting upon SIB1 request.

RAN2 rely on R1 for Kssb setting upon SIB1 request. No R2 specification impact is foreseen.

FFS Co-existence of SDT and OD-SIB1

One open issue was raised during the CR discussion, i.e. FFS Co-existence of SDT and OD-SIB1. In the legacy, the OSI request cannot be initiated during the SDT procedure, as

- Before initiating the SDT procedure, UE would have acquired all essential SIBs needed for operating in a

camped cell.

- The SI request may lead to UL carrier switching (UL carrier selected at the time of initiation of SDT can be different from UL carrier selected at the time of SI request) or BWP switching (in case SDT is performed on non-initial BWP) which interrupts the ongoing SDT procedure.

So the key issues above

- Also applies to OD-SIB1 for serving cell;

- Similarly, for OD-SIB1 for neighboring cell for cell reselection reason, the UL/DL transmission in serving cell for SDT will be impacted if the UE is not able to perform simultaneous Tx/Rx with serving and neighouring cell (for OD-SIB1 request and RAR, SIB1 reading). Besides, separate RACH may be initiated for the two reasons, and at least the RACH collision has to be avoided since “There is only one Random Access procedure ongoing at any point in time in a MAC entity”.

So, a simple solution is to exclude the co-existence, or consider that there might be space for UE implementation to handle that

- For OD-SIB1 for serving cell, if both SDT and OD-SIB1 happens at NUL and initial BWP;

- For OD-SIB1 for neighboring cell, if the UE has enough Tx/Rx chains, to support simultaneous operation;

When SDT is on-going, RAN2 discuss to 1) either not support OD-SIB1 request; 2) or support OD-SIB1 request but leave it to UE implementation to handle the collision, if any.

Conclusion

We have the following proposals:

Proposal 1	In Rel-19 NES, do not include cell access information originally provided in SIB1 in the UL-WUS configuration.

Proposal 2	RAN2 rely on R1 for Kssb setting upon SIB1 request. No R2 specification impact is foreseen.

Proposal 3	When SDT is on-going, RAN2 discuss to 1) either not support OD-SIB1 request; 2) or support OD-SIB1 request but leave it to UE implementation to handle the collision, if any.

Reference

[1] RP-242354 Revised WID: Enhancements of network energy savings for NR