3GPP TSG-RAN WG2 Meeting #130                                  R2-2504025

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda item:	6.1.3.1

Title:	Discussion on eDRX operation with emergency PDU session

Source:	ZTE Corporation

Document for:	Discussion and Decision

Introduction

In RAN2#128 meeting, RAN2 sent an LS [1] to SA2, asking about the handling of eDRX when UE is in RRC_INACTIVE state with emergency service. Regarding the question, SA2 provided the following feedback in their reply LS [2].

SA2 analysed the identified issue above and would like to provide the following feedback: - Existing specifications have specified that each PLMN/operator reserves a specific ARP value for emergency PDU session and provide to RAN node. See related text in TS 23.501 clause 5.16.4.3, 5.16.4.7, 5.16.4.9 and TS 23.502 clause 6.1.3.10. Therefore, the RAN node can be aware of an emergency PDU session based on the reserved ARP value when the PDU session context exist in the RAN node. - In addition, there is inactivity monitoring for the presence of user plane resources for the emergency PDU session in SMF as defined in TS 23.502 clause 4.4.2.2 and TS 23.503 clause 6.1.3.10. Based on the above, some companies in SA2 believe that the RAN node is capable through configuration to not release the emergency PDU session user plane resources based on existing implementation, and to not release the UE to RRC_INACTIVE state/RRC_IDLE state when the UE has an emergency PDU session. Therefore, the issue highlighted in RAN2 LS may not exist. However, there is no consensus in SA2 whether the UP of emergency PDU session can be deactivated, or whether to introduce new signalling to NG-RAN. SA2 would like to ask RAN2 to provide feedback.

The reply LS was discussed in last RAN3 meeting, companies in RAN3 endorsed the stage 2 CR and approved LS [3] with below content:

RAN3 thanks SA2 for the LS on emergency call back and paging. RAN3 has discussed the scenario raised by RAN2 and checked the response from SA2. According to SA2’s clarification, RAN3 agrees that the NG-RAN is able to be aware of the presence of emergency PDU session when PDU session resources are established in the NG-RAN node based on the received special ARP value from CN. Regarding the open issues, RAN3 has endorsed the attached CRs to TS 38.300.

In last RAN2 meeting, we also discussed the reply LS from SA2, but considering the progress in RAN3, the discussion in RAN2 was suspended and the paper regarding UE behavior was postponed. In this contribution, we further discussed this issue and shared our views.

Discussion

According to the LSs from SA2 and RAN3, they both confirm that RAN-node can be aware of the presence of emergency PDU session based on the ARP value, which means that, in real deployment, each PLMN/operator can reserve a specific ARP value for emergency PDU session, and the ARP value will be sent to RAN when the PDU session is established, so, the gNB can recognize the emergency service based on the received ARP value.

Observation 1: According to reply LSs, both SA2 and RAN3 confirm that RAN node can be aware of the presence of emergency PDU session based on the reserved ARP value.

Based on above assumption, RAN3 agreed to add below Note in TS 38.300. The intention is to disallow network to configure RAN eDRX when releasing the UE to RRC_INACTIVE if the UE is configured with emergency PDU session.

NOTE: If emergency PDU session resources are established, the gNB should not configure the Extended DRX when sending the UE to RRC_INACTIVE state. The gNB recognizes the emergency PDU session resources based on special ARP value as specified in TS 23.501 [3].

Observation 2: Based on RAN3 endorsed stage 2 CR, if the UE is configured with emergency PDU session, RAN node cannot release the UE to RRC_INACTIVE state with RAN eDRX configuration.

For UE in RRC_IDLE state, it is already captured in TS 23.501 that UE shall ignore CN eDRX when the UE has a PDU session associated with emergency service.

In last RAN2 meeting, some companies suggest to adopt the same principle for RRC_INACTIVE UEs, which means the RRC_INACTIVE UE shall ignore RAN eDRX when the UE has emergency service and the gNB configures RAN eDRX when releasing the UE to RRC_INACTIVE state. However, we don’t think this is needed, because:

- According to the endorsed RAN3 CR, the situation mentioned above won’t happen, because if the gNB is aware of the emergency PDU session, the gNB shall not configure RAN eDRX when the gNB decides to release the UE to RRC_INACTIVE state.

- Even if the gNB mistakenly releases the UE to RRC_INACTIVE state with RAN eDRX configured, it is meaningless for the UE to ignore the RAN eDRX configuration, because, from network perspective, if the gNB cannot recognize the emergency PDU session, the gNB will anyway follow the RAN eDRX configuration it provides. Then, when the gNB receives downlink data from CN, the gNB will wait for RAN eDRX paging occasions and page the UE. In this case, even if the UE ignores the RAN eDRX and follows legacy DRX paging occasions, it is impossible for the UE to receive RAN paging over those paging occasions, the UE can only receive RAN paging over RAN eDRX paging occasions. So, ignoring RAN eDRX just wastes UE’s power.

Observation 3: Even if the gNB cannot recognize emergency PDU session and mistakenly releases the UE to RRC_INACTIVE state with RAN eDRX configured, there is no benefit to ask the UE to ignore the RAN eDRX configuration and it will waste UE’s power.

Based on above analysis, UE-based solution won’t work for RRC_INACTIVE UEs, it may also be NBC for existing Rel-17 UE implementations. If the network implements according to RAN3 endorsed CR, the UE won’t be configured with RAN eDRX when the UE has emergency PDU session and be released to RRC_INACTIVE state.

If unfortunately, the CN and/or the gNB haven’t implemented the emergency PDU session awareness function (e.g. no reserved ARP value), then the UE can just follow the network configuration, which means if gNB releases the UE to RRC_INACTIVE state and gNB configures RAN eDRX, the UE can still follow the RAN eDRX even if the UE has suspended emergency PDU session. So, the paging delivery/reception behavior is the same at both gNB side and UE side. Although this may cause long delay for emergency callback, there is no other good way out. From implementation point of view, it is operator and CN/NW vendor’s responsibility to upgrade if they want to avoid long delay in emergency callback.

Proposal 1: 	RAN2 confirm that the RRC_INACTIVE UE does not need to ignore the RAN eDRX (if configured) when the UE has emergency PDU session (no spec change is needed).

On the other hand, in last RAN2 meeting, some companies think to ensure fast emergency call fallback, the network should be allowed to keep the UE in RRC_CONNECTED mode, if the network is aware of the establishment of emergency PDU session, we think this is straightforward, and it is not forbidden by the endorsed RAN3 CR), because in the CR, it clarifies “the gNB should not configure the Extended DRX when sending the UE to RRC_INACTIVE state”, it does not say the gNB must send the UE to RRC_INACTIVE state.

So, in summary, when the UE is configured with emergency PDU session and there is no active date transmission, the below network behaviors are supported:

- NW behavior 1: Keep the UE in RRC_CONNECTED state;

- NW behavior 2: Release the UE to RRC_IDLE state, in this case, if the UE was configured with IDLE eDRX, both the UE and the CN will ignore IDLE eDRX as specified in TS 23.501.

- NW behavior 3: Release the UE to RRC_INACTIVE state, in this case, the gNB should not configure RAN eDRX to the UE.

Proposal 2: 	RAN2 confirm that if gNB is able to recognize the presence of emergency PDU session, the gNB can also keep the UE in RRC_CONNECTED state for fast emergency callback (this is not forbidden by the endorsed RAN3 CR).

Considering the decision in RAN2 may have impact on SA2 specification, a reply LS to SA2 is needed, as also indicated in SA2’s reply LS, they expect RAN2 to provide feedback. Based on above analysis, we have prepared a draft reply LS to both SA2 and RAN3 in [4].

Proposal 3: 	Approve the reply LS to SA2 and RAN3 in [4].

Conclusion

Based on the above, RAN2 is requested to discuss and agree on the following observations and proposals:

Observation 1: According to reply LSs, both SA2 and RAN3 confirm that RAN node can be aware of the presence of emergency PDU session based on the reserved ARP value.

Observation 2: Based on RAN3 endorsed stage 2 CR, if the UE is configured with emergency PDU session, RAN node cannot release the UE to RRC_INACTIVE state with RAN eDRX configuration.

Observation 3: Even if the gNB cannot recognize emergency PDU session and mistakenly releases the UE to RRC_INACTIVE state with RAN eDRX configured, there is no benefit to ask the UE to ignore the RAN eDRX configuration and it will waste UE’s power.

Proposal 1: 	RAN2 confirm that the RRC_INACTIVE UE does not need to ignore the RAN eDRX (if configured) when the UE has emergency PDU session (no spec change is needed).

Proposal 2: 		RAN2 confirm that if gNB is able to recognize the presence of emergency PDU session, the gNB can also keep the UE in RRC_CONNECTED state for fast emergency callback (this is not forbidden by the endorsed RAN3 CR).

Proposal 3: 	Approve the reply LS to SA2 and RAN3 in [4].

References

- R2-2411041 LS on emergency services and eDRX    from RAN2

- R2-2501752/S2-2502427 Reply LS on emergency call back and paging    from SA2

- R2-2503318	 Reply LS on emergency call back and paging (R3-252402; contact: ZTE)  from RAN3

- R2-2504026 [draft] Reply LS on emergency call back and paging   from RAN2