3GPP TSG-RAN WG2 Meeting #130	R2-2504030

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda item:	8.20.1

Title:	Discussion on Rx BSF optimization

Source:	ZTE Corporation

Document for:	Discussion and Decision

Introduction

In RAN2#130 meeting, RAN2 receives an LS from RAN4 [1] regarding FR2-1 measurement enhancement by optimizing Rx BSF (beam sweeping factor), the LS contains below RAN4 agreements.

Agreement in RAN4#113: - RAN4 to introduce serving cell’s signal quality (i.e., based on RSRP and/or RSRQ threshold(s)) based triggering condition to activate/deactivate multi-Rx for L3 measurement. Agreement in RAN4#114: - Adopt FBS triggering condition of serving cell is worse than a threshold, including RSRP, RSRQ threshold. Agreement in RAN4#114bis: - For the baseline scenario agreed in RAN4 #112, limit the serving cell to a FR2 serving cell. Agreement in RAN4#112: - Baseline: L3 delay enhancements in Rel-19 by optimizing Rx BSF for UE supporting multi-Rx simultaneous reception are applicable provided that: - the target carrier(s) to be measured: only one carrier in the single FR2-1 band is configured for L3 SSB measurement and - UE serving carrier(s): UE is configured with single carrier on FR2-1 band, i.e. FR2-1 PCell without CA/DC. Note: Target and serving carrier frequency can be the same or different. Agreement in RAN4#112bis: - RAN4 agree that the criteria/threshold(s) are configured by RRC signaling.

In this contribution, we discussed the RAN4 LS and the potential RAN2 impact.

Discussion

The motivation of the enhancement is for reduce FR2-1 measurement latency for multi-Rx UE, such UE can use 2 antenna panels to receive DL signals simultaneously. Compared with legacy UE requirements of FR2 RRM measurements, multi-Rx UE can perform measurements more efficiently, so, the measurement period can be reduced by reducing the Rx beam sweeping factor (BSF).

However, activating 2 antenna panels consumes more power, so, RAN4 assumes this is only needed in urgent mobility cases. To achieve this, RAN4 agreed to activate and deactivate the multi-Rx L3 measurements only when the serving cell quality fulfills a condition.

Applicable scenario:

Agreement in RAN4#112: - Baseline: L3 delay enhancements in Rel-19 by optimizing Rx BSF for UE supporting multi-Rx simultaneous reception are applicable provided that: - the target carrier(s) to be measured: only one carrier in the single FR2-1 band is configured for L3 SSB measurement and - UE serving carrier(s): UE is configured with single carrier on FR2-1 band, i.e. FR2-1 PCell without CA/DC. Note: Target and serving carrier frequency can be the same or different.

Based on the above RAN4 agreement, the multi-Rx L3 measurement is applicable for both intra-frequency measurement and inter-frequency measurement. To enable the function, the UE can only be configured with one PCell and at most one FR2-1 MeasObject is configured. In addition, the discussion in RAN4 only focus on SSB-based L3 measurements, it does not touch CSI-RS based L3 measurements.

Proposal 1: 	Multi-Rx L3 measurement is only applicable when the UE is configured with FR2-1 PCell only, and there is only one SSB-based MO configured, the MO is FR2-1 carrier and can be different from PCell’s frequency.

Activation condition:

Agreement in RAN4#113: - RAN4 to introduce serving cell’s signal quality (i.e., based on RSRP and/or RSRQ threshold(s)) based triggering condition to activate/deactivate multi-Rx for L3 measurement.

Based on above RAN4 agreement, the network can configure either RSRP threshold or RSRQ threshold, or both. If network configures RSRP threshold and RSRQ threshold at the same time, in our understanding, it means the UE activates multi-Rx L3 measurements when both thresholds are satisified, similar to “not-at-cell-edge” evaluation in RRM relaxation (i.e. PCell’s RSRP is lower than the RSRP threshold and PCell’s RSRQ is lower than the RSRQ threshold).

Proposal 2: 		Separate RSRP threshold and RSRQ threshold are introduced. If both are configured, it means the UE activates multi-Rx L3 measurement when both of the thresholds are satisfied.

In addition, in RAN4, there was a hot discussion on whether to introduce TTT-based mechanism and hysteresis, the intention is to avoid Ping-pong effect. However, no consensus was made in RAN4, and it is assumed that RAN2 can discuss this and make decision.

Discussion in RAN4#114bis Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF - Issue a: Whether to limit the serving cell to FR2 serving cell: - Issue b: Whether to add the hysteresis and TimeToTrigger parameters: For Issue b: Option 1: the hysteresis and TTT is beneficial from RAN4 perspective (QC) Option 2: only the hysteresis is beneficial from RAN4 perspective (Nokia, E///, HW, Samsung) Option 3: leave to RAN2 (MTK, OPPO, ZTE, CATT, vivo, LGE, Apple)

From network perspective, although introducing TTT can avoid activating multi-Rx measurement due to one fluctuated measurement sample, this may also delay the action, then, mobility triggering may not be benefit from it. In our view, the whole mechanism is quite similar to s-Measure, that UE changes its measurement behavior based on only threshold (no TTT), so, we think it is fine to not define TTT for this.

Proposal 3: 	Do not consider TimeToTrigger and Hysteresis in activation condition.

Deactivation condition:

RAN4 indicates that RAN4 is still discussing the de-activation condition and re-triggering mechanism, from RAN2 perspective, at least deactivation should be triggered when the PCell quality does not meet the threshold anymore, however, considering the detailed mechanism will be specified in RAN4 spec. We can wait for RAN4 if additional signaling is needed.

Proposal 4: 	RAN2 assumes the UE at least deactivates multi-Rx L3 measurement when the threshold becomes not satisfied. FFS on other conditions, pending RAN4.

Based on above proposals, we provide the draft TP in Annex, for UE capability, we can wait for the feature list from RAN4.

Proposal 5: 	Adopt the RRC TP in Annex.

Conclusion

Based on the above, RAN2 is requested to discuss and agree on the following observations and proposals:

Proposal 1: 	Multi-Rx L3 measurement is only applicable when the UE is configured with FR2-1 PCell only, and there is only one SSB-based MO configured, the MO is FR2-1 carrier and can be different from PCell’s frequency.

Proposal 2: 		Separate RSRP threshold and RSRQ threshold are introduced. If both are configured, it means the UE activates multi-Rx L3 measurement when both of the thresholds are satisfied.

Proposal 3: 	Do not consider TimeToTrigger and Hysteresis in activation condition.

Proposal 4: 	RAN2 assumes the UE at least deactivates multi-Rx L3 measurement when the threshold becomes not satisfied. FFS on other conditions, pending RAN4.

Proposal 5: 	Adopt the RRC TP in Annex.

References

- R2-2503326/R4-2504962  LS on Rx BSF optimization for NR RRM Ph5    from RAN4

Annex---draft TP

#### –	MeasConfig

The IE MeasConfig specifies measurements to be performed by the UE, and covers intra-frequency, inter-frequency and inter-RAT mobility as well as configuration of measurement gaps.

MeasConfig information element

-- ASN1START

-- TAG-MEASCONFIG-START

MeasConfig ::=                      SEQUENCE {

measObjectToRemoveList              MeasObjectToRemoveList                                    OPTIONAL,   -- Need N

measObjectToAddModList              MeasObjectToAddModList                                    OPTIONAL,   -- Need N

reportConfigToRemoveList            ReportConfigToRemoveList                                  OPTIONAL,   -- Need N

reportConfigToAddModList            ReportConfigToAddModList                                  OPTIONAL,   -- Need N

measIdToRemoveList                  MeasIdToRemoveList                                        OPTIONAL,   -- Need N

measIdToAddModList                  MeasIdToAddModList                                        OPTIONAL,   -- Need N

s-MeasureConfig                     CHOICE {

ssb-RSRP                            RSRP-Range,

csi-RSRP                            RSRP-Range

}                                                                                             OPTIONAL,   -- Need M

quantityConfig                      QuantityConfig                                            OPTIONAL,   -- Need M

measGapConfig                       MeasGapConfig                                             OPTIONAL,   -- Need M

measGapSharingConfig                MeasGapSharingConfig                                      OPTIONAL,   -- Need M

...,

[[

interFrequencyConfig-NoGap-r16      ENUMERATED {true}                                         OPTIONAL    -- Need R

]],

[[

effectiveMeasWindowConfig-r18       SetupRelease {MeasWindowConfig-r18}                       OPTIONAL    -- Need M

]],

[[

multiRxMeasConfig-r19               SEQUENCE {

ssb-RSRP-r19                        RSRP-Range                                            OPTIONAL,    -- Need R

ssb-RSRQ-r19                        RSRQ-Range                                            OPTIONAL     -- Need R

}                                                   																 OPTIONAL     -- Cond PCell

]]

}

MeasObjectToRemoveList ::=              SEQUENCE (SIZE (1..maxNrofObjectId)) OF MeasObjectId

MeasIdToRemoveList ::=                  SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasId

ReportConfigToRemoveList ::=            SEQUENCE (SIZE (1..maxReportConfigId)) OF ReportConfigId

-- TAG-MEASCONFIG-STOP

-- ASN1STOP

MeasConfig field descriptions

effectiveMeasWindowConfig Used to setup and release effective measurement window in NR for E-UTRA measurements.

interFrequencyConfig-NoGap-r16 If the field is set to true, UE is configured to perform SSB based inter-frequency measurement without measurement gaps when the inter-frequency SSB is completely contained in the active DL BWP of the UE, as specified in TS 38.133 [14], clause 9.3. Otherwise, the SSB based inter-frequency measurement is performed within measurement gaps. In NR-DC, the field can only be configured in the measConfig associated with MCG, and when configured, it applies to all the inter-frequency measurements configured by MN and SN.

measGapConfig Used to setup and release measurement gaps in NR.

measIdToAddModList List of measurement identities to add and/or modify.

measIdToRemoveList List of measurement identities to remove.

measObjectToAddModList List of measurement objects to add and/or modify.

measObjectToRemoveList List of measurement objects to remove.

multiRxMeasConfig-r19 Threshold for NR PCell SSB RSRP and RSRQ measurement controlling when the UE is required to perform multi-Rx based L3 measurement, as specified in TS 38.133 [14], clause X.

reportConfigToAddModList List of measurement reporting configurations to add and/or modify.

reportConfigToRemoveList List of measurement reporting configurations to remove.

s-MeasureConfig Threshold for NR SpCell RSRP measurement controlling when the UE is required to perform measurements on non-serving cells. Choice of ssb-RSRP corresponds to cell RSRP based on SS/PBCH block and choice of csi-RSRP corresponds to cell RSRP of CSI-RS. This field is also applicable to L1 measurements configured with ltm-CSI-ResourceConfigToAddModList-r18. This field is not configured to a UE configured with event(s) related to the L2 U2N Relay UE.

measGapSharingConfig Specifies the measurement gap sharing scheme and controls setup/ release of measurement gap sharing.

| Conditional Presence | Explanation |
| --- | --- |
| PCell | This field is optional present, Need R, when the UE is only configured with PCell operating on FR2-1 and there is only one measObjectNR configured which is associated with SSB carrier of FR2-1. Otherwise, it is absent, Need R. |