3GPP TSG-RAN WG2 #130                                          R2-2504032

Malta, MT, May 19h – 23rd, 2025

Agenda Item:	8.7.5 RLC AM Enhancements

Source:	NEC

Title:	Remaining details on RLC AM enhancements

Document for:	Discussion, Decision

# 1.Introduction

In this document, we discuss following remaining details:

- Discard the outdated PDUs even if its transmission has been attempted, for specification clarity

- independent or joint capability/configuration for DL and UL regarding stopping retransmission

- RETX-COUNT and RLF

- Efficient polling

# 2 Discussion

## 2.1 Discard the outdated PDUs

Because legacy RLC AM never give up retransmission unless it is ACKed or RLF is triggered, as shown below, when discard indication of a SDU is received from PDCP, the transmitting side cannot discard the SDU if its transmission has been attempted. However, if stop retransmission for outdated SDU is enable, this condition shall be lifted. There is no point to store them

Legacy text:

## 5.4 SDU discard procedures When indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, the transmitting side of an AM RLC entity or the transmitting UM RLC entity shall discard the indicated RLC SDU, if neither the RLC SDU nor a segment thereof has been submitted to the lower layers. The transmitting side of an AM RLC entity shall not introduce an RLC SN gap when discarding an RLC SDU.

Suggested TP:

## 5.4 SDU discard procedures When indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, the transmitting side of an AM RLC entity or the transmitting UM RLC entity shall: If [x] is not configured: - discard the indicated RLC SDU, if neither the RLC SDU nor a segment thereof has been submitted to the lower layers. The transmitting side of an AM RLC entity shall not introduce an RLC SN gap when discarding an RLC SDU. Else if [x] is configured - discard the indicated RLC SDU (consequently, this RLC SDU and any segment thereof are not pending for initial transmission or retransmission)

Once these outdated PDUs are discarded, they will not be considered for transmission/retransmission, polling, data volume calculation for BSR/DSR report. Otherwise, many places in specification have to be updated with exceptional sentence, one example is as follows:

## 5.3.3.4 Expiry of t-PollRetransmit Upon expiry of t-PollRetransmit, the transmitting side of an AM RLC entity shall: - if both the transmission buffer and the retransmission buffer are empty (excluding transmitted RLC SDU or RLC SDU segment awaiting acknowledgements and excluding RLC SDUs or RLC SDU segments for which the transmission and retransmission are stopped); or - if no new RLC SDU or RLC SDU segment can be transmitted (e.g. due to window stalling): - consider the RLC SDU with the highest SN among the RLC SDUs submitted to lower layer for retransmission; or - consider any RLC SDU which has not been positively acknowledged for retransmission. - include a poll in an AMD PDU as described in clause 5.3.3.2.

With the understanding that RLC AM will not retransmit the discarded/outdated RLC SDU, these are not SDUs pending for retransmission. They will not be taken into account for data volume calculation, polling, retransmission etc.

Proposal 1: Update the section “SDU discard procedures” to allow the transmitting side of an AM RLC entity to discard the indicated RLC SDU without exception, consequently outdated SDU is discarded, not pending for retransmission, and not for triggering polling etc.

## 2.2 Capability and configuration on stopping retransmission

It is obvious this feature is only for RLC AM DRB with very short latency requirements, and a UE usually can be configured with multiple DRBs. So, the feature should be enabled per AM DRB under NW control.

One question is whether we should report and enable this feature in DL and UL independently or jointly? With independent way:

- In DL direction, UE first reports if it supports this feature in DL direction, NW then configures the new timer for Rx operation at UE. This will enable the feature in DL direction.

- In UL direction, UE first reports if it supports this feature in UL direction. Because there is neither new timer nor new threshold configuration is needed, as captured in running CRs, a enable bit for UL direction is added.

If joint enabling is agreed, there is no need to have enable bit. Presence of the new timer would enable the feature in both DL and UL direction, and also one capability bit would be enough.

Notice that RLC AM entity is bi-direction entity, but the configuration of UL and DL direction are independently currently. So independent way is aligned with other configurations, while joint way will be simpler. The question would be: is it possible to enable this feature in one direction but not in another direction for the same AM DRB? A step further into essence, is it a possible case that the short delay requirement outweighs lossless transmission in one direction, while lossless transmission outweighs short delay requirement in another direction for the same AM DRB?

Proposal 2: RAN2 discuss if the feature “skip outdated/abandoned SDU” should be supported by UE/enabled by NW jointly or independently for DL and UL directions:

- If go with independent way: a enable bit is needed for UL direction, two capability bits are needed

- If go with joint way: enable bit is not needed, presence of new timer configuration will implicitly enable the feature in both DL and UL directions and one capability bit is enough

## 3.3 RETX_COUNT and RLF

RLF could be tiggered when RETX_COUNT reaches maxRetxThreshold. RETX_COUNT is maintained per RLC SDU. RETX_COUNT is initiated as 0 when the RLC SDU is considered for retransmission for the first time, and will increment whenever another retransmission is triggered/considered as following:

When an RLC SDU or an RLC SDU segment is considered for retransmission, the transmitting side of the AM RLC entity shall: - if the RLC SDU or RLC SDU segment is considered for retransmission for the first time: - set the RETX_COUNT associated with the RLC SDU to zero. - else, if it (the RLC SDU or the RLC SDU segment that is considered for retransmission) is not pending for retransmission already and the RETX_COUNT associated with the RLC SDU has not been incremented due to another negative acknowledgment in the same STATUS PDU: - increment the RETX_COUNT. - if RETX_COUNT = maxRetxThreshold: - indicate to upper layers that max retransmission has been reached.

Following two open issues are relevant to each other:

- Whether RLF upon maximum retransmission should be disabled for enhanced RLC AM

- whether RETX_COUNT should increment as well when auto-retransmission is triggered/considered.

Upon maximum times of ARQ retransmissions reaches, it means a RLC PDU cannot be successfully delivered after (x times of ARQ retransmissions * y times of HARQ retransmissions). This was considered as a rare case and RLF can be triggered, because firstly we assume that x * y would not be a small number, secondly RLC AM never advance transmission window and give up retransmission unless successful reception of the RLC PDU is confirmed i.e. aim for lossless delivery. These assumptions have been changed with enhanced AM RLC: firstly x * y could be a small value for the short PDB service which we are working on; secondary, RLC AM may stop retransmission when PDB of a RLC SDU runs out and hence the maximum retransmissions times will never reaches. It becomes questionable whether RLF should be triggered at all for enhanced AM RLC entities.

If RLF is skipped, then we do not need to maintain RETX_COUNT if enhanced AM RLC featured is configured.

If RLF is still considered, then we need to discuss the second issue. Auto retransmission will speed up retransmission, it impacts how fast the maximum retransmissions will reaches, or in another word it will impact how many retransmissions it can do within its PDB. NW could configure a higher maximum retransmission time if auto retransmission is counted. Because we agreed auto retransmission is only triggered once, it is simple to adjust the configuration. If auto-retransmission is not counted, and maximum retransmission times configuration could be kept the same, we slightly prefer later way for simplicity.

Proposal 3: RAN2 discuss following options regarding RLF and RETX_COUNT for enhanced AM RLC entity

Option1: RETX_COUNT and RLF trigger part is skipped (our preference)

Option2: keep RETX_COUNT and RLF trigger part, auto-retransmission is not counted into RETX_COUNT

Option3: keep RETX_COUNT and RLF trigger part, auto-retransmission is counted into RETX_COUNT

## 3.4 Efficient polling

Some companies concerned excessive polling based on remaining time e.g.:

- Each RLC SDU in TX window maintains a different remaining time, so they may trigger polling one after another.

- Remaining time of a RLC SDU will keep being lower than the threshold once it happens, this RLC SDU can keep on polling

Excessive polling itself will not cause extra signalling overhead, because it is just a bit setting between 0 and 1. However it may result in excessive SR. Currently SR frequency is limited by two factors a) HARQ reordering time and b) t-StatusProhibit:

Triggers to initiate STATUS reporting include: - Polling from its peer AM RLC entity: - When an AMD PDU with SN = x and the P field set to "1" is received from lower layer, the receiving side of an AM RLC entity shall: - if the AMD PDU is to be discarded as specified in clause 5.2.3.2.2; or - if x < RX_Highest_Status or x >= RX_Next + AM_Window_Size: - trigger a STATUS report. - else: - delay triggering the STATUS report until x < RX_Highest_Status or x >= RX_Next + AM_Window_Size. NOTE 1: This ensures that the RLC Status report is transmitted after HARQ reordering. ///////////skip irrelevant//// When STATUS reporting has been triggered, the receiving side of an AM RLC entity shall: - if t-StatusProhibit is not running: - at the first transmission opportunity indicated by lower layer, construct a STATUS PDU and submit it to lower layer. - else: - at the first transmission opportunity indicated by lower layer after t-StatusProhibit expires, construct a single STATUS PDU even if status reporting was triggered several times while t-StatusProhibit was running and submit it to lower layer.

Assuming Tx side keep polling as long as any RLC SDU’s remaining time is lower than the threshold, and polling bit will be sent for every AMD PDU transmission, but SR will not be sent more frequently than every prohibit time or every HARQ reordering time. it seems ok to keep polling as long as a faster SR is required:

Proposal 4: RAN2 confirm that excessive polling will lead to neither excessive SR nor excessive signalling overhead

Besides, not all RLC SDU or RLC SDU segment having remaining time lower than the polling threshold should trigger polling. For these RLC SDU or RLC SDU segments which has been ACKed or already pending for retransmission, further feedback is not necessary, hence:

proposal 5: only for these RLC SDUs or RLC SDU segments waiting for acknowledgment in the TX buffer, remaining time-based polling could be triggered

One more FFS as below is added during email discussion:

FFS how to handle the case “ if there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, there is no SDU to retransmit the poll with”

Firstly as proposed in P5, in above case, polling is not needed because no SDUs in the buffer is waiting for acknowledgment.

Secondly, this case has exists even in legacy procedure. As follows, polling will be included only when there is transmission opportunity and there is buffered SDU submitted for transmission or retransmission. In case there is no buffered SDU for transmission or retransmission, even if polling/polling retransmission is triggered, TX will not be able to send out polling bit.

Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission such that the AMD PDU contains either a not previously transmitted RLC SDU or an RLC SDU segment containing not previously transmitted byte segment, the transmitting side of an AM RLC entity shall: - increment PDU_WITHOUT_POLL by one. - increment BYTE_WITHOUT_POLL by every new byte of Data field element that it maps to the Data field of the AMD PDU; - if PDU_WITHOUT_POLL >= pollPDU; or - if BYTE_WITHOUT_POLL >= pollByte: - include a poll in the AMD PDU as described below. Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission, the transmitting side of an AM RLC entity shall: - if both the transmission buffer and the retransmission buffer becomes empty (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) after the transmission of the AMD PDU; or - if no new RLC SDU can be transmitted after the transmission of the AMD PDU (e.g. due to window stalling); - include a poll in the AMD PDU as described below.

The case mentioned in FFS in our understanding is more about TX window stalling, if we need to solve it ,we should allow the Tx window move forward considering the SDU whose transmission stops.

Proposal 6: In case “there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, there is no SDU to retransmit the poll with”, RAN2 confirm no need to poll or retransmit the poll for any acknowledgement (no spec change)

# 3 Conclusion

In this contribution, we provided the following observations and proposals regarding the details of RLC AM enhancement:

Proposal 1: Update the section “SDU discard procedures” to allow the transmitting side of an AM RLC entity to discard the indicated RLC SDU without exception, consequently outdated SDU is discarded, not pending for retransmission, and not for triggering polling etc.

Proposal 2: RAN2 discuss if the feature “skip outdated/abandoned SDU” should be supported by UE/enabled by NW jointly or independently for DL and UL directions:

- If go with independent way: a enable bit is needed for UL direction, two capability bits are needed

- If go with joint way: enable bit is not needed, presence of new timer configuration will implicitly enable the feature in both DL and UL directions and one capability bit is enough

Proposal 3: RAN2 discuss following options regarding RLF and RETX_COUNT for enhanced AM RLC entity

Option1: RETX_COUNT and RLF trigger part is skipped (our preference)

Option2: keep RETX_COUNT and RLF trigger part, auto-retransmission is not counted into RETX_COUNT

Option3: keep RETX_COUNT and RLF trigger part, auto-retransmission is counted into RETX_COUNT

Proposal 4: RAN2 confirm that excessive polling will lead to neither excessive SR nor excessive signalling overhead

proposal 5: only for these RLC SDUs or RLC SDU segments waiting for acknowledgment in the TX buffer, remaining time-based polling could be triggered

Proposal 6: In case “there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, there is no SDU to retransmit the poll with”, RAN2 confirm no need to poll or retransmit the poll for any acknowledgement (no spec change)