3GPP TSG-RAN WG2 Meeting #130	R2-2504090

Malta, Malta, 19th – 23rd May, 2025

Agenda item:	8.9.2

Source:	Samsung

Title:	Open issues on Store and Forward operation

WID/SID:	IoT_NTN_Ph3-Core

Document for:	Discussion and Decision

# Introduction

In RAN#107 the latest WID for IoT NTN Phase 3 was agreed [1], where S&F objective is the following:

The aim of this WI is enhancement of IoT-NTN with the following objectives: - Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - Specify necessary enhancements for full eNB as regenerative payload e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

In this contribution, we provide some discussions related Store and Forward.

# Discussion

- In the last meeting (RAN2#129bis), the following was agreed:

Agreements:

1.	For UEs supporting S&F, the UE AS indicates the information on transition time (if any) from current "S&F operation mode" to "normal mode" (if received) to the upper layers. Whether/how it is used by the upper layers is up to CT1.

2.	We introduce an indication in system information for the normal mode to S&F mode transition, at least for NAS use. FFS on the details (e.g. whether we can link this to other existing information). The information on transition time for the normal mode to S&F mode transition is sent from AS to NAS

3.	We don’t introduce any new release cause related to S&F operation

## On SA2 S&F Monitoring list

Over the last couple of meetings, RAN2 has discussed the meaning of the S&F monitoring list introduced by SA2. The last 23.401 partly explains how the S&F monitoring list is supposed to be used:

--------------------------- 23.401 V19.2.0 ---------------------------

For a UE which indicates support of Store and Forward Satellite operation and when an MME is operating in S&F Mode:

-	If the MME cannot complete a NAS procedure with the information currently available on the satellite e.g. when the MME does not have UE security context or, if the MME needs to retrieve UE-specific authentication vectors or subscription information from the ground network, it shall reject the NAS procedure. In this case, the MME shall include a reject cause indicating the NAS rejection is due to Store and Forward Satellite operation.

-	If the UE is rejected with a reject cause indicating it is due to S&F operation, the UE's EMM state shall remain unchanged.

-	The MME may provide to the UE a S&F Wait Timer, a S&F Monitoring List or both when accepting or rejecting a NAS procedure.

NOTE 4:	How the MME determines the S&F Wait Timer and S&F Monitoring List is up to MME implementation, e.g. based on feeder link (un)availability period, service link (un)availability period, UE power saving requirements, Communication Pattern parameters, UE location, UE mobility, etc.

-	When the S&F Wait Timer expires, the UE may perform a NAS procedure, which can be a subsequent NAS procedure or a reattempt of a NAS procedure previously rejected with a S&F reject cause.

NOTE 5:	When the S&F Wait Timer is running, the power consumption optimization behaviours, if any, are left for UE implementation e.g. whether to listen to paging or deactivate its Access Stratum functions.

The S&F Monitoring List includes satellite(s) which belong to the same PLMN and indicates the satellite(s) that the UE may (re)attempt NAS procedures or receive MT data from.

-	The MME may indicate to the UE that it should delete any previously provided S&F Monitoring List for the current PLMN. When the S&F Monitoring List is deleted then the UE may use any satellite(s).

NOTE 6:	The S&F Wait Timer or S&F Monitoring List doesn't affect the UE when accessing an eNodeB that does not broadcast an indication of operating in S&F Mode.

NOTE 7:	How UE behaves when receiving the S&F Monitoring List is up to UE implementation. When a UE receives a S&F Monitoring List and the UE access a satellite that supports Store and Forward Satellite operation that is not on the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure. The UE can continue to use the previously provided S&F Monitoring List, if the MME did not send one and the UE has previously been provided with one.

--------------------------- 23.401 V19.2.0 ---------------------------

- In other words, the UE is not prevented from performing any type of action. Instead the UE can use the list in order to find satellites where it can perform NAS procedures and expect MT data. From the point of view of AS, the important parts of how a UE operates can be found in the Notes 5 and 7. Both Note 5 and 7 state that it is mostly up to UE implementation how to operate when receiving the S&F Wait timer and S&F Monitoring List. However, we do not think that it can be fully up to UE implementation how the UE behaves when receiving the UE implementation, at least not in AS layers. This is because AS layers have requirements, for instance idle mode operation that cannot be up to UE implementation.

Observation 1: It cannot be fully up to UE implementation in the AS layer how to behave during S&F when S&F monitoring list is received.

- We believe that S&F monitoring list shall allow a UE to sleep when a satellite passes by that is not part of the S&F monitoring list. According to current specifications, there are no mechanisms to allow for this. We believe that this type of behaviour shall be similar to discontinuous coverage. In discontinuous coverage, the UE is provided with a set of TLE Ephemeris elements and other assistance information that describe how a satellite and its cells move. This is then used by the UE to determine when to wake up in order to camp or to perform MO or MT access. When the UE has determined that there is no coverage, the UE may not perform the AS procedures related to NTN.

- Thus we believe that the S&F Monitoring List signalled over NAS shall be used by the AS layer to determine when to wake up for NAS and MT access, i.e for the UE to perform discontinuous coverage. As an example, the MME may provide an S&F Monitoring List indicating satellite IDs 3, 4, 7, 10. In the current cell that the UE is connected to, the cell may broadcast SIB32 where the satellite IDs 1, 3, 4 and 6 are indicated. Then the UE may perform discontinuous coverage to wake up to camp on satellite with satellite ID 3 and 4, but may also wake up for satellite with satellite ID 1 and 6, for instance to receive Long Term Ephemeris for more satellites. An example of this can be seen in Figure 1. In line with this, we propose:

Proposal 1: The S&F monitoring list shall allow a UE to not perform its NTN idle mode tasks if a UE only has coverage from a satellite that is not part of the S&F monitoring list. When the UE is covered by a satellite that is a part of the S&F monitoring list, the UE performs the NTN idle mode tasks.

Proposal 2: The S&F monitoring list is used in AS for UE to perform discontinuous coverage operation on the satellites indicated in the monitoring list.

Figure 1. Using the S&F Monitoring list with discontinuous coverage.

## Open issues on Store and Forward

- Neighbour cell assistance information

- It has been suggested by several companies to allow for introducing a new indication related to the S&F operation of a neighbouring cell. However, in our view this would be difficult to maintain. Typically the neighbour cell system information does not change quickly, so it may be challenging for a cell to maintain information that is likely to change rather dynamically. Furthermore, the UE cannot be expect to reacquire neighbour cell system information too often to check whether a cell is operating in S&F mode or not. For this reason we do not think it would be particularly useful to have neighbour cell signalling related to Store and Forward operation.

Proposal 3: Do not introduce neighbour cell signalling related to Store and Forward.

- Paging monitoring

- Some enhancements related to Paging monitoring to allow for a UE to save power have been proposed. For instance, it has been suggested that if the S&F satellite does not have any downlink data, then the UE may not need to monitor for paging. This however ignores the reality that paging is not only monitored for downlink data, but also for system information update, which is expected to occur often in a non-terrestrial network due to the moving satellites. Also, we believe that power saving should come from allowing the UE to sleep based on the S&F monitoring list as explained in Section 2.1.

Observation 2: Paging is not only monitored for downlink data, but also for system information update.

Proposal 4: Do not introduce enhancements specific to paging monitoring for S&F.

# Conclusion

In this contribution, we discussed issues related to S&F network:

Observation 1: It cannot be fully up to UE implementation in the AS layer how to behave during S&F when S&F monitoring list is received.

Observation 2: Paging is not only monitored for downlink data, but also for system information update.

Proposal 1: The S&F monitoring list shall allow a UE to not perform its NTN idle mode tasks if a UE only has coverage from a satellite that is not part of the S&F monitoring list. When the UE is covered by a satellite that is a part of the S&F monitoring list, the UE performs the NTN idle mode tasks.

Proposal 2: The S&F monitoring list is used in AS for UE to perform discontinuous coverage operation on the satellites indicated in the monitoring list.

Proposal 3: Do not introduce neighbour cell signalling related to Store and Forward.

Proposal 4: Do not introduce enhancements specific to paging monitoring for S&F.

# Reference

RP-250472, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, Mediatek, RAN#107, Incheon, ROK, March 2025.