3GPP TSG-RAN WG2 Meeting #130	R2-2504101

St. Julian’s, Malta, 19 – 23 May 2025

Agenda item:	8.1.3

Source:	Nokia

Title:	Data Collection for Training of network-side ML models

WID/SID:	NR_AIML_air-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

This contribution elaborates on the open issues related to data collection for network-side models for beam management. The issues addressed are related to open points from the previous RAN2 meetings as well as significant ones from the running CR for TS 38.331.

# 2	Discussion

## 2.1	Open Issues

### 2.1.1	(RRC-19): Reporting assistance information related to logged measurements

Issue description: It is not yet clear what the otherConfig should contain to enable the UE to report assistance information via UAI, related to logging of radio measurements. For instance, should the low power, buffer full, and buffer threshold reached indications be all configured with a single bit, or should the configurations be separated?

The issue is quite straightforward as UE’s internal status, regardless of low power state or buffer issue, would impact the ongoing data collection process for network side models, it is better to configure all with a single bit. Thus, network could take necessary actions based on whether UE is indicating low power state and buffer full concurrently or UE is indicating low power state and buffer threshold reached separately.

Proposal 1: Allow reporting of all assistance information (low power state, buffer full, buffer threshold reached indicators) related to logged measurements to be configured with a single bit.

### 2.1.2	(RRC-20): Further procedures for UE assistance information related to logging

Issue description: It has not yet been discussed whether further procedures for UE reporting assistance information related to logging is need. Such further procedures may be, e.g. prohibit timers, indication that battery state is not low any longer, indication that the memory is not full any longer, etc. The issue is captured as an editor’s note in the running CR, in clause 5.7.4.2 and 5.7.4.3.

UE sends a data availability indication (such as threshold reached, buffer full, low power) to assist network with further actions. There are multiple ways that can be adapted to avoid too frequent indications:

In one scenario, UE can only notify the network once per indication. That is, UE can indicate in a sequence such as, buffer threshold exceeds, buffer is full, and low power state. UE may not send further indication until the network may take actions, such as retrieve the data or deconfigure the UE of data collection configuration. Alternatively, UE can itself decide when to send the next indication. Another approach could be, UE can use a prohibit timer configured by network. This allows UE to explicitly wait until the timer expires.

Proposal 2: To avoid too frequent sending of UE assistance information related to logging, adapt one of the following options:

a) UE can only notify the network once per indication and wait until network takes an action.

b) UE can itself decide when to send the next indication.

c) Introduce a prohibit timer.

In Release 19, if UE is in low power, then UE indicates to the gNB a low power indication. gNB can then de-configure the UE. If UE is able to resume data collection, it can send UAI message without low power indication, which indicates to the gNB that UE is not in lower power state.

Observation 1: UE can send a UAI message without a low power state indication if UE’s power state is no longer low.

Regarding the buffer full indication, whenever the UE sends logged data to the gNB, the UE releases the sent data from its buffer and can continue data collection. Therefore, no separate indication is needed to explicitly inform the gNB that the memory is no longer full.

Proposal 3: No additional indication needed to indicate that the buffer is not full.

### 2.1.3	(RRC-23) Cell ID stored with logged data for NW-side data collection

Issue description: It has not been clarified what type of cell ID the UE needs to log along with the logged data, in order to unambiguously identify the cell in which the UE performed the data logging, e.g. CGI, PCI-ARFCN etc.

The following agreements are related to this issue.

- RAN2#129 - UE retains logged data during handover (HO).  FFS if there is scenarios where the UE needs to release the data and how does the UE know and if control from network is needed

- RAN2#129 - UE indicates availability of logged data during handover (i.e., within the RRCReconfigurationComplete message) (if data is retained in the UE).

- RAN2#129b - Introduce 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration during/before HO.  Source gNB decides whether the data should be kept.  The indication is provided in RRCReconfiguration (i.e. not in RRC Reconfiguration from target cell).   FFS signaling details.

Thus far, we have agreed that a target cell can retrieve logged data for network-side data collection from a UE after handover and transmit that logged data to the source cell. We have not discussed anything about multiple hops of this logged data. Since the data collected could be as small as 64KB, we do not see the reason that a target cell would not have sufficient time to retrieve and forward the data.

Observation 2: After handover, the target cell should have sufficient time to retrieve and transfer the logged data for network-side data collection from a UE to the source cell.

While waiting for the target cell to transfer logged data for network-side data collection from a UE, the source cell must retain the context of the UE which was handed over, and it should not have to do so indefinitely.

Observation 3: While waiting for the target cell to transfer logged data for network-side data collection from a UE, the source cell must retain the context of the UE which was handed over, and it should not have to do so indefinitely.

Proposal 4: Only the immediate target cell can retrieve logged data for network-side data collection from a UE after handover.

Because the only cell which should transmit the logged data for network-side data collection to the source cell is the immediate target cell, the cell ID is not needed to be stored with the logged data.

Observation 4: The UE will already store the serving cell index with its data samples to differentiate data collected for different serving cells, e.g., in the case of carrier aggregation.

Observation 5: Since the target cell knows the identity of the source cell, a cell ID is not required to be stored with logged data for network-side data collection.

Proposal 5: The cell ID is not included in the logged data for network-side data collection.

### 2.1.4	(RRC-24) Where to include logging configuration from NW to UE

Issue description: It is FFS whether the logging configuration is included in the CSI framework (whether in CSI-ReportConfig or directly under CSI-MeasConfig) or at L3.

There was a strong desire to use a unified procedure for all AI/ML based use cases for logging configurations. From RAN1#116bis, it is clarified that existing CSI framework can provide necessary support for configuration of network-side AI/ML model for BM-Case1 and BM-Case2. It was agreed, in RAN1#120 that for data collection for UE-side models for beam management, to support CSI-ReportConfig without generating a CSI report. In RAN1#120bis meeting it was agreed, for training data collection for UE-side model for CSI prediction use case, to support the use of CSI-ReportConfig to configure resources for data collection. Here we observe that different CSI related use cases can use a common CSI-ReportConfig for logging configuration. Therefore, we propose to adhere to a common approach for resource configurations for both network side and UE-side data collection that is to support CSI-ReportConfig without a CSI report.

Proposal 6: To harmonize different CSI use cases, use CSI-ReportConfig for network-side data collection within CSI-MeasConfig.

To differentiate between the UE-side data collection and NW-side data collection, the appropriate reportQuantity would be used, or we could use a new IE such as loggingQuantity and use separate quantities for UE-side and network-side data collection.

Proposal 7: Include logging configurations for network-side DC inside of CSI-ReportConfig, using new reportQuantity related to logging or using a new type of quantity, e.g., loggingQuantity, either of which would differentiate data collection between UE-side models and network-side models.

### 2.1.5 	(RRC-26) Multiplexing legacy logged data and AIML logged data in new SRB

Issue description: RAN2#129bis agreed that

2.	New SRB can be configured for NW-side data collection  (with lower priority)

Given the agreement on the new SRB for the transmission of the UEInformationResponse, Rapporteur’s understanding is that the new SRB may be used also for the transmission of the legacy SON/MDT reports (e.g. logged MDT measurements, RLF-Report, RA-reports, successful HO reports, etc.), whenever the UEInformationResponse carries both a legacy SON/MDT report and AIML logged data. However, this would impact how legacy SON/MDT reporting is performed which can only be at the moment on SRB1 or SRB2.

The running CR and this open issue imply that the same UEInformationResponse should be able to include logged data configured by SON/MDT and for network-side data collection. It should be first noted that SON/MDT logged data would typically be retrieved nearly immediately after a UE transitions from IDLE or INACTIVE to CONNECTED. On the other hand, logged data for network-side data collection would typically be retrieved by the same cell which configured UE or by a target cell attempting to retrieve logged data for network-side data collection on behalf of the source cell. Therefore, it isn’t necessarily likely that both logged data for SON/MDT and logged data for network-side data collection would become available at the same time.

Observation 6: It is unlikely that both logged data for SON/MDT and logged data for network-side data collection would become available at the same time.

Therefore, we propose to disallow the simultaneous request for logged data for SON/MDT and logged data for NW-side data collection.

Proposal 8: Disallow the simultaneous request for logged data for SON/MDT and logged data for network-side data collection.

### 2.1.6	(RRC-27) Logging periodicity

Regarding the editor’s note: “Editor's Note: FFS whether the periodicity of the logging is configurable” in the post email discussion, the motivation of having a configurable periodicity for the logging is not clear. Our understanding is that the periodicity of the logging of measurements can be the same as the RS transmission periodicity. This is straightforward and has no additional impact on the UE behaviour. Thus, this can be achievable in Rel-19 framework.

Proposal 9: No logging periodicity to be introduced in Release 19. The UE should assume the same logging periodicity as the reference signal transmission periodicity received in the measurement configuration.

### 2.1.7	(RRC-29): Whether data availability indication should be sent when the UE has data below the threshold and low power state is sent, and what cause should be included then

Whether data availability indication should be sent when the UE has data and low power state is sent and what cause should be included then. The UE may have some data when indicating low power state. Even though this data volume may be lower than full buffer/threshold, it is still useful to let the network know about that so that the data can be fetched.

When UE is in low power state, RAN2 agreed to indicate a low power state flag to the network. The issue above states that UE may have some data in the buffer and the buffer limits have not reached a threshold or full size. This data might be useful for the network. Now, the question is whether the low power state indication to be acted as an implicit indication of data availability or not. In our opinion, the UE may assume a threshold equal to one sample of data which can be indicated via the buffer threshold has been exceeded. This would allow network to know that UE has low power and the data is available. Therefore, a separate data availability cause might not be required along with low power state.

Proposal 10: If the UE is in a low power state AND it has data in its buffer, even below the threshold, UE can still indicate both low power state and buffer threshold met indications. That is, no additional cause or flag is required.

## 2.2	Logged Measurements Transmission

### 2.2.1	Logged Data Handling During Handover

The legacy immediate MDT solution does not support of continuing and reporting MDT after a handover. Therefore, the decision of keeping the data at HO has RAN3 impact due to the potentially necessary data forwarding and may have SA5 impact due to potential immediate MDT impacts. Thus, we think that RAN2 should inform RAN3 and SA5 about this decision.

Proposal 11: Send an LS to RAN3/SA5 providing the background information and the RAN2 agreements of retaining logged data at handover and asking RAN3/SA5 to introduce the necessary enhancements in their specifications in Rel-19 that enable the support of the RAN2 agreements. (See draft LS in Annex.)

### 2.2.2	Discarding of Logged Data upon Receiving Same Measurement Configuration(s) as for Logged Data

While a UE is configured with a logged measurement configuration for network-side data collection, the gNB knows the context of the reports being generated by the ID through the reference to the logged measurement configuration ID. However, we must consider what happens when the gNB releases the configuration from the UE. On the gNB side, once the configuration is released, normally, the gNB would lose the context of the configuration, rendering the referenced to the logged measurement configuration ID in the measurement log un-interpretable.

Observation 7: The gNB may lose the context of measurements logged and reported by the UE if it releases the measurement logging configuration from the UE.

Releasing the configuration should not cause the discarding of measurements as we have agreed to support handover. However, if a new logged measurement configuration is configured in place of an old one and is identified by the same logged measurement configuration ID, the UE shall discard the old samples because it would be nearly impossible for the gNB to properly identify which samples were of an old or new logged measurement configuration with the same ID.

Proposal 12: If a new logged measurement configuration is configured in the UE with the same ID as an existing logged measurement configuration, the UE shall discard samples belonging to the old configuration.

# 3	Conclusion

This document has made the following observations:

Observation 1:

Observation 1: UE can send a UAI message without a low power state indication if UE’s power state is no longer low.

Observation 2: After handover, the target cell should have sufficient time to retrieve and transfer the logged data for network-side data collection from a UE to the source cell.

Observation 3: While waiting for the target cell to transfer logged data for network-side data collection from a UE, the source cell must retain the context of the UE which was handed over, and it should not have to do so indefinitely.

Observation 4: The UE will already store the serving cell index with its data samples to differentiate data collected for different serving cells, e.g., in the case of carrier aggregation.

Observation 5: Since the target cell knows the identity of the source cell, a cell ID is not required to be stored with logged data for network-side data collection.

Observation 6: It is unlikely that both logged data for SON/MDT and logged data for network-side data collection would become available at the same time.

Observation 7: The gNB may lose the context of measurements logged and reported by the UE if it releases the measurement logging configuration from the UE.

And proposed the following:

Proposal 1: Allow reporting of all assistance information (low power state, buffer full, buffer threshold reached indicators) related to logged measurements to be configured with a single bit.

Proposal 2: To avoid too frequent sending of UE assistance information related to logging, adapt one of the following options:

a) UE can only notify the network once per indication and wait until network takes an action.

b) UE can itself decide when to send the next indication.

c) Introduce a prohibit timer.

Proposal 3: No additional indication needed to indicate that the buffer is not full.

Proposal 4: Only the immediate target cell can retrieve logged data for network-side data collection from a UE after handover.

Proposal 5: The cell ID is not included in the logged data for network-side data collection.

Proposal 6: To harmonize different CSI use cases, use CSI-ReportConfig for network-side data collection within CSI-MeasConfig.

Proposal 7: Include logging configurations for network-side DC inside of CSI-ReportConfig, using new reportQuantity related to logging or using a new type of quantity, e.g., loggingQuantity, either of which would differentiate data collection between UE-side models and network-side models.

Proposal 8: Disallow the simultaneous request for logged data for SON/MDT and logged data for network-side data collection.

Proposal 9: No logging periodicity to be introduced in Release 19. The UE should assume the same logging periodicity as the reference signal transmission periodicity received in the measurement configuration.

Proposal 10: If the UE is in a low power state AND it has data in its buffer, even below the threshold, UE can still indicate both low power state and buffer threshold met indications. That is, no additional cause or flag is required.

Proposal 11: Send an LS to RAN3/SA5 providing the background information and the RAN2 agreements of retaining logged data at handover and asking RAN3/SA5 to introduce the necessary enhancements in their specifications in Rel-19 that enable the support of the RAN2 agreements. (See draft LS in Annex.)

Proposal 12: If a new logged measurement configuration is configured in the UE with the same ID as an existing logged measurement configuration, the UE shall discard samples belonging to the old configuration.

# Annex: Draft LS to RAN3/SA5

3GPP TSG-RAN WG2 Meeting #130	R2-250xxxx

St. Julians, Malta, 19 – 23 May 2025

Title:	[DRAFT] Logged Data Handling During Handover

Response to:	-

Release:	Release 19

Work Item:	NR_AIML_air-Core

Source:	Nokia [TSG RAN WG2]

To:	RAN3, SA5

Cc:

Contact Person:

#### Name:	First Last

E-mail Address:	first.last@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

Regarding the handling of logged data collected for NW-side data collection at handover RAN2 has made the following agreements:

At RAN2#129:

- UE retains logged data during handover (HO).  FFS if there is scenarios where the UE needs to release the data and how does the UE know and if control from network is needed

- UE indicates availability of logged data during handover (i.e., within the RRCReconfigurationComplete message) (if data is retained in the UE).

At RAN2#129b:

Introduce 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration during/before HO.  Source gNB decides whether the data should be kept.  The indication is provided in RRCReconfiguration (i.e. not in RRC Reconfiguration from target cell).   FFS signaling details.

RAN2 has also agreed that a UE does not retain logged data for a source cell when a subsequent handover occurs; e.g., the UE collects data for gNB1, hands over to gNB2, and then to gNB3. gNB3 is not expected to forward logged data to gNB1.

RAN2 would like to clarify that the data collection configuration could originate in the source gNB or from OAM. When the configuration originates in the gNB, the final destination of the collected data is the source gNB. When the configuration originates in OAM, the final destination of the collected data is the Trace Collection Entity (TCE). Note that this data collection only happens when the UE is in RRC_CONNECTED state and this data collection is unrelated to legacy logged MDT.

RAN2 respectfully asks RAN3 and SA5 to introduce the necessary enhancements in their specifications in Rel-19 that enable the support of the above RAN2 agreements, if feasible. More specifically RAN2 would like to request RAN3 and SA5 to specify that after a handover the target gNB retrieves the logged data collected by a UE configured to log data in RRC_CONNECTED state at the source gNB and then the target gNB forwards the logged data towards its final destination.

2. Actions:

To RAN3, SA5

ACTION: 	RAN2 respectfully asks RAN3 and SA5 to introduce the necessary enhancements in their specifications in Rel-19 that enable the support of the above RAN2 agreements if feasible.

3. Date of Next TSG-RAN WG2 Meetings:

RAN2#131	from 2025-08-25	to 2025-08-29		Bengaluru, IN

RAN2#131bis	from 2025-10-13	to 2025-10-17		Prague, CZ