3GPP TSG-RAN WG2 Meeting #130					           R2-2504110

St. Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:		8.3.5

Source:			Huawei, HiSilicon

Title:			Discussion on performance monitoring

Document for:		Discussion and Decision

# 1   Introduction

In RAN2#129bis meeting, agreements related to performance monitoring were reached as follow:

For UE sided model, the performance monitoring considered in AIML air interface can be studied as a baseline for AIML aided mobility, i.e. NW-side and UE-sided performance monitoring. FFS metrics per use case

In this Tdoc, we provide our views on performance monitoring, including metric for each use case and performance monitoring mechanism options.

# 2   Discussion

## 2.1 Performance monitoring procedure

For BM use case, RAN1 initially considered the following options for prediction performance monitoring:

- NW based performance monitoring (i.e. metrics calculated at NW side based on the measurement from UE)

- UE assisted performance monitoring (i.e. metrics calculated at UE side and reported to NW for functionality management)

- UE based performance monitoring (i.e. metrics calculated at UE side and functionality management is also done in UE)

Further in the WI phase discussions, RAN1 decided to exclude an option were the UE performs functionality management decisions and only NW based and UE assisted performance monitoring are being specified for BM use case. Even though UE based performance monitoring was excluded for BM use case by RAN1, some companies wanted to consider it for AIML based mobility. Eventually, in RAN2#129bis, RAN2 agreed that both NW-side and UE-sided performance monitoring can be further studied.

However, we would like to note that, for some use cases, it is not feasible to allow the UE to make functionality management decisions by itself. Taking an example, one of the purposes of temporal domain case B prediction is to allow RS overhead reduction at the network side. If some RS(es) are turned off at the NW side for saving RS resources, the UE cannot simply switch to non-AIML based measurements as the network needs to turn on the required signals first. The same problem exists for spatial domain prediction where some RS(es) may be turned off. In case of inter-frequency measurements on the other hand, the UE needs to be configured with measurement gaps which need to be provided by the network, as they are not required for frequency domain prediction. Hence, in these cases it is not possible to apply UE based performance monitoring.

Observation 1: In case of temporal domain case B or spatial domain prediction, some RS(es) may be turned off at NW side in case the UE performs prediction for certain RS(es) or time instances.

Observation 2: In case of frequency domain prediction, the UE is not using measurement gaps to predict another frequency RSRP.

Based on the above, we can see that for majority of use cases UE based performance monitoring where the UE is allowed to make functionality management decisions by itself is infeasible. We then propose the following:

Proposal 1: As a baseline, functionality management decisions based on performance monitoring are made at the network side. Metrics calculation at both network side and UE side should be supported (as for BM use case).

Regarding temporal domain case A prediction, the UE autonomous decision about whether to apply predicted results or actual measurements can be in theory supported as the RS(es) will be transmitted continuously. Furthermore, the HO is a delay sensitive procedure, so it may be actually better if the UE does not wait for the network to switch to the regular measurements in case prediction accuracy deteriorates.

Observation 3: For temporal domain Case A prediction, it is feasible for the UE to autonomously switch from predicted measurements to regular measurements as all required RS(es) are continuously provided by the network.

Observation 4: The HO performance may deteriorate if the UE waits for the NW to switch from predicted measurements to regular measurements.

Based on these observations, we propose the following:

Proposal 2: RAN2 to discuss whether for temporal domain Case A prediction, UE can autonomously switch from predicted to regular measurements in case of prediction performance deterioration.

## 2.2 Performance metrics

For current scope of AI mobility in Rel-19, use cases can be classified as follows:

- RRM measurement prediction, including temporal domain, frequency domain and spatial domain

- Indirect event prediction

- Direct event prediction

Generally, both RRM measurement prediction and indirect event prediction are related to RSRP prediction, thus it is straightforward to use RSRP difference or similar metric (e.g. RMSE) as the metric for performance monitoring of RRM measurement prediction and indirect event prediction. If the prediction is performed at a cell-level, the metric should be cell level L3 RSRP difference while for beam-level prediction, the metric should be beam level L3 RSRP difference. Furthermore, such metric can be used for both UE-side and NW-side monitoring.

Proposal 3: For RRM measurement prediction and indirect event prediction, RSRP difference or similar metric (e.g. RMSE) should be used as the metric for performance monitoring for both cell-level and beam level measurement predictions. The details of the metric should be defined by RAN4.

During the simulation work for event prediction use case, RAN2 analyzed other metrics as well, i.e., F1 score and system level KPIs. However, these metrics are not available in real time and by nature they need to be calculated over a long period of time. Taking an example, the F1 score calculation should be performed using at least several instances of predicted measurement event, and the system level KPIs calculation should be performed at least over several HO executions (and ideally it should be dozens of events/HOs). Therefore, if such metrics were used alone, i.e., without RSRP difference metric, the UE or the NW may still perform prediction for a very long time before realizing that the prediction accuracy is actually unacceptable. Furthermore, it does not seem feasible for F1 score and system level KPIs to be calculated at the UE side. Similarly as today, system KPIs such as HOF rate can be monitored by the network, e.g. in order to determine a long-term effect of using predictions on the system performance.

Observation 5: It is not feasible for the UE to measure F1 score and system level KPIs.

Observation 6: As today, such metrics should be monitored by the network on a long-term basis, but are not suitable for AIML functionality management.

Proposal 4: F1 score and system level KPIs are not used for instantaneous AIML model performance monitoring.

However, for direct event prediction, there is no predicted RRM measurement result and in the simulations only F1 score and SLS level KPIs were derived for direct event prediction. This leads to a question how performance monitoring can be performed when the UE uses direct event prediction. Without having a mechanism to monitor the performance of the model, the network would have to rely solely on long-term metrics which means that a certain UE may experience a very bad mobility performance before the UE/network realizes that predictions are inaccurate. Such situation is obviously not acceptable for cellular systems and unless a proper way to verify model performance for direct method is identified, we believe direct event prediction at the UE side should be excluded from further work.

Proposal 5: Unless a way to monitor the performance of direct AIML prediction is identified, direct event prediction should be excluded from further work.

# 3   Summary

In the paper, the performance monitoring on AIML aided mobility are discussed, and the following observations and proposals are provided:

Performance monitoring procedure

Observation 1: In case of temporal domain case B or spatial domain prediction, some RS(es) may be turned off at NW side in case the UE performs prediction for certain RS(es) or time instances.

Observation 2: In case of frequency domain prediction, the UE is not using measurement gaps to predict another frequency RSRP.

Observation 3: For temporal domain Case A prediction, it is feasible for the UE to autonomously switch from predicted measurements to regular measurements as all required RS(es) are continuously provided by the network.

Observation 4: The HO performance may deteriorate if the UE waits for the NW to switch from predicted measurements to regular measurements.

Proposal 1: As a baseline, functionality management decisions based on performance monitoring are made at the network side. Metrics calculation at both network side and UE side should be supported (as for BM use case).

Proposal 2: RAN2 to discuss whether for temporal domain Case A prediction, UE can autonomously switch from predicted to regular measurements in case of prediction performance deterioration.

Performance metrics

Observation 5: It is not feasible for the UE to measure F1 score and system level KPIs.

Observation 6: As today, such metrics should be monitored by the network on a long-term basis, but are not suitable for AIML functionality management.

Proposal 3: For RRM measurement prediction and indirect event prediction, RSRP difference or similar metric (e.g. RMSE) should be used as the metric for performance monitoring for both cell-level and beam level measurement predictions. The details of the metric should be defined by RAN4.

Proposal 4: F1 score and system level KPIs are not used for instantaneous AIML model performance monitoring.

Proposal 5: Unless a way to monitor the performance of direct AIML prediction is identified, direct event prediction should be excluded from further work.