3GPP TSG-RAN WG2 Meeting #130				   		        R2-2504111

Saint Julian’s, Malta, 19 – 23 May, 2025

Title: 	Discussion on NW-sided data collection for training

Source: 	Huawei, HiSilicon

Agenda item:	8.1.3

Document for:	Discussion/Decision

# Introduction

In the previous meetings, the following agreements were achieved for NW-sided data collection:

Agreements from RAN2#127bis

1	Periodic logging is supported for training data collection procedure in R19

2	Event-triggered data logging will be supported.  At least radio condition based event triggered logging will be supported.  FFS the details of radio condition based event.  FFS if other events are supported.

3	Periodic reporting of logged data is not supported.

4	On-demand reporting of the logged measurements will be specified

5	UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection.   FFS of details of the message

6	The UE can indicates the availability of logged data to the network to assist network to trigger UEInformationRequest.  FFS trigger/definition of availability indication.   and FFS how data availability indication is sent to the network.

7	Low priority SRB will be used.    FFS new SRB or use of SRB4

8	For data collection for both NW-sided/UE sided BM model training, at least L1-RSRPs and/or beam-IDs needs to be collected by UE.  FFS if other data needs to be collected based on RAN1 progress.

Agreements from RAN2#128

1. Focus on the following three radio condition event based logging

§ L3 serving cell measurement based (e.g. X1/X2 similar to A1/A2)

§ Beam based events (e.g. beam becomes top-1 beam and number of measurements is less than configured value)

§ L1 beam level measurement

2. Measurements on aperiodic CSI resources are not reported for NW sided data collection.

3. Data collection is controlled by the network.   The UE will not autonomously stop when low power state is detected.

4. The UE reports to the network when the power state is low.  We will not specify how the UE determines low power state.   The network should de-configure the data collection (this can be captured in stage 2).

5. The UE reports to the network when buffer is or may become full.  FFS when it reports (before and/or after).

6. The UE can report the reason for triggering of indication for the status (e.g. low power state, low memory).  FFS how this is signalled and if the reporting can be part of availability indication.

7. Duration is not supported

Agreements from RAN2#129

1. Support the use of L3 measurement event triggered (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE performs logging or not.  L1 measurement event triggered will not be supported.    FFS what to log

2. Low power bit indication is supported

3. Data availability indication is supported.  FFS when this would be triggered

4. As baseline, the UEInformationResponse contains one or more logged measurement entries in chronological order (i.e. starting from the oldest measurement entries stored in the UE memory), and an availability indication if there are further data available for transmission. Same principles as for logged MDT.

5. UE retains logged data during handover (HO).  FFS if there is scenarios where the UE needs to release the data and how does the UE know and if control from network is needed

6. UE indicates availability of logged data during handover (i.e., within the RRCReconfigurationComplete message) (if data is retained in the UE).

7. FFS how to handle idle/inactive and RLF cases and whether we have a unified.

Agreements from RAN2#129bis

- RAN2 assumes that for OAM centric, only adopt management based immediate MDT for NW-side data collection

- For NR-DC, the configuration and reporting of NW sided data collection can only be via MN

Agreements on Idle/inactive and HO

- Introduce 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration during/before HO.  Source gNB decides whether the data should be kept. The indication is provided in RRCReconfiguration (i.e. not in RRC Reconfiguration from target cell). FFS signaling details.

- Upon going to RRC_IDLE, RLF, or RRC_INACTIVE, UE discards any logged data

Agreements on availability indication

- Availability indication can be triggered due to:

o Full buffer being reached (if configured)

o Buffer threshold being reached (if configured).

o Low power (if configured)

- The UE send a UAI that indicates:

o Data is available

o Reason for trigger (full buffer, threshold)

o Low power indication

- The encoding of the data is available/UAI and the cause value is FFS

NOTE: it is up to UE Implementation how buffer threshold reached and low power is determined

Agreements on data collection configuration

- The measurement configuration of AI/ML data collection can configure measurements for multiple sets of resources and use cases (e.g. BM, Mobility, etc)

Agreements

- For temporal domain, the network is made aware whether there is a gap between two consecutive samples. FFS amount of gap and whether this is implicit or explicit

- New SRB can be configured for NW-side data collection (with lower priority)

This paper analyzes the network-sided data collection topic further, with a focus on the FFS points mentioned above.

# Discussion

## Event-triggered logging details [RRC-21]

In the last meeting, the following agreements were made for event triggered logging:

1. Support the use of L3 measurement event triggered (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE performs logging or not. L1 measurement event triggered will not be supported. FFS what to log

As mentioned above, L3 serving cell measurements becoming worse/better than a threshold for TTT can be used as the triggering event which corresponds to the definitions of existing events A1 and A2. Thus, RAN2 should confirm that the definitions of events A1 and A2 should be reused for event triggered logging.

Proposal 1: For the event-triggered logging, RAN2 confirms that the definitions of events A1 and A2 will be reused.

In the last meeting, RAN2 agreed that the NW needs to be aware whether there is a gap between two consecutive samples.

1. For temporal domain, the network is made aware whether there is a gap between two consecutive samples. FFS amount of gap and whether this is implicit or explicit

In our understanding the gap is just a by-product of the event-based logging, not something that we want to use to train models, so the knowledge about the amount of gap is not indispensable for the model training purposes. On the other hand, some signalling needs to be added to the logged results anyway for the network to know that the gap exists, so it may be straightforward to signal the amount of gap actually. This could be either achieved via signalling the amount of gap explicitly and adding this value to a first sample of a new “block” of samples, or via adding an explicit time stamp to a first sample of each “block” of samples. In the latter case, the amount is not known explicitly but can be derived by the network rather easily.

Proposal 2: (RRC-21) When the gap occurs in the logged measurements, the length of the gap should be indicated to the network either via:

- Explicitly reporting gap length in the first sample of each “block” of samples; or

- Adding a time stamp to a first sample of each “block” of samples.

## Data Availability Indication [RRC-19, RRC-20]

As agreed in the previous meeting, the network can provide the UE with multiple sets of configurations, each corresponding to different scenario or set of resources. These configurations or scenarios may have different data collection requirements. The UE performs data collection based on the provided configuration and logs the collected data locally. When the stored data volume meets the specified requirements, the UE reports a data availability indication. Since the speed of data collection may vary for each configuration or scenario, we believe that the data availability indication triggering threshold should be configured and indicated separately for each data collection configuration. This allows the network to obtain more complete information about the data availability and request the data collected by the UE for the specific configuration or scenario.

Proposal 3: (RRC-19) Data availability triggering threshold should be configurable per data collection configuration.

Proposal 4: (RRC-20) The UE should include the corresponding configuration identifier in the data available indication to support NW to request a certain data collected by the UE.

Proposal 5: (RRC-20) The network should be able to request the UE to send collected data for a specific data collection configuration.

For the open issue RRC-8: Reporting assistance information related to logged measurements

Issue description: It is not yet clear what the otherConfig should contain to enable the UE to report assistance information via UAI, related to logging of radio measurements. For instance, should the low power, buffer full, and buffer threshold reached indications be all configured with a single bit, or should the configurations be separated?

There is also an FFS on the encoding of data available in the last meeting.

Agreements on availability indication - Availability indication can be triggered due to: - Full buffer being reached (if configured) - Buffer threshold being reached (if configured). - Low power (if configured) - The UE send a UAI that indicates: - Data is available - Reason for trigger (full buffer, threshold) - Low power indication - The encoding of the data is available/UAI and the cause value is FFS NOTE: it is up to UE Implementation how buffer threshold reached and low power is determined

Based on the agreements in the last meeting, the separate indicators are considered, i.e., data available indication for full buffer, data available indication for buffer threshold, and low power indication. As for how the NW controls the indication report, we believe there is no need for a too granular configuration. In our view full buffer indication can be assumed to be enabled by default when the UE is configured with data collection configuration. For the UE to be able to use data threshold, this threshold needs to be configured by the network, so its presence in the configuration is used to determine whether data availability based on threshold is enabled. When it comes to low power state indication, we believe a prohibit timer is needed to avoid too frequent power state updates from the UE, so this prohibit timer can be also an indication that low power state reporting is enabled.

Proposal 6: (RRC-19) The indications reported by the UE as part of UAI for data collection purpose are controlled by the network in the following way:

- Full buffer indication is enabled by default when the UE is configured with data collection

- Data threshold is enabled by including data threshold in the configuration

- Low power state reporting is enabled by including a corresponding prohibit timer in the configuration

## Low power state indication [RRC-19, RRC-20, RRC-29]

In the previous meeting, RAN2 agreed that the UE reports to the network when its power state is low and then the NW should de-configure the data collection [1].

After receiving the low power indication from the UE, there may be two ways for the NW to de-configure the data collection job. Firstly, the network may remove the measurement and logging configuration on the UE side through the RRC reconfiguration message.

Proposal 7: (RRC-20) When the NW receives the low power state indication from a UE, the NW may use RRC Reconfiguration to either release the data collection or suspend it without removing it from UE’s RRC configuration.

There is an FFS about low power indication in running CR as follow:

Editor's Note: FFS the need to introduce further procedures, e.g. prohibit timers, indication that battery state is not low any longer, etc.

The NW may indicate the UE to suspend the running data collection procedure and recover it when the UE exits low power state. In case the UE recovers from the low power state quickly, then the NW would not have to configure the data collection configuration again when the UE exits the low power state, so a process can be continued.

Proposal 8: (RRC-20) When the UE exits low power state, it indicates it to the network and the network may either configure a new data collection configuration or resume a suspended configuration.

Since the low power state determination is up to UE implementation, the network needs to have a possibility to control the amount of related signalling, Therefore, it is necessary to specify a prohibit timer for the low power state indications.

Proposal 9: (RRC-19) Prohibit timer for low power state indications is introduced.

Open issue RRC-29 on the other hand discusses whether data availability indication should be sent when the UE sends low power state indication. In our understanding, when the UE indicates low power state, it may have collected some data already, but this fact might not have been indicated to the network before in case neither the data threshold nor full buffer has been reached. In this situation, data availability indication can also be included with another cause (current CR captures only ‘full buffer’ and ‘data threshold’ causes). Without this addition, the network will release the configuration form the UE and the data collected so far will be lost.

Proposal 10: (RRC-29) When the UE sends low power state indication to the network and it has already collected some data, it should also include data availability indication, e.g. with ‘low power’ cause.

## Configuration details [RRC-23, RRC-25, RRC-27, RRC-30]

Regarding whether dynamic activation/deactivation of data collection should be supported. Data collection for model training is normally a continuous process and it is unclear what the purpose would be of activating and deactivating it dynamically. If the network no longer requires more data from the UE for a certain data collection configuration, the network may simply de-configure it using RRC signaling. Another potential reason to have dynamic activation/deactivation would be to allow the network to rely on semi-persistent CSI resources for data collection. Related to this, RAN2 made the following agreement during RAN2#128:

2. Measurements on aperiodic CSI resources are not reported for NW sided data collection.

Similarly, we do not see a reason to rely on semi-persistent resources and believe periodic CSI resources fit data collection purpose best. Based on the above consideration, we propose the following:

Proposal 11: (RRC-25) (RRC-30) Only periodic CSI resources are used for NW sided data collection, i.e. neither aperiodic nor semi-persistent CSI resources are supported for this purpose.

There is an FFS in the running CR about whether periodicity of the logging should be configurable as follows:

Editor's Note: FFS whether the periodicity of the logging is configurable.

It should be noted that the UE gathers data through RS measurements, and it is most efficient to align the logging periodicity with the CSI resource periodicity. If the network requires data with a specific periodicity, it can configure the CSI resource accordingly and there is no need for configuring logging periodicity separately.

Proposal 12: (RRC-27) There is no need to configure logging periodicity separately, i.e. the measurements are logged according to the periodicity of the CSI resource used for data collection.

In the open issue list, there is an issue about cell ID in data collection procedure.

Open issue RRC-12: Cell ID stored with logged data for NW-side data collection Issue description: It has not been clarified what type of cell ID the UE needs to log along with the logged data, in order to unambiguously identify the cell in which the UE performed the data logging, e.g. CGI, PCI-ARFCN etc.

In our understanding, CGI can be used for data collection. This is because CGI serves as a unique identifier in the NW, while a single PCI can be assigned to different cells. In the case of HO, the target gNB needs to identify which cell the collected data comes from. Therefore, CGI is suitable for data collection purposes.

Proposal 13: (RRC-23) Cell ID stored with logged data for NW-side data collection should be a CGI.

## SRB for data reporting [RRC-26]

In the last meeting, it was agreed that new SRB can be configured for NW-side data collection.

New SRB can be configured for NW-side data collection (with lower priority)

In the open issue list, there is an issue about whether legacy logged data can be carried over a new SRB in case the UE needs to report both legacy and AIML data to the network:

Open issue RRC-26: Multiplexing legacy logged data and AIML logged data in new SRB Issue description: RAN2#129bis agreed that 2. New SRB can be configured for NW-side data collection (with lower priority). Given the agreement on the new SRB for the transmission of the UEInformationResponse, Rapporteur’s understanding is that the new SRB may be used also for the transmission of the legacy SON/MDT reports (e.g. logged MDT measurements, RLF-Report, RA-reports, successful HO reports, etc.), whenever the UEInformationResponse carries both a legacy SON/MDT report and AIML logged data. However, this would impact how legacy SON/MDT reporting is performed which can only be at the moment on SRB1 or SRB2.

We believe it can be up to UE implementation whether to put both legacy SON/MDT and new data collection reports in the same response message or use separate ones for this purpose. If both the legacy SON/MDT report and AIML logged data are included in a single UEInformationResponse, the UE should select a new SRB when AIML logged data is present.

Proposal 14: (RRC-26) If both the legacy SON/MDT report and AIML logged data are included in a single UEInformationResponse, the UE should select a new SRB to transmit such message.

## Collected data handling during HO/CHO/LTM [RRC-18, RRC-36, RRC-37]

RAN2 agreed previously that the UE retains logged data during handover:

5. UE retains logged data during handover (HO). FFS if there is scenarios where the UE needs to release the data and how does the UE know and if control from network is needed 6. UE indicates availability of logged data during handover (i.e., within the RRCReconfigurationComplete message) (if data is retained in the UE).

As we can see, RAN2 agreed UE indicates availability of logged data during handover. However, RAN2 did not discuss what the network does based on such indication. Depending on whether gNB-centric or OAM-centric data collection was performed, the collected data needs to be either forwarded to the source gNB or to the OAM. To achieve this, after receiving the data available indication from the UE, the target gNB should fetch the data and forward it to the source gNB or OAM. It seems RAN3 involvement is needed to handle this aspect.

Proposal 15: (RRC-36) After receiving the data available indication from the UE, the target gNB should fetch the data from the UE and forward it to the source gNB or OAM. RAN3 should discuss and specify the details.

Furthermore, in the last meeting, RAN2 agreed to introduce one-bit indication to indicate the UE to retain or release the logged data.

1. Introduce 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration during/before HO. Source gNB decides whether the data should be kept. The indication is provided in RRCReconfiguration (i.e. not in RRC Reconfiguration from target cell). FFS signaling details.

Regarding the FFS on the signaling details, it is also included in the open issue RRC-7: NW control on retaining logged data at HO.

The issue is captured as an editor’s note in the running CR, in clause 5.3.5.3 and 6.2.2. It the rapporteur’s view, it should be clarified Whether/how the 1-bit indication can be sent during or before HO, taking into account that the source gNB decides whether the data should be kept. From the rapporteur perspective, it is not possible for the source gNB to add the 1-bit indication during HO, in the same RRCReconfiguration that encapsulates the RRCReconfiguration from the target gNB. Thus, the rapporteur sees the following two possible solutions for this issue: - The source gNB sends the 1-bit indication to the UE before HO. - The source gNB decides if the 1-bit indication is needed and, if so, sends it to the target gNB, which includes it in the RRCReconfiguration sent to the UE during HO. No RAN3 impact is expected if the transmission of the 1-bit indication is limited to the case in which the target gNB is from the same vendor as the source gNB.

Currently, a HO command is an RRCReconfiguration message generated entirely by the target gNB and provided to the source gNB which then forwards it transparently to the UE as it is (i.e. not as a container in another RRCReconfiguration message). Hence, we agree with the rapporteur evaluation for this open issue that the source gNB is not able to add data retain/discard indication in the HO command directly. Out of the two options proposed by the rapporteur, we believe that the second one where the indication is added to the HO command by the target gNB contradicts the purpose of this mechanism which was to address a scenario where the target gNB cannot be fully trusted. Therefore, we propose the following:

Proposal 16: (RRC-18) The source gNB sends the 1-bit data discard/retain indication to the UE in a separate RRCReconfiguration message sent before HO command.

Such message needs to be sent by the source gNB only in case the UE has some data already collected as sending these messages blindly would increase an overhead unnecessarily. Also, the source gNB may actually be able to fetch the data before the HO is executed in case there is sufficient time to do this.  However, at the time of handover, the triggers for sending data availability indication agreed thus far (data threshold, full buffer) may not be met. Hence, it should be possible for the data availability indication to be sent in the measurement report sent for handover purposes, e.g., based on event A3 or A5. In this case, the indication should be sent regardless of the amount of stored data. When the source gNB receives the indication, it can indicate the UE to delete data before sending the handover command if inter-vendor handover occurs, or if there is sufficient time, the source gNB may still try to fetch the data.

Proposal 17: (RRC-37) The UE should include the logged data availability indication in the L3 measurement report.

So far RAN2 focused only on legacy handover procedure, but it may also happen that the UE executed CHO or LTM HO while configured with data collection. Unlike legacy HO scenario, during CHO/LTM, there is no possibility to send an indication to the UE to retain or discard logged data as the HO execution is performed by the UE automatically (for CHO) or via a switching command sent via MAC CE (LTM). On the other hand, the source gNB is aware of which cells can be subject to CHO/LTM, so the data retain/discard indication can be pre-configured together with CHO/LTM configuration for each candidate cell.

Proposal 18: (RRC-18) For CHO/LTM, a collected data retain/discard indication can be pre-configured by the gNB for each CHO/LTM candidate cell.

# Conclusion

Based on the discussion in the Tdoc the following, we provided the following observations and proposals:

Event-triggered logging details [RRC-21]

Proposal 1: For the event-triggered logging, RAN2 confirms that the definitions of events A1 and A2 will be reused.

Proposal 2: (RRC-21) When the gap occurs in the logged measurements, the length of the gap should be indicated to the network either via:

- Explicitly reporting gap length in the first sample of each “block” of samples; or

- Adding a time stamp to a first sample of each “block” of samples.

Data Availability Indication [RRC-19, RRC-20]

Proposal 3: (RRC-19) Data availability triggering threshold should be configurable per data collection configuration.

Proposal 4: (RRC-20) The UE should include the corresponding configuration identifier in the data available indication to support NW to request a certain data collected by the UE.

Proposal 5: (RRC-20) The network should be able to request the UE to send collected data for a specific data collection configuration.

Proposal 6: (RRC-19) The indications reported by the UE as part of UAI for data collection purpose are controlled by the network in the following way:

- Full buffer indication is enabled by default when the UE is configured with data collection

- Data threshold is enabled by including data threshold in the configuration

- Low power state reporting is enabled by including a corresponding prohibit timer in the configuration

Low power state indication [RRC-19, RRC-20, RRC-29]

Proposal 7: (RRC-20) When the NW receives the low power state indication from a UE, the NW may use RRC Reconfiguration to either release the data collection or suspend it without removing it from UE’s RRC configuration.

Proposal 8: (RRC-20) When the UE exits low power state, it indicates it to the network and the network may either configure a new data collection configuration or resume a suspended configuration.

Proposal 9: (RRC-19) Prohibit timer for low power state indications is introduced.

Proposal 10: (RRC-29) When the UE sends low power state indication to the network and it has already collected some data, it should also include data availability indication, e.g. with ‘low power’ cause.

Configuration details [RRC-23, RRC-25, RRC-27, RRC-30]

Proposal 11: (RRC-25) (RRC-30) Only periodic CSI resources are used for NW sided data collection, i.e. neither aperiodic nor semi-persistent CSI resources are supported for this purpose.

Proposal 12: (RRC-27) There is no need to configure logging periodicity separately, i.e. the measurements are logged according to the periodicity of the CSI resource used for data collection.

Proposal 13: (RRC-23) Cell ID stored with logged data for NW-side data collection should be a CGI.

SRB for data reporting [RRC-26]

Proposal 14: (RRC-26) If both the legacy SON/MDT report and AIML logged data are included in a single UEInformationResponse, the UE should select a new SRB to transmit such message.

Collected data handling during HO/CHO/LTM [RRC-18, RRC-36, RRC-37]

Proposal 15: (RRC-36) After receiving the data available indication from the UE, the target gNB should fetch the data from the UE and forward it to the source gNB or OAM. RAN3 should discuss and specify the details.

Proposal 16: (RRC-18) The source gNB sends the 1-bit data discard/retain indication to the UE in a separate RRCReconfiguration message sent before HO command.

Proposal 17: (RRC-37) The UE should include the logged data availability indication in the L3 measurement report.

Proposal 18: (RRC-18) For CHO/LTM, a collected data retain/discard indication can be pre-configured by the gNB for each CHO/LTM candidate cell.

# References

- [1] RAN2#128 meeting minutes

- [2] RAN2#127bis meeting minutes

- [3] RAN2#127 meeting minutes