3GPP TSG-RAN WG2 Meeting #130	R2- 2504113

St. Julian's, Malta, 19 - 23 May, 2025

Agenda item:	8.7.4.2

Source: 	Huawei, HiSilicon

Title: 	Remaining open issues on DSR enhancements in XR

Document for:	Discussion and Decision

# 1. Introduction

For DSR enhancements, RAN2 made the following agreements in RAN2#129bis:

- To avoid the case that there is no delay-reporting data for DSR, at least one configured reporting threshold should be no lower than the DSR triggering threshold. This means we do not have to address “empty DSR” issue, but it is FFS whether we need to capture this restriction in specifications - During DSR data volume calculation the remaining time of retransmitted is not considered, i.e. it is always put either in the smallest configured or smallest reported threshold. FFS which one. - Clarify RAN2#128 agreement as “the UE may also support including non-delay-reporting data ahead of delay-reporting data for buffer size calculation of Rel-19 DSR, based on the capability indication” (the exact terminology to be discussed as part of CR review) - We will try to find a way to describe how the UE determines non-delay-reporting data ahead of delay-reporting data for delay-reporting data volume calculation. The aim is to have consistent UE behaviour to avoid fairness issues, but also consider different UE implementations.

In this contribution, we will further discuss the remaining issues of DSR enhancement according to the current progress.

# 2.	Discussion

## 2.1	Buffer size calculation for control signalling and retransmission [PDCP-1]

During last RAN2#129bis meeting, the issues related to buffer size calculation with respect to PDCP and RLC related control signalling and retransmission were discussed and RAN2 agreed that the remaining time of the retransmitted PDUs is not considered for buffer size calculation.

- During DSR data volume calculation the remaining time of retransmitted is not considered, i.e. it is always put either in the smallest configured or smallest reported threshold. FFS which one.

The only issue remaining is on where the PDCP and RLC related control signalling and retransmission data should be considered between:

Option A) the smallest reported threshold or

Option B) the smallest configured reporting threshold:

This is listed as Open Issue 1 in the PDCP running CR [1]:

- Issue 1: It is FFS which delay-reporting PDCP data volume shall consider PDCP Control PDUs, the PDCP SDUs to be retransmitted, and the PDCP Data PDUs to be retransmitted.

For Option A, the control PDU and retransmitted data of PDCP and RLC are reported together with the data with shortest reported threshold included in the DSR for the LCG. As on the issue of LCG only having control PDU and retransmitted data, it is our understanding that this should not happen with the following text in Rel-18:

3GPP TS 38.321 V18.5.0 6.1.3.72 Delay Status Report MAC CE …… The DSR MAC CE shall include delay information of all LCGs which have pending DSRs when the MAC PDU containing this DSR MAC CE is to be built.

Furthermore, it is also agreed as follow that there will not be any cases where there is no delay-reporting data in an LCG in the DSR by ensuring that at least one configured reporting threshold is not lower than the triggering threshold.

- To avoid the case that there is no delay-reporting data for DSR, at least one configured reporting threshold should be no lower than the DSR triggering threshold. This means we do not have to address “empty DSR” issue, but it is FFS whether we need to capture this restriction in specifications

One of the concerns for including control PDUs and/or retransmitted data in the shortest reported threshold of the LCG is that the UE needs to determine the smallest reported threshold and include all the control PDUs and retransmitted data in the buffer status calculation for that threshold. Another concern is on the specification of this. For the former, whenever a DSR is triggered, the UE will anyway need to determine the reporting threshold where there are delay reporting data, and the total amount of control PDUs and retransmissions can then be added to the buffer size calculation for the smallest reported threshold. Hence we do not see any further complexity for the UE. For the latter, it will just be adding the total amount of control PDUs and retransmissions to the smallest reported threshold (i.e. smallest ith reported threshold).

For Option B, the control PDU and retransmitted data of PDCP and RLC are reported in the smallest configured reporting threshold. No matter whether there is delay-reporting data in the smallest configured reporting threshold, UE always includes the data volume of control PDU and retransmitted data of PDCP and RLC in the smallest configured reporting threshold. This is simple from the UE implementation and complexity point of view and it is also easy to specify (which the rapporteur has already included this in the PDCP/RLC specification). However, this may increase the overhead if the configured shortest reporting threshold is not the shortest reported threshold included in the DSR.

We consider MAC signalling overhead is an important factor and hence prefer Option A (i.e. smallest reported threshold).

Proposal 1:	(PDCP-1) The Option A (i.e. option of “smallest reported threshold”) should be applied, where all the PDCP/RLC control PDU and PDCP/RLC retransmitted PDU/SDU are calculated to the buffer size for the smallest reported threshold.

## 2.2	DSR cancellation in Dual Connectivity [MAC-03]

The DSR cancellation in Dual Connectivity scenario related issues were discussed in last RAN2#129 meeting, and it is agreed that “No clarifications needed for Rel-18”. For DSR cancellation in DC, companies provided different views during the offline discussion as summarized in [2]. It is suggested that the DSR cancellation in DC scenario related issues can be considered to make the specifications clearer for DSR in Rel-19. This is listed as an open issue.

As discussed in our previous contribution [3], in DC scenario, DSR will be triggered both in primary and secondary MAC entity, when the remaining time of a PDCP SDU buffered in PDCP entity becomes below the remainingTimeThreshold of the primary MAC entity and the remainingTimeThreshold of the secondary MAC entity. For DSR cancellation, as summarized in the offline discussion [2], the cancellation of a DSR is specified as follows:

A PDCP SDU is considered to be associated with a DSR if it has not been transmitted in any MAC PDU and is a delay-critical PDCP SDU (as defined in TS 38.323 [3]) associated with the logical channel which triggered the DSR. A MAC PDU shall contain at most one DSR MAC CE. A MAC PDU shall not contain a DSR MAC CE if it includes all PDCP SDUs associated with all the pending DSRs. After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, when all the PDCP SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR (as described in the clause 6.1.3.72), or when a MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR. NOTE 2: It is up to UE implementation whether the MAC entity includes a DSR MAC CE in a MAC PDU if the MAC PDU can accommodate all PDCP SDUs associated with all the pending DSRs but is not sufficient to additionally accommodate the DSR MAC CE plus its subheader.. Therefore, there are three conditions upon which the DSR is cancelled: - when all the PDCP SDUs associated with the DSR have been discarded - when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR (as described in the clause 6.1.3.72) - when a MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR

According to the offline discussion [2], some companies think that the 38.321 specification can be understood as a per UE specification, or one MAC entity can cancel the pending DSR according to an event happening in another MAC entity based on UE implementation. However, from our understanding, MAC specification specifies the behaviour from a MAC entity perspective instead of a UE perspective, for example, in the Power Headroom Reporting clause, “other MAC entity” or “any MAC entity” is used for DC scenario as shown below:

### 5.4.6 Power Headroom Reporting The Power Headroom reporting procedure is used to provide the serving gNB with the following information: - Type 1 power headroom: the difference between the nominal UE maximum transmit power and the estimated power for UL-SCH transmission per activated Serving Cell; - Type 2 power headroom: the difference between the nominal UE maximum transmit power and the estimated power for UL-SCH and PUCCH transmission on SpCell of the other MAC entity (i.e. E-UTRA MAC entity in EN-DC, NE-DC, and NGEN-DC cases); ……ommited…… A Power Headroom Report (PHR) shall be triggered if any of the following events occur: - phr-ProhibitTimer expires or has expired and the path loss has changed more than phr-Tx-PowerFactorChange dB for at least one RS used as pathloss reference for one activated Serving Cell of any MAC entity of which the active DL BWP is not dormant BWP since the last transmission of a PHR in this MAC entity when the MAC entity has UL resources for new transmission;

Therefore, it can be seen that the MAC specification specifies the behaviour from a MAC entity perspective, and one MAC entity cannot cancel the pending DSR according to an event happening in another MAC entity, meaning that the pending DSR cannot be cancelled according to the above condition 3.

Observation 1:	The MAC specification specifies the behaviour from a MAC entity perspective, and one MAC entity cannot cancel the pending DSR according to an event happening in another MAC entity.

Furthermore, if left to UE implementation to cancel the pending DSR, e.g., based on some inter-MAC entity interactions, the UE complexity will be severely impacted. Taking the following Figure (2) as example, if SDU#1 is sent by MN MAC at T2, then MN MAC needs to inform SN MAC, based on which SN MAC can cancel its pending DSR at T2. From UE implementation perspective, this needs a lot of inter-MAC interaction between MN MAC and SN MAC. Moreover, MN MAC doesn't know whether SN MAC triggers DSR or not, e.g., whether the delay-critical data that MN MAC just sent also triggered a DSR at SN MAC or not, so MN MAC needs to indicate to SN MAC as long as a delay-critical data is sent by MN MAC, which leads to lots of unnecessary interactions between the MAC entities and cause sever complexity to UE implementation.

Figure (2) example of DSR procedure for split bearer when MN and SN MAC have same remainingTimeThreshold

Observation 2:	One MAC entity cancels the pending DSR according to an event happening in another MAC entity by inter-MAC interaction can cause sever complexity to UE implementation.

Therefore, to specify the DSR cancellation in DC scenario, the solution of left to UE implementation, or based on inter-MAC entity interactions between MAC entities (Option 2 from [3]) should be avoided, e.g., specifying the DSR cancellation condition in MAC as the Option 2 from [3] “when a MAC PDU is transmitted by any MAC entity and this MAC PDU includes all the PDCP SDUs associated with the DSR.” is not good for UE implementation.

From our understanding, the following condition as proposed in Option 3 of [3] can be considered for MAC entity to cancel the pending DSR in DC scenario.

Option 3 After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, when all the PDCP SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR (as described in the clause 6.1.3.72), or when a MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR, or if the volume of delay-critical data associated with the DSR is zero even though no MAC PDU including PDCP SDUs associated with the DSR was transmitted.

Specifically, taking the above Figure (2) as an example. At T2, a MN MAC PDU including PDCP SDU#1 is sent. For example, the MN MAC obtains an UL grant and informs the RLC entity and PDCP entity to submit UL data available for transmission to generate the MAC PDU, so a MN MAC PDU including PDCP SDU#1 is sent at T2. Then MN MAC will cancel the pending DSR according to the cancelling condition “when a MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR”, as copied above. However, SN MAC cannot cancel SN’s pending DSR based on this condition, because PDCP SDU#1 is sent by the MN path, not SN path. Later on, if the SN MAC has an UL grant, the SN MAC may send a SN MAC PDU at T3. Since a DSR has been triggered previously and not cancelled, this MAC PDU would include a DSR MAC CE, even though the data which triggered and was supposed to be included in the DSR had been already sent via MN path. Furthermore, at T3, SN MAC cannot cancel the pending DSR even after sending the DSR, because the DSR cancelation condition (highlighted in blue above) cannot be satisfied, because the PDCP SDU#1 which is the PDCP SDU associated with SN’s pending DSR has been sent by the other MAC entity and PDCP SDU#1 is not available for transmission in SN MAC. In other words, the DSR cannot contain information about the “PDCP SDUs associated with the DSR” as they no longer exist, which makes it impossible to meet the current DSR cancellation conditions.

To solve the above issue, a condition “if the volume of delay-critical data associated with the DSR is zero even though no MAC PDU including PDCP SDUs associated with the DSR was transmitted” can be introduced for the MAC entity to cancel the pending DSR. For example, SN MAC can cancel the pending DSR if a DSR MAC CE with delay-critical data volume = 0 is sent, even though the SN MAC didn't perform any MAC PDU transmission carrying delay-critical data after T1 and before T3, i.e., after DSR is triggered and before DSR MAC CE is sent.

Observation 3:	The solution for DSR cancellation in DC should not impact on UE complexity.

Proposal 2:	(MAC-03) In Dual Connectivity case, a MAC entity can cancel the pending DSR if the volume of delay-critical data associated with the DSR is zero even though no MAC PDU including PDCP SDUs associated with the DSR was transmitted.

Furthermore, as summarized in [2], companies have concern on impact of BSR cancellation. According to the current spec as below, the BSR can be cancelled as long as a BSR MAC CE carrying the latest buffer status is sent. That is to say, there is no limitation of “data associated with BSR” as in DSR. The main reason why DSR cannot be cancelled is because that the “PDCP SDU associated with the DSR does not exist”, and it is not reasonable for MAC entity to report the information of data which doesn't exist. However, there is no such limitation for BSR, so BSR can always be cancelled if a BSR MAC CE is sent. For example, in the above Figure (2), SN MAC can cancel the pending BSR if BSR MAC CE is sent at T3.

All triggered BSRs may be cancelled when the UL grant(s) can accommodate all pending data available for transmission but is not sufficient to additionally accommodate the BSR MAC CE plus its subheader. All BSRs triggered prior to MAC PDU assembly shall be cancelled when a MAC PDU is transmitted and this PDU includes a Long, Refined Long, Extended Long, Short, or Extended Short BSR MAC CE which contains buffer status up to (and including) the last event that triggered a BSR prior to the MAC PDU assembly. NOTE 3: MAC PDU assembly can happen at any point in time between uplink grant reception and actual transmission of the corresponding MAC PDU. BSR and SR can be triggered after the assembly of a MAC PDU which contains a BSR MAC CE, but before the transmission of this MAC PDU. In addition, BSR and SR can be triggered during MAC PDU assembly.

Proposal 3:	(MAC-03) In current spec, for Dual Connectivity case, the BSR can already be cancelled if a BSR MAC CE is sent, and there is no spec impact for BSR.

## 2.3	Co-existence between Rel-18 and Rel-19 DSR MAC CE [RRC Open Issue 7]

During the MAC running CR review after RAN2#129bis [4], companies had different views on how to configure the Rel-19 DSR as shown below, where some companies’ understanding is that NW always configures all the LCG of UE with triggering threshold also with reporting threshold(s) to use Rel-19 DSR, while some other companies’ understanding is that it is possible for some LCGs of UE with triggering threshold may not be configured with reporting threshold(s) (i.e., some LCGs can be configured with triggering threshold only without reporting threshold(s) configured).

The reason for the different understanding is that there are the following conflicting agreements from last RAN2#129 meeting:

2. FFS New DSR MAC CE will (always) be used when at least one LCG is configured with multiple thresholds. 5. If UE is configured to use R19 DSR, then any LCG with a triggering threshold shall be configured with at least one reporting threshold

Based on our understanding, agreement 2 tends to declare that the Rel-19 DSR MAC CE will be used as long as any one LCG is configured with reporting threshold. While the result of the agreement 5 implies that the Rel-19 DSR MAC CE will be used when all LCGs are configured with reporting threshold.

In our view, it is not needed for the network to configure all the LCGs with a triggering threshold also with at least one reporting threshold. Triggering threshold by itself is also a reporting threshold in Rel-18. This same condition can be reused.

It can be left to NW implementation to decide whether to configure some LCGs with Rel-19 reporting threshold(s) and some LCGs only with Rel-18 DSR triggering threshold, or to configure all the LCGs with Rel-19 reporting threshold(s). From the UE side, as long as at least one LCG is configured with Rel-19 reporting threshold(s), the new Rel-19 DSR MAC CE is used. In the new DSR MAC CE, UE can include the delay information of each LCG based either on the configured Rel-19 reporting threshold(s), or on the configured Rel-18 DSR triggering threshold, which can simply follow existing PDCP and RLC specifications.

Observation 4:	Since Rel-18 triggering threshold by itself is also a reporting threshold, it is sufficient for the UE to use Rel-19 new DSR MAC CE as long as at least one LCG is configured with Rel-19 reporting threshold(s). There is no impact to PDCP and RLC specifications.

Proposal 4:	(RRC-7) New DSR MAC CE will (always) be used when at least one LCG is configured with Rel-19 reporting threshold(s) (rather than all LCGs are configured with multiple reporting thresholds).

Proposal 5:	(RRC-7) When the R19 DSR MAC CE is used which includes LCG(s) configured only with triggering threshold, the triggering threshold is reused as the reporting threshold for these LCGs.

## 2.4	UE capability for DSR enhancement [UE Cap-1]

In the open issue list of the running CR of UE capability [6], there is one open issue related to DSR enhancement:

Open issue 1: the pre-requisite of enhanced delay status report This was discussed in RAN2#129bis meeting (as shown below) and an FFS was captured: “FFS A UE supporting this feature shall also indicate support of delayStatusReport-r18.” - LGE, QCM thinks we do not need to couple R18 and R19 DSR. Vivo thinks reasonable UE will implement R18 if implements R19, but does not want to couple in specs. - Huawei does not think it is possible to support R19 without R18

In our view, there is no additional complexity for UE supporting Rel-19 DSR enhancement to also support Rel-18 DSR, considering that the mechanism and procedure of Rel-19 DSR enhancement is based on Rel-18 DSR, e.g., DSR triggering and procedure are the same between Rel-18 and Rel-19. The only additions are on the reporting of multiple buffer status for the different reporting thresholds and the possibility of taking into account of non-delay reporting data ahead of delay reporting data. Furthermore, it provides more flexibility from the network point of view to support Rel-19 DSR enhancement with triggering threshold separated from the reporting threshold(s), or support R18 DSR only which combines both triggering and reporting threshold to reduce RRC signalling overhead.

Proposal 6:	(UE-Cap-1) UE supporting enhancedDelayStatusReport-r19 supports delayStatusReport-r18 as pre-requisite.

## 2.5	Rel-19 DSR on BSR cancellation

The newly introduced DSR MAC CE is capable of reporting delay information with finer granularity for an LCG, where the delay reporting data with remaining time above the triggering threshold can be included in the DSR MAC CE. However, this may lead to redundant report between BSR and DSR if there is only delay-reporting data (and if pduDiscard is not configured) in all LCHs of the UE. In a simple example, when there is new data arrived at the LCH configured with reporting threshold and all other LCHs are empty and the remaining time of the new data are within the reporting threshold, a BSR and Rel-19 DSR will be triggered. Assuming that an available UL resource is, both the BSR and DSR MAC CE will be sent in the same MAC PDU, which contain the same data volume for the LCH. Since DSR contains not just the data volume but also the smallest remaining time, the triggered BSR should be cancelled as the DSR provides more information.

Proposal 7:	(Others) RAN2 to specify that all triggered BSRs can be cancelled if amount of data to be reported in the BSR is already indicated by the DSR included in the same MAC PDU, as given in Appendix.

The corresponding TP for the above Proposal 7 is shown in Appendix.

# 3. Conclusion

In this contribution, we analyzed the potential aspects for DSR enhancements. The following observations and proposals were made:

Buffer size calculation for control signalling and retransmission [PDCP Issue 1]

Proposal 1:	(PDCP-1) The Option A (i.e. option of “smallest reported threshold”) should be applied, where all the PDCP/RLC control PDU and PDCP/RLC retransmitted PDU/SDU are calculated to the buffer size for the smallest reported threshold.

DSR cancellation in Dual Connectivity [Open issue no.: MAC-03]

Observation 1:	The MAC specification specifies the behaviour from a MAC entity perspective, and one MAC entity cannot cancel the pending DSR according to an event happening in another MAC entity.

Observation 2:	One MAC entity cancels the pending DSR according to an event happening in another MAC entity by inter-MAC interaction can cause sever complexity to UE implementation.

Observation 3:	The solution for DSR cancellation in DC should not impact on UE complexity.

Proposal 2:	(MAC-03) In Dual Connectivity case, a MAC entity can cancel the pending DSR if the volume of delay-critical data associated with the DSR is zero even though no MAC PDU including PDCP SDUs associated with the DSR was transmitted.

Proposal 3:	(MAC-03) In current spec, for Dual Connectivity case, the BSR can already be cancelled if a BSR MAC CE is sent, and there is no spec impact for BSR.

Co-existence between Rel-18 and Rel-19 DSR MAC CE [RRC Open Issue 7]

Observation 4:	Since Rel-18 triggering threshold by itself is also a reporting threshold, it is sufficient for the UE to use Rel-19 new DSR MAC CE as long as at least one LCG is configured with Rel-19 reporting threshold(s). There is no impact to PDCP and RLC specifications.

Proposal 4:	(RRC-7) New DSR MAC CE will (always) be used when at least one LCG is configured with Rel-19 reporting threshold(s) (rather than all LCGs are configured with multiple reporting thresholds).

Proposal 5:	(RRC-7) When the R19 DSR MAC CE is used which includes LCG(s) configured only with triggering threshold, the triggering threshold is reused as the reporting threshold for these LCGs.

UE capability for DSR enhancement [Open Issue 1 in UE cap running CR]

Proposal 6:	(UE-Cap-1) UE supporting enhancedDelayStatusReport-r19 supports delayStatusReport-r18 as pre-requisite.

Rel-19 DSR on BSR cancellation

Proposal 7:	(Others) RAN2 to specify that all triggered BSRs can be cancelled if amount of data to be reported in the BSR is already indicated by the DSR included in the same MAC PDU, as given in Appendix.

# 4. Reference

[1]. R2-2503696 Discussion of [POST129bis][504][XR] PDCP running CR (LGE) St. Julians, Malta, May 19 – May 23, 2025

[2]. R2-2501350 Report of [AT129][014][XR] DSR cancellation (Huawei) Athens, Greece, 17th – 21st February, 2025

[3]. R2-2500894 Discussion for DSR cancellation in DC Huawei, HiSilicon Athens, Greece, 17th – 21st February, 2025

[4]. R2-250xxxx R19 XR MAC running CR_v19_rapp (final) Qualcomm Incorporated

[5] R2-250xxxx- Open issues of Rel-19 XR UE capabilities Xiaomi  St Julian’s, Malta, 19 – 23 May 2025

# 5.	Appendix

### 5.4.5	Buffer Status Reporting

…… Omitted……

All triggered BSRs may be cancelled when the UL grant(s) can accommodate all pending data available for transmission but is not sufficient to additionally accommodate the BSR MAC CE plus its subheader. All BSRs triggered prior to MAC PDU assembly shall be cancelled when a MAC PDU is transmitted and this PDU includes a Long, Refined Long, Extended Long, Short, or Extended Short BSR MAC CE which contains buffer status up to (and including) the last event that triggered a BSR prior to the MAC PDU assembly. All triggered BSRs may be cancelled if amount of data to be reported in the BSR is indicated by the DSR included in a MAC PDU.