3GPP TSG-RAN WG2 Meeting #130						       	   		R2-254114

St Julian’s, Malta, 19th – 23nd May, 2025

Title: 	Applicability report and Inference for UE-sided model for Beam Management and CSI prediction

Source: 	Huawei, HiSilicon

Agenda item:	8.1.2.2

Document for:	Discussion/Decision

# Introduction

In RAN2 #129-bis meeting, RAN2 made the following agreements for the UE-side model for BM case related to inference and applicability reporting:

Agreements for direction B 1 RAN2 assumes UE receives RRCReconfiguration message including one set or multiple sets of inference related parameters via OtherConfig for option B. This assumption can be confirmed (i.e., whether to reconsider CSI-ReportConfig) after receiving Option B inference related parameters (e.g., in RAN1 RRC parameters list). Potential aspects to consider if RAN2 revisit: - To reconsider CSI-ReportConfig for option B, for example, if the list of inference related parameters is fully contained within existing CSI-ReportConfig. - to take into accounts UE behaviors when confirming the assumption e.g., whether option A and option B result in different UE behavior

- Agreements for applicability report - Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability FFS how to define this simple cause related to model availability and how we capture it in the spec - Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly. - No prohibit timer is introduced

In this contribution, we discuss the FFSs highlighted in the above box and other open issues identified during the post email discussion on the running RRC CR.

# Discussion

## Applicability reporting procedure for Option B (RRC-7)

In RAN2 #129bis, RAN2 agreed as a working assumption to include the related signaling for Option B in the OtherConfig

- RAN2 assumes UE receives RRCReconfiguration message including one set or multiple sets of inference related parameters via OtherConfig for option B.

With this working assumption, it is clear that the applicability reporting for Option B will be in UAI.

Observation 1: (RRC-7) Since it is agreed as a working assumption that the one set or multiple sets of inference related parameters for Option B is in OtherConfig, it is obvious that the subsequent applicable reporting for Option B is in UAI

The question then is whether the initial applicable reporting should be in the RRCReconfigurationComplete message.

In RAN2#127bis, RAN2 agreed that the applicable reporting can be in UAI or in the RRC Reconfiguration Complete message and the aim is to align the report of applicability for both messages.

UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality.   We should aim to align the design on how the applicable functionality are signaled.   FFS on the applicability reporting content.

The above agreement is in our understanding applied to both Options A) and B).

In RAN2#129 meeting agreement, RAN2 agrees the following procedure based on Option A.

- Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability.

Regardless of whether Option A) and/or B) is applied in Step 3, we think the purpose of Step 4 is the same, which is to report the applicability of the full inference configuration in Option A or of the sets of inference related parameters for Option B. The agreement based on Option A in RAN2#129 should be extended to Option B as proposed below.

Proposal 1: (RRC-7) Upon receiving a full inference configuration in CSI-ReportConfig or a set of inference parameters in OtherConfig, the UE always reports the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability report only if the applicability status changes afterwards.

The procedure of Option B is shown as following:

In the running CR, the UE reports CSI-ReportConfig IDs corresponding to the inference configuration to indicate the applicability of the configuration for Option A. Similarly, if gNB provides Option B) where one set or multiple sets of inference related parameters are signaled for applicability report, the UE indicates the set of the inference related parameters of the applicable functionalities (e.g. a InferenceParameterSet ID).

Proposal 2: (RRC-7) For Option B, the applicability reporting in the UAI message or RRCReconfigurationComplete indicates which set of inference parameters is applicable/inapplicable (e.g., using inferenceParameterSetID).

Based on the above, if agreed, the following TPs underlined can be added to the running RRC spec for the UE actions on transmission of UEAssistanceInformation and RRCReconfiguratioComplete messages respectively:

Section 5.7.4.3: Action related to transmission of UEAssistanceInformation message

<<Omitted>> 1> if transmission of the UEAssistanceInformation message is initiated to report assistance information about the applicability of configurations subject to applicability determination procedure according to 5.7.4.2: 2> include applicabilityReportList in this UEAssistanceInformation message; 2> for each serving cell configured with at least one reportConfigId associated to a CSI-ReportConfig including a configuration for measurement predictions for which the applicability information has changed: 3> include an entry in applicabilityReportList in the UEAssistanceInformation message, and set the content as follows: 4> set the applicabilityCellId to the serving cell index of the cell; 4> for each configured reportConfigId associated to a CSI-ReportConfig including a configuration for measurement predictions: 5> include an entry in the applicabilityReportConfigIdList and set the content as follows: 6> set the applicabilityReportConfigId to the corresponding reportConfigId; 6> set the applicabilityStatus to the applicability status of the configuration for measurement predictions corresponding to the applicabilityReportConfigId; 6> if the applicabilityStatus is set to inapplicable: 7> set the inapplicabilityCause for the configuration for measurements predictions to the cause of inapplicability; 2> for each set of inference parameters configured in the OtherConfig for which the applicability information has changed: 3> include an entry in applicabilityReportConfigIdList in the UEAssistanceInformation message, and set the content as follows: 4> set the applicabilityReportConfigId to the corresponding set of inference parameters (e.g. inferenceParameterID); 4> set the applicabilityStatus to the applicability status of set of inference parameters for measurement predictions corresponding to the applicabilityReportConfigId; 4> if the applicabilityStatus is set to inapplicable: 5> set the inapplicabilityCause for the configuration for measurements predictions to the cause of inapplicability; <<Omitted>>

Section 5.3.5.3: Reception of an RRCReconfiguration by the UE

<<Omitted>> 2> for each serving cell configured with at least one reportConfigId associated to a CSI-ReportConfig including a configuration for measurement predictions for which the applicability information has changed: 3> include an entry in applicabilityReportList in the UEAssistanceInformation message, and set the content as follows: 4> set the applicabilityCellId to the serving cell index of the cell; 4> for each configured reportConfigId associated to a CSI-ReportConfig including a configuration for measurement predictions: 5> include an entry in the applicabilityReportConfigIdList and set the content as follows: 6> set the applicabilityReportConfigId to the corresponding reportConfigId; 6> set the applicabilityStatus to the applicability status of the configuration for measurement predictions corresponding to the applicabilityReportConfigId; 6> if the applicabilityStatus is set to inapplicable: 7> set the inapplicabilityCause for the configuration for measurements predictions to the cause of inapplicability; 2> for each set of inference parameters configured in the OtherConfig for which the applicability information has changed: 3> include an entry in applicabilityReportList in the UEAssistanceInformation message, and set the content as follows: 4> set the applicabilityReportConfigId to the corresponding set of inference parameters (e.g. inferenceParameterID); 4> set the applicabilityStatus to the applicability status of set of inference parameters for measurement predictions corresponding to the applicabilityReportConfigId; 4> if the applicabilityStatus is set to inapplicable: 5> set the inapplicabilityCause for the configuration for measurements predictions to the cause of inapplicability; <<Omitted>>

## Applicability reporting contents (RRC-1/2/7/16)

There is the following Editor’s Note on the applicability reporting contents:

Editor's Note: FFS the structure of the IE ApplicabilityReportList and other content in ApplicabilityReport, e.g. for option B

In the following, we further discuss other applicability reporting contents.

Applicable reporting of the Associated ID

Associated ID is agreed as a working assumption by RAN1 to be used to ensure the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2, when needed. The network side additional condition alignment can be done in the following ways.

- First, the UE trains the model based on the dataset and associates it with the related associated ID, if provided by the network.

- The UE reports the applicable functionality to the NW, which may be associated with the NW-side additional condition identified by the associated ID.

- NW selects the applicable functionality and determines the inference configuration based on the NW-side additional condition.

In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - the associated ID Note: this doesn’t imply the associated ID is mandatory

From RAN1 #119 agreement highlight above, the NW-side additional condition can be indicated in step 3 optionally in Option A and B for applicability report.

For Option B, the associated ID can also be optionally indicated by the UE to the NW in Step 4. In our view, associated ID is optional for the whole LCM process, i.e., even for training phase the NW might not provide it and the UE should still be able to determine the applicability of the functionality based on implementation, but this needs to be concluded by RAN1 based on the above reply from RAN1.

Observation #2: (RRC-16) Based on the RAN1’s agreement for Options A and B, the NW-side additional condition can be indicated in Step 3 optionally. For Option B, the associated ID can also be optionally indicated by the UE to the NW in Step 4.

Based on Observation#2, the UE reporting behavior can be as follows depending on whether the network indicates or not the NW-side additional condition in Step 3 in all options or whether the UE indicates back or not in Step 4 for Options 2:

- Case 1: If the network indicates the associated ID, the network can indicate the network-side additional condition it is interested in. In this case, the UE reports only the applicable functionalities corresponding to the indicated associated ID(s).

- Case 2: If the network does not indicate the associated ID, the UE can determine which of the functionalities are applicable from UE perspective considering model availability and UE side additional conditions and report those functionalities together with the corresponding associated IDs, if available. Based on this, the network can select and configure the selected applicable functionality for inference according to the current network-side addition condition.

For Case 1, NW may indicate the interested associated IDs when requesting the applicable functionality reporting, and then the UE only needs to report the applicable functionalities associated with the indicated associated IDs. In this case, the UE can do the filtering based on the associated ID information, so that it does not need to report all the applicable functionality information stored. However, if the NW-side additional condition changes to the one not requested earlier from the UE, the network will have to ask the UE to report the applicable functionalities again, with the new associated ID.

For Case 2, when the NW side additional condition changes, the NW can do an update based on the associated ID reported earlier. However, in some situations, it may cause some signaling overhead in case the network is able to currently support only a very few NW-side additional conditions/associated IDs. For, example, if the UE has applicable functionality for associated ID #1, #2, and #3, but the current NW supports only ID#4, then the UE reporting of other IDs is meaningless.

Based on the above analysis, both Case 1 and Case 2 are useful and the choice between them will depend on the NW implementation and current NW-side additional conditions. We think it should be left to NW implementation which option to use at a certain time. If the NW indicates associated ID(s) to UE, the UE checks the indicated associated ID for determination of applicable functionalities; otherwise, the UE only checks UE-side additional conditions and model availability and reports applicable functionalities together with the supported associated IDs, if available.

Proposal 3: (RRC-16) The NW-side additional condition is optional in Step 3. The NW can decide whether to provide the NW-side additional condition by implementation, and the applicable functionalities determination can be supported in the following ways:

- In case the network does not provide the UE with associated IDs, the UE performs applicable functionality filtering without considering NW-side additional conditions and reports the (UE-considered) applicable functionalities together with their associated IDs, if available.

- In case the network provides the UE with associated IDs, the UE performs applicable functionality filtering considering them and only reports the functionalities meeting the NW-side additional condition.

Inclusion of simple inapplicability cause

Another issue is related to the FFS in RAN2 #129bis meeting agreement:

- Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability FFS how to define this simple cause related to model availability and how we capture it in the spec

Our understanding is that this is a compromise to have simple cause for inapplicability for the case where the model is not available and this is to indicate to the gNB that the applicability configurations (Option A or B) will not be possible at all in the near future (e.g. the model is not available and has to be downloaded or trained) and the gNB can release the applicability configuration.  The simple cause can be just to indicate the gNB to release the non-applicable configurations for both Option A and B.

Proposal 4: (RRC-1) The simple cause for inapplicability status can be just a value of ‘release’ for both Option A and B. Absence of the cause value means that it is just not applicable currently.

Since this is related to model unavailable, it can only occur at the initial reporting (i.e. when the applicable configuration is configured in CSI-ReportConfig for Option A or OtherConfig for Option B if proposal 2 (i.e. Option B can use RRCReconfigurationComplete for applicable reporting) is agreed), the simple cause is only applied at RRCReconfigurationComplete message.

Proposal 5: (RRC-1) Non-applicable cause is only in initial inapplicability report via RRCReconfigurationComplete message (i.e. add a condition that the inapplicabilityCause-r19 is only for RRCReconfigurationComplete message).

Support of multiple serving cells

Another FFS/Editor’s note is on the indication of which serving cell an applicable configuration is for in a CA/DC scenario in the applicability reporting:

FFS if applicability reporting is supported for multiple serving cells and applicabilityCellId is needed to unambiguously identify CSI report configurations for prediction.

It makes sense that applicability report is supported for multiple serving cells, which depends on UE capability (e.g. how many serving cells can the UE support for inference etc.). Combined with the RAN1 #119 agreement below where the applicability report is based on Options A) and B), we observe:

- In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - B) One set or multiple sets of inference related parameters for applicability report only (not for inference)

Observation #3: (RRC-2) In Option A, NW will send CSI-Reconfiguration for inference configuration, which is specific to a serving cell. In Option B, since the OtherConfig is UE specific, sets of inference related parameters in Option B in OtherConfig is also UE specific (i.e. not specific to a serving cell).

Proposal 6: (RRC-2) For Option B, the gNB does not need to indicate the serving cell(s) in which a set of inference parameters are related to. In the reporting, the UE does not need to indicate which serving cells a set of inference parameters are applicable for in the applicability reporting via RRCReconfigurationComplete and UAI.

Other possible reporting contents

Initial KPI of applicable functionality can be provided as part of applicable functionality reporting. For example, prediction accuracy can be calculated during training by using data for the ground truth as reference against the trained model. This prediction accuracy can be defined as initial KPI. It would be beneficial for the network to know how well the applicable functionality has been trained (by the OTT server) before it configures the UE to perform prediction on the functionality or trigger the data collection for training to improve on the KPI or perform performance monitoring before activating the applicable functionality. Without such initial KPI information of a reported applicable functionality by UE, the network may activate an applicable functionality where it may not meet its expectation and requiring starting the performance monitoring immediately after activation.

Observation #4: (RRC-7) Without initial KPI information of a reported applicable functionality by UE, the network may activate an applicable functionality where it may not meet its expectation and requiring starting the performance monitoring immediately after activation.

Proposal 7: (RRC-7) Initial KPI of trained functionality can be part of applicable functionality reporting.

## Any interaction between Option A and B for applicable functionality determination (RRC-8)

It is necessary to consider whether Options A and B can be configured at the same time. We think the selection between Option A and Option B depends on the network's strategy for performing applicability reporting. For example, gNB may use Option A if network does not mind configuring blindly and having the additional overhead on configuring the full inference configuration.  On the other hand, if the network wants to check the applicability first before configuring the inference configuration, Option B may be preferred.

Observation #5: (RRC-8) Selection between Option A and B depends on the network strategy for performing applicability reporting (e.g. gNB may use Option A if network does not mind configuring blindly and having the additional overhead on configuring the full inference configuration, while using Option B if it wants to check the applicability first before configuring the inference configuration)

As usual, UE will indicate whether it supports Option A and/or Option B and network can configure to apply Option A or Option B

Proposal 8: (RRC-8) UE can only be configured to perform applicability reporting based on Option A or Option B but not both.

## Activation of a periodic CSI report configuration upon change from inapplicable to applicable (RRC-4)

For Option A) where one or more CSI-ReportConfig is provided in Step 3 for inference, RAN1 agrees that:

- Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete.

Also, RAN1 has conclusion below, if CSI-ReportConfig for inference configuration is provided in Step 5:

- Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig

In RAN2 #129bis meeting, we agree that:

- Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly.

However, according to RAN1 note in #119 meeting agreement, UE is not expected to be configured with a CSI-ReportConfig for a non-applicable CSI-ReportConfig. Our understanding is that, this conclusion applies to all reporting types of non-applicable CSI-ReportConfig (i.e.periodic, aperiodic and semi-persistent reporting). From UE perspective, after initial reporting for a configuration of Option A and if the configuration is non-applicable, the UE assumes that network will always release the non-applicable configuration regardless of the reporting types for Option A.

Proposal 9a: (RRC-4) RAN2 confirms that RAN1 conclusion that UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig

Proposal 9b: (RRC-4) The gNB should release the non-applicable CSI-ReportConfig once the UE indicates that it is not applicable.

## Reporting behaviour of inapplicable beam prediction configuration (RRC-5)

In RAN2#129, the reporting of the changes of the applicability of a functionality is discussed with the following agreement that the UE reports explicitly applicability/inapplicability in initial report (i.e. in the response of the RRCReconfigurationComplete where the functionality is configured for Direction A or B), and any subsequent changes of the configured functionalities, UE only reports the functionalities with applicability status change.

Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. FFS if we report explicit cause

However, this agreement does not capture whether the UE ‘shall’ or ‘may’ report and the running CR currently leave it as ‘may’:

Running CR: A UE capable of providing assistance information related to the applicability of configurations subject to the applicability determination procedure may initiate the procedure in several cases, including upon being configured to report assistance information about the applicability of configurations related radio measurement predictions and upon change of the applicability of the configurations related to radio measurement predictions.

We think this should be a ‘shall’ as UE not reporting this may result in performance degradation. For example, if an activated functionality becomes non-applicable, UE has to report it so that the NW can deactivate/de-configure the functionality to avoid any system performance degradation). As on whether the UE should continue L1 reporting, with the ‘shall’ on the status change, the network can then ignore the L1 reporting and/or release to the configuration

Proposal 10: (RRC-5) When the status of the functionality which has been previously requested by the network changes from applicable to non-applicable or vice-versa, UE shall report the status change information to network. The NW can then use this information to decide on whether to ignore the L1 reporting or release the inference configuration.

## Handling inference/applicability configurations when UE changes RRC state (RRC-6)

RAN2 made following agreements in RAN2#129bis meeting:

Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly.

It is not clear whether UE releases inference/applicability configurations when UE goes to RRC_IDLE/CONNECTED state or upon network configuration via RRCRelease, etc.

For UE entering RRC_IDLE, the inference/applicability configurations contain dedicated RRC resources/configurations and will be released as normal.

For UE entering RRC_Inactive, it is possible for the inference/applicability configuration to be kept just in case the UE enters the same cell. However, it is also possible for the gNB upon UE resuming to reconfigure/release the configuration when it is needed and this should handle in the same as legacy L1 configuration/configuration in OtherConfig. The UE should be able to indicate the applicability reporting in the RRCResumeComplete message.

Similarly, for RRC re-establishment, the L1 configuration/configuration in OtherConfig can be reconfigured/released during RRC re-establishment by the target gNB if needed as per legacy.

Proposal 11: (RRC-6) For UE entering RRC_IDLE, the inference/applicability configurations contain dedicated RRC resources/configurations and will be released as per legacy.

Proposal 12: (RRC-6) For UE entering RRC Inactive, the inference/applicability configurations can be kept just in case the UE enters the same cell. Upon UE resumption of the connection, the gNB can reconfigure/release the configurations when it is needed and this should handle in the same as per legacy for existing L1 configuration/configuration in OtherConfig. The UE should be able to indicate the applicability reporting in the RRCResumeComplete message.

Proposal 13: (RRC-6) For re-establishment, the L1 configuration/configuration in OtherConfig can be reconfigured/released during RRC re-establishment by the target gNB if needed as per legacy.

## Discussion on LCM impacts due to CSI prediction

In the WID RP-250792, we have a use case CSI prediction:

- CSI feedback enhancement, encompassing [RAN1/RAN2]:

  - CSI prediction (UE-sided model):

    - Functionality-based LCM leveraged from other use cases, when necessary and applicable,

    - Study, and if necessary, specify consistency of training/inference,

    - Note: Normative work in RAN1 for CSI prediction will start from Q1 of 2025

This UC is only about UE-sided model, and RAN1 has made some progress. Details can be found in Section 5. We observe some potential RAN2 impacts as below:

- Applicability reporting

- UE request for data collection for training

- In addition, the UC may also impact stage-2 and stage-3 running CRs

For applicability reporting, we checked the latest RAN1 progress. Firstly, RAN1 has not made any conclusions on it. Secondly, in their FL summary R1-2503059, there were some discussions on the details of applicability reporting, e.g. Option A) and B) However there is no conclusion andRAN1 may down-select between Option A) and B). From RAN2 point of view, we think it is better to wait for more RAN1 progress (including L1 parameter list), and then RAN2 could check detailed specification impacts.

For data collection configuration for training, we provide our analysis in another paper [3].

For inference and monitoring aspects, we think RAN1 is still discussing them, and RAN2 can wait for the L1 parameter list and then check for possible RAN2 impacts, which should be straightforward.

In summary, we have the following observations:

Observation #7: For applicability reporting, inference and monitoring for CSI prediction, RAN2 can wait for more RAN1 progress on signalling details.

# Conclusion

In this paper, our observations and proposals are summarized as follows listed by the number of RRC open issues:

Open issue RRC-1

Proposal 4: (RRC-1) The simple cause for inapplicability status can be just a value of ‘release’ for both Option A and B.

Proposal 5: (RRC-1) non-applicable cause is only in initial inapplicability report via RRCReconfigurationComplete message (i.e. add a condition that the inapplicabilityCause-r19 is only for RRCReconfigurationComplete message).

Open issue RRC-2

Observation #2: (RRC-2) In Option A, NW will send CSI-Reconfiguration for inference configuration, which is specific to a serving cell. In Option B, since the OtherConfig is UE specific, sets of inference related parameters in direction B in OtherConfig is also UE specific.

Proposal 6: (RRC-2) For Option B, the gNB does not need to indicate the serving cell(s) in which a set of inference parameters are related to if the sets of inference parameters are distinguished by multiple serving cells. In the reporting, the UE does not need to indicate for the applicable reporting in the RRCReconfigurationComplete and UAI which serving cells a set of inference parameters are applicable for.

Open issue RRC-4

Proposal 9a: (RRC-4) RAN2 confirms that RAN1 conclusion that UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig.

Proposal 9b: (RRC-4) the gNB should release the non-applicable CSI-ReportConfig once the UE indicates that it is not applicable.

Open issue RRC-5

Proposal 10: (RRC-5) When the status of the functionality which has been previously requested by the network changes from applicable to non-applicable or vice-versa, UE shall report the status change information to network. The NW can then use this information to decide on whether to ignore the L1 reporting or release the inference configuration.

Open issue RRC-6

Proposal 11: (RRC-6) For UE entering RRC_IDLE, the inference/applicability configurations contain dedicated RRC resources/configurations and will be released as per legacy.

Proposal 12: (RRC-6) For UE entering RRC Inactive, the inference/applicability configurations can be kept just in case the UE enters the same cell. Upon UE resumption of the connection, the gNB can reconfigure/release the configurations when it is needed and this should handle in the same as per legacy for existing L1 configuration/configuration in OtherConfig. The UE should be able to indicate the applicability reporting in the RRCResumeComplete message.

Proposal 13: (RRC-6) For re-establishment, the L1 configuration/configuration in OtherConfig can be reconfigured/released during RRC re-establishment by the target gNB if needed as per legacy.

Open issue RRC-7

Observation 1: (RRC-7) Since it is agreed as a working assumption that the one set or multiple sets of inference related parameters for Option B is in OtherConfig, it is obvious that the subsequent applicable reporting for Option B is in UAI

Proposal 1: (RRC-7) Upon receiving a full inference configuration in CSI-ReportConfig or a set of inference parameters in OtherConfig, the UE always reports the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability report only if the applicability status changes afterwards.

Proposal 2: (RRC-7) For Option B, the applicability reporting in the UAI message or RRCReconfigurationComplete indicates which set of inference parameters is applicable/inapplicable (e.g., using inferenceParameterSetID).

Observation #3: (RRC-7) Without initial KPI information of a reported applicable functionality by UE, the network may activate an applicable functionality where it may not meet its expectation and requiring starting the performance monitoring immediately after activation.

Proposal 7: (RRC-7) Initial KPI of trained functionality can be part of applicable functionality reporting.

Open issue RRC-8

Observation #4: (RRC-8) Selection between Option A and B depends on the network strategy for performing applicability reporting (e.g. gNB may use Option A if network does not mind configuring blindly and having the additional overhead on configuring the full inference configuration, while using Option B if it wants to check the applicability first before configuring the inference configuration)

Proposal 8: (RRC-8) UE can only be configured to perform applicability reporting based on Option A or Option but not both.

Open issue RRC-16

Observation #2: (RRC-16) Based on the RAN1’s agreement for Options A and B, the NW-side additional condition can be indicated in Step 3 optionally. For Option B, the associated ID can also be optionally indicated by the UE to the NW in Step 4.

Proposal 3: (RRC-16) The NW-side additional condition is optional in Step 3. The NW can decide whether to provide the NW-side additional condition by implementation, and the applicable functionalities determination can be supported in the following ways:

- In case the network does not provide the UE with associated IDs, the UE performs applicable functionality filtering without considering NW-side additional conditions and reports the (UE-considered) applicable functionalities together with their associated IDs, if available.

- In case the network provides the UE with associated IDs, the UE performs applicable functionality filtering considering them and only reports the functionalities meeting the NW-side additional condition.

LCM impacts due to CSI prediction

Observation #5: For applicability reporting, inference and monitoring for CSI prediction, RAN2 can wait for more RAN1 progress on signalling details.

# References

- R2-2407848, LS on applicable functionality reporting for beam management UE-sided model

- R1-2410898, Reply LS on applicable functionality reporting for beam management UE-sided model

- R2-2504239, Discussion on training data collection for UE-sided model for BM and CSI prediction, Huawei, HiSilicon

# Annex - RAN1 progress on CSI prediction

## RAN1#120bis agreements

Agreement

Introduce a dedicated AI/ML PU for AI/ML features for UE,

- The AI/ML PU is used at least for quantifying the simultaneous processing of multiple CSI reports subject to CSI-related AI/ML use case(s), e.g., CSI compression (if supported), CSI prediction, BM spatial prediction, BM temporal prediction.

Conclusion

For CSI prediction using UE-side model, for data collection for training, aperiodic CSI-RS resource for CMR is not supported.

Agreement

For CSI prediction using UE-side model, for training data collection, support

- CSI-ReportConfig can used for configuring the resources for data collection purpose without CSI report

  - FFS on how to indicate without CSI report in CSI-ReportConfig

Agreement

For CSI prediction using UE-side model, for performance monitoring, support UE assisted performance monitoring subject to an additional UE capability, and UE assisted performance monitoring is based on Type 3 performance monitoring

Agreement

For CSI prediction using UE-side model, for performance monitoring type 3, support SGCS as a performance metric.

Agreement

For the definition of SGCS,

Note: How to handle layer mapping mismatch, if any, is up to UE implementation.

Agreement

For CSI prediction using UE-side model, for reporting contents of UE assisted performance monitoring, down-select one alternative by RAN1#121.

- Alt 1: one SGCS is calculated based on predicted CSI for one inference reporting, ground truth CSI

- Alt 2: one SGCS is calculated based on predicted CSI for one inference reporting, and ground truth CSI, another SGCS is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance.

- Alt 3: statistic of reporting contents (e.g., mean, x% CDF of SGCS values) of inference reporting instances within configured monitoring window

  - Monitoring window can be configured by NW

    - FFS on signalling details

- FFS on whether to report per prediction instance, selected prediction instance(s), averaged over prediction instances

- FFS on report frequency granularity (e.g., per wideband or per subband or averaged over subband or selected subband)

- FFS on whether to report per layer, the first layer, averaged over layer

- FFS on how to quantize and report mechanism

Agreement

For CSI prediction using UE side model, for inference, consider following options for potential down selection

- Option 1: only dedicated AI/ML PU (OAPU) is occupied

- Option 2: only legacy CPU (OCPU) is occupied

- Option 3: both dedicated AI/ML PU (OAPU) and legacy CPU (OCPU) are occupied

- FFS whether OAPU and OCPU are based on defined rule and/or reported by UE

- Note: The supported option(s) by UE is reported by UE capability, if multiple options are supported.

The total number of dedicated AI/ML PU for AI/ML is reported by UE capability.

- Note: The total number of Use case specific dedicated AI/ML PU could be discussed separately.

Agreement

For CSI prediction using UE-side model, at least for inference, introduce new RRC parameter for CSI report configuration to distinguish CSI report of AI-CSI prediction and non-AI CSI prediction.

- Note: terminology of “AI-CSI prediction” and “non-AI CSI prediction” is separate discussion

- Detailed parameter name is upto RAN2

## RAN1#120 agreements

R1-2501528	Summary #1 of CSI prediction	Moderator (LG Electronics)

From Wednesday session

Agreement

For CSI prediction using UE-side model, at least for inference, Rel-18 CSI framework is reused.

- For CSI-RS resource type for CMR, periodic, semi-persistent, and aperiodic CSI-RS are supported.

- For inference report,

  - for N4>=1, support Rel-18 codebook ('typeII-Doppler-r18')

R1-2501529	Summary #2 of CSI prediction	Moderator (LG Electronics)

R1-2501530	Summary #3 of CSI prediction	Moderator (LG Electronics)

From Thursday session

Agreement

For CSI prediction using UE-side model, if performance monitoring type 1 or 3 is supported, for calculation of monitoring metric, support

- Based on intermediate KPI

  - Down select between SGCS and NMSE by RAN1#120bis

  - FFS on the definition of SGCS/NMSE and how to calculate monitoring metric

Agreement

For CSI prediction using UE-side model, for CSI processing criteria and timeline, at least for inference further study on

- Whether the CPU should be shared or separately counted between legacy CSI reporting and AI/ML-based CSI reporting

- Whether the Processing Unit should be shared or separately counted among AI/ML related features/functionalities.

- Whether new timeline is needed/updated for inference, and whether a different timeline is needed when functionality switches/activates.

- Whether legacy framework for active CSI-RS resource and port counting can be reused

Note: Strive to study CSI processing criteria considering both BM and CSI case, and take the existing solutions as starting point.

R1-2501610	Summary #4 of CSI prediction	Moderator (LG Electronics)

Presented in Friday session

Agreement

For CSI prediction using UE-side model, for data collection for training,

- For resource configuration,

  - For CSI-RS resource type for CMR, periodic and semi-persistent are supported.

    - FFS on support of aperiodic CSI-RS resource

  - FFS on whether to separately configure CSI-RS resource(s) used for measurements for model input and CSI-RS resource(s) used for measurements for ground-truth CSI

  - FFS whether/how to enhance CSI-RS resource configuration to reduce CSI-RS signalling overhead.

- FFS on CSI report configuration

Final summary in R1-2501626.