3GPP TSG-RAN WG2 Meeting #130	R2-2504117

St.Julian's, Malta, 19-23 May, 2025

Agenda item:	8.20.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on LS S4-250739 for RTP retransmission

Document for:	Discussion and Decision

1. Introduction

SA4 has some discussion on network awareness for RTP retransmission. Several solutions to distinguish the source and retransmission PDUs were considered. SA4 sent an LS in S4-250739 [1] to RAN2 to check whether there are any potential benefits to RAN if RAN is provided with the application layer retransmission information. In this contribution, we will provide our views based on the background and the solutions mentioned in the LS.

A draft reply LS is provided as the annex.

2. Discussion

In S4-250739, SA4 explained the following requirements for RTP retransmission PDUs:

- the RTP retransmission PDUs should have stricter delay requirements than the source PDUs since the application layer needs to ensure the decoding deadlines of corresponding ADUs.

- In case of PDU set handling, the retransmission PDUs and the source PDUs should not be included in the same PDU set. Otherwise, the network cannot handle them differently.

The content of the LS is copied below.

1. Overall Description: During the FS_5G_RTP_Ph2 study, SA4 looked into a candidate solution that aims to enable network awareness for RTP retransmission (as specified in RFC 4588) with the goal of improved PDU Set handling in the 5G network, as documented in clause 6.9 of TR 26.822. According to RFC 4588, source and retransmission PDUs associated to a media flow are transmitted in separate RTP streams. Hence, they may be mapped by the 5GC into the same or different QoS flows. SA4 discussed different PDU Set marking options for source and retransmission PDUs and identified some approaches which may be preferable depending on the QoS flow mapping. One such approach is to assign source and retransmission streams to the same QoS flow and configure an RTP sender to apply PDU Set marking only to source PDUs of a media flow. SA4 understands that this would result in retransmission PDUs being marked by the 5GC into new PDU Sets containing a single PDU, as they would be N6-unmarked PDUs. SA4 believes that marking source and retransmission PDUs into the same PDU Set would be detrimental for the network. If an RTP sender uses PDU Set marking both for source and retransmission PDUs within the same QoS flow, they should be marked into different PDU Sets to avoid maintaining state in the RAN, which (according to the SA4 understanding) may make the RAN operation more complex. Retransmission PDUs have stricter delay requirements to ensure meeting the decoding deadlines of corresponding ADUs at the receiver. One identified approach to address this requirement is configuring an RTP sender to mark source and retransmission PDUs into different PDU Sets, which the 5GC maps then into distinct QoS flows. This approach can improve the timely delivery of retransmission PDUs by providing better QoS. Currently, there is no mechanism to inform the 5G network whether an application uses RTP retransmission (e.g. a flag in the RTP HE for PDU Set marking) or any related information, such as how long source packets are kept in the RTP sender buffer for potential retransmission. Question to SA2: Considering the options identified above, are there any benefits to providing application-layer retransmission information to the 5GC when PDU Set based handling is enabled? If so, SA4 would like to receive feedback on what type of application-layer retransmission information would be beneficial for PDU Set based handling in the 5GC. Question to RAN2: Considering the options identified above (including potential mapping of source and retransmission PDUs into distinct QoS flows in the 5GC), are there any potential additional benefits to the RAN from receiving application-layer retransmission information when PDU Set based handling is enabled? If so, SA4 would like to receive feedback on what type of application-layer retransmission information would be beneficial for PDU Set based handling in the RAN. 2. Actions: To SA2, RAN2 ACTION: SA4 kindly asks SA2 and RAN2 to take the above information into account and provide answers to the above questions. SA4 welcomes any additional feedback on potential usage and value of RTP retransmission related information in the 5GC and/or RAN.

According to the LS, SA4 has identified several solutions to separate the retransmission PDUs and the source PDUs into different PDU sets. The solutions are summarized as following.

- Option A. The RTP sender applies PDU Set marking only to the source PDUs and leaves the retransmission PDUs as unmarked in N6, so that the retransmission PDUs will be marked as new PDU Sets containing a single PDU at the 5GC.(As blue highlighted in the excerption.)

- Option B. The RTP sender marks the retransmission PDUs and the source PDUs into different PDU Sets within the same QoS flow. (As green highlighted in the excerption.)

- Option C. The RTP sender marks the retransmission PDUs and the source PDUs into different PDU Sets and the 5GC maps them into distinct QoS flows. (As red highlighted in the excerption.)

From RAN2 perspective, the delay budget is guaranteed consistently for different PDU Sets within the same QoS flow when PDU Set based handling is required. If the retransmission PDUs and the source PDUs are mapped to the same QoS flow, even if they are separated into different PDU Sets at NG-RAN, it is not feasible to meet the stricter delay budget for the retransmission PDUs based on the existing mechanisms. So that option A and option B cannot meet the different delay requirements mentioned by SA4.

Observation 1: If the retransmission PDUs and the source PDUs are mapped to the same QoS flow, RAN cannot guarantee different delay requirements.

Mapping the retransmission PDUs and the source PDUs to different QoS flows will be better to satisfy different delay requirements. As long as the 5GC maps the retransmission PDUs to a distinct QoS flow, it can set different PDB/PSDB for the different QoS flows, e.g., it can set a shorter PDB/PSDB for the QoS flow containing the retransmission PDUs, so that the retransmission PDUs will be transmitted with shorter delay. With such solution, there is no need for RAN to get the information of application layer retransmission any more since it is already distinguished at the 5GC and the current QoS mechanism can be reused. SA4’s requirement can be met without RAN impacts.

Observation 2: If the retransmission PDUs and the source PDUs are mapped to different QoS flows, RAN can guarantee different delay requirements based on existing QoS mechanism.

With the above analysis, RAN2 can reply to SA4 that if the retransmission PDUs and the source PDUs are mapped to different QoS flows, there is no additional benefit for RAN to get the application layer retransmission information, and RAN can guarantee different delay requirements naturally based on the existing QoS mechanism. Otherwise, if the retransmission PDUs and the source PDUs are mapped to the same QoS flow, RAN has no capability to meet the different delay requirements unless some additional enhancements are supported, e.g., the application layer information is provided to RAN, based on which some differentiated handling for the retransmission PDUs is specified at RAN. This would bring more complexity to RAN.

Proposal 1: RAN2 to reply to SA4 that there is no additional benefit to RAN from receiving application layer retransmission information if the retransmission PDUs and the source PDUs are mapped to distinct QoS flows and no RAN impact is needed.

3. Conclusion

In this contribution, we discussed the LS from SA4 on RTP retransmission. The following proposals were made:

Proposal 1: RAN2 to reply to SA4 that there is no additional benefit to RAN from receiving application layer retransmission information if the retransmission PDUs and the source PDUs are mapped to distinct QoS flows and no RAN impact is needed.

4. Reference

[1]. S4-250739, 3GPP TSG SA WG4, LS on RTP retransmission.

Annex A. Draft reply LS for S4-250739

3GPP TSG-RAN WG2 Meeting #130	   R2-25xxxxx

St.Julians, Malta, 19-23 May, 2025

Title:	[Draft] Reply LS on RTP retransmission

Response to:	S4-250739

Release:	Rel-19

Work Item:	5G_RTP_Ph2

Source:	Huawei, HiSilicon [To be RAN2]

To:	SA4

Cc: 	SA2

# Contact Person:

# Name:

# Email:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1. Overall Description:

RAN2 thanks SA4 for the LS on network awareness for RTP retransmission. RAN2 has discussed the potential solutions identified by SA4 and would like to provide the following feedback.

From RAN2 perspective, if the RTP retransmission PDUs and the source PDUs are mapped to distinct QoS flows, RAN can guarantee the different delay requirements based on the existing QoS mechanism and No RAN impact is needed. There is no additional benefit to RAN from receiving the application-layer retransmission information in this case.

# 2. Actions:

# To SA4, SA2:

ACTION: RAN2 kindly asks SA4 and SA2 to take the above feedback into consideration.

# 3. Date of Next RAN2 Meetings:

RAN3#131	25th Aug – 29th Aug 2025				Bangalore, India

RAN3#131bis	13th Oct – 17th Oct 2025				Prague, Czechia