3GPP TSG-RAN WG2 Meeting #130	R2-2504121

St. Julian's, Malta, 19-23 May, 2025

Agenda item:	8.7.3

Source: 	Huawei, HiSilicon

Title: 	Discussion on UAI for MG skipping

Document for:	Discussion and Decision

1. Introduction

RAN4 has worked on what kind of UAI is needed for MG skipping during the previous meetings The latest conclusion agreed to introduce a ratio of gap occasions which is recommended for cancellation during a period of time. In this contribution, we will analyse the RAN2 impacts to support it, e.g., how to trigger the report of the UAI on gap ratio.

2. Discussion

There is an LS from RAN4 in R4-2504972 [1], asking RAN2 to implement the agreements on UAI related to MG skipping. The main part of the LS is excerpted below.

# 1 Overall description RAN4 thanks RAN1 for the LS on UE assistance information (R4-2411007/R1-2405736). Regarding UE assistance information related to measurement occasions, the following was agreed on RAN4#114 (R4-2502700): ## Issue 5-1: General on UAI <Agreement>: - Agree to define UAI based on the following assumptions, and further discuss the content of UAI - RAN4 will not specify any explicit or implicit requirement or expectation on gNB behaviour in response to any XR UAI (from RAN4 perspective) - Not define UE behaviour and the corresponding requirements based on UAI - Supporting of UAI is optional for UE with optional UE capability.. And the following was agreed in RAN4 #114bis (R4-2504911): - The information in UAI: a ratio (R) of gap occasions that is recommended for cancellation during a time period of 1 s. - R ∈ {0, 20, 40, 60}%. # 2 Actions To RAN2 ACTION: RAN4 respectfully asks RAN2 to implement the signaling of the above mentioned UAI reporting for XR.

2.1. UE capability [UE capability-03]

In the LS, RAN4 explained the UAI includes the information of a ratio which indicates the recommended percentage of MG occasions to be cancelled during a time period of 1 s. The UAI is optionally supported by the UE with an optional capability. Although RAN4 didn't define any UE behaviour for the UE, it can be reasonably assumed that the UE can determine the ratio based on the measurement performance requirement and the traffic load. There is no different requirement on UE capability to evaluate these factors on different bands. Thus, the capability should be defined as a per UE capability.

Proposal 1: (UE capability-03) Introduce an optional per UE capability for the UAI of recommended gap cancellation ratio.

2.2. Configuration and triggering for the UAI report [RRC-05, RRC-06]

From RAN2 perspective, it should be specified when the UE should report the UAI. The common triggering conditions used in the other UAI procedures can be considered. Typically, upon receiving the configuration for the UAI, the UE should report the information for the first time. After that, the information can be reported again when it changes from the last reported content. This mechanism can be reused for the gap ratio report.

Considering RAN4 is not going to specify the UE behaviour for determining the reported ratio, it can be left to the UE implementation to decide whether the ratio has changed.

Proposal 2: (RRC-05) UE reports the assistance information of recommended gap cancellation ratio when the configuration is received or when the assistance information changes since the last report.

Proposal 3: (RRC-05) It is left to UE implementation to decide whether the recommended gap cancellation ratio changes.

To prevent too frequent reporting, a prohibit timer can be introduced. Similar to the legacy design, a prohibit timer can be configured together with the UAI configuration for gap ratio report. It would be start by the UE upon transmitting UEAssistanceInformation message including the gap ratio. When the timer is running, the UE shall not report another UEAssistanceInformation message including the gap ratio. In this way, even the change of the gap ratio is determined based on UE implementation, the UE is prohibited from reporting the information too frequently, so that the signalling overhead can be acceptable.

Proposal 4: (RRC-06) A prohibit timer is used to limit frequent transmission of the UAI with recommended gap cancellation ratio.

2.3. Granularity of the UAI

Another issue is the granularity to report the assistance information, e.g., whether the gap ratio recommended for cancellation should be reported per UE or per FR. This may be related to the UE measurement performance requirements. For example, if it is possible that a UE requires more frequent measurement on FR2 than FR1, it may make some sense to let the UE report the gap ratio separately for FR1 and FR2. However, such discussion is out of RAN2 scope. RAN4 should further consider the details of the new UAI and address the above issue. RAN2 can wait for RAN4 progress before determining specific RAN2 design.

Proposal 5: RAN2 to wait for RAN4 progress on the granularity (e.g., per FR or per UE) for the recommended gap cancellation ratio.

To show the above impacts more clearly, we provide potential TPs for TS 38.306 and TS 38.331 separately in the annex part.

3. Conclusion

In this contribution, we analyzed the RAN2 impacts to support UAI on gap skipping ratio. The following proposals were made:

Proposal 1: (UE capability-03) Introduce an optional per UE capability for the UAI of recommended gap cancellation ratio.

Proposal 2: (RRC-05) UE reports the assistance information of recommended gap cancellation ratio when the configuration is received or when the assistance information changes since the last report.

Proposal 3: (RRC-05) It is left to UE implementation to decide whether the recommended gap cancellation ratio changes.

Proposal 4: (RRC-06) A prohibit timer is used to limit frequent transmission of the UAI with recommended gap cancellation ratio.

Proposal 5: RAN2 to wait for RAN4 progress on the granularity (e.g., per FR or per UE) for the recommended gap cancellation ratio.

4. Reference

[1]. R4-2504972, 3GPP TSG RAN WG4, Reply LS on UE assistance information.

[2]. R1-2501663, 3GPP TSG RAN WG1, Introduction of XR (eXtended Reality) for NR Phase 3