3GPP TSG-RAN WG2 #130	R2-2504124

# St. Julian’s, Malta, 19th – 23rd May 2025

Agenda Item:	7.0.2.22

Source:	Ericsson

Title:	Summary of proposals for feMob

Document for:	Discussion

# 1	Introduction

In this contribution is provide a summary of the proposals for FeMob with some possible conclusion for each of them.

# 2	Discussion

## 2.1	Reply LS from RAN4 in R2-2503325

There are three contributions about on how to implement what RAN4 has replied in their LS:

R2-2503471	Clarification on source band definition and UE reporting unsupported target band in pdcch-RACH-AffectedBandsList-r18	MediaTek Inc.	CR	Rel-18	38.306	18.5.0	1274	-	F	NR_Mob_enh2-Core

R2-2504123	Clarification of pdcch-RACH-AffectedBandsList-r18 and related capabilities	Ericsson	discussion	Rel-18	NR_Mob_enh2-Core

R2-2504136	Corrections to Rel-18 LTM (interruptions during RACH and LTM Recovery to the source cell)	Nokia	discussion	Rel-18	NR_Mob_enh2-Core

Rapporteur’s comment: The contributions in R2-2503471 and R2-2504123 propose to clarify the description of the UE capability according to what has been clarified by RAN4. The contribution in R2-2504123 further propose to clarify the other PDCCH-order related capability in light of what RAN4 has clarified, which make sense since all these capabilities are linked to each other. The R2-2504136 proposes instead to add reference to RAN4 spec, but probably this is not enough since it does not really clarify how this capability should be intepreted by the network.

The contribution in R2-2503471 and R2-2504123 can be taken as a baseline for how to capture in 38.306 the information received by RAN4 in the reply LS in R2-2503325.

## 2.2	Correction to LTM cell switch failure handling (R2-2503738)

R2-2503738	Correction to LTM cell switch failure handling	Google	CR	Rel-18	38.331	18.5.1	5346	-	F	NR_Mob_enh2-Core

Rapporteur’s comment: This could be an issue only if the timer T304 expires before the RACH procedure fails, which does not sound a very common case, considering that here we are talking about CFRA. Further, some to double check is how UE treat information received within a MAC CE. The understanding is that such information are typically “one-shot” information, and thus they are not stored by the UE (or re-used later). Finally, we have agreed that for the case of LTM recovery only CBRA is used and therefore the issue described should never happen.

RAN2 to discuss if the UE can indeed re-use CFRA information received within the LTM cell switch MAC CE. If this is the case, RAN2 to agree on the intention of the CR in R2-2503738.

## 2.3	Data loss issue on fast LTM recovery (R2-2503743)

R2-2503743	Data loss issue on fast LTM recovery	Fujitsu	discussion	Rel-18	NR_Mob_enh2-Core

Rapporteur’s comment: It seems that this is a corner case, and one of the proposals is anyway to leave this to the network implementation. So probably nothing is needed from this point of view. Also, this does not seems a correction but rather an enhancement to the current LTM procedure.

RAN2 does not pursue enhancement on data loss issues (if any) during the fast LTM recovery.

## 2.4	LTM fast recovery to source cell (R2-2504136)

R2-2504136	Corrections to Rel-18 LTM (interruptions during RACH and LTM Recovery to the source cell)	Nokia	discussion	Rel-18	NR_Mob_enh2-Core

Rapporteur’s comment: So far, we did not forbid the case where the UE select the source cell when performing the fast LTM recovery procedure and I don’t think that this causes confusion at the network, since the network knows which candidates has configured. One it may argue why a UE selects a cell which is the previous failed cell, but I think that we can rely on the UE to be smart enough.

RAN2 to confirm that the UE can select the previous SpCell during the fast LTM recovery procedure (no specification impact).

## 2.5	Handling of radio and RLC bearers in LTM with NR-DC (R2-2504144)

R2-2504144	Handling of radio and RLC bearers in LTM with NR-DC	Huawei, HiSilicon	discussion	Rel-18	NR_Mob_enh2-Core

Rapporteur’s comment: (Proposal 1) How the UE handles the radio bearer upon an LTM cell switch procedure is indeed an issue and this need to be fixed. The proponent decribes two options on how to solve this and both as valid. Option 1 is UE-specific while option 2 seems to be network-specific. The Rapporteur has a slight preference for Option 1.

RAN2 to discuss how to solve the radio bearer handling during an LTM cell switch procedure:

  - Modify the UE behaviour so that at MCG LTM cell switch execution, the UE may only release MCG RLC bearers (unless the SCG is explicitly released) and radio bearers using the master key, and at SCG LTM cell switch execution, the UE may only release SCG RLC bearers and radio bearers using the secondary key

  - MN and SN exchange the MN/SN radio bearer configurations when LTM is configured/updated.

(Proposal 2) About the keystream re-use, it seems that network can already handle this case and therefore there is no potential impact on any spec.

RAN2 confirms that when a DRB is only in a subset of LTM candidate configurations, the network ensures that keystream reuse cannot happen by changing the key before triggering LTM cell switch that will establish this bearer, and by not configuring the UE to perform LTM recovery (no spec impact).

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	The contribution in R2-2503471 and R2-2504123 can be taken as a baseline for how to capture in 38.306 the information received by RAN4 in the reply LS in R2-2503325.

Proposal 2	RAN2 to discuss if the UE can indeed re-use CFRA information received within the LTM cell switch MAC CE. If this is the case, RAN2 to agree on the intention of the CR in R2-2503738.

Proposal 3	RAN2 does not pursue enhancement on data loss issues (if any) during the fast LTM recovery.

Proposal 4	RAN2 to confirm that the UE can select the previous SpCell during the fast LTM recovery procedure (no specification impact).

Proposal 5	RAN2 to discuss how to solve the radio bearer handling during an LTM cell switch procedure:

a.	Modify the UE behaviour so that at MCG LTM cell switch execution, the UE may only release MCG RLC bearers (unless the SCG is explicitly released) and radio bearers using the master key, and at SCG LTM cell switch execution, the UE may only release SCG RLC bearers and radio bearers using the secondary key

b.	MN and SN exchange the MN/SN radio bearer configurations when LTM is configured/updated.

Proposal 6	RAN2 confirms that when a DRB is only in a subset of LTM candidate configurations, the network ensures that keystream reuse cannot happen by changing the key before triggering LTM cell switch that will establish this bearer, and by not configuring the UE to perform LTM recovery (no spec impact).