3GPP TSG-RAN WG2 #130	R2-2504132

St. Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	8.3.4 Data collection

Source:	Ericsson

Title:	Data collection for network sided and UE sided models

Document for:	Discussion

# 1	Introduction

The study item on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR was approved in RAN plenary #102 [1]. One of the objectives included in the SID is the Life Cycle Management (LCM) of the AI/ML model:

- Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2]

This contribution discusses the aspects related to the data collection for both NW sided and UE sided AI/ML model, as part of the LCM framework.

# 2	Discussion

## 2.1 Introduction

In RAN2#129bis meeting, the following agreements have been taken:

As a baseline, use the AI ML PHY NW-side data collection procedures, configuration and reporting framework.

Study UE side data collection configuration, taking AI ML PHY relevant procedure as baseline.   Postpone UE side data collection transfer discussion until further progress is made in AI ML PHY in R20.

Based on these agreements, this contribution divides the data collection subject into three parts where the first is related to overall principles for data collection, the second part is related to data collection for network side models and the final part is related to data collection for UE side models.

Data collection needs to be performed to provide input to AI/ML models for model training or inference. The data collection could consist of performing measurements which are used as input to the model.

Data collection may interfere with other actions or events in the network, and it should therefore be transparent what data collection procedures the UE is doing since it can influence the rest of the network. The network should therefore also be able to terminate and initiate data collection procedures to limit the effect of data collection on other parts of the network.

The network configures the UE to initiate/terminate data collection procedures.

On the same topic, if data collection measurement collides with other RRM measurements and both cannot be carried out, RRM measurement should have higher priority than data collection measurements. User plane data should also have higher priority than data collection.

Priority between data collection and RRM measurements or user plane data needs to be considered.

## 2.2 Data collection for network sided model

### 2.2.1 Data collection for inference

Data collection for inference purpose requires the UE to send data when in RRC_CONNECTED state to the network. The network therefore needs to configure the data collection and the relevant reporting for the UE, in a similar way as done for the legacy RRM measurements. For this reason, the simplest way to implement that, in terms of standardization effort, is to reuse the RRM measurement framework.

RRM measurement framework is used for configuration of data collection and reporting of the collected data from UE to the NW, for inference purpose.

Depending on the scenarios, the network may need to receive data from the UE periodically (e.g. to provide it as input to the AI/ML model for continuous inference) or it may need to receive the collected data only when a certain event is fulfilled (e.g. in case of handover, to predict the quality of neighbor cells). For this reason, both periodical and event triggered reporting can be supported for the data collection from the UE.

Event triggered report and periodical report are used for data collection for inference purposes.

### 2.2.2 Data collection for training

To ensure that the training of network-side machine learning models benefit from comprehensive and representative data, it is important to collect data from the UE regardless of its state. Additionally, we need flexibility in the data collection process by allowing the UE to perform data logging and collection in both RRC_CONNECTED and RRC_IDLE/INACTIVE states, maximizing the coverage while minimizing the signaling impact.

Data collection for training purpose of network side model should be supported to be done by the UE in RRC_IDLE/INACTIVE state or RRC_CONNECTED state.

In scenarios where the UE is in RRC_IDLE or RRC_INACTIVE state, minimizing network signaling while still enabling useful data collection would be useful. Hence, reusing the established procedures of logged Minimization of Drive Tests (MDT), which allow the UEs to collect and store measurement data for later retrieval by the network, offers a low-overhead mechanism for data collection for training.

For data collection when the UE is in RRC_IDLE/INACTIVE state, a solution based on logged MDT principles can be reused.

For UEs in RRC_CONNECTED state, it is important to collect data without disrupting ongoing sessions. It is also important to think about reducing unnecessary uplink traffic and enable the network to retrieve the data at an optimal time, maintaining efficiency and session continuity. Therefore, the UE should be able to locally log relevant measurement data and simply notify the network about the availability of such logs.

For data collection when the UE is in RRC_CONNECTED state, the UE logs the data and reports the availability of data to the network.

The simplest solution for retrieval of the data and which require the least specification impacts is to reuse the UE Information Request/Response procedure.

The UE Information Request/Response procedure is used to retrieve the logged data from the UE.

It should be investigated if assistance information should be provided by the UE to facilitate and improve the quality of the inference and training of the network sided model. Since the study item is almost completed, this can be done in the work item phase.

Possible assistance information from the UE to the network can be considered in the work item.

## 2.3 Data collection for the UE sided model

Data collection for UE-side models is similar to network sided models in the way that it can be divided into data collection for training and data collection for inference.

### 2.3.1 Data collection for inference

Data collection for inference of UE side models are discussed in another Ericsson contribution submitted to this meeting and is therefore left out here and readers are referred to [2].

### 2.3.2 Data collection for training

Collection of data for training should preferably be done in all RRC state to maximize the opportunities for data collection. The collection can be divided into two parts where one part is collection of data in RRC_IDLE/INACTIVE state and the other one is collection of data in RRC_CONNECTED state.

Data collection for training purposes of UE-side models can be done in either RRC_IDLE/INACTIVE or RRC_CONNECTED state.

#### 2.3.2.1 Data collection in IDLE/INACTIVE state

Deta collection for UEs in RRC_IDLE/INACTIVE states can be done without any specific configuration required from the network, since the UE actions in RRC_IDLE/INACTIVE are to a large extent up to UE implementation. The data collection in RRC_IDLE/INACTIVE states can most likely be up to UE implementation too. Possibly, the requirements on the measurements may need to be looked into.

Data collection for UE sided models in RRC_IDLE/INACTIVE state is mainly up to UE implementation.

The measurement requirements for RRC_IDLE/INACTIVE can be checked for data collection purposes.

#### 2.3.2.2 Data collection in RRC_CONNECTED state

When a UE is in RRC_CONNECTED state and collects measurements from its serving frequency, the data collection for model training may often not impact ongoing procedures or measurements. However, if a lot of data needs to be collected, it could be cumbersome for the UE to perform the data collection during ongoing transmissions/receptions and other measurements. A possibility for the UE could be to perform the data collection during inactive periods in RRC_CONNECTED when there are no ongoing data transmissions/receptions.

The UE preferably performs data collection in RRC_CONNECTED when there are no ongoing data transmissions/receptions.

When a UE is in RRC_CONNECTED state and collects measurements from its serving frequency, no additional configuration is needed from the network as no measurement gaps are required.

UEs can perform intra-frequency measurement collection for UE side models without any configuration update from the network.

However, measurements on other frequencies may require periods, such as measurement gaps, when the UE is not required to monitor PDCCH or perform data transmissions/reception on the serving frequency. It is proposed that RAN2 discusses whether measurement gaps can be configured for the UE to perform data collection for model training and/or inference.

Discuss whether measurement gaps can be configured for the purpose of data collection and/or model training.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Observation 1	UEs can perform intra-frequency measurement collection for UE side models without any configuration update from the network.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The network configures the UE to initiate/terminate data collection procedures.

Proposal 2	Priority between data collection and RRM measurements or user plane data needs to be considered.

Proposal 3	RRM measurement framework is used for configuration of data collection and reporting of the collected data from UE to the NW, for inference purpose.

Proposal 4	Event triggered report and periodical report are used for data collection for inference purposes.

Proposal 5	Data collection for training purpose of network side model should be supported to be done by the UE in RRC_IDLE/INACTIVE state or RRC_CONNECTED state.

Proposal 6	For data collection when the UE is in RRC_IDLE/INACTIVE state, a solution based on logged MDT principles can be reused.

Proposal 7	For data collection when the UE is in RRC_CONNECTED state, the UE logs the data and reports the availability of data to the network.

Proposal 8	The UE Information Request/Response procedure is used to retrieve the logged data from the UE.

Proposal 9	Possible assistance information from the UE to the network can be considered in the work item.

Proposal 10	Data collection for training purposes of UE-side models can be done in either RRC_IDLE/INACTIVE or RRC_CONNECTED state.

Proposal 11	Data collection for UE sided models in RRC_IDLE/INACTIVE state is mainly up to UE implementation.

Proposal 12	The measurement requirements for RRC_IDLE/INACTIVE can be checked for data collection purposes.

Proposal 13	The UE preferably performs data collection in RRC_CONNECTED when there are no ongoing data transmissions/receptions.

Proposal 14	Discuss whether measurement gaps can be configured for the purpose of data collection and/or model training.

# References

RP-234055,	SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

R2-2504131 - Inference and report configuration of UE sided model